3GPP TSG-RAN WG2 Meeting #127                                                      R2-2407102

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	Qualcomm Incorporated

Title:	Control procedure for multi-hop L2 based U2N relay

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay was approved as Rel-19 item, and this contributions discuss the contron plane issues.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

## 2.1 Features to be supported

In Rel-17 L2 U2N relay, the basic features are supported as following. For L2 multiple U2N relay, these features should also be supported.

- Remote UE NAS/RRC connection and E2E PDU Sessions/DRBs management

- AS security

- System Informaiton relay

- Paging monitoring and relay

- Radio link management, e.g. RLF detection/notification, RRC re-establishment

- Access control, location area update

- Positioning

- The following basic functions should be supported in Multi-hop U2N relay.

- Remote UE NAS/RRC connection and E2E PDU Sessions/DRBs management

- AS security

- System Informaiton relay

- Paging monitoring and relay

- Radio link management, e.g. RLF detection/notification, RRC re-establishment

- Access control, location area update

- Positioning

## 2.3 Protocol Architecture

For protocol architectures, there could be two possible ways considering whether the Intermediate Relay UE has no Uu RRC layer to the gNB.

Option 1: The Intermediate Relay UE has no Uu RRC layer to the gNB, as shown in Figure 1.

In this option, the Intermediate Relay UE does not have Uu RRC layer and when the Remote UE is in CONNECTED state the Intermediate Relay UE does not need to have RRC connection. And the Intermediate Relay UE does not need perform Relay UE Uu functionality. The PC5 configuration between the Intermediate Relay UE and the U2N Relay UE can be provided by the gNB and forwarded to the Intermediate Relay UE using PC5 RRC message.

Figure 1

Option 2 : The Intermediate Relay UE has Uu RRC layer to the gNB, as shown in Figure 2.

In this option, the Intermediate Relay UE has Uu RRC layer to the gNB and when the Remote UE is in CONNECTED state the Intermediate Relay UE is also in CONNECTED state and has RRC connection. The benefit of this option is the Intermediate Relay UE is visible to the gNB and can be totally controlled by the gNB. However, when the Intermediate Relay UE is out of coverage, the Intermediate Relay UE has to perform the functionality of the Remote UE.

Figure 2

In option 1, the Intermediate Relay UE is not controlled by the gNB and the gNB cannot authorize the Intermediate Relay UE since there is no connection between the Intermediate Relay UE and the gNB, the Intermediate Relay UE is more like a UE-to-UE Relay UE.

In Option 2, the Intermediate Relay UE can be controlled by the gNB and gNB can authorize the Intermediate Relay UE as today, and most of existing U2N mechanism can reused for Intermediate Relay UE. However, when the Intermediate Relay UE is out of coverage, the Intermediate Relay UE has to perform the functionality of the Remote UE.

- RAN2 discusses the following two protocol architecture options for multi-hop U2N relay protocol architecture.

Option 1: The Intermediate Relay UE has no Uu RRC layer to the gNB. i.e.

- the Intermediate Relay UE does not need to be in CONNECTED state when the Remote UE is in CONNECTED state

- the Intermediate Relay UE does not need perform relay Uu functionalities for the Remote UE

- the Intermediate is not visible and authorized by the gNB

Option 2: The Intermediate Relay UE has Uu RRC layer to the gNB

- the Intermediate Relay UE needs to be in CONNECTED state when the Remote UE is in CONNECTED state

- the Intermediate Relay UE may need perform relay Uu functionalities for the Remote UE, which depends on the solution

- the Intermediate can be not visible and authorized by the gNB.

## 2.1 Supported Scenarios and topology

From the WID description, RAN2 will firstly look into one additional hop relay, then this contribution focus on one additional hop relay topology.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

Considering the potential topology architecture for one additional hop relay, there could be the following cases

Case 1: The Intermediate Relay UE only connects with one U2N Relay UE.In this case, it is limited that the Intermediate Relay UE only connects with one U2N Relay UE at the same time.

Case 2: One Intermediate Relay UE can connect with multiple U2N Relay UEs, and the multiple U2N Relay UEs belong to the same cell.

Case 3: One Intermediate Relay UE can connect with multiple U2N Relay UEs and the multiple U2N Relay UEs belong to the different cells within the same gNB.

Case 4: One Intermediate Relay UE can connect with multiple U2N Relay UEs and the multiple U2N Relay UEs belong to different cells of different gNBs.

Which cases can be supported depends on wehther the Intermediate Relay UE needs to be controlled by the gNB, i.e. in CONNECTED state whent the Remote UE is in CONNECTED state.

Case 1 is the most simple case, can support the basic multi-hop U2N Relay functionality, but there is limitation that one Intermediate Relay UE. Case 2 should be possible to be supported since the different U2N Relay UEs belong to the same cell and the same gNB.

For case 4, if the Intermediate Relay UE needs to be controlled by the gNB, then case 4 cannot be supported so far since the Intermediate Relay UE cannot be controlled by different gNBs.

For case 3, the gNB ID is not visible to the UE so the Relay UEs cannot distinguish case 3 and case 4, then case 3 is also difficult to be supported if  wehther the Intermediate Relay UE needs to be controlled by the gNB.

Observation 1: Case 3 and Case 4 is hard to be supported if Option 2 in proposal 2 is selected.

- All the cases can be supported if Option 1 in proposal 2 (i.e. The Intermediate Relay UE has no Uu RRC layer to the gNB ) is selected

It is proposed to treat case 1 and case 2 as first priority if Option 2 in proposal 2 (i.e,The Intermediate Relay UE has Uu RRC layer to the gNB) is selected.

As today, the PC5 RLC Channel between the Intermediate Relay UE and U2N Relay UE can be shared by different Remote UEs.the PC5 link between the Intermediate Relay UE and the U2N Relay UE can be shared by different Remote UEs. The Uu link also can be shared by different Remote UEs.

- The Uu link between the Intermediate Relay UE and U2N Relay UE can be shared by different Remote UEs.

- The PC5 link between the Intermediate Relay UE and the U2N Relay UE can be shared by different Remote UEs as today.

# Conclusion

- The following basic functions should be supported in Multi-hop U2N relay.

- Remote UE NAS/RRC connection and E2E PDU Sessions/DRBs management

- AS security

- System Informaiton relay

- Paging monitoring and relay

- Radio link management, e.g. RLF detection/notification, RRC re-establishment

- Access control, location area update

- Positioning

- RAN2 discusses the following two protocol architecture options for multi-hop U2N relay protocol architecture.

Option 1: The Intermediate Relay UE has no Uu RRC layer to the gNB. i.e.

- the Intermediate Relay UE does not need to be in CONNECTED state when the Remote UE is in CONNECTED state

- the Intermediate Relay UE does not need perform relay Uu functionalities for the Remote UE

- the Intermediate is not visible and authorized by the gNB

Option 2: The Intermediate Relay UE has Uu RRC layer to the gNB

- the Intermediate Relay UE needs to be in CONNECTED state when the Remote UE is in CONNECTED state

- the Intermediate Relay UE may need perform relay Uu functionalities for the Remote UE, which depends on the solution

- the Intermediate can be not visible and authorized by the gNB.

- All the cases can be supported if Option 1 in proposal 2 (i.e. The Intermediate Relay UE has no Uu RRC layer to the gNB ) is selected

It is proposed to treat case 1 and case 2 as first priority if Option 2 in proposal 2 (i.e,The Intermediate Relay UE has Uu RRC layer to the gNB) is selected.

- The Uu link between the Intermediate Relay UE and U2N Relay UE can be shared by different Remote UEs.

- The PC5 link between the Intermediate Relay UE and the U2N Relay UE can be shared by different Remote UEs as today.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet