3GPP TSG-RAN WG2 Meeting #127	R2-2407103

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:                Qualcomm Incorporated

Title:	Discussion on MP relay capabilities

Agenda Item:	7.9.6

Document for:	Discussion and Decision

# Introduction & Background

This contribution clarifies UE capabilities on multi-path U2N relay.

# Discussion

RAN2 had discussion on UE capabilities for multi-path U2N relay for split bearer.

In RAN2#125 meeting, RAN2 agreed the following capabilities for multi-path U2N relay for split DRB and SRB based on contribution R2-2401158.

Introduce new capabilities to indicate whether the UE supports PDCP duplication over split DRB and split SRB in MP operation.

In RAN2#125bis meeting, RAN2 had achieved the progress on the following capabilities based con contribution R2-2403139.

Agreements:

Postpone the discussion of new UE capability to indicate whether UE supports UL transmission via direct path and DL reception via either direct path or indirect path for split SRB.

Introduce new UE capability to indicate whether UE supports UL transmission via both direct path and indirect path for the split DRB.

In RAN2 #126 meeting, RAN2 continued to discuss the highlight part capability based on contribution R2-2405458, and achieved the following agreements.

Agreements:

No capability is introduced to indicate that the UE supports only one path in UL while two paths in DL for MP split RB.

If the UE does not support duplication with split SRB, the network cannot configure DL-only duplication for this UE.

After the RAN2 #126 meeting, there are some post-meeting discussion about the following two agreements since companies have different understanding.

Agreement 1: Introduce new UE capability to indicate whether UE supports UL transmission via both direct path and indirect path for the split DRB.

Agreement 2: No capability is introduced to indicate that the UE supports only one path in UL while two paths in DL for MP split RB.

Some companies think these two agreements are conflict and think agreement 2 implies the UE is mandated to support UL transmission via both direct path and indirect path for split SRB and split DRB and reverts Agreement 1. This is incorrect understanding considering the aspects below.

- The capability of supporting only one path in UL while two paths in DL for MP split RB in Agreement 2 is only proposed for split SRB, not for split DRB. In DC, this capability was introduced to enable DL only duplication for SRB in case that the UE does not support UL duplication for SRB. So, Agreement 2 means that DL only duplication cannot be enabled if the UE does not support UL duplication or UE is mandated to support only one path in UL while two paths in DL for MP split RB. But we had further agreement in RAN2 #126 that If the UE does not support duplication with split SRB the network cannot configure DL-only duplication for this UE, which clarifies the former one.

- The capability in Agreement 1 is used for DRB UL transmission split. Similar with DC, with this capability, the gNB can enable UL transmission splitting into both of indirect path and direct path. So this is different scenario and capability with the one in Agreement 2.

- RAN2 already had agreed to introduce new capabilities to indicate whether the UE supports PDCP duplication over split DRB and split SRB in MP operation. That means UE is optional to support UL thansmission on both direct path and indirect path

Observation 1: The capability in Agreement 1 is used for DRB UL transmission split on both direct path and indirect path.

Observation 2: Agreement 2 means there is no scenario for one path in UL while two paths in DL for MP split RB or the UE supports one path in UL while two paths in DL for MP split RB by default.

Observation 3: Agreement 1 and Agreement 2 have different meanings and used in different scenarios, they are not conflict.

Therefore, the understanding that Agreement 2 means the UE is mandated to UL transmission via both direct path and indirect path for split SRB and split DRB is not correct.

Proposal 1: Agreement 2 does not impact Agreement 1 and RAN2 confirms to introduce new UE capability to indicate whether UE supports UL transmission via both direct path and indirect path for the split DRB.

# Conclusion

Observation 1: The capability in Agreement 1 is used for DRB UL transmission split on both direct path and indirect path.

Observation 2: Agreement 2 means there is no scenario for one path in UL while two paths in DL for MP split RB or the UE supports one path in UL while two paths in DL for MP split RB by default.

Observation 3: Agreement 1 and Agreement 2 have different meanings and used in different scenarios, they are not conflict.

Agreement 1: Introduce new UE capability to indicate whether UE supports UL transmission via both direct path and indirect path for the split DRB.

Agreement 2: No capability is introduced to indicate that the UE supports only one path in UL while two paths in DL for MP split RB.

Proposal 1: Agreement 2 does not impact Agreement 1 and RAN2 confirms to introduce new UE capability to indicate whether UE supports UL transmission via both direct path and indirect path for the split DRB.

# Reference