3GPP TSG-RAN WG2 Meeting #127		R2-240xxxx

Maastricht, Netherlands, Aug 19 – 23, 2024

Agenda Item:	8.10.2

Source: 	LG Electronics

Title:  	MRO for R18 Mobility

Document for:	Discussion

Introduction

In this contribution, we focus on the Rel-18 mobility MRO based on the last agreements.

Discussion

CHO+CPAC MRO

    - Near failure logging

The near failure scenario will always be considered only in the case of handover success. Thus, the UE can report the related information via SHR (Successful Handover Report). In other words, although PSCell change can be performed together, CPC is an additional condition based on CHO and the configuration is the same as CHO+SCG, so there is no need to consider SPR (Successful PSCell change or addition Report).

Proposal 1: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

Additionally, to simplify the SHR logging condition, we think that the existing SHR conditions can be used as it is. The network may prepare the CHO-only configuration and other execution conditions for candidate cells, so if the CHO+CPC execution condition is not met, most other CHO execution conditions will likely be met. Thus, we think there is no need to consider additional new conditions for SHR logging due to associated CPC/CPA execution condition.

Proposal 2: For CHO with associated CPA/CPC, the UE uses the existing condition of SHR to log the near failure case to the network.

If the CHO is successfully completed but a failure occurs in the associated CPA/CPC procedure, the UE sends SCG failure information according to the current specification. Thus, in this case, sending the SCG failure information for the associated CPA/CPC is not a new procedure. We think that the UE should send the SCG failure information without considering additional information. This is because if the SCG is failed in a situation that is not the near failure scenario, it is similar to a legacy failure rather than new scenario requiring additional information for CHO with the associated CPC/CPA.

Proposal 3: For CHO with associated CPA/CPC, the UE uses the SCG failure information to inform the network if only the associated CPC/CPA fails. However, no further information needs to be included in the SCG failure information if the SHR can be adequately enhanced.

Then, let's consider what information can be included in the SHR. Currently, the following information is included in the SHR [1]:

SuccessHO-Report-r17 ::= SEQUENCE { sourceCellInfo-r17 SEQUENCE { sourcePCellId-r17 CGI-Info-Logging-r16, sourceCellMeas-r17 MeasResultSuccessHONR-r17 OPTIONAL, rlf-InSourceDAPS-r17 ENUMERATED {true} OPTIONAL}, targetCellInfo-r17 SEQUENCE { targetPCellId-r17 CGI-Info-Logging-r16, targetCellMeas-r17 MeasResultSuccessHONR-r17 OPTIONAL}, measResultNeighCells-r17 SEQUENCE { measResultListNR-r17 MeasResultList2NR-r16 OPTIONAL, measResultListEUTRA-r17 MeasResultList2EUTRA-r16 OPTIONAL } OPTIONAL, locationInfo-r17 LocationInfo-r16 OPTIONAL, timeSinceCHO-Reconfig-r17 TimeSinceCHO-Reconfig-r17 OPTIONAL, shr-Cause-r17 SHR-Cause-r17 OPTIONAL, ra-InformationCommon-r17 RA-InformationCommon-r16 OPTIONAL, upInterruptionTimeAtHO-r17 UPInterruptionTimeAtHO-r17 OPTIONAL, c-RNTI-r17 RNTI-Value OPTIONAL, ..., [[ eutraTargetCellInfo-r18 SEQUENCE { targetPCellId-r18 CHOICE { cellGlobalId-r18 CGI-Info-Logging-r16, pci-arfcn-r18 PCI-ARFCN-EUTRA-r16}, targetCellMeas-r18 MeasQuantityResultsEUTRA OPTIONAL } OPTIONAL, measResultServCell-RSSI-r18 RSSI-Range-r16 OPTIONAL, measResultNeighFreqList-RSSI-r18 MeasResultNeighFreqList-RSSI-r18 OPTIONAL, eutra-C-RNTI-r18 EUTRA-C-RNTI OPTIONAL, timeSinceSHR-r18 TimeSinceSHR-r18 OPTIONAL ]]}

We consider additional information that could be included based on the existing SHR information.

- First satisfied execution condition

It should be possible to determine which execution condition was satisfied first between the execution conditions provided for CHO and associated SCG. This is necessary to identify which condition was not satisfied or was satisfied later.

- Satisfaction time of the first execution condition

Information about the exact time when the first satisfied execution condition was met could be helpful. We believe the UE can easily report the network of this information using the existing time stamp information.

- Satisfaction status of the remaining execution condition

It is necessary to provide information about whether the remaining execution condition was satisfied. Even if the UE successfully performs handover, the handover may not be initiated by CHO+CPC. Thus, it would be helpful to know if the remaining execution condition was also met.

- Time gap between two execution conditions

Information about how much time elapsed between the two execution conditions is required. This information inform the network to determine how much time passed after the first execution condition was satisfied till the both execution conditions were met.

Additionally, this information can be also used to inform the network of the time between the first satisfied execution condition and the handover if the rest if execution condition is not met and another handover is initiated.

- Source PSCell signal quality

Information about the signalling quality of the source PSCell could be helpful. Currently, A3 or A5 is not used for the execution conditions of CPC for CHO+CPC configuration. Thus, even if the source PSCell is still good when performing CHO+CPC, PSCell change can happen. If the target PCell can maintain the source PSCell, the signal quality of the source PSCell is information that helps the network achieve more robust CHO+CPC.

- Candidate PSCell signal quality

Information about the signalling quality of the other candidate PSCell could be also helpful. Through this information, the network can figure out which other candidate PSCell are still considered or are not longer considered as candidate SCG for other UEs.

Based on the above information, we propose the following additional information to be included in the SHR:

Proposal 4. For CHO with associated CPA/CPC, the UE newly includes the below information in SHR as the information of the near failure:

- First satisfied execution condition

- Satisfaction time of the first execution condition

- Satisfaction status of the remaining execution condition

- Time gap between two execution conditions

- Source PSCell measurement result

- Candidate PSCell measurement results

    - Failure logging

Since RAN2 agreed that the UE use the RLF-report in case of the failure logging, let's consider what information can be included in the RLF-report. Currently, the following information is included in the RLF-report [2]:

RLF-Report-r16 ::= CHOICE { nr-RLF-Report-r16 SEQUENCE { measResultLastServCell-r16 MeasResultRLFNR-r16, measResultNeighCells-r16 SEQUENCE { measResultListNR-r16 MeasResultList2NR-r16 OPTIONAL, measResultListEUTRA-r16 MeasResultList2EUTRA-r16 OPTIONAL } OPTIONAL, c-RNTI-r16 RNTI-Value, previousPCellId-r16 CHOICE { nrPreviousCell-r16 CGI-Info-Logging-r16, eutraPreviousCell-r16 CGI-InfoEUTRALogging } OPTIONAL, failedPCellId-r16 CHOICE { nrFailedPCellId-r16 CHOICE { cellGlobalId-r16 CGI-Info-Logging-r16, pci-arfcn-r16 PCI-ARFCN-NR-r1 }, eutraFailedPCellId-r16 CHOICE { cellGlobalId-r16 CGI-InfoEUTRALogging, pci-arfcn-r16 PCI-ARFCN-EUTRA-r16}}, reconnectCellId-r16 CHOICE { nrReconnectCellId-r16 CGI-Info-Logging-r16, eutraReconnectCellId-r16 CGI-InfoEUTRALogging } OPTIONAL, timeUntilReconnection-r16 TimeUntilReconnection-r16 OPTIONAL, reestablishmentCellId-r16 CGI-Info-Logging-r16 OPTIONAL, timeConnFailure-r16 INTEGER (0..1023) OPTIONAL, timeSinceFailure-r16 TimeSinceFailure-r16, connectionFailureType-r16 ENUMERATED {rlf, hof}, rlf-Cause-r16 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, beamFailureRecoveryFailure, lbtFailure-r16, bh-rlfRecoveryFailure, t312-expiry-r17, spare1}, locationInfo-r16 LocationInfo-r16 OPTIONAL, noSuitableCellFound-r16 ENUMERATED {true} OPTIONAL, ra-InformationCommon-r16 RA-InformationCommon-r16 OPTIONAL, ..., [[ csi-rsRLMConfigBitmap-v1650 BIT STRING (SIZE (96)) OPTIONAL ]], [[ lastHO-Type-r17 ENUMERATED {cho, daps, spare2, spare1} OPTIONAL, timeConnSourceDAPS-Failure-r17 TimeConnSourceDAPS-Failure-r17 OPTIONAL, timeSinceCHO-Reconfig-r17 TimeSinceCHO-Reconfig-r17 OPTIONAL, choCellId-r17 CHOICE { cellGlobalId-r17 CGI-Info-Logging-r16, pci-arfcn-r17 PCI-ARFCN-NR-r16 } OPTIONAL, choCandidateCellList-r17 ChoCandidateCellList-r17 OPTIONAL ]], [[ pSCellId-r18 CHOICE { cellGlobalId-r18 CGI-Info-Logging-r16, pci-arfcn-r18 PCI-ARFCN-NR-r16 } OPTIONAL, mcgRecoveryFailureCause-r18 ENUMERATED {t316-Expiry, scgDeactivated, spare2, spare1} OPTIONAL, scgFailureCause-r18 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-ReconfigFailure, srb3-IntegrityFailure, scg-lbtFailure-r16, beamFailureRecoveryFailure-r16, t312-Expiry-r16, bh-RLF-r16, beamFailure-r17, spare3, spare2, spare1 } OPTIONAL, elapsedTimeSCGFailure-r18 ElapsedTimeSCGFailure-r18 OPTIONAL, voiceFallbackHO-r18 ENUMERATED {true} OPTIONAL, measResultLastServCell-RSSI-r18 RSSI-Range-r16 OPTIONAL, measResultNeighFreqList-RSSI-r18 MeasResultNeighFreqList-RSSI-r18 OPTIONAL, bwp-Info-r18 AttemptedBWP-Info-r18 OPTIONAL, elapsedTimeT316-r18 ElapsedTimeT316-r18 OPTIONAL ]] }, eutra-RLF-Report-r16 SEQUENCE { failedPCellId-EUTRA CGI-InfoEUTRALogging, measResult-RLF-Report-EUTRA-r16 OCTET STRING, ..., [[ measResult-RLF-Report-EUTRA-v1690 OCTET STRING OPTIONAL ]] } }

We believe that since the UE also uses the MCG RLF-report for handover failure, No information, which seems to be added or omitted compared to the “Near failure” case in SHR. Thus, we propose that the following information should be considered for MCG RLF-report.

Proposal 5. For CHO with associated CPA/CPC, the UE includes the below information in RLF-report as the information of the failure:

- First satisfied execution condition

- Satisfaction time of the first execution condition

- Satisfaction status of the remaining execution condition

- Time gap between two execution conditions

- Source PSCell signal quality

- Candidate PSCell signal quality

LTM MRO

    - Near failure logging

The first point to consider is what information is newly needed compared to the existing reporting.

A significant different point of LTM cell switch compared to existing PCell handover is the early synchronization procedure for RACH-less cell switch. This early synchronization procedure involves detailed steps for DL sync and UL sync. Except for the UE-based UL synchronization step (i.e. TA acquisition), all other steps are things where the network can already know the information before the UE reporting.

The reason why we think only the UE-based UL synchronization information is needed is because, even though the early synchronization procedure includes DL synchronization and PDCCH order-based UL synchronization, the DL synchronization and PDCCH order-based UL synchronization information is initiated by the network to the UE, meaning the network already knows this information with start time and results.

Thus, we think the most critical aspect is to report the UE-based synchronization to the network to provide more chance of the RACH-less cell switch in LTM.

Additionally, even though the UE has acquired the UE-based synchronization information, if TA values are provided in the cell switch command, the UE performs the RACH-less cell switch using the provided TA information instead of the UE-based synchronization information. Thus, informing whether the UE had acquired the UE-based synchronization information even in cases of performing the RACH-less cell switch becomes more helpful for the network to consider in the subsequent cell switch.

Furthermore, informing which candidate cells the UE acquired the UE-based synchronization information when performing the cell switch also becomes helpful information for the network to consider in the subsequent cell switch.

Based on the above points, for successful cell switch scenarios for MCG LTM, we can propose that the UE-based synchronization information can be mainly considered as new information for the UE reporting.

Proposal 6. In successful scenarios for MCG LTM, the UE logs new information related to the UE based synchronization in SHR to inform the network of the RACH-less cell switch logging in MCG LTM.

Proposal 7. In successful scenarios for MCG LTM, the UE doesn’t need to log information related to DL synchronization and PDCCH order-based UL synchronization in SHR that the network already knows without reporting.

Next is how the UE will report the SHR to the network. Except the UE based UL synchronization, we discuss what other information can be considered in SHR.

First, we think that Random Access information performed for LTM should be included. According to the current specifications, the RACH report only includes the last successful or failed RACH information, which limits its usefulness as it cannot directly help the network support subsequent mobility in LTM.

Second, similar to the existing logging report, it would be beneficial to include the measurement results of the last serving cell, neighboring cells, and candidate cells. Since LTM is a mobility procedure based on L1 measurements, it would be helpful to include both L3 and L1 measurement results.

Based on the above information, we propose that the following information be included in the SHR.

Proposal 8. In successful scenarios for MCG LTM, the UE includes the below other information in SHR as the information of the near failure:

- Source PSCell L3 and/or L1 measurement results

- Candidate PSCell L3 and/or L1 measurement results

- Other neighbour cell L3 and/or L1 measurement results

- Random access trial results if RACH-less isn’t allowed

Lastly, we think there is no need to log time-related information regarding when the UE received the LTM configuration and when the UE performed the LTM cell switching. This is because the LTM isn’t executed by the UE decision but by the network decision, and unlike conditional mobility, the network can fully track this sort of time information.

Proposal 9. In successful scenarios for MCG LTM, the UE does not include time-related information, such as when the UE received the LTM configuration and when the UE performed the LTM.

    - Failure logging

We think that for failure logging, only the information that the network cannot already know should be newly considered, just as in the case of SHR. Thus, we propose the following, as we are proposing for SHR:

Proposal 10. In failure scenarios for MCG LTM, the UE includes the below information in RLF-report as like SHR:

- Source PSCell L3 and/or L1 measurement results

- Candidate PSCell L3 and/or L1 measurement results

- Other neighbour cell L3 and/or L1 measurement results

- Random access trial results if RACH-less isn’t allowed

- the UE based synchronization information

In addition, we think beam information should be newly considered in RLF-report

The network triggers the RACH-less LTM, the network needs to indicate which beam to be applied for the RACH-less LTM. For the beam to be applied for the mobility, the network can indicate SSB index or TCI state ID associated with the target cell. For the association, the network needs to be pre-configured the UE with the SSB configuration/TCI state configuration of the target cell. Then, the UE applies the received beam information, during the RACH-less mobility, for monitoring of PDCCH at the target cell for receiving dynamic grant for transmitting mobility completion message or the received beam information is applied for transmission of mobility completion message via configured grant. The Network may indicate TA value to be applied for the mobility or may indicate the UE to apply the UE-calculated TA value for the mobility.

The problem is that the received beam information may not be suitable for the UE due to, e.g., the beam information is outdated, or the UE’s local movement makes the beam information unsuitable. If such unsuitable beam is used for the RACH-less LTM, the mobility fails, i.e. the received TA or the UE calculated TA value may not be suitable for the UE, and in that case, the mobility fails. In case LTM failure happens, the current RLF-report cannot provide any information of the beam information used by the UE for the mobility. Hence, it is difficult for network to analyse the failure reason.

Thus, we propose to include the beam information for LTM trial in RLF-report.

Proposal 11. In failure scenarios for MCG LTM, the UE newly include the beam information what is used for LTM in RLF-report.

SCPAC MRO

SCPAC are not very different from the existing CPAC. Thus, we think that in most cases, existing information can be simply reused for both near-failure and failure cases of SCPAC.

However, the newly introduced multiple security keys need to be considered as new logging information.

The UE receives multiple security keys in advance for each cell group (i.e., per gNB) to perform SCPAC. When the UE performs SCPAC and the target cell belongs to a different cell group (i.e., during inter-gNB mobility), the UE selects the unused security key of the cell group that includes the target cell to perform a security update. In this case, there might be cases where the network and the UE have a mismatch in the information regarding the used security keys, or the network may have failed to appropriately update the multiple security keys, resulting in no unused security key being available for the security update.

Therefore, it will be helpful for the network if the network knows which security keys have been used and when the security keys have been selected to prevent further security failure in various conditional mobility scenarios.

Proposal 12. For, SCPAC MRO, the UE performs logging for multiple security key status to RLF-report or SHR to help resolve the security key mismatch issue that occurs during SCPAC.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

Proposal 2: For CHO with associated CPA/CPC, the UE uses the existing condition of SHR to log the near failure case to the network.

Proposal 3: For CHO with associated CPA/CPC, the UE uses the SCG failure information to inform the network if only the associated CPC/CPA fails. However, no further information needs to be included in the SCG failure information if the SHR can be adequately enhanced.

Proposal 4. For CHO with associated CPA/CPC, the UE newly includes the below information in SHR as the information of the near failure:

- First satisfied execution condition

- Satisfaction time of the first execution condition

- Satisfaction status of the remaining execution condition

- Time gap between two execution conditions

- Source PSCell measurement result

- Candidate PSCell measurement results

Proposal 5. For CHO with associated CPA/CPC, the UE includes the below information in RLF-report as the information of the failure:

- First satisfied execution condition

- Satisfaction time of the first execution condition

- Satisfaction status of the remaining execution condition

- Time gap between two execution conditions

- Source PSCell signal quality

- Candidate PSCell signal quality

Proposal 6. In successful scenarios for MCG LTM, the UE logs new information related to the UE based synchronization in SHR to inform the network of the RACH-less cell switch logging in MCG LTM.

Proposal 7. In successful scenarios for MCG LTM, the UE doesn’t need to log information related to DL synchronization and PDCCH order-based UL synchronization in SHR that the network already knows without reporting.

Proposal 8. In successful scenarios for MCG LTM, the UE includes the below other information in SHR as the information of the near failure:

- Source PSCell L3 and/or L1 measurement results

- Candidate PSCell L3 and/or L1 measurement results

- Other neighbour cell L3 and/or L1 measurement results

- Random access trial results if RACH-less isn’t allowed

Proposal 9. In successful scenarios for MCG LTM, the UE does not include time-related information, such as when the UE received the LTM configuration and when the UE performed the LTM.

Proposal 10. In failure scenarios for MCG LTM, the UE includes the below information in RLF-report as like SHR:

- Source PSCell L3 and/or L1 measurement results

- Candidate PSCell L3 and/or L1 measurement results

- Other neighbour cell L3 and/or L1 measurement results

- Random access trial results if RACH-less isn’t allowed

- the UE based synchronization information

Proposal 11. In failure scenarios for MCG LTM, the UE newly include the beam information what is used for LTM in RLF-report.

Proposal 12. For, SCPAC MRO, the UE performs logging for multiple security key status to RLF-report or SHR to help resolve the security key mismatch issue that occurs during SCPAC.

References

- 3GPP TS 38.331 V18.2.0 (2024-06)