3GPP TSG-RAN WG2 Meeting #127	R2-2407108

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.6.2

Source: 	China Telecom

Title:	Discussion on Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding the general aspects for inter-CU LTM, the following agreements were reached.

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

However, there are still some open issues on the details of inter-CU LTM left for further discussion. In this contribution, we would like to share our opinions on support of inter-CU LTM.

- Discussion

Stage-2 procedure for inter-CU LTM

In previous RAN2 meeting, the details of stage-2 signalling flows and procedures were discussed and many agreements were reached. Based on the agreements, we try to give a potential stage-2 signalling flow for inter-CU LTM in NR SA scenario as illustrated in Fig.1, which includes LTM preparation, the early synchronization, LTM cell switch execution and LTM cell switch completion. To make it clearer for inter-CU LTM, we only consider inter-CU LTM in the following stage-2 procedure. Whether to have a separate procedure for inter-CU LTM or mix it with intra-CU LTM can be further discussed in the CR drafting phase.

The procedure for inter-CU LTM is as follows:

1. The UE sends a MeasurementReport message to the gNB.

2. The source gNB decides to configure inter-CU LTM and initiates LTM preparation.

3. The source gNB sends the LTM request related information (such as the candidate cell information, early sync request information, reference configuration) to the candidate gNB(s) through Handover Request message.

4. The candidate gNB performs access control, when it accepts the LTM request, it responds the Handover Request Acknowledge message including the generated LTM configuration(s) for the accepted candidate cell.

5. The source gNB determines the final candidate cell(s) and related LTM configurations.

6. LTM configuration update/modification procedure may be needed to support LTM configuration update or modification between the source gNB and candidate gNBs. (It is up to RAN3 discussion.)

7. The source gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

8. The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the gNB.

9a. The UE performs DL synchronization with the candidate cell(s) before receiving the cell switch command.

9b. UL synchronization with the candidate cell(s). For inter-CU LTM, early TA related information might be transmitted from candidate gNBs to the source gNBs, which is also up to RAN3 discussion.

10. The UE sends the L1 measurement result to the source gNB, and the source gNB decides to execute LTM.

11. The source gNB sends message to the target gNB to indicate the initiation of the LTM Cell Switch Command to the UE. (Up to RAN3 discussion.)

12. The source gNB sends the LTM Cell Switch Command MAC CE to the UE.

13. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell.

14. When target gNB detects the UE access, the target gNB sends the ACCESS SUCCESS message to the source gNB including target cell ID.

15. The UE sends an RRCReconfigurationComplete message to the target gNB.

Proposal 1: RAN2 to consider the stage-2 signalling flow for inter-CU LTM in non-DC scenario, as illustrated in Fig.1.

Fig.1 Stage-2 signalling flow for inter-CU LTM in NR SA scenario

Configurations for inter-CU LTM

In RAN2#125bis meeting, it is agreed that UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells. Regarding how to configure the LTM configuration when inter-CU LTM is involved may need further discussion. Potential options are listed as below.

- Option 1: Having separate LTM configurations for intra-CU and inter-CU candidates respectively.

- Option 2: Having one LTM configuration including both intra-CU and inter-CU candidates.

Since different options may have different impacts on how to associate L1 measurement results and the content of inter-node messages, as the leading group of WI, it is better for RAN2 to discuss this issue at an early stage to provide information or guidance to other working group involved.

From signalling design’s perspective, either of the above options can work. The key point is the granularity of the reference configuration. For Option 1, the LTM configuration is per-CU level, which may include a reference configuration together with a list of candidate cell configurations within the same CU. The NW would provide multiple lists of LTM configurations to the UE. For Option 2, the NW would provide UE with only one list of LTM configuration similar to Rel-18, which includes only one reference configuration and different candidate cells belonging to different CUs. Considering the motivation of introducing reference configuration in Rel-18 is to reduce potential signalling overhead in Uu interface, we prefer to remain a single reference configuration for inter-CU LTM case as well. If the reference configuration is per-CU configured, the gain of having reference configuration will be reduced from the perspective of signalling overhead. Besides, for the case where is little common configuration between different CUs, the network can fall back to use the complete configuration instead.

Proposal 2: Use a single reference configuration for the case that the UE is configured with a mixture of intra-CU and inter-CU candidate LTM cells.

If only one reference configuration is used for configuring a mixture of intra-CU and inter-CU candidate LTM cells, there seems no need to have separate LTM configurations for intra-CU and inter-CU candidates respectively.

Proposal 3: The RRC signalling structure and modelling for Rel-18 LTM could be reused in Rel-19 LTM, i.e. having one LTM configuration including both intra-CU and inter-CU candidates.

In RAN2#125bis meeting, the following agreements were reached on LTM cell switch execution phase.

RAN2#125bis agreements on LTM cell switch execution phase

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

For Rel-18 intra-CU LTM, UE may have different behaviors upon executing different types of LTM, which involves intra-DU LTM and inter-DU LTM within the same CU. To help UE to determine whether a L2 reset is needed or not upon an LTM cell switch procedure, an ID is introduced in Rel-18.

For Rel-19, since the UE will have different behaviors upon performing inter-CU LTM from intra-CU LTM, the UE needs to know the type of candidate cell and perform L2 operation accordingly, if RAN2 agrees to have one LTM configuration including both intra-CU and inter-CU candidates. Similar to Rel-18, a new ID could be introduced to indicate whether an inter-CU LTM is expected to be executed.

Proposal 4: Similar to Rel-18, a new ID could be introduced to indicate whether an inter-CU LTM is expected to be executed.

Inter-CU LTM in NR-DC case

In Rel-18, the supported Dual connectivity scenarios includes PCell and MCG SCell(s) change and intra-SN PSCell and SCG SCell(s) change without MN involvement. LTM for simultaneous PCell and PSCell change is not supported [2].

As per the WID, inter-CU LTM in NR standalone scenario will be prioritized in Rel-19, and inter-CU LTM in some of the NR-DC scenarios will also be supported as secondary priority. Regarding inter-CU LTM in NR-DC scenarios, the supported cases listed in the WID are limited to inter-CU SN LTM with MCG unchanged and inter-CU MN LTM with SCG unchanged or SCG is released.

For Rel-19 inter-CU SN LTM with MCG unchanged scenarios, both MN initiated inter-CU SN LTM and SN initiated inter-CU SN LTM could be considered. To only support LTM in one of the potential scenarios may significantly limit the opportunities to use LTM. Since both scenarios involve MN, the procedure of Rel-18 intra-SN LTM without MN involvement could not be reused directly. The details could be further discussed.

Proposal 5: For inter-CU SN LTM with MCG unchanged scenarios, both MN initiated inter-CU SN LTM and SN initiated inter-CU SN LTM could be supported.

For Rel-19 inter-CU MN LTM with SCG unchanged or SCG is released, one scenario that may need further clarification is whether to support change SN into MN by inter-CU MN LTM procedure. In our understanding, for inter-CU LTM in NR-DC scenario, the current PSCell or SCG SCell(s) could also be configured as candidate cell(s). Then changing the current PSCell or SCG SCell into PCell by LTM procedure might be treated as inter-CU MN LTM with SCG released, which means the SCG will be released before triggering inter-CU MN LTM.

Proposal 6: For inter-CU MN LTM with SCG unchanged or SCG is released, changing the current SCG PSCell or SCell into PCell could be supported by inter-CU MN LTM procedure with SCG released.

- Conclusion

In this contribution, we discuss inter-CU LTM related aspects in Rel-19. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 to consider the stage-2 signalling flow for inter-CU LTM in non-DC scenario, as illustrated in Fig.1.

Proposal 2: Use a single reference configuration for the case that the UE is configured with a mixture of intra-CU and inter-CU candidate LTM cells.

Proposal 3: The RRC signalling structure and modelling for Rel-18 LTM could be reused in Rel-19 LTM, i.e. having one LTM configuration including both intra-CU and inter-CU candidates.

Proposal 4: Similar to Rel-18, a new ID could be introduced to indicate whether an inter-CU LTM is expected to be executed.

Proposal 5: For inter-CU SN LTM with MCG unchanged scenarios, both MN initiated inter-CU SN LTM and SN initiated inter-CU SN LTM could be supported.

Proposal 6: For inter-CU MN LTM with SCG unchanged or SCG is released, changing the current SCG PSCell or SCell into PCell could be supported by inter-CU MN LTM procedure with SCG released.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

3GPP TS 38.300 18.1.0: " NR; NR and NG-RAN Overall Description; Stage 2".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB