3GPP TSG-RAN WG2 Meeting #127	R2-2407109

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.6.3

Source: 	China Telecom

Title:	Discussion on measurement event evaluation for LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made on measurement enhancements for LTM. Regarding the general aspects for event-triggered L1 measurement reporting for LTM, the following agreements were reached.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we would like to share our opinions on further details of measurement event evaluation for LTM.

- Discussion

Events to trigger L1 measurement reporting for LTM

In last RAN2 meeting, Event LTM2~Event LTM5 are agreed to be used for triggering L1 measurement report for LTM. Whether to introduce Event LTM1 remains FFS. In our opinion, Event LTM1 is also useful for LTM. In case that the current serving beam suffers a sudden drop of quality, it is necessary to trigger a L1 measurement report when the beam quality recovers to avoid unnecessary LTM cell switch.

Proposal 1: To support event-triggered L1 measurement reporting for LTM, Event LTM1 i.e. Beam of serving cell becomes better than absolute threshold, could be introduced.

As agreed in last RAN2 meetim of serving cell becomes worseng, beam level measurement result is used for LTM event evaluation. Another open issue is what beam(s) of the serving cell and candidate cell is used for event evaluation. Potential options are listed as below.

- Option 1: all the configured beams of the serving cell or candidate cell;

- Option 2: a certain number of beams of the serving cell or candidate cell, e.g. the best X beams or the worst Y beams;

- Option 3: a single beam of the serving cell or candidate cell, e.g. the best beam, the worst beam or a new beam.

For Option 1, it seems too strict to trigger the L1 measurement report only when all the configured beams are good or bad enough. It may take a long time to meet the trigger conditions and the NW might not get the available L1 measurement results timely. For Option 3, only a single beam is used for evaluating the event conditions, which lead to more L1 measurement reports. Considering the motivation of introducing event-triggered L1 measurement report is to reduce the L1 overhead for measurement reporting, Option 3 is not recommended. Option 2 could be a trade-off solution, which can reduce the L1 reporting overhead as well as guarantee the L1 measurement results to be reported timely. Only when a certain number of beams of the serving cell or candidate cell meet the trigger conditions of LTM events, the L1 measurement report might be triggered.

Proposal 2: For event evaluation for LTM, a certain number of beams of the serving cell or candidate cell, e.g. the best X beams or the worst Y beams, could be used.

Regarding whether to introduce cell-level events for L1 measurement reporting for LTM as well, we see no clear motivation since the legacy L1 measurement reports for LTM only include beam-level measurement results. If cell-level events are introduced for event-triggered L1 measurement reporting of LTM, then how to report the corresponding measurement results may be not clear. Thus, we suggest RAN2 first focus on defining beam-level events for event-triggered L1 measurement reporting of LTM.

Proposal 3: RAN2 assumes that Cell-level events for triggering L1 measurement report for LTM are not needed.

Moreover, in legacy Layer 3 mobility, S-MeasureConfig is used to avoid unnecessary measurements on non-serving cells when the quality of the current serving cell is good enough. For event-triggered L1 measurement reporting for LTM, similar mechanism could be considered as well to avoid unnecessary measurements on candidate beams if the current beam(s) of the serving cell is good enough.

Proposal 4: To avoid unnecessary measurements on candidate beams in LTM, S-Measure like mechanism could be considered for event-triggered L1 LTM measurement report.

Configuration of event-triggered L1 measurement reporting for LTM

In RAN2#125bis meeting, the following agreement on configuration of L1 LTM measurement event was reached.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

Based on the above agreement, L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signalling. The detailed association between the L1 LTM measurement event configuration and L1 measurement resource configuration may need further discussion.

For L3 measurement configurations, there are multiple reporting configurations with different Reportconfig IDs associated with different measurement objectives of different MesureObject IDs. For configuration of event-triggered L1 measurement reporting for LTM, the similar framework could be followed. A configuration ID for L1 LTM measurement event configuration could be introduced to identify an L1 LTM measurement event configuration.

Proposal 5: For L1 LTM measurement event configuration, a configuration ID could be introduced to identify an L1 LTM measurement event configuration.

To associate a configuration ID of L1 LTM measurement event configuration with L1 measurement resource configuration, there may be different options with different granularities.

- Option 1: a configuration ID of L1 LTM measurement event configuration is associated with one beam;

- Option 2: a configuration ID of L1 LTM measurement event configuration is associated with one cell.

For Option 1, different beams of a candidate cell may have different L1 measurement event configurations. Then the candidate beam in the event definition refers to the associated beam. For Option 2, different beams of a candidate cell have the same L1 event configuration. The configuration ID of L1 event configuration is associated with a candidate ID. Then the definition of candidate beam could be any beam, best beam or some specific beam of the candidate cell.

Since no strong motivation is foreseen to support per-beam level event configuration, RAN2 can assume per-cell level event configuration is used.

Proposal 6: Regarding how to associate a configuration ID of L1 LTM measurement event configuration with L1 measurement resource configuration, a configuration ID of L1 LTM measurement event configuration is associated with one candidate cell and/or one serving cell.

- Conclusion

In this contribution, we discuss L1 measurement event evaluation for Rel-19 LTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: To support event-triggered L1 measurement reporting for LTM, Event LTM1 i.e. Beam of serving cell becomes better than absolute threshold, could be introduced.

Proposal 2: For event evaluation for LTM, a certain number of beams of the serving cell or candidate cell, e.g. the best X beams or the worst Y beams, could be used.

Proposal 3: RAN2 assumes that Cell-level events for triggering L1 measurement report for LTM are not needed.

Proposal 4: To avoid unnecessary measurements on candidate beams in LTM, S-Measure like mechanism could be considered for L1 LTM measurement.

Proposal 5: For L1 LTM measurement event configuration, a configuration ID could be introduced to identify an L1 LTM measurement event configuration.

Proposal 6: Regarding how to associate a configuration ID of L1 LTM measurement event configuration with L1 measurement resource configuration, a configuration ID of L1 LTM measurement event configuration is associated with one candidate cell and/or one serving cell.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB