3GPP TSG-RAN WG2 Meeting #127	R2-2407110

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.6.4

Source: 	China Telecom

Title:	Discussion on measurement reporting for LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and some progress has been made on measurement event evaluation for R19 LTM. Regarding the general aspects on measurement reporting for event-triggered L1 measurement for LTM, the following open issues remain FFS.

- What information in the measurement report?

- L1 or L2 based measurement report.

- Whether to support measurement report once leaving condition is met?

In this contribution, we would like to share our opinions on the above open issues on measurement reporting for LTM.

- Discussion

What information in the measurement report?

In Rel-18, LTM is introduced to reduce the handover latency and interruption time compared to Layer 3 based mobility. The network may trigger LTM cell switch based on the L1 measurement report(s) from the UE. In legacy Layer 3 mobility, the Layer 3 measurement report is event triggered, which can not only reduce signalling overhead but also provide the available measurement in time. However, UE evaluated events for triggering of L1 measurement reports is not supported for Rel-18 LTM. In Rel-19, event-triggered L1 measurement reporting for LTM will be supported to reduce signalling overhead for CSI reporting.

In Rel-18 LTM, CSI report procedure is used for reporting L1 measurements for LTM, which could be reported in periodic/semi-Persistent/aperiodic way according to the network configuration. The UE can report a fixed number of L1 measurement results within a single L1 measurement report instance, which is configured by the parameters of nrOfReportedRS-PerCell and nrOfReportedCells. And the UE can only report the L1 measurement result of the current SpCell if the parameter of spCellInclusion is configured when the current SpCell is configured as an SpCell of an LTM candidate configuration.

In last RAN2 meeting, the following events for LTM are introduced.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

As per the above events for LTM, not only the L1 measurement results of candidate cells but also serving cell are needed to be reported when meeting the trigger conditions. It is obvious that the Rel-18 L1 measurement report cannot be reused directly. Take Event LTM3 and Event LTM5 as example, the L1 measurement reports shall include the L1 measurement results of both the candidate cells and the current serving cells within a single L1 measurement report. For Event LTM2, only the L1 measurement results of serving cells are needed, while for Event LTM4, only the L1 measurement results of candidate cells are needed.

Proposal 1: Regarding the event-triggered L1 measurement reporting for LTM, a single measurement report could include the following L1 measurement results.

- Only the results of serving cell;

- Only the results of candidate cell(s);

- Both candidate cell(s) and serving cell results.

Besides, on top of reducing signalling overhead for reporting, we shall also consider how to provide enough valid L1 measurement results to the gNB in time to guarantee the mobility performance and user experience. Considering different Events may have different number of L1 measurement results to be reported, it is better to have the event- triggered L1 measurement report with a flexible length.

Proposal 2: For LTM event-triggered L1 measurement reporting, the L1 measurement report could have a flexible length.

L1 or L2 based measurement report

Regarding how to report the L1 measurement results for event-triggered measurement enhancements, there are two potential options as follows.

- Option 1:	L1 based measurement report, e.g. UCI;

- Option 2:	L2 based measurement report, e.g. MAC CE.

For Option 1, UCI has less latency than Option 2. However, as analysed in section 2.1, the L1 measurement report may have different results of serving cell and/or candidate cell(s), which may require to have a flexible length. It may be hard to have a CSI report with a flexible length to report L1 measurement results. From RAN2 perspective, Option 2 seems a better choice.

However, as per the WID, we may also need to align with Rel-19 MIMO Enh where the MIMO feature serves the purpose of Mobility. In last few RAN1 meetings, RAN1 has made much progress on event-triggered L1 measurement report in Rel-19 FeMIMO WI. The UCI-based L1 measurement report for event-triggered measurement reporting is under discussion in RAN1. Besides, RAN1 will start the discussion on R19 LTM in this meeting. Whether to use UCI or MAC CE for event-triggered L1 measurement reporting is also related to RAN1. To avoid duplication discussion, RAN2 can wait for more RAN1 progress and further down-select the above reporting options later.

Proposal 3: Regarding whether to use UCI or MAC CE for event-triggered L1 measurement reporting, RAN2 can wait for more RAN1 progress and further down-select the above reporting options later.

Whether to support measurement report once leaving condition is met

In last RAN2 meeting, the following agreement is reached on LTM event evaluation. Whether to support measurement report once leaving conditions is met remains FFS.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In legacy L3 measurement reporting, whether to report measurement results when meeting the leaving condition is indicated by an RRC parameter configured per event. For L1 event-trigger measurement reporting, supporting reportOnLeave can help the NW to know the latest beam quality of serving cell and/or candidate cell(s) timely, which is also useful for LTM decision. Similar to L3 measurement reporting, whether to support measurement report upon meeting the leaving condition could be configured per-event level to have more flexibility.

Proposal 4: For event-triggered L1 measurement reporting, support L1 measurement reporting once leaving conditions is met, which could be configured by RRC parameter in per-event level.

- Conclusion

In this contribution, we discuss event-triggered L1 measurement reporting for Rel-19 LTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Regarding the event-triggered L1 measurement reporting for LTM, a single measurement report could include the following L1 measurement results.

- Only the results of serving cell;

- Only the results of candidate cell(s);

- Both candidate cell(s) and serving cell results.

Proposal 2: For LTM event-triggered L1 measurement reporting, the L1 measurement report could have a flexible length.

Proposal 3: Regarding whether to use UCI or MAC CE for event-triggered L1 measurement reporting, RAN2 can wait for more RAN1 progress and further down-select the above reporting options later.

Proposal 4: For event-triggered L1 measurement reporting, support L1 measurement reporting once leaving conditions is met, which could be configured by RRC parameter in per-event level.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB