3GPP TSG-RAN WG2 Meeting #127	R2-2407111

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.13.3

Source: 	China Telecom

Title:	Discussion on control plane procedures for NR sidelink multi-hop relay

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

- Introduction

In RAN#104, a new WI on NR sidelink multi-hop relay was approved in RP-241609 [1]. In Rel-17, single hop U2N relay was introduced to further extend the coverage of Uu and sidelink. Additional sidelink relay enhancements are needed to support some remaining (and new) requirements for Proximity Services captured in TS 22.278 and TS 22.261, e.g. multi-hop support for UE-to-Network Relay. Based on that, the following objective will be specified in Rel-19 NR sidelink multi-hop relay WI.

Copied from latest WID in RP-241609 - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

In this contribution, we would like to share our opinions on control plane procedures related issues for multi-hop U2N relay.

- Discussion

General aspects on supporting multi-hop U2N relay

For single hop U2N relay, the following aspects on RRC state of U2N relay UE are specified in TS 38.300[2].

Copied from TS 38.300 A U2N Relay UE shall be in RRC_CONNECTED to perform relaying of unicast data. For L2 U2N Relay operation, the following RRC state combinations are supported: - Both L2 U2N Relay UE and L2 U2N Remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data; and - The L2 U2N Relay UE can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

Regarding multi-hop U2N relay, the related U2N relays on the relay path shall follow the same principle as single hop U2N relay scenario. All the U2N Relay UEs on the relay path shall be in RRC_CONNECTED to perform relaying of unicast data. All the related multi-hop U2N relays and U2N remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data. The multi-hop U2N relay can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) or L2 U2N relay(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

Proposal 1: For multi-hop U2N relay, all the related multi-hop U2N relay UEs on the relay path shall be in RRC_CONNECTED to perform relaying of unicast data.

Proposal 2: For multi-hop U2N relay, all the related multi-hop U2N relays and U2N remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data.

Proposal 3: For multi-hop U2N relay, the multi-hop U2N relay can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) and/or all the L2 multi-hop U2N relay(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

Control plane procedures on supporting multi-hop U2N relay

Radio link failure

For radio link failure in multi-hop U2N relay scenario, the Uu RLM and RLF handling for remote UE and relay UEs can follow the same behaviour as single hop U2N relay scenario. For the intermediate relay UE, upon detecting PC5 RLF, some enhancement might be needed to indicate the RLF to the related nodes in the other direction that are not suffering the RLF. PC5-RRC message could be used to carry the indication of PC5 RLF between the relay UEs and the remote UE. The NotificationMessageSidelink message could be reused in this case. Whether to use the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3 CR drafting.

Proposal 4: For the multi-hop U2N relay UE, upon detecting PC5 RLF, the relay UE may send the NotificationMessageSidelink message to the remote UE or next hot relay UE in other direction to indicate the PC5 RLF.

Proposal 5: Whether to reuse the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3.

System information

For single hop U2N relay, the in-coverage L2 U2N Remote UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to L2 U2N Relay UE. The L2 U2N Remote UE can also receive the system information from the L2 U2N Relay UE after PC5 connection establishment with L2 U2N Relay UE. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform L2 U2N Relay UE of its requested SIB type(s) via PC5-RRC message. Then, L2 U2N Relay UE triggers on-demand SI/SIB acquisition procedure according to its own RRC state (if needed) and sends the acquired SI(s)/SIB(s) to L2 U2N Remote UE via PC5-RRC message.

For multi-hop U2N relay, the same principle can be followed. One thing may need further discussion is which relay to require the system information for the remote UE, e.g. the first relay or the last relay.

Proposal 6: For system information in multi-hop U2N relay case, RAN2 discuss which node is responsible for requiring the system information for the remote UE, e.g. the first relay or the last relay.

Paging

Similar to system information for multi-hop U2N relay scenario, it may need further discussion on which relay is responsible for monitoring paging occasions and delivering the paging for the remote UE, e.g. the first relay or the last relay.

Proposal 7: For paging in multi-hop U2N relay case, RAN2 discuss which node is responsible for monitoring paging occasions and delivering the paging for the remote UE, e.g. the first relay or the last relay.

- Conclusion

In this contribution, we discuss control plane procedures related issues for multi-hop U2N relay. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For multi-hop U2N relay, all the related multi-hop U2N relay UEs on the relay path shall be in RRC_CONNECTED to perform relaying of unicast data.

Proposal 2: For multi-hop U2N relay, all the related multi-hop U2N relays and U2N remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data.

Proposal 3: For multi-hop U2N relay, the multi-hop U2N relay can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) and/or all the L2 multi-hop U2N relay(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

Proposal 4: For the multi-hop U2N relay UE, upon detecting PC5 RLF, the relay UE may send the NotificationMessageSidelink message to the remote UE or next hot relay UE in other direction to indicate the PC5 RLF.

Proposal 5: Whether to reuse the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3.

Proposal 6: For system information in multi-hop U2N relay case, RAN2 discuss which node is responsible for requiring the system information for the remote UE, e.g. the first relay or the last relay.

Proposal 7: For paging in multi-hop U2N relay case, RAN2 discuss which node is responsible for monitoring paging occasions and delivering the paging for the remote UE, e.g. the first relay or the last relay.

- Reference

RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

3GPP TS 38.300 18.2.0: "NR; Radio Resource Control (RRC) protocol specification".