3GPP TSG-RAN WG2 Meeting #127	R2-2407112

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.2.5

Source: 	China Telecom

Title:	Discussion on Topology 2 related aspects for Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions

Document for:	Discussion and Decision

- Introduction

In last few RAN2 meetings, the Rel-19 WI of Ambient IoT was widely discussed and much progress has been made on the general aspects for Ambient IoT Topology 1. As per the WID, Topology 2 where UE is as an intermediate node under NW control shall be supported as well. Based on the progress of Topology 1, RAN2 could start to discuss the Topology 2 specific issues for supporting inventory and/or command use cases.

In this contribution, we would like to share our opinions on Topology 2 related aspects for Ambient IoT.

- Discussion

General aspects on the support of A-IoT Topology 2

In RAN2#125bis meeting, RAN2 reached the following agreements related to Topology 2.

Agreements - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

As per the above agreements, there is no specific issue on the interface between the reader and A-IoT device in Topology 2. For Topology 2, we can focus on the potential interactions between the UE reader and the NW, which may include the following aspects.

- Authorization of UE reader;

- UE reader selection;

- UE reader configuration;

- A-IoT data reporting by the UE reader.

For the authorization of UE reader, the CN is the responsible node for the authorization procedure, which is similar to the legacy authorization procedure introduced for sidelink. Thus, the detail solution for authorization of UE reader shall be decided by SA2 and RAN3.

For the selection of UE reader, it could be the CN and/or gNB to determine a suitable UE reader upon an inventory and/or command service. Since this issue is under discussion in RAN3 and SA2, RAN2 can wait for more progress and focus on the potential Uu impacts if identified by RAN3/SA2.

For the configuration of UE reader and A-IoT data reporting, they are more RAN2 issues that need further discussion on the detailed solutions.

Proposal 1: To support Topology 2 for A-IoT, RAN2 first focus on the specific issues in the Uu interface, such as the configuration of UE reader and A-IoT data reporting by the UE reader.

Proposal 2: For the selection of UE reader, RAN2 wait for more progress in RAN3/SA2 and further discuss the potential impacts if identified by RAN3/SA2.

Topology 2 specific issues

As per the WID, the UE reader shall be controlled by the NW. For the air interface, the related radio configuration for A-IoT service between A-IoT device and the UE reader shall be controlled and configured by the gNB. The UE reader will follow the configuration to trigger A-IoT service. Otherwise, it might lead to interference in the air interface and also increase the implementation complexity of the UE reader.

Proposal 3: For A-IoT Topology 2, the related radio configuration for A-IoT service between A-IoT device and the UE reader shall be controlled and configured by the gNB. The UE reader will follow the configuration to trigger A-IoT service.

Moreover, before sending the A-IoT radio configuration to a UE reader, the gNB shall get the UE radio capability of A-IoT and decide the configuration accordingly. Regarding the A-IoT UE radio capability as a UE reader, RAN2 can further discuss the details in the normative work phase.

Proposal 4: For A-IoT Topology 2, the gNB shall get the UE radio capability of A-IoT before sending the A-IoT related configuration. Further details could be discussed in the normative work phase.

In order to support the A-IoT service by the UE reader for Topology 2, the UE reader shall be in RRC connected state to receive the service request and the radio configuration from the NW. After gathering the A-IoT data from A-IoT devices, the UE reader need to send the related data to the NW, which may require the UE in RRC connected stated as well. If an idle mode or inactive mode UE is selected as a UE reader by the NW, it could be paged in the legacy way and enter into RRC connected mode accordingly.

Proposal 5: For A-IoT Topology 2, the UE reader shall be in RRC connected state when receiving the A-IoT related configuration from the gNB or when reporting the A-IoT data to the NW.

Proposal 6: For A-IoT Topology 2, if an idle mode or inactive mode UE is selected as a UE reader by the NW, it could be paged in the legacy way and enter into RRC connected mode accordingly.

Regarding whether to support the UE reader to fallback to idle mode or inactive mode during the A-IoT procedure between the UE reader and A-IoT devices, it might be a further enhancement compared to keep the UE reader in RRC connected state. In the SI phase, we suggest RAN2 focus on the basic function for supporting A-IoT Topology 2. Whether to support idle mode or inactive mode during the A-IoT procedure could be further discussed in WI phase.

Proposal 7: For A-IoT Topology 2, RAN2 assumes that the UE reader keeps in RRC connected mode during the A-IoT procedure between the UE reader and A-IoT device.

- Conclusion

In this contribution, we discuss Topology 2 specific issues for Ambient IoT. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: To support Topology 2 for A-IoT, RAN2 first focus on the specific issues in the Uu interface, such as the configuration of UE reader and A-IoT data reporting by the UE reader.

Proposal 2: For the selection of UE reader, RAN2 wait for more progress in RAN3/SA2 and further discuss the potential impacts if identified by RAN3/SA2.

Proposal 3: For A-IoT Topology 2, the related radio configuration for A-IoT service between A-IoT device and the UE reader shall be controlled and configured by the gNB. The UE reader will follow the configuration to trigger A-IoT service.

Proposal 4: For A-IoT Topology 2, the gNB shall get the UE radio capability of A-IoT before sending the A-IoT related configuration. Further details could be discussed in the normative work phase.

Proposal 5: For A-IoT Topology 2, the UE reader shall be in RRC connected state when receiving the A-IoT related configuration from the gNB or when reporting the A-IoT data to the NW.

Proposal 6: For A-IoT Topology 2, if an idle mode or inactive mode UE is selected as a UE reader by the NW, it could be paged in the legacy way and enter into RRC connected mode accordingly.

Proposal 7: For A-IoT Topology 2, RAN2 assumes that the UE reader keeps in RRC connected mode during the A-IoT procedure between the UE reader and A-IoT device.

- Reference

RP-240826	Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR 		CMCC, Huawei, T-Mobile USA

3GPP TR 38.848 v18.0.0: " Study on Ambient IoT (Internet of Things) in RAN".

R2-xxxxxx	draft Report of 3GPP TSG RAN WG2 meeting #125bis, 126