3GPP TSG-RAN WG2 Meeting #126	R2-2407116

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	7.9.3

Source:	Nokia

Title:	Correction in U2U relay sidelink DRB addition/modification

WID/SID:	NR_SL_relay_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The specification the procedure for L2 U2U DRB addition/modification in clause 5.8.9.1a.2.2 (Sidelink DRB addition/modification operations) of TS 38.331 is not complete for UEs in RRC_CONNECTED.

# 2	Discussion

In clause 5.8.9.1a.2.2 (Sidelink DRB addition/modification operations) the procedure for L2 U2U DRB addition is the following:

5.8.9.1a.2.2 Sidelink DRB addition/modification operations For the sidelink DRB, whose sidelink DRB addition conditions are met as in clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall: <***** text omitted ******> 1> for an end-to-end sidelink DRB (i.e. the UE is acting as L2 U2U Remote UE or L2 U2U Relay UE): 2> if the UE is in RRC_CONNECTED: 3> (re)configure the SRAP entity for the sidelink DRB, in accordance with the sl-SRAP-ConfigU2U included in sl-ConfigDedicatedNR, received from RRCReconfiguration; 2> else if the UE is in RRC_IDLE or RRC_INACTIVE, or out of coverage: 3> perform the PC5 Relay RLC channel addition/modification as specified in clause 5.8.9.7.2 if needed, according to the derived PC5 Relay RLC channel configuration as described in clause 5.8.9.11; 3> consider the PC5 Relay RLC channel applying the derived PC5 Relay RLC channel configuration as the egress PC5 Relay RLC channel; 3> configure the egress PC5 Relay RLC channel for this end-to-end sidelink DRB to SRAP; NOTE 1: When a sidelink DRB addition is due to the configuration by RRCReconfigurationSidelink, it is up to UE implementation to select the sidelink DRB configuration as necessary transmitting parameters for the sidelink DRB, from the received sl-ConfigDedicatedNR (if in RRC_CONNECTED), SIB12 (if in RRC_IDLE/INACTIVE), SidelinkPreconfigNR (if out of coverage) with the same RLC mode as the one configured in RRCReconfigurationSidelink.

In clause 5.8.9.1a.2.2 (Sidelink DRB addition/modification operations) the procedure for L2 U2U DRB modification is the following:

For the sidelink DRB, whose sidelink DRB modification conditions are met as in clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall: <***** text omitted ******> 1> for an end-to-end sidelink DRB (i.e. the UE is acting as L2 U2U Remote UE or L2 U2U Relay UE): 2> if the UE is in RRC_CONNECTED: 3> reconfigure the SRAP entity for the sidelink DRB, in accordance with the sl-SRAP-ConfigU2U received in sl-ConfigDedicatedNR, if included; 2> else if the UE is in RRC_IDLE or RRC_INACTIVE, or out of coverage: 3> if the derived PC5 Relay RLC channel configuration of this end-to-end sidelink DRB as described in clause 5.8.9.11 is changed: 4> if there is no other end-to-end sidelink DRB(s) associated with this RLC channel: 5> perform the PC5 Relay RLC channel release as specified in 5.8.9.7.1 or 5.8.9.7.2; 4> else: 5> perform the PC5 Relay RLC channel addition/modification as specified in 5.8.9.7.2; 4> consider the PC5 Relay RLC channel applying the PC5 Relay RLC channel configuration as the egress PC5 relay RLC channel; 4> reconfigure the SRAP entity with the the egress PC5 Relay RLC channel for the end-to-end sidelink DRB.

Observation: The description for the case when the UE is in RRC_CONNECTED does not include the operations related to the RLC channels.

In principle the difference between these cases (UE is in RRC_CONNECTED vs UE is not in RRC_CONNECTED) is where the DRB configuration (including RLC and SRAP) is coming from, but the steps should be similar. Therefore, the RLC channel related steps should also specified in this clause when the UE is RRC_CONNECTED.

Proposal 1: The RLC channel related operations should be specified for UEs in RRC_CONNECTED in clause 5.8.9.1a.2.2.

Proposal 2: If proposal 1 is agreed then adopt the specification changes proposed in the Annex.

# 3	Conclusion

This document has made the following observations and proposals:

Observation: The description for the case when the UE is in RRC_CONNECTED does not include the operations related to the RLC channels.

Proposal 1: The RLC channel related operations should be specified for UEs in RRC_CONNECTED in clause 5.8.9.1a.2.2.

Proposal 2: If proposal 1 is agreed then adopt the specification changes proposed in the Annex.

# Annex: TP for 38.331

<***** Start of modifications ******>

5.8.9.1a.2.2	Sidelink DRB addition/modification operations

For the sidelink DRB, whose sidelink DRB addition conditions are met as in clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

<***** text omitted ******>

1>	for an end-to-end sidelink DRB (i.e. the UE is acting as L2 U2U Remote UE or L2 U2U Relay UE):

2>	if the UE is in RRC_CONNECTED:

3>	perform the PC5 Relay RLC channel addition/modification as specified in clause 5.8.9.7.2 if needed according to the PC5 Relay RLC channel configuration received in sl-ConfigDedicatedNR included in the RRCReconfiguration;

3>	(re)configure the SRAP entity for the sidelink DRB, in accordance with the sl-SRAP-ConfigU2U included in sl-ConfigDedicatedNR, received from RRCReconfiguration;

2>	else if the UE is in RRC_IDLE or RRC_INACTIVE, or out of coverage:

3>	perform the PC5 Relay RLC channel addition/modification as specified in clause 5.8.9.7.2 if needed, according to the derived PC5 Relay RLC channel configuration as described in clause 5.8.9.11;

3>	consider the PC5 Relay RLC channel applying the derived PC5 Relay RLC channel configuration as the egress PC5 Relay RLC channel;

3>	configure the egress PC5 Relay RLC channel for this end-to-end sidelink DRB to SRAP;

NOTE 1:	When a sidelink DRB addition is due to the configuration by RRCReconfigurationSidelink, it is up to UE implementation to select the sidelink DRB configuration as necessary transmitting parameters for the sidelink DRB, from the received sl-ConfigDedicatedNR (if in RRC_CONNECTED), SIB12 (if in RRC_IDLE/INACTIVE), SidelinkPreconfigNR (if out of coverage) with the same RLC mode as the one configured in RRCReconfigurationSidelink.

For the sidelink DRB, whose sidelink DRB modification conditions are met as in clause 5.8.9.1a.2.1, the UE capable of NR sidelink communication that is configured by upper layers to perform NR sidelink communication shall:

<***** text omitted ******>

1>	for an end-to-end sidelink DRB (i.e. the UE is acting as L2 U2U Remote UE or L2 U2U Relay UE):

2>	if the UE is in RRC_CONNECTED:

3> 	if the received PC5 Relay RLC channel configuration of this end-to-end sidelink DRB as described in clause 5.8.9.11 is changed:

4>	perform the PC5 Relay RLC channel addition/modification as specified in clause 5.8.9.7.2 or release as specified in clause 5.8.9.7.1 according to the PC5 Relay RLC channel configuration received in sl-ConfigDedicatedNR included in the RRCReconfiguration;

4>	reconfigure the SRAP entity for the sidelink DRB, in accordance with the sl-SRAP-ConfigU2U received in sl-ConfigDedicatedNR, if included;

2>	else if the UE is in RRC_IDLE or RRC_INACTIVE, or out of coverage:

3>	if the derived PC5 Relay RLC channel configuration of this end-to-end sidelink DRB as described in clause 5.8.9.11 is changed:

4>	if there is no other end-to-end sidelink DRB(s) associated with this RLC channel:

5>	perform the PC5 Relay RLC channel release as specified in 5.8.9.7.1 or 5.8.9.7.2;

4>	else:

5>	perform the PC5 Relay RLC channel addition/modification as specified in 5.8.9.7.2;

4>	consider the PC5 Relay RLC channel applying the PC5 Relay RLC channel configuration as the egress PC5 relay RLC channel;

4>	reconfigure the SRAP entity with the the egress PC5 Relay RLC channel for the end-to-end sidelink DRB.

<***** End of modifications ******>