3GPP TSG-RAN WG2 Meeting #127	R2-2407117

# Maastricht, Netherlands, 19 – 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | 0136 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 alignments with stage 3 specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Give the reason(s) for change: - There is an incomplete paragraph in 5.1.3: “or if none of the MDT measurements in the MDT configurations are subject to user consent.” Based on CR0135 approved in RP-241544 this sentence should not be in the specification. - RLF report can contain two consecutive failures in case one or both failures are related to CHO. In case of consecutive failures, the UE stores and reports both failure related information in the RLF report. The consecutive failure scenarios concern the sequence of events defined in 5.4.1.2. However, the cases a, b, c description does not differentiate detailed steps of CHO recovery procedure that covers storing CHO configuration, CHO execution or CHO recovery excution. This introduces ambiguity,which particular steps of the procedure are considered for storing (consecutive) RLF reports. - Stage 2 description for MDT support in the legacy RATs (UTRAN and E-UTRAN) clearly defines dedicated sections for “Detailed location information”. CR0077 introducing MDT support for NR (R2-2002002) did not introduce similar section, refering to generic description on Immediate MDT procedures (5.1.2), where general requirement on including detailed location informaiton is placed. However, this introduces ambiguity, which MDT measurements should be tagged with GNSS dataFirst reason for change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The incorrect paragraph is removed from 5.1.3. - Detailed description for storing RLF report for CHO in clause 5.4.1.2 is added, and reference is corrected - Detailed Location Information clause is added under NR solutions for RRC Connected Impact analysis Impacted functionality: RLF report generation and Immediate MDT Inter-operability: - If the network is implemented according to the CR and the UE is not there should be no interoperabilty issues. - If the UE is implemented according to the CR and the network is not there should be no interoperabilty issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 37.320 is not aligned with 38.331 and thus specifications remain ambiguous. Moreover an incorrect paragraph remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3, 5.4.1.2, 5.4.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.1.3	MDT Initiation

There are two cases that RAN should initiate a MDT measurements collection task. One is that the MDT task is initiated without targeting a specific UE by the cell traffic trace, i.e. management based trace function from OAM. The other is that the MDT task is initiated towards a specific UE by the signalling trace activation messages from CN nodes, i.e. the Initial Context Setup message, the Trace Start message or the Handover request message in E-UTRAN or NR, the CN Invoke Trace message in UTRAN. The detailed procedures to transfer the MDT configurations to RAN are specified in TS 32.422 [6].

For signalling based MDT within a PLMN, the CN shall not initiate MDT towards a particular user unless it is allowed.

For management based MDT within a PLMN, the CN indicates to the RAN whether MDT is allowed to be configured by the RAN for this user considering e.g. user consent and roaming status (see TS 32.422 [6]), by providing management based MDT allowed information. For MDT measurements that are not subject to user consent, the RAN may initiate MDT towards a particular UE independently of user consent. For E-UTRAN/UTRAN, the MDT allowed information consists of the Management Based MDT Allowed indication and optionally the Management Based MDT PLMN List. For NR, the MDT allowed information only consists of the Management Based MDT PLMN List. The management based MDT allowed information propagates during inter-PLMN handover or inter-PLMN UE context retrieval if the Management Based MDT PLMN List is available and includes the target PLMN.

A UE is configured with an MDT PLMN List from AMF if user consent is valid for the RPLMN, or a UE is configured with an MDT PLMN List from OAM if any of the MDT measurements in the MDT configuration is configurable and not subject to user consent, as specified in TS 32.422 [6].

User consent does not apply if the UE is served by an SNPN.

Next Modified Subclause

#### 5.4.1.2	Radio Link Failure report

The Radio Link Failure report contains information related to the latest connection failure experienced by the UE. The connection failure can be Radio Link Failure (RLF), or Handover Failure (HOF). The contents of the RLF report and the procedure for retrieving it by a gNB are specified in TS 38.331 [15], including failure information related to CHO, or DAPS Handover Failure (DAPS HOF). In case of consecutive connection failures associated to CHO or DAPS, the UE stores and reports both failure related information in the RLF report.

RLF report can contain latest two consecutive failures in case one or both failures are related to CHO. In case of consecutive failures, the UE stores and reports both failure related information in the RLF report. The consecutive failure scenarios concern the following sequence of events:

a.	A UE that has CHO configuration (as specified in TS 38.331 [15]) detects RLF in the source cell. The UE selects a configured candidate CHO target cell for CHO recovery via re-establishment procedure. The UE fails to re-establish to the selected CHO candidate cell when performing conditional reconfiguration.

b.	A UE that has CHO configuration, executes the CHO towards the target cell upon fulfilling the configured condition and experiences a HO failure. The UE selects a configured candidate CHO target cell for CHO recovery via re-establishment procedure. The UE fails to re-establish to the selected CHO candidate cell when performing conditional reconfiguration.

c.	A UE that has CHO configuration executes the normal HO towards the target cell and experiences a HO failure. The UE selects a configured candidate CHO target cell for CHO recovery via re-establishment procedure. The UE fails to re-establish to the selected CHO candidate cell when performing conditional reconfiguration.

For DAPS, two consecutive failure information concern the following scenarios:

a.	A UE detects a connection failure at the source (RLF) while performing access to DAPS target cell and fails to access the target (HOF).

b.	A UE detects a connection failure at the target cell (HOF) and fails to perform fallback (RLF at source).

In addition, for RACH Optimization solutions, NR RLF report may include part of the RACH report with:

-	preamble configuration and indications specific to features (e.g. RedCap, Small Data Transmission or Slicing) or their combinations, associated to the RA procedure, as specified in TS 38.331 [15].

-	additional information required for Self-Optimisation solutions, as specified in TS 38.300 [22].

NR RLF report content required for MDT includes:

-	Latest radio measurement results of the serving and neighbouring cells, including SSB/CSI-RS index and associated measurements in the serving and neighbouring cells;

NOTE:	The measure quantities are sorted through the same RS type depending on the availability, according to the following priority: RSRP, RSRQ, SINR.

-	For CHO, UE includes the latest radio measurement results of the candidate target cells;

-	WLAN and Bluetooth measurement results, if were configured prior RLF and are available for reporting;

-	"No suitable cell is found" flag when T311 expires;

-	Indication per SSB/CSI-RS beams reporting whether it is configured to RLM purpose;

-	Available sensor information;

-	Available detailed location information;

-	RACH failure report (in case, the cause for RLF is random access problem or Beam Failure Recovery failure):

-	Tried SSB/CSI-RS index and number of Random Access Preambles transmitted for each tried SSB/CSI-RS in chronological order of attempts;

-	Contention detected as per RACH attempt;

-	Indication whether the selected SSB is above or below the rsrp-ThresholdSSB threshold, as per RACH attempt;

-	TAC of the cell in which the UE performs the RA procedure;

-	Frequency location related information of the RA resources used by the UE as specified in TS 38.331 [15];

-	For 2-step RA, the following information can be additionally included:

-	The measured RSRP of DL pathloss reference obtained just before performing RACH procedure (per RA procedure);

-	Indication that fallback from 2-step RA to 4-step RA was performed by the UE, as per RACH attempt;

-	Indication of RA switching point (as defined by the field msgA-TransMax in TS 38.331 [15]);

-	The payload size available in the UE buffer at the time of initiating the 2-step RA procedure, without considering the padding (per RA procedure);

-	MSGA PUSCH resources for 2-step RACH as specified in TS 38.331 [15] can be included in case the UE uses random access resources configured with CFRA.

If detailed location information (e.g. GNSS location information) is available the reported location information in rlf-Report consists of:

-	Latitude, longitude (mandatory);

-	Altitude (conditional on availability);

-	Velocity (conditional on availability);

-	Uncertainty (conditional on availability);

-	Confidence (conditional on availability);

-	Direction (conditional on availability).

If sensor information is available, the sensor information may convey uncompensated barometric pressure, UE speed, and UE orientation.

In addition, the RLF report may include additional information required for MRO solutions, as specified in TS 38.300 [22].

Next Modified Subclause (new clause)

#### 5.4.1.x	Detailed Location Information

The M1 measurements are tagged by the UE with location data in the following manner:

-	Detailed location information (e.g. GNSS location information) is included if available in the UE when the measurement was taken. If detailed location information is available, the reporting consists of information as specified in TS 38.331 [15]

End of Changes