3GPP TSG-RAN WG2 Meeting #127	R2-2407118

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	7.13.2

Source:	Nokia

Title:	RAN2 impacts of RAN3 reply LS on MDT for NPN (R2-2406218/R3-243892)

WID/SID:	NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 has received a reply LS on MDT for NPN in R2-2406218 (R3-243892). This paper discusses the RAN2 specification impacts RAN3 answers.

# 2	Discussion

## 2.1	Number of PNI-NPN identities

RAN3 replied the following on this issue:

RAN3 answer: RAN3 agreed to keep the number of PNI-NPN identities (PLMN ID+CAG) supported in RAN3 specifications to 256. However to alleviate the UE memory consumption issue highlighted by RAN2, for logged MDT, RAN3 specifications have been enhanced with amendments in the semantics of the CAG List for MDT , stating that this list contains a maximum of 144 Public Network Integrated NPNs with a maximum of 12 different PLMN identities where a PLMN ID may be repeated more than once (see attached CRs).

The current specification regarding to the maximum number of PNI-NPN identities for logged MDT configuration is the following:

CAG-ConfigList-r18 ::=           SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-Config-r18

CAG-Config-r18 ::=               SEQUENCE {

plmn-Identity-r18                PLMN-Identity,

cag-IdentityList-r18             SEQUENCE (SIZE (1..maxNPN-r16)) OF BIT STRING (SIZE (32))

}

maxNPN-r16                              INTEGER ::= 12      -- Maximum number of NPNs broadcast and reported by UE at establishment

This specification meets the RAN3 requirement that there can be maximum 12 PLMN IDs and all together there can be maximum 144 PNI-NPN listed in the MDT configuration. In 38.331 there is an additional limitation:

cag-IdentityList The cag-IdentityList contains one or more CAG IDs. All CAG IDs associated to the same PLMN ID are listed in the same cag-IdentityList entry.

As RAN3 explicitly stated that a PLMN ID may be repeated more than once (see text highlighted by green), this limitation should be removed, and a clarification sentence should be added the PLMN ID may be repeated.

Proposal 1: Remove the limitation that “All CAG IDs associated to the same PLMN ID are listed in the same cag-IdentityList entry.” and add a clarification that the same PLMN ID may be present more than once within the CAG-ConfigList.

Proposal 2: Adopt the change below in 38.331 according to proposal 1:

cag-IdentityList The cag-IdentityList contains one or more CAG IDs. A PLMN ID may be present more than once within CAG-ConfigList.

## 2.2	Applicability of existing plmn-IdentityList for logged MDT measurements in SNPN networks

RAN3 replied the following on this issue:

RAN3 answer: No, plmn-IdentityList applies only to public networks and it shall not be used for SNPNs.

Our understanding is that this means that no additional specification is needed in RAN2 in this area.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: Remove the limitation that “All CAG IDs associated to the same PLMN ID are listed in the same cag-IdentityList entry.” and add a clarification that the same PLMN ID may be present more than once within the CAG-ConfigList.

Proposal 2: Adopt the change below in 38.331 according to proposal 1:

cag-IdentityList The cag-IdentityList contains one or more CAG IDs. A PLMN ID may be present more than once within CAG-ConfigList.