3GPP TSG-RAN WG2 Meeting #127	R2-2407119

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.10.2

Source:	Nokia

Title:	MRO for LTM

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion and Decision

# Introduction

The following agreements were reached on the topic of MRO for LTM at RAN2#125bis and RAN2#126:

- For LTM MRO, RAN2 considers the following three connection failure cases:

- -	Too late LTM

- -	Too early LTM

- -	LTM to wrong cell

- For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

- -	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

- -	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

- For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

- LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

- RAN2 considers SHR, RA report and RLF for MCG LTM SON.

- RAN2 will start work on MCG LTM.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

This paper discusses clarifications of the agreed LTM MRO scenarios and further LTM failure cases referring to the other important phases (like preparation or early synchronization phase) beyond the LTM cell switch execution phase.

# 2	Discussion on LTM MRO scenarios

## 2.1	Clarification on the already agreed LTM MRO scenarios

The agreed scenarios from RAN2#125bis represent an adaptation of classic MRO scenarios as defined in the past for other mobility mechanisms. However, the current wording is at times unclear or lacks essential details. In the following we will attempt to clarify the overall goals and propose rewording of each scenario.

If legacy definitions for Too Late, Too Early and Wrong Cell are to be kept then it is our understanding that, for correct root cause analysis, only the cell the UE finally reestablishes radio connection to, either via re-establishment or successful LTM recovery, is of interest.

Proposal 1: RAN2 to agree that root cause analysis is based on the cell the UE finally reestablishes radio connection to, either via re-establishment or successful LTM recovery.

### 2.1.1	Too Late LTM

Legacy definition in 38.300: Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

The defining characteristic of a too late handover is that the UE was long enough in a cell as to be configured with a possible handover or it was configured but the handover was not executed. As a result of running out of coverage of the serving cell, an RLF is detected, and re-establishment is attempted to a different cell other than the source cell. This holds true also in the case that an LTM recovery is attempted after the initial failure and prior to the re-establishment.

Agreed scenario 1a seems to cover this exact scenario. However, without specifying to which cell the re-establishment is attempted, it cannot be clearly classified as a too late handover. Therefor we propose the following re-wording:

- Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and attempts re-establishment in a cell other than the source cell.

Proposal 2: RAN2 to agree the update of case 1a description as follows: 
Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and attempts re-establishment in a cell other than the source cell.

Case 1b mentions that the UE ‘selects an LTM candidate cell’, which means the UE immediately attempts LTM recovery to a candidate cell, but the recovery fails due to T304 expiry. The current definition of the scenario is however incomplete as after LTM recovery failure, the UE may also attempt re-establishment.

Observation 1: The current definition of case 1b ends with a failed LTM fast recovery without knowing the final target cell, i.e., where UE will re-establish or reconnect.

If the selected cell for re-establishment is a cell other than the source cell, this scenario can be mapped to a too late handover. But if the selected cell for re-establishment is the source cell again, this scenario sounds more like coverage hole with LTM fast recovery attempt to wrong cell. To clarify this scenario, we propose the following rewording that also helps to distinguish between regular/initial LTM execution and LTM recovery:

Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell and attempts LTM recovery, which fails, then attempts re-establishment in a cell other than the source cell.

Proposal 3: RAN2 to agree the update of case 1b description as follows: 
Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell and attempts LTM recovery, which fails, then attempts re-establishment in a cell other than the source cell.

Case 1c is similar to case 1b with the slight difference that the LTM recovery to the selected LTM candidate cell is successful. In order to further clarify the scenario definition, we propose the following rewording:

Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM recovery with an LTM candidate cell other than the source cell.

Proposal 4: RAN2 to agree the update of case 1c description as follows: 
Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM recovery with an LTM candidate cell other than the source cell.

### 2.1.2	Too early LTM

Legacy definition in 38.300: Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

Case 2a follows the legacy definition and we propose the following clarification of the text:

Case 2a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in the source cell.

Proposal 5: RAN2 to agree the update of case 2a description as follows: 
Case 2a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in the source cell.

In our understanding, case 2b, should reflect a similar scenario as case 1b: first mobility related failure which is HOF or RLF in the target cell shortly after the cell switch followed by a failed LTM fast recovery and ending up with a re-establishment. In this scenario the source cell is explicitly selected as candidate for the LTM recovery attempt and fails. Where it finally re-establishes is left open. This is a highly unprobeable scenario for too early handover as a reestablishment to the source cell would have to follow and be successful. As such we propose that case 2b be removed.

Proposal 6: RAN2 to remove case 2b from too early LTM use case.

We think that the final re-establishment / reconnection cell is to be used for scenario categorization, i.e., irrespective which cell was tried for LTM recovery, the one where UE ends up after re-establishment or reconnection is the real target. Therefore, the re-establishment cell needs to be specified. We propose the following new case definition which covers a scenario similar to case 1b that can be correctly mapped to too early LTM:

Case 2d: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell and attempts LTM recovery, which fails, then attempts re-establishment in the source cell.

Proposal 7: RAN2 to add case 2d to the too early LTM use cases: 
Case 2d: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell and attempts LTM recovery, which fails, then attempts re-establishment in the source cell.

Case 2c is complementary to the new case 2d but here the LTM recovery is successful. Only minor additions for clarity are needed to distinguish between regular/initial LTM execution and LTM recovery:

Case 2c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the source cell which is also an LTM candidate cell.

Proposal 8: RAN2 to agree the update of case 2c description as follows: 
Case 2c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the source cell, which is also an LTM candidate cell.

### 2.1.3	Wrong cell LTM

Legacy definition in 38.300: Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

Case 3a, in its corrected form after RAN2#126, needs only minor clarification to reflect the classic scenario.

Case 3a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in a cell other than the source cell and the target cell.

Proposal 9: RAN2 to agree the update of case 3a description as follows: 
Case 3a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in a cell other than the source cell and the target cell.

Case 3b seems to be missing an important part of its description, namely to which cell does the UE attempt re-establishment after the failed LTM recovery. As this detail is vital to the correct determination of the KPI, we propose the following rewording:

- Case 3b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell and attempts LTM recovery, which fails, and then attempts re-establishment in a cell other than source or target cell.

Proposal 10: RAN2 to agree the update of case 3b description as follows: 
Case 3b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell and attempts LTM recovery, which fails, and then attempts re-establishment in a cell other than source or target cell.

Finally, scenario 3c, complementary to scenario 3b, needs only minor additions:

Case 3c: the UE detects HOF during LTM cell switch or /RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the selected LTM candidate cell which is different from the source or target one.

Proposal 11: RAN2 to agree the update of case 3b description as follows: 
Case 3c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the selected LTM candidate cell which is different from the source or target one.

## 2.2	 Early TA acquisition optimization

The main rational behind and strength of LTM is the RACH-less cell change, which is only possible with knowing the timing advance (TA) when cell switch is triggered. One critical part of the LTM is therefore the phase called early TA acquisition, which the UE may be instructed to perform before receiving the cell switch command. If the ETA is successful, the interruption time during the cell switch will be significantly reduced as RACH can be skipped. If the UE received the cell switch command that does not include a valid TA previously acquired for the target cell, the UE can still access the cell via regular RACH procedure, but the benefits of LTM compared to the legacy L3 handover types are lost.

In case of the fully defined network-triggered early TA acquisition, the two trigger criteria for early TA acquisition and cell switch command need to be properly synced for a successful RACH-less LTM. Firstly, the TA once acquired may become invalid because of changed UE position or changed propagation conditions with respect to the candidate cell and the network will need to trigger early TA acquisition again. Secondly, the early TA acquisition procedure should not be performed too often as it is associated with some interruption time. It is thus important that the early TA acquisition trigger criterion is cell-pair individually and mutually with the LTM cell switch command trigger criterion optimized.

Observation 2 Timely and correct early TA acquisition is a crucial part of LTM to guarantee the benefits of LTM, namely a RACH-less cell switch for minimum handover interruption time.

Proposal 12: RAN2 to study what additional reports could enable the optimization of early TA acquisition in addition to the already agreed scenarios.

## 2.3	 Enhancement of SHR

Successful HO report (SHR) was introduced in Rel-17 to capture near failure events that happen during a successful HO. SHR is triggered based on running timers (T310/T312/T304) exceeding a preconfigured threshold. In some cases, additional SHR triggering conditions have been defined. For example, in case of DAPS HO, SHR is also generated if the UE experiences an RLF on the source link during the DAPS HO as this is not the desired behaviour.

Similarly, in the context of LTM, if the UE is not able to access the target cell in a RACH-less manner and is using RACH-based (due to the absence of a valid TA in the MAC CE command) or in case of RACH-less the ACK for the first UL transmission taking longer than expected would constitute a similar undesired behavior (near failure scenario). The LTM handover will still be successful (e.g. by means of fallback to RACH-based access or since the number of RLC retransmissions remain below the maximum admissible number) but the interruption time would be significantly increased.

Observation 3: Missing RACH-less and fallback to RACH-based access to target cell or the ACK for first UL transmission taking longer than expected during an LTM HO represents an undesired behaviour.

Proposal 13 RAN2 to define additional SHR triggering conditions specific to LTM near failure cases.

Another possible optimization aspect with respect to LTM is the unnecessary configurations. As a reminder, in LTM, the UE may receive multiple configurations but will only execute one of them once it received the cell switch command. Moreover, the UE does not release the configuration and the prepared candidate PCells after LTM cell switch execution or LTM recovery execution. This enables fast triggering of cell changes and reduces the signalling overhead that is associated with the preparation of the LTM configurations.

There is currently however no way for the network to detect any unnecessary configurations and implicitly blocked resources. Information that would help the network to identify these cases could be added to the SHR.

Proposal 14: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: RAN2 to agree that root cause analysis is based on the cell the UE finally reestablishes radio connection to, either via re-establishment or successful LTM recovery.

Proposal 2: RAN2 to agree the update of case 1a description as follows: 
Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and attempts re-establishment in a cell other than the source cell.

Observation 1: The current definition of case 1b ends with a failed LTM fast recovery without knowing the final target cell, i.e., where UE will re-establish or reconnect.

Proposal 3: RAN2 to agree the update of case 1b description as follows: 
Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell and attempts LTM recovery, which fails, then attempts re-establishment in a cell other than the source cell.

Proposal 4: RAN2 to agree the update of case 1c description as follows: 
Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM recovery with an LTM candidate cell other than the source cell.

Proposal 5: RAN2 to agree the update of case 2a description as follows: 
Case 2a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in the source cell.

Proposal 6: RAN2 to remove case 2b from too early LTM use case.

Proposal 7: RAN2 to add case 2d to the too early LTM use cases: 
Case 2d: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell and attempts LTM recovery, which fails, then attempts re-establishment in the source cell.

Proposal 8: RAN2 to agree the update of case 2c description as follows: 
Case 2c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the source cell, which is also an LTM candidate cell.

Proposal 9: RAN2 to agree the update of case 3a description as follows: 
Case 3a: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch and attempts re-establishment in a cell other than the source cell and the target cell.

Proposal 10: RAN2 to agree the update of case 3b description as follows: 
Case 3b: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, selects an LTM candidate cell and attempts LTM recovery, which fails, and then attempts re-establishment in a cell other than source or target cell.

Proposal 11: RAN2 to agree the update of case 3b description as follows: 
Case 3c: the UE detects HOF during LTM cell switch or RLF in the LTM target cell shortly after the LTM cell switch, and successfully completes LTM recovery with the selected LTM candidate cell which is different from the source or target one.

Observation 2 Timely and correct early TA acquisition is a crucial part of LTM to guarantee the benefits of LTM, namely a RACH-less cell switch for minimum handover interruption time.

Proposal 12: RAN2 to study what additional reports could enable the optimization of early TA acquisition in addition to the already agreed scenarios.

Observation 3: Missing RACH-less and fallback to RACH-based access to target cell or the ACK for first UL transmission taking longer than expected during an LTM HO represents an undesired behaviour.

Proposal 13 RAN2 to define additional SHR triggering conditions specific to LTM near failure cases.

Proposal 14: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.