3GPP TSG-RAN WG2 Meeting #127	R2-2407120

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.10.2

Source:	Nokia

Title:	MRO for CHO with candidate SCG

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were reached on the topic of MRO for CHO with candidate SCG at RAN2#125bis and RAN2#126:

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

Similar agreements were reached in RAN3#124bis as well:

MRO for CHO with candidate SCG failure and near failure cases.

RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

R19 SON/MDT solution discussion is based on R18 work.

Agreements from RAN3#124 exclude certain scenarios from being handled in this release:

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

This paper discusses further details on the possible objectives for MRO for CHO with candidate SCG.

# 2	Enhancements for MRO for CHO with candidate SCGs

As already agreed, both in RAN2 and RAN3, MRO for CHO with candidate SCG(s) should consider both failure and near failure scenarios.

A UE configured for CHO with candidate SCG(s) is carrying out a joint evaluation of both CHO and CPAC, where both conditions need to be fulfilled before a concerted CHO and CPC is executed. The criterion (condExecutionCondPSCell) for the candidate PSCell can only be a condEventA4, where the currently serving PSCell can be considered as candidate. The criterion for MCG CHO called condExecutionCond in the configuration field is typically a condEventA3.

Mobility failure scenarios would be those in which one or both execution conditions (i.e. CHO and/or CPAC) are not fulfilled. This would result in a failure on MCG and/or SCG being detected by the UE before an execution is possible. It is important in this case to know which execution condition did not trigger in order to decide on the appropriate corrective actions. RAN2 already agreed to enhance the RLF report in case of MCG failure, but there was no decision on enhancements for SCGFailureInformation which is generated in case of SCG failure. While it is true that an MCG failure carries more weight, it is nevertheless important to also prevent failures of the SCG as this can have a direct impact on the throughput of the user.

Proposal 1: RAN2 to agree to also enhance SCGFailureInformation with additional information regarding the state of the two execution conditions in case of SCG failure, when the UE is configured CHO with candidate SCG(s).

Near failure scenarios in the context of CHO with candidate SCGs would be those in which both conditions are fulfilled but with a big gap in timing between them which inevitably introduced interruption time and service degradation. In these cases, it is also important for the network to know which execution event caused the delay in the handover execution or indication of misalignment between configurations.

Proposal 2: RAN2 to enhance SHR and SPR with new triggers (e.g. based on time gap between condition fulfilment) and content that allows the network to identify the execution condition that delayed the handover.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: RAN2 to agree to also enhance SCGFailureInformation with additional information regarding the state of the two execution conditions in case of SCG failure, when the UE is configured CHO with candidate SCG(s).

Proposal 2: RAN2 to enhance SHR and SPR with new triggers (e.g. based on time gap between condition fulfilment) and content that allows the network to identify the execution condition that delayed the handover.