3GPP TSG RAN WG2 #127	R2-2407121

Maastricht, The Netherlands, August 19 - 23, 2024

Agenda Item:	8.9.3 
Source:	Toyota ITC
Title:	Discussion on Contention Resolution Diversity Slotted ALOHA

Document for:	Discussion and Decision

# Introduction

In the RAN #103 meeting, the work item for Rel-19 Non-Terrestrial Networks (NTN) targeting Internet of Things (IoT) Phase 3 was approved [1], with one of key objectives being the enhancement of uplink.

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

Subsequently, at the RAN2#126 meeting, an approach termed Diversity Slotted ALOHA (DSA) was presented [2] which falls under the scope of this work item [1]. This concept was shown to significantly outperform the traditional slotted ALOHA method [3], offering substantial performance gains. RAN2 reached a consensus to further explore DSA, along with its advanced variant, Contention Resolution Diversity Slotted Aloha (CRDSA), to assess their potential benefits and impacts.

Agreements: 1. RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) 3. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

The aim of this document is to evaluate CRDSA's implications on the existing 3GPP specifications, ensuring that any integration aligns with and enhances the current standard.

# Discussion

The core principle of Diversity Slotted ALOHA (DSA) involves transmitting multiple copies of the same message across various randomly selected sub-frames within a radio frame. This strategy increases the probability of at least one replica being received without interference, thereby enhancing the chances of successful message delivery. However, a notable drawback of DSA is the potential for system congestion at lower traffic volumes compared to traditional Slotted ALOHA (SA), as the transmission of multiple replicas can lead to a higher likelihood of collisions.

An advancement over DSA is the Contention Resolution DSA (CRDSA) technique, which incorporates a set of unique identifiers for each replica in the form of subframe numbers. It means that all replicas convey a set of randomly generated sub-frame numbers sent within the radio frame. Upon receiving at least one collision-free replica, the receiver employs successive interference cancellation (SIC), utilizing the subframe numbers to resolve contention and thereby maximize the number of collision-free transmission attempts. The practical application and effectiveness of CRDSA have been demonstrated in the satellite Digital Video Broadcasting - Return Channel via Satellite 2 (DVB-RCS2) standard [4]. A current requirement of CRDSA is the inclusion of sub-frame number information in each data unit, indicating the locations of the replicas within the radio frame.

Observation 1: A current requirement of contention resolution diversity slotted ALOHA is the inclusion of sub-frame information in each data unit, indicating the locations of the replicas within the radio frame.

Deployment considerations also affect the use of sub-frame numbers. These sub-frame numbers are only necessary if the receiver uses successive interference cancellation to enhance communication. For receivers with DSA, sub-frame numbers are superfluous, and they merely increase overhead. In contrast, for receivers using CRDSA, sub-frame numbers are vital for effective contention resolution. Hence, the transmitting user equipment must know whether the network supports DSA or CRDSA to determine the need for sub-frame numbers unless sub-frame numbers are consistently included, which would result in unnecessary overhead.

Observation 2: The user equipment must know whether the network supports DSA or CRDSA unless the sub-frame numbers are always included.

The overhead associated with sub-frame headers for 3 replicas and 12 sub-frames can be more clearly explained as follows: Each sub-frame number is represented by 4 bits. For 3 replicas, this results in a 12-bit overhead for each replica to carry the sub-frame information. Consequently, the total overhead for all 3 replicas is 36 bits. This can be visualized as each replica having a 12-bit 'header' that, when combined, totals to a 36-bit overhead across all replicas

The root cause of the difficulty is that random number generation occurs locally within the transmitter and therefore the sub-frame numbers need to be included in the data units. However, this is not the only possible CRDSA approach, as the data units already contain user-specific random content, such as user identity, that is known by both the transmitter and receiver, and conveyed in the data units.

Therefore, to reduce signaling overhead, this contribution proposes an alternative approach to infer the sub-frame numbers from this mutual knowledge, enabling both parties to determine the sub-frame numbers of replicas without needing to append them in the data units nor generating them locally in the transmitter. This means that transmission can be initiated without differentiating between ordinary DSA and CRDSA, nor adding sub-frame numbers for replicas. The advantages are twofold: it avoids signaling overhead and renders the receiver's implementation transparent to the transmitter, ensuring consistent transmitter behavior regardless of the receiver's implementation.

Observation 3: It is possible to skip appending sub-frame numbers because messages already contain user-specific random content, such as user identity, that is known by both the transmitter and receiver, and/or conveyed in the data units.

An example of a user identity is the STemporary Mobile Subscriber Identity (S-TMSI) [5] which uniquely defines the subscriber unit in a tracking area and is included, e.g., in early data transmission message [6] as shown below.

RRCEarlyDataRequest message

-- ASN1START

RRCEarlyDataRequest-r15 ::=		SEQUENCE {

criticalExtensions				CHOICE {

rrcEarlyDataRequest-r15			RRCEarlyDataRequest-r15-IEs,

criticalExtensionsFuture		CHOICE {

rrcEarlyDataRequest-5GC-r16		RRCEarlyDataRequest-5GC-r16-IEs,

criticalExtensionsFuture-r16	SEQUENCE {}

}

}

}

RRCEarlyDataRequest-r15-IEs ::=	SEQUENCE {

s-TMSI-r15						S-TMSI,

establishmentCause-r15			ENUMERATED {mo-Data, delayTolerantAccess},

dedicatedInfoNAS-r15				DedicatedInfoNAS,

nonCriticalExtension				RRCEarlyDataRequest-v1590-IEs			OPTIONAL

}

The user identity leads to static sub-frame usage until the S-TMSI is periodically reallocated, e.g. during registration updates [7]. To introduce more variability, the system frame number (SFN) can be utilized to alternate sub-frame selections over time. The SFN is included in the master information block [6] as shown below.

MasterInformationBlock

-- ASN1START

MasterInformationBlock ::=			SEQUENCE {

dl-Bandwidth						ENUMERATED {

n6, n15, n25, n50, n75, n100},

phich-Config						PHICH-Config,

systemFrameNumber					BIT STRING (SIZE (8)),

schedulingInfoSIB1-BR-r13			INTEGER (0..31),

systemInfoUnchanged-BR-r15			BOOLEAN,

partEARFCN-r17					CHOICE {

spare							BIT STRING (SIZE (2)),

earfcn-LSB						BIT STRING (SIZE (2))

},

spare								BIT STRING (SIZE (1))

}

-- ASN1STOP

Another possible ingredient is to make use of other system parameters. One such system parameter is the physical cell identity (PCI) [6] as shown below. If neighboring and/or nearby cells are allocated different cell identities, the parameter creates randomization based on the transmitter’s physical location.

SystemInformationBlockType1 message

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

Additionally, the channel quality indicator (CQI) from NB-IoT early data transmission request message [6], which varies over time and is known to both transmitter and receiver because it is conveyed in the message, can be used for randomization. The CQI value, derived from downlink control channel measurements, reflects the radio environment and propagation conditions, thus providing a dynamic element.

RRCEarlyDataRequest-NB message

-- ASN1START

RRCEarlyDataRequest-NB-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcEarlyDataRequest-r15				RRCEarlyDataRequest-NB-r15-IEs,

later								CHOICE {

rrcEarlyDataRequest-r16				RRCEarlyDataRequest-5GC-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

}

RRCEarlyDataRequest-NB-r15-IEs ::=	SEQUENCE {

s-TMSI-r15							S-TMSI,

establishmentCause-r15				ENUMERATED {mo-Data, mo-ExceptionData, delayTolerantAccess, mt-Access-v1610},

cqi-NPDCCH-r15						CQI-NPDCCH-NB-r14						OPTIONAL,

dedicatedInfoNAS-r15				DedicatedInfoNAS,

nonCriticalExtension				RRCEarlyDataRequest-NB-v1590-IEs		OPTIONAL

}

This approach means that the transmitter calculates sub-frame numbers for replica transmissions based on the above-described message content and system parameters. The receiver decodes a message replica to determine sub-frame numbers for other replicas within the radio frame. This process involves arithmetic operations like multiplication, addition, and modulo operations, which combine message contents and system parameters to generate a pseudo-random number known to both the transmitter and receiver. Similar kinds of functions may be employed like those used in specifying, e.g., paging occasions and security parameter derivations.

In one approach, sub-frame number combinations for a set number of replicas in a radio frame are predefined in a table. For instance, with 3 replicas in a 12-subframe radio frame, there are 220 possible combinations, as 3 sub-frames can be selected from 12 in 220 unique ways. Each combination can be indexed from 0 to 219, as shown in Table 1 for some example combinations. A pseudo-random number serves as the index.

Table 1: Example of subframe combinations

| Index | Combination | Index | Combination | Index | Combination |
| --- | --- | --- | --- | --- | --- |
| 0 | (0,1,2) | 3 | (0,1,5) | 6 | (0,1,8) |
| 1 | (0,1,3) | 4 | (0,1,6) | 7 | (0,1,9) |
| 2 | (0,1,4) | 5 | (0,1,7) | 8 | (0,1,10) |
| … | … | … | … | … | … |
|  |  |  |  | 219 | (10,11,12) |

It is important to note that the combinations in Table 1 are only examples. Other combinations, such as (0,2,3) or (9,10,11), are feasible. The table's content can be permuted to maximize randomness and minimize correlation between adjacent entries. For example, the combination (0,1,2) could be placed at any index from 0 to 219, and the combination (0,1,3) need not be adjacent to (0,1,2).

In this example, the function could be defined, for example, in the following “ad hoc” manner where all input parameters are considered with their integer value:

Index = (S-TMSI + SFN + PCI) modulo 220.

It can be argued that the above proposed (or any other similar kind of implicit) randomization method may not completely match the robustness of a local random number generator within user equipment in all scenarios. Nonetheless, its standardization impact is negligible, as it only necessitates an equation, a procedure, and one table, all without the need for any modifications to signaling formats and without any overhead. Furthermore, the details of the receiver's implementation remain hidden from the user equipment, which is also worthwhile considering. Therefore, this contribution concludes with the following proposal and encourages discussion on the proposed approach.

Proposal 1: Take advantage of already existing system parameters and user-specific random content conveyed in the message to infer sub-frame numbers instead of appending sub-frame numbers.

# Conclusion

The observations and proposals of this contribution are summarized as follows:

Observation 1: A current requirement of contention resolution diversity slotted ALOHA is the inclusion of sub-frame information in each data unit, indicating the locations of the replicas within the radio frame.

Observation 2: The user equipment must know whether the network supports DSA or CRDSA unless the sub-frame numbers are always included.

Observation 3: It is possible to skip appending sub-frame numbers because messages already contain user-specific random content, such as user identity, that is known by both the transmitter and receiver, and/or conveyed in the data units.

Proposal 1: Take advantage of already existing system parameters and user-specific random content conveyed in the message to infer sub-frame numbers instead of appending sub-frame numbers.

# Reference

- 3GPP Tdoc RP-240776, "Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3", March 18-21, 2024.

- 3GPP Tdoc R2-2405202, "Discussion on EDT enhancements", May 20th - 24th, 2024.

- Raphael Rom, Moshe Sidi, "Multiple Access Protocols", Springer-Verlag New York Inc., 1990, ISBN 978-1-4612-3402-9.

- ETSI EN 301 545-2, "Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard"

- 3GPP TS 23.003 V18.5.0, "Numbering, addressing and identification".

- 3GPP TS 36.331 V18.1.0, "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

- 3GPP TS 24.501 V18.7.0, “Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”