3GPP TSG-RAN WG2 #127	R2-2407124

Maastricht, Netherlands, 19 – 23 Aug 2024

Agenda item:		8.6.3	Measurement event evaluation

Title:		Event evaluation of L1 measurement reporting

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#126, there was good progress on the event triggered L1 measurement reporting with the agreements below [1], where there are some FFS issues related to the event evaluation.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we discuss the event evaluation and provide our views.

# 2. Discussion

2.1	Event evaluation

In the last meeting, four events below were agreed as the L1 LTM measurement events:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

The events above were agreed but it is still unclear what beam(s) of the serving cell and LTM candidate cell is used for event evaluation. The simplest way is to compare the best beam of the serving cell and the best beam of the candidate cell. However, unlike the cell level quality in the legacy mobility, L1 beam quality may not be so stable and can be changed more dynamically. For example, a few beams may have similar quality in a cell and actual best beam among those beams may be changed during TTT, even after applying L1 filtering. This might be the case for the serving cell as well as the candidate cell. Thus, we expect that it would be good to consider more than one beam in a cell.

For the serving cell, the UE is normally using the best beam, which is selected by the UE via RA or indicated by the gNB via TCI state indication. On the other hand, the UE may not be using actual best beam at some point in time due to dynamic change of beam quality (as shown in the Fig.1 as example). For example, when the condition of the event (e.g. Event LTM2/3/5) is met for the first time, the UE is using the best beam of the serving cell. During the TTT, the best beam may be changed to another beam. A question is how to continue event evaluation for the serving cell in this case. One way is to keep using the same beam (i.e. previous best beam) regardless of whether it is best or not later. This is simple, while it may not be desirable if the beam quality of this original best beam suddenly goes worse and worse, which means the UE may underestimate the quality of the serving cell.

Other way is that the UE changes beams for event evaluation to keep using best beams. The other way implies that the event evaluation is continued without restarting the TTT even after changing the beams for evaluation. This TTT continuation (as in Option A in the Fig.1) was slightly discussed in the last meeting, but not yet be concluded. Yet another way is to use the “current” beam of the UE, which is normally controlled by the network. The UE keeps using the current beam regardless of whether actual beam is changed or not after the event condition is met.

Considering the situation where best beam could be changed during event evaluation, from the event evaluation perspective it would be better to use “current” beam instead of the best beam to ensure the UE to evaluate the event based on the beam as the representative beam of the serving cell.

Proposal 1: For serving cell, the UE uses “current” beam for LTM event evaluation.

Proposal 2: For serving cell, the UE continues TTT evaluation, even if the current beam is changed after the event condition is met for the first time.

Fig.1: Example of beam changes in serving cell

For candidate cell, there would be similar situation as the serving cell discussed above. Best beam of candidate cell could be changed even after applying L1 filtering. For some events (e.g. Event LTM3/4/5), the simplest way is to use individual beams of the candidate cell. In this case, the TTT evaluation is also performed individually.

For the Event LTM3, the UE compares the best or current beam of the serving cell with individual beams of the candidate cell. Given that there are multiple beams per candidate cell, it would be better to consider a certain UE complexity reduction in event evaluation. For example, only best M beams can be target of the event evaluation. If one or more beams among best M beams are replaced by other new beams, the event evaluation should be reset and restarted for those new beams.

For the Event LTM4/5, the UE compares the individual beams with absolute threshold. Similar to the Event LTM3/5, in order to reduce UE complexity, best M beams can be considered.

Proposal 3: For candidate cells, the UE uses individual beams for LTM event evaluation.

Proposal 4: For candidate cells, the UE uses only best M beams for LTM event evaluation, where M is configured by the network explicitly.

Based on the discussions and the proposals above, we consider the following rules can be assumed for LTM event evaluation.

Proposal 5: L1 LTM measurement events can be further refined as follows:

- Event LTM2: Current beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Any beam among best M beams of candidate cell becomes amount of offset better than current beam of serving cell;

- Event LTM4: Any beam among best M beams of candidate cell becomes better than absolute threshold;

- Event LTM5: Current beam of serving cell becomes worse than absolute threshold1 AND Any beam among best M beams of candidate cell becomes better than another absolute threshold2

2.2	Need of Event LTM1

In the last meeting, there was one more potential event proposed in [2], which is now called the Event LTM1:

- Event LTM1: Beam of serving cell becomes better than absolute threshold;

The need of this event is one of the remaining issues. At first, the two purposes of event triggered L1 measurement reports were agreed:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

Considering these purposes to introduce LTM event condition, the Event LTM1 does not seem to fit them (i.e. neither of two motivates the need). We do not see any use case for this event.

Proposal 6: Event LTM1 is not defined.

2.3	Need of cell level measurements

In addition to the beam level measurements, it is FFS whether cell level measurement is also considered for LTM event evaluation.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

The cell level measurements would be more stable or reliable than the beam level measurements. However, it is not very clear how or why the cell level measurement is used for the L1 measurement reporting purpose. Even for Rel-18 LTM, the network can determine whether (and which candidate cell) the LTM is executed based on the L1 measurement reporting. Although the network by implementation may also take into account a kind of cell level quality based on the reported beam level quality, it is not a part of LTM procedure from specification perspective. A possible approach may be to allow the network to request the UE to report cell level quality as well when the event triggered L1 measurement is reported. Otherwise, we do not see need of cell level measurements.

Proposal 7: RAN2 to not introduce cell level measurements for L1 LTM event evaluation and discuss need of cell level measurements in the event triggered L1 measurement report.

2.4	Handling of L1 measurement event evaluation

Currently L1 measurements for LTM is reported via UCI on either PUCCH or PUSCH. A question is whether the same way is applied for the event triggered L1 measurements or a different way is adopted. This aspect will be discussed in the other sub-agenda (8.6.4 Measurement reporting). Before discussing that issue, we discuss which sub layer handles the event.

In legacy L3 measurement reporting, the event is handled by RRC. For L1 measurement reporting, there may be some options, e.g. PHY, MAC or RRC. For PHY layer, although the measurement is still based on L1 measurement reporting, PHY layer is expected to do more dynamic and real-time processing. Thus, it would not be appropriate for PHY layer to perform event triggered reporting. For MAC layer, although there is no path to receive the L1 measurement results from PHY layer, there are already much information exchanged between PHY and MAC layers. It would be feasible for PHY layer to inform the L1 measurement results to MAC layer and the MAC layer handle the event based on those measurement results with a configured event. The processing delay can be much less than that in the case of RRC layer option.

Proposal 8: MAC sublayer handles the event evaluation for event triggered L1 measurement reporting.

# 3. Conclusion

In this contribution we discussed event evaluation for L1 measurement reporting and made the following proposals.

Proposal 1: For serving cell, the UE uses “current” beam for LTM event evaluation.

Proposal 2: For serving cell, the UE continues TTT evaluation, even if the current beam is changed after the event condition is met for the first time.

Proposal 3: For candidate cells, the UE uses individual beams for LTM event evaluation.

Proposal 4: For candidate cells, the UE uses only best M beams for LTM event evaluation, where M is configured by the network explicitly.

Proposal 5: L1 LTM measurement events can be further refined as follows:

- Event LTM2: Current beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Any beam among best M beams of candidate cell becomes amount of offset better than current beam of serving cell;

- Event LTM4: Any beam among best M beams of candidate cell becomes better than absolute threshold;

- Event LTM5: Current beam of serving cell becomes worse than absolute threshold1 AND Any beam among best M beams of candidate cell becomes better than another absolute threshold2

Proposal 6: Event LTM1 is not defined.

Proposal 7: RAN2 to not introduce cell level measurements for L1 LTM event evaluation and discuss need of cell level measurements in the event triggered L1 measurement report.

Proposal 8: MAC sublayer handles the event evaluation for event triggered L1 measurement reporting.

# References

[1] R2-2405701, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

[2] R2-2404677, Measurement enhancements for LTM, Apple