3GPP TSG-RAN WG2 #127	R2-2407125

Maastricht, Netherlands, 19 – 23 Aug 2024

Agenda item:		8.6.4	Measurement reporting

Title:		Details of event triggered L1 measurement report

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#126, there was good progress on the event triggered L1 measurement reporting with the agreements below [1], where there are some remaining issues including FFS on the need of measurement reporting once leaving condition is met.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we discuss further details of event triggered L1 measurement reporting and provide our views.

# 2. Discussion

2.1	Contents of event triggered L1 measurement report

In the last meeting, there were some progresses for L1 measurement reporting. We discuss further details of the event evaluation in our tdoc [2] of the other sub-agenda (8.6.3 Measurement event evaluation).

Regarding the content of L1 measurement reporting, the content should include at least L1 measurements of serving cell and that of candidate cell for which the event condition is met. The former should include the beam level measurements of current beam of the serving cell. The latter should include the indication of the corresponding candidate cell and the beam level measurements of beam(s) of the candidate cell, which was used for event evaluation.

Proposal 1: RAN2 to agree the event triggered L1 measurement report includes following information:

- Beam level measurement of current beam of the serving cell

- Candidate cell information for which the event condition is met

- Beam level measurement of beam(s) of the candidate cell, which was used for event evaluation

In [2], we discuss to introduce a concept of best M beams of candidate cell for event evaluation. The network configures how many beams (from the best) should be considered for event evaluation. If this (i.e. “M”) is configured, the UE includes the beam level measurements of those “M” best beams.

Proposal 2: RAN2 to assume that if the best M beams of candidate cell is considered for event evaluation, the beam level measurements of M beams are also included in the event triggered L1 measurement report.

Another aspect is about the cell level measurements. The need of cell level measurement was discussed as a part of L1 measurement event evaluation. We do not see strong need of cell level measurements for event evaluation as discussed in [2]. On the other hand, we see potential usage of cell level measurements.

For Rel-18 LTM, the network can determine whether (and which candidate cell) the LTM is executed based on the L1 measurement reporting. Indeed, the cell level quality would be more stable or reliable than the beam level quality. Thus, the network by implementation may also take into account a kind of cell level quality based on the reported beam level quality in LTM, although it is not a part of LTM procedure from specification perspective. A possible approach may be to allow the network to request the UE to report cell level quality as well when the event triggered L1 measurement is reported.

Proposal 3: RAN2 to discuss need of cell level measurements in the event triggered L1 measurement report, instead of using the cell level measurements for event evaluation.

2.2	Need of measurement reporting once leaving condition is met

Another remaining issue is the need of measurement reporting when the leaving condition is met for LTM events. One of the agreed purposes of event triggered L1 measurement report is to select the candidate beam/cell to trigger early synchronization. For example, the network can set relatively lower or smaller value in some events so that the L1 measurement report is triggered earlier. In order for the network to know the L1 measurement status, the measurement reporting when the leaving condition is met would be also useful. This is similar to the legacy L3 measurement report by setting the reportOnLeave.

Proposal 4: RAN2 to support measurement reporting once leaving condition is met for event triggered L1 measurement reporting.

2.3	L1 measurement report on UCI or MAC CE

Currently L1 measurements for LTM is reported via UCI on either PUCCH or PUSCH. A question is whether the same way is applied for the event triggered L1 measurements or a different way is adopted.

Before discussing this issue, we discuss which sub layer handles the event in our tdoc [2] of the other sub-agenda (8.6.3 Measurement event evaluation). In short, we assume that the MAC sublayer handles the event triggered L1 measurement reporting, because the processing delay can be much less than that in the case of RRC layer (as in legacy mobility) and PHY layer is not appropriate to handle this kind of evaluation.

Given that the MAC layer handles the event, it seems reasonable to assume introducing a new MAC CE to report L1 measurement reporting. One concern may be a certain delay for sending the new MAC CE especially when no UL resource is available. However, this expected delay is related to the very first report only. If the event triggered periodic reporting is assumed, it would not be critical issue.

Proposal 5: RAN2 to agree a new MAC CE is used to carry the event triggered L1 measurement report.

Note that for the P5, we co-sign the joint contribution from multiple companies in [3].

# 3. Conclusion

In this contribution we discussed some details of L1 event triggered measurement reporting and made the following proposals.

Proposal 1: RAN2 to agree the event triggered L1 measurement report includes following information:

- Beam level measurement of current beam of the serving cell

- Candidate cell information for which the event condition is met

- Beam level measurement of beam(s) of the candidate cell, which was used for event evaluation

Proposal 2: RAN2 to assume that if the best M beams of candidate cell is considered for event evaluation, the beam level measurements of M beams are also included in the event triggered L1 measurement report.

Proposal 3: RAN2 to discuss need of cell level measurements in the event triggered L1 measurement report, instead of using the cell level measurements for event evaluation.

Proposal 4: RAN2 to support measurement reporting once leaving condition is met for event triggered L1 measurement reporting.

Proposal 5: RAN2 to agree a new MAC CE is used to carry the event triggered L1 measurement report.

Note that for the P5, we co-sign the joint contribution from multiple companies in [3].

# References

[1] R2-2405701, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

[2] R2-2407124, Event evaluation of L1 measurement reporting, NEC

[3] R2-2406921, Discussion on which layer reports the event triggered L1 measurements, Ericsson, Honor, Huawei, InterDigital Inc., MediaTek Inc., NEC, Nokia, NTT DOCOMO, Oppo, T-Mobile USA, Vivo, ZTE corporation