3GPP TSG-RAN WG2 Meeting #R2-127		R2-2407128 Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	7.3.2

Source: 	Rakuten

Title:  	Coexistence of Cell DTXDRX and RACH-less LTM

Document for:	Discussion and decision

# Introduction

Agreements - R2#125bis

4	FFS if we will support the coexistence of the cell DTX/DRX and RACH-less LTM and whether there is any spec impact.

Agreements – R2#126

Understanding is that an LTM target is not a serving cell and hence it will not apply CELL DTX/DRX until the switch is completed.   Nothing is optimized or specified for RACHless.

The interworking between LTM and Cell DTX/DRX was mentioned as an FFS in RAN2 #125bis and it was partially addressed in R2#126, as shown above. We address the remaining aspect in this contribution.

# Discussion

The impact of Cell DTX/DRX on LTM could occur either at serving cell or at target cell. In R2#126, it was agreed that no changes are specified for CELL DTX/DRX from an LTM target cell perspective. In this paper, we analyse the impact of CELL DTX/DRX on LTM in a serving cell and provide our views.

## Cell DTX/DRX at serving cell

For a UE configured with LTM, if Cell DTX/DRX is enabled at the serving cell, the interworking between Cell DTX/DRX and LTM has to consider preparation of an LTM candidate cell and execution of LTM cell switch when Cell DTX/DRX is configured in the serving cell.

Assuming that the cell is under DTX/DRX at the same time when a UE is configured to report periodic L1 measurements for LTM and also execution of LTM cell switch, the following are the key steps involved in the interworking.

Key step 1: UE reporting L3 measurements (UL) of discovered cells to the serving gNB-CU.

Key step 2: UE reporting L1 measurements (UL) of the configured LTM candidate/target cells to the serving gNB-DU.

Key step 3 : Serving gNB-DU sending LTM cell switch command in a MAC CE (DL) to the UE.

Issues in the key-steps

- Key step 1 : UE cannot send L3 measurement report  to the serving gNB-CU when cell DRX is enabled. But gNB can configure the UE to align the sending of L3 measurement reports during active duration of cell DRX cycle.

- Key step 2 : UE cannot send L1 measurement report  to the serving gNB-DU when cell DRX is enabled. But gNB can configure the UE to align the sending of L1 measurement reports during active duration of cell DRX cycle.

- Key step 3 : gNB-DU can’t send LTM cell switch command to the UE since cell DTX is enabled in the cell. Delaying LTM cell switch command until completion of the Cell DTX cycle may cause unacceptable delays followed by RLF. Network cannot wait to send the cell switch command, especially for a UE configured with RACH-less LTM as it is expected to be a fast handover procedure compared to baseline L3 HO.

When Cell DTX/DRX is configured in the serving cell, a serving gNB-DU cannot send the LTM cell switch command.

Here, we think that the serving gNB-DU will be able to anticipate the LTM cell switch based on the L1 measurement reports from the UE. Based on this, the gNB-DU may configure the UE with an extended active duration or permit PDCCH monitoring irrespective of Cell DTX.

When a serving gNB-DU expects a UE to undergo LTM cell switch, the UE is configured with an extended active duration or permitted to monitor PDCCH even during Cell DTX.

RAN2 agrees that a serving gNB-DU can perform LTM cell switch even during CELL DTX/DRX and specifies the behavior.

# Conclusion

We have the following observations:

Observation 1	When Cell DTX/DRX is configured in the serving cell, a serving gNB-DU cannot send the LTM cell switch command.

We have the following proposals:

Proposal 1	When a serving gNB-DU expects a UE to undergo LTM cell switch, the UE is configured with an extended active duration or permitted to monitor PDCCH even during Cell DTX.

Proposal 2	RAN2 agrees the a serving gNB-DU can perform LTM cell switch even during CELL DTX/DRX and specifies the behavior.

# Reference

RP-223540, “Network energy savings for NR” – Rel 18.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

RAN2 Chairman Notes R2_125bis_ChairNotes

RAN2 Chairman Notes R2_126_ChairNotes