3GPP TSG-RAN WG2 Meeting #127	R2-2407129

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Idle mode considerations for downlink coverage enhancements

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Idle mode considerations for downlink coverage enhancements

# 1	Introduction

The WID for NTN phase 3 has been updated as below to specifically mention the SSB periodicity enhancements of NTN as a potential objective.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

As it is RAN1s task to report on the potential backward compatibility issues, and RAN2 already sent an LS concerning this to RAN1 this paper handles potential impacts to UE behaviour under the assumption that agreements to such enhancements are only agreed if the reported, potential, backward compatibility impacts have been accepted by RAN Plenary.

# 2	Discussion

Since RAN1 is now discussing the SSB periodicity extension, it is only natural to have RAN2 follow this process in parallel. Furthermore, given the LS to RAN1 sent from RAN2, information on multiple topics have been queried and the most reasonable progress in RAN2#127 would be to only handle SSB periodicity enhancements.

Proposal 1: Until RAN1 reply has been received, only SSB periodicity extension should be studied in RAN2.

The SSB periodicity enhancement, enables cells to be at least in a cell DTX state. Our understanding is that the SSB periodicity increase is for covers RRC_IDLE/RRC_INACTIVE by transmitting the SSBs/SIBs more sparsely, and firstly, RAN2 should agree on the worst-case scenario with respect to periodicity. In RAN1 it is defined that the number of active beams can be just 16, and the number of total beam footprints are 1058, requiring approximately 66 beam hopping steps to cover each footprint at least once. Since a step would be assumed to be at least a single half frame, and the SSB/SIB periodicity must be indicated in the power of 2, a SSB periodicity of 320 ms would be sufficient to achieve a coverage ratio of 96.8%. However, in this case, all resources are used to transmit within a single half frame, and there will not be much freedom for any active communication with any UE. Thus, we would propose to have RAN2 studying extension to 640 ms, allowing 100% coverage ratio, and significant freedom to transmission of other data.

Observation 1: SSB periodicity of 320 will only provide a 96.8% coverage ratio, with little to no DL resources for UE communication.

Observation 2: SSB periodicity of 640 will provide a 100% coverage ratio, with DL resources available for UE communication.

Proposal 2: RAN2 to study a SSB periodicity of up to 640 ms.

Considering extended SSB periodicities not only impacts reception of serving cell’s SSB but there are some further considerations related to neighbor cells measurements. The serving cell provides SMTC windows and measurement gaps (MG) to UEs based on the neighboring cell is transmitting the SSB with a certain periodicity. Even if this periodicity is statically configures, it may be challenging for a cell to configure the measurement gaps for the UEs within the cells. Furthermore, if the SSB frequency is dynamic, i.e. changes occasionally, reconfiguring the SMTC windows and measurement gaps may lead to extra signaling overhead.

Observation 3: Adjusting the SMTC window and measurement gaps e.g. in case of changing SSB frequency will increase the signalling overhead.

One way to avoid this overhead can be to instead provide the UE with information about the neighboring cell SSB configuration, such the UE can adjust its behavior, like skipping a measurement gap when it knows the neighboring cells have gone to a lower SSB periodicity. Likewise, the cell could provide information on for how long the neighbor cells are in a certain state (common signal only, or active with UL/DL traffic), allowing a UE to make reasonable decisions about when to read the SSBs considering the UEs own state.

Proposal 3: Serving cell provides neighbor cell activity such as SSB pattern information. FFS on state information i.e. whether to include information on when cells and/or beams are on or off. FFS on how to signal i.e. on/off pattern and/or duration, or delta signaling for patterns in case of dynamic configuration.

Proposal 4: Irrespectively of the selected RAN2 solution, RAN2 should strive for a low signaling overhead solution.

Proposal 5: Due to the nature of the configuration complexity, RAN2 include in the study to allow UE implementation to read SSB occasions.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: Until RAN1 reply has been received, only SSB periodicity extension should be studied in RAN2.

Observation 1: SSB periodicity of 320 will only provide a 96.8% coverage ratio, with little to no DL resources for UE communication.

Observation 2: SSB periodicity of 640 will provide a 100% coverage ratio, with DL resources available for UE communication.

Proposal 2: RAN2 to study a SSB periodicity of up to 640 ms.

Observation 3: Adjusting the SMTC window and measurement gaps e.g. in case of changing SSB frequency will increase the signalling overhead.

Proposal 3: Serving cell provides neighbor cell activity such as SSB pattern information. FFS on state information i.e. whether to include information on when cells and/or beams are on or off. FFS on how to signal i.e. on/off pattern and/or duration, or delta signaling for patterns in case of dynamic configuration.

Proposal 4: Irrespectively of the selected RAN2 solution, RAN2 should strive for a low signaling overhead solution.

Proposal 5: Due to the nature of the configuration complexity, RAN2 include in the study to allow UE implementation to read SSB occasions.