3GPP TSG-RAN WG2 Meeting #127	R2-2407131

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	7.15.2

Source:	Nokia

Title:	Carrier selection for some MAC CEs

WID/SID:	NR_SL_enh2 - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#126 a postponed issue was whether to specify carrier selection for IUC and DRX MAC CEs.

R2-2405698 Discussion on carrier selection for SL MAC CE(s) LG Electronics Inc. discussion NR_SL_enh2 Proposal 1-1. UE selects a carrier on which the SL IUC Request was received as a carrier for transmission of a SL IUC Information MAC CE. Proposal 1-2. Carrier selection of SL IUC Request MAC CE, Condition based SL IUC Information MAC CE and SL DRX command MAC CE uses the same procedure as the carrier selection procedure of logical channel data. - P1-1 is agreed. - Revisit P1-2 next meeting.

This paper discusses the topic

# 2	Discussion

From our understanding, the postponed proposal P1-2 suggests appending to the Tx carrier (re-)selection procedure a check whether any Sidelink IUC MAC CEs are available on a sidelink logical channel as to consider this carrier as a candidate carrier for Tx carrier (re-)selection.

The main reasoning behind this suggestion seems to be, that since the MAC entity shall now select the carrier on which the SL IUC Request was received, this must be within the candidate set of carriers. However, it seems reasonable to assume that any carrier where data is available would be within the preferred, or outside the non-preferred resource set and thus relying on legacy procedures to handle transmission of the IUC MAC CEs in such case should be sufficient.

Furthermore, in order to determine whether such change to the Tx carrier (re-)selection procedure, the following should be taken into account:

- The sidelink IUC procedure can be seen as non-critical information, as it only assists UE resource selection and can follow legacy procedure.

- For the Sidelink IUC Request, the carrier on which it is transmitted may not be the preferred one for the UE providing the Sidelink IUC Information, and thus the carrier selected will not impact the IUC operation as such.

- For the SL IUC information transfer triggered by a condition, it can be assumed that the carriers selected based on the sidelink logical channels are somehow within the IUC preferred, or outside the IUC non-preferred set, and thus the Tx carrier (re-)selection procedure for these should be sufficient to provide a correct selection.

- Nothing differentiates Sidelink IUC MAC CEs from other MAC CEs, other than CSI MAC CE. Hence if Tx carrier (re-) selection procedure for Sidelink IUC MAC CE is an issue, it should be brought up for all MAC CEs, which will be a TEI.

Observation 1: Sidelink IUC procedure is not a delay sensitive operation and the carriers selected by the UE transmitting the Sidelink IUC information can be assumed to be within the preferred or outside the non-preferred resource set.

Proposal 1: Enhancements to Tx carrier (re-)selection for SL IUC Request MAC CE, Condition based SL IUC Information MAC CE should not be considered.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Sidelink IUC procedure is not a delay sensitive operation and the carriers selected by the UE transmitting the Sidelink IUC information can be assumed to be within the preferred or outside the non-preferred resource set.

Enhancements to Tx carrier (re-)selection for SL IUC Request MAC CE, Condition based SL IUC Information MAC CE should not be considered.