3GPP TSG-RAN WG2 Meeting #R2-127		R2-2407133 Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.6.2

Source: 	Rakuten

Title:  	Fast LTM recovery in DC scenarios

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

The WID objective includes configuring LTM for UEs in Dual Connectivity.

In this contribution, we describe LTM failure in a DC setup and discuss a reliable and fast mechanism to recover from LTM failure.

# Discussion

## LTM Failure in Dual Connectivity

The dual connectivity can be configured to a UE in two variants as shown below.

Figure 1: NR-DC over Xn (Variant 1)

Figure 2: NR-DC over F1(Variant 2)

NR-DC can be configured over Xn where MN and SN roles are assigned to two different gNBs and the Pcell configured in the MN gNB belongs to Master Cell Group (MCG) and the PSCell configured in the SN belongs to the Secondary Cell Group (SCG). NR-DC can also be configured over F1 where MN and SN roles are assigned to the same gNB. The Pcell belongs to one gNB-DU and that belongs to Master Cell Group (MCG) and the PSCell may belong to the same or a different gNB-DU and belongs to the Secondary Cell Group (SCG).

The MCG is configured with SRB 0/1/2 and SCG is configured with SRB3.

In case of LTM cell switch failure, following is the behaviour (from TS 38.300)

-	When initial LTM execution attempt fails or HO fails, the UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts LTM execution once, otherwise re-establishment is performed.

This implies the following in our understanding

- Whenever there is an LTM cell switch failure, the UE performs a cell selection based on the prevalent radio conditions. If the selected cell happens to be an LTM candidate cell, the UE performs another LTM execution, post failure (only if the network has configured).

- This is attempted only once and if failure occurs again, rrc re-establishment is initiated.

- RRC re-establishment will mean both MCG and SCG bearers will have to be released and set-up again.

The LTM failure recovery is autonomous in nature and doesn’t require an LTM switch command (DL MAC CE) from the network.

In the context of NR-DC the LTM failure recovery is possible to be addressed differently than what is specified in R18. In a DC setup, unless there is a simultaneous MCG and SCG failure, MCG SRB is available whenever there is an SCG LTM failure and SCG SRB is available whenever there is an MCG LTM failure.

Barring a simultaneous MCG and SCG failure, MCG SRB is available whenever there is an SCG LTM failure and SCG SRB is available (if SRB3 is configured) whenever there is an MCG LTM failure.

Since the whole objective of an LTM failure recovery is to avoid RRC re-establishment, the active and available radio link can be used to recover the failed link i.e after the LTM recovery attempt as specified in R18, instead of triggering RRC re-establishment, the active SRB (MCG SRB in case of SCG LTM failure and SCG SRB in case of MCG LTM failure) could be configured to be used by the UE to send the latest measurement reports to the gNB-CU. This could be forwarded to the MN/SN which underwent an LTM failure. The MN/SN that underwent failure could prepare a new candidate cell based on the latest measurement reports and send the new LTM candidate cell configuration to the UE using the active radio link. The LTM cell switch command could also be sent together with the new LTM candidate cell configuration as well.

Post LTM failure and recovery, the latest measurement results are sent from UE to the gNB-CU which are forwarded to the MN/SN that underwent failure.

Latest measurement results are used to prepare a fresh LTM candidate/target cell.

The newly prepared LTM candidate/target cell configuration and the LTM cell switch command for the new candidate/target cell are delivered to the UE using an RRC message over the active radio link.

The UE performs an LTM cell switch to the newly prepared candidate/target cell and applies the candidate/target cell configuration delivered using the RRC message. Thus RRC re-establishment is avoided and the UE recovers from LTM failure.

# Conclusion

We have the following observations:

Observation 1	Barring a simultaneous MCG and SCG failure, MCG SRB is available whenever there is an SCG LTM failure and SCG SRB is available (if SRB3 is configured) whenever there is an MCG LTM failure.

We have the following proposals:

Proposal 1	Post LTM failure and recovery, the latest measurement results are sent from UE to the gNB-CU which are forwarded to the MN/SN that underwent failure.

Proposal 2	Latest measurement results are used to prepare a fresh LTM candidate/target cell.

Proposal 3	The newly prepared LTM candidate/target cell configuration and the LTM cell switch command for the new candidate/target cell are delivered to the UE using an RRC message over the active radio link.

# References

RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024

RAN2 Chairman Notes R2_125bis_ChairNotes

3GPP TS 38.300. NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)