3GPP TSG-RAN WG2 Meeting #127	     R2-2407135

Maastricht, Holland, Aug 19th – 23rd, 2024

Agenda Item	:	8.7.2

Title	:	Multi-modality support for XR

Source	:	Google

Document for	:	Discussion and Decision

# Introduction

In RAN#102 [1], the following Rel-19 objective has been agreed to study the multi-modality for XR:

- Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g., synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2].

  - Note: Check in RAN#105 (check also other WG involvement if needed).

In this Tdoc, we share our views and our inputs on how better synchronization can be studied and specified for XR application with Multiple QoS flows.

# Enhancement for multi-modal XR traffic

In RAN2#126, the following agreements have been made regarding the multi-modality awareness at RAN:

Support Multi-Modality awareness in RAN in Rel-19 for UL and DL.

Ask SA2 whether/what information could be provided to RAN

Clarify in the LS how RAN2 considers to use this information, e.g. coordinated handling of multi-modal flows.

Can consider while drafting the LS to indicate the potential benefits

In RAN2#125bis, the following agreement has been made regarding the assumption on the UE and the gNB sides to have multi-modal information.

For the purpose of study, RAN2 assumes that UE and gNB have some kind of multi-modal information. FFS what information is needed/useful, e.g. just multi-modal ID, association between the flow, synchronization requirement etc.

Enabling Multi-Modality awareness in both the UE and RAN is essential to provide a comprehensive view of multi-modal XR traffic flows. This allows for coordinated handling of XR data streams, enabling the network to optimize resource allocation, scheduling, and synchronization for a seamless user experience, joint admission control ensuring all flows associated with a multi-modal service are either accepted or rejected together, preventing partial service failures.

For UL, UE can provide Multi-Modality awareness information about UL QoS flows association to the network through Uplink Assistance Information. UE can have multi-modality awareness by implementation. For DL, RAN can receive Multi-Modality awareness information (Multi-Modality Service ID, flow dependencies, synchronization thresholds) from the Core Network.

Proposal 1: For UL, the UE could have multi-modality awareness about the association between the UL QoS flows by implementation. The UE could provide Multi-Modality awareness information about UL QoS flows association to the network through Uplink Assistance Information

Proposal 2: For DL, RAN can have multi-modality awareness about the association between the DL QoS flows from Core Network.

Regarding the synchronization and QoS requirements, RAN needs to be aware of synchronization requirements between different modalities. This would allow the RAN to prioritize and schedule transmissions to meet these requirements. The indication can be explicit and can come directly from Core Network. Alternatively, if the synchronization and QoS requirements are not explicitly signalled, then the RAN and UE need to study and design techniques to meet the existing SA2 requirements.

Proposal 3: Awareness of synchronization requirements would enable the RAN to make informed scheduling decisions, improving the quality and reliability of multi-modal XR applications.

RAN can use the dependency information between the mutli-modal flows (Multi-modal identifier indication, flows dependency and type of dependency, synchronization thresholds, synchronization error/delay, …) to further optimize the scheduling and for better discard and prioritization. For example, if QoS Flow 1 and QoS Flow 2 belong to the same multi-modal service, the network and the UE can discard some specific packets from QoS Flow 1 if some associated packets from QoS Flow 2 are discarded or lost.

Proposal 4: Study the optimization of PDU-Set discard and prioritization across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows and the unmet synchronization requirements.

The discard of packets if synchronization requirements are not met can be signalled to RAN/UE and enabled/disabled by core network.

Proposal 5: RAN/UE to be indicated to discard packets if synchronization requirements are not met.

As assistance information, the UE can indicate to the network the flows belonging to the same multi-modal XR service. For example, a multi-modal XR service ID can be signalled by the UE to the network for the UL multi-modal flows to indicate which traffic flows are correlated/dependent.

Proposal 6: UE could provide UL assistance information to the network including:

- Multi-Modality awareness information about UL QoS flows association

- Granular dependency information (e.g., at the PDU Set level)

- synchronization requirements/thresholds

Rel-18 introduced the concept of delay status reporting (DSR) which is used to provide the gNB with delay status of an LCG. This delay status for an LCG includes remaining time which is the smallest remaining value of the running PDCP discard timers among SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU.

In the scenario where each QoS flow of the multi-modal traffic is mapped to a different LCG, the network can derive an estimation of the synchronization delay using the reported DSRs.

The UE however should signal additional information to the network for the network to identify the mapping of different QoS flows to the different LCGs and to know the associated PDUs/PDU-Sets sequence numbers. i.e., the network should know the delays but also the data associated with these delays.

To optimize handling of multi-modal XR traffic, the gNB must distinguish between DSRs intended for different flows, for example video and those for haptic data. Further, the gNB must recognize that specific video and haptic DSRs are correlated with corresponding flows within the same XR service. Crucially, the gNB requires awareness of the association between individual video frames and corresponding groups of haptic packets for fine-grained synchronization.

Proposal 7: Study the enhancement of Rel-18 DSR reporting for synchronization purposes.

Proposal 8: the gNB/UE could recognize that specific DSRs associated with multi-modal flows are correlated allowing it to schedule their transmission in a way that minimizes synchronization errors

A new concept of Multi-Modal Data Set (MMDS) can be introduced. An MMDS is composed of one or more PDUs and/or PDU-Set(s) carrying information payload from multiple multi-modal QoS flows that are associated with the same XR service. MMDS can be considered as the unit of data used to calculate the synchronization error/delay and it contains data from across multiple QoS flows of a multi-modal service. For example, MMDS can be composed of multiple PDU-Sets associated to video, multiple PDUs associated with audio and haptic data flows.

For example, it can be composed of a video frame and the associated audio, haptic packets required by the user at the same time instant.

Proposal 9: Study the concept of Multi-modal data set which can be defined as a unit of information across multiple multi-modal QoS flows that are associated with the same XR service and can be used to calculate the synchronization error.

MMDS can also be used for better scheduling, prioritization and discard, e.g., if one or multiple packets of the MMDS are dropped or lost the entire MMDS is discarded.

The network should ensure that all PDUs or/and PDU-Sets belonging to the same MMDS are delivered with similar delay to reduce the synchronization error.

The synchronization error can be defined using the MMDS concept. It can be defined as the maximum arrival temporal delay between packets belonging to the same MMDS where the delay should be between packets belonging to the same MMDS but belonging to different modality flows.

To improve the synchronization, if synchronization delay is above a synchronisation defined threshold, the priority of late PDU(s) or PDU-Set(s) could be increased.

A discarding threshold can also be specified, and if synchronization delay exceeds some synchronization budget, then the entire MMDS can be dropped.

Proposal 10: If synchronization delay is above a synchronisation threshold, the priority of late PDU(s) or PDU-Set(s) is increased

The granularity of the synchronization can be at the QoS flow level or at the packet level. Using the concept of Multi-modal data set, the synchronization at the packet level is needed and can be done via dynamic marking in the GTP-U header.

Proposal 11: RAN2 to study packet level synchronization via dynamic marking in the GTP-U header.

Regarding enhancements to C-DRX, the multi-modal XR applications demand flexible power-saving mechanisms that can adapt to diverse traffic patterns. Current C-DRX limitations hinder optimal power efficiency and latency performance. Multiple active DRX configurations, tailored to individual flows, offer a promising solution. This approach allows the UE to closely follow the irregular sleep opportunities created by multi-modal traffic, leading to significant power saving gains without sacrificing latency. By aligning DRX configurations with the specific characteristics of each flow, we can achieve a balance between power efficiency and responsiveness, enhancing the overall user experience for immersive XR applications.

Proposal 12: RAN2 should investigate the feasibility of supporting multiple active DRX configurations per serving cell for multi-modal XR flows, including mechanisms for configuration, signalling, and interaction with other power-saving features.

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: For UL, the UE could have multi-modality awareness about the association between the UL QoS flows by implementation. The UE could provide Multi-Modality awareness information about UL QoS flows association to the network through Uplink Assistance Information

Proposal 2: For DL, RAN can have multi-modality awareness about the association between the DL QoS flows from Core Network.

Proposal 3: Awareness of synchronization requirements would enable the RAN to make informed scheduling decisions, improving the quality and reliability of multi-modal XR applications.

Proposal 4: Study the optimization of PDU-Set discard and prioritization across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows and the unmet synchronization requirements.

Proposal 5: RAN/UE to be indicated to discard packets if synchronization requirements are not met.

Proposal 6: UE could provide UL assistance information to the network including:

- Multi-Modality awareness information about UL QoS flows association

- Granular dependency information (e.g., at the PDU Set level)

- synchronization requirements/thresholds

Proposal 7: Study the enhancement of Rel-18 DSR reporting for synchronization purposes.

Proposal 8: the gNB/UE could recognize that specific DSRs associated with multi-modal flows are correlated allowing it to schedule their transmission in a way that minimizes synchronization errors

Proposal 9: Study the concept of Multi-modal data set which can be defined as a unit of information across multiple multi-modal QoS flows that are associated with the same XR service and can be used to calculate the synchronization error.

Proposal 10: If synchronization delay is above a synchronisation threshold, the priority of late PDU(s) or PDU-Set(s) is increased

Proposal 11: RAN2 to study packet level synchronization via dynamic marking in the GTP-U header.

Proposal 12: RAN2 should investigate the feasibility of supporting multiple active DRX configurations per serving cell for multi-modal XR flows, including mechanisms for configuration, signalling, and interaction with other power-saving features.

# 4		References

- RP-234014 ‘New WID: XR (eXtended Reality) for NR Phase 3, MediaTek (Moderator), Edinburgh, Scotland, December 11-15, 2023, 3GPP TSG RAN Meeting #102

- TS 23.501 ‘System architecture for the 5G System (5GS)’, v.18.3.0.

- TR 23.700-60 ‘Study on XR (Extended Reality) and media services’ v.18.0.0.

- TR 26.926 ‘Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems’ v.18.0.0.

- TR 22.847 ‘Study on supporting tactile and multi-modality communication services’ v.18.0.0.

- TS 22.261 ‘Service requirements for the 5G system’ v19.5.0

- RP-232340, ‘XR enhancements in Rel-19’, Huawei, Bangalore, September 11-15, 2023, RAN Meeting #101.