3GPP TSG-RAN WG2 #127	R2-2407136

Maastricht, Netherlands, 19 – 23 August 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on A-IoT paging message

Document for:	Discussion and decision

Agenda Item:	8.2.3

- Introduction

In the last meeting, RAN2 reached following agreements on contents of A-IoT paging message [1].

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

This contribution is to discuss remaining aspects in terms of A-IoT paging message.

- Discussion

  - “Command only” use case

RAN2 agreed to study “Command only” use case where the command is included in the A-IoT paging message [1].

Agreements (omit) 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

The command message is generated in the CN and delivered in upper layer. From AS perspective, the command message is regarded as upper layer data.

We would like to note that we are asking SA2 and SA3 whether to introduce security protection in upper layer in the LS [2]. RAN2 is still waiting for the reply but we think we could assume the presence of upper layer security considering that the upper layer messages can contain data important for secure operation, e.g., command to control Ambient IoT devices or Device ID to track devices.

As legacy, the security protection is an end-to-end protection. In other words, two terminated nodes share a security key which is hidden to any other nodes to enable encrypt/integrity protection and decrypt/integrity checking only in the terminated nodes. If the security protection in upper layer is supported in the same way, as shown in Figure 1, the command message sent in A-IoT paging message can be decoded by only one device.

Figure 1. Upper layer security and A-IoT paging message with command

Therefore, we propose to assume that the “Command only” use case is only when one device is targeted by A-IoT aging message. We are still waiting for SA2/SA3 reply, thus it is open to revisit taking their answers into account after receiving replies.

Observation 1.	Given that security protection is supported in upper layer, upper layer data for a command message provided in an A-IoT paging message cannot be decoded by more than one Ambient IoT devices since the upper layer security is E2E protection.

Proposal 1.	RAN2 assume that only one device can be targeted in A-IoT paging message in “Command only” use case. RAN2 can revisit based on SA2/SA3 conclusion.

  - Multi-reader paging

Given the scenario where multiple Ambient IoT Readers are physically located close to one another, e.g., indoor deployment scenario, it is possible that one Ambient IoT Device is in a coverage area to receive A-IoT paging messages from multiple Ambient IoT Readers.

In the case above, if the CN indicates multiple readers to send an A-IoT paging message targeting the device or targeting any device, the device will receive multiple A-IoT paging messages from multiple readers. Then, as depicted in Figure 2, the device will send A-IoT Msg1 for the same time as received A-IoT paging messages. In other words, the number of A-IoT Msg1 from one device is multiplied by the number of nearby readers.

Figure 2. Device sends A-IoT Msg1 for the number of readers nearby.

On top of that there should be other devices that send A-IoT Msg1 for multiple times for the same reason. Therefore, the possibility to occur contention increases when readers are densely deployed.

Observation 2.	Ambient IoT device will send A-IoT Msg1 for times of the number of nearby Ambient IoT readers, which increases possibility of contention.

This issue can be resolved by introducing Paging ID, which is identical for one attempt of paging from the CN, in the A-IoT paging message. The CN indicates the same Paging ID to readers for one attempt. Then, as depicted in Figure 3, the readers transmit A-IoT paging messages attached with the same Paging ID. The Device can discard A-IoT paging messages with the Paging ID already received to avoid unnecessary transmission of A-IoT Msg1.

Figure 3. Paging ID enables to avoid unnecessary transmission of A-IoT Msg1.

Observation 3.	Duplicated transmission of A-IoT Msg1 can be avoided via discarding A-IoT paging messages attached with the same Paging ID.

Proposal 2.	Introduce Paging ID to be attached to the A-IoT paging message. Ambient IoT devices have to respond only once to A-IoT paging messages with the same Paging ID.

If we introduce Paging ID described above, the Device will send preamble message only once. However, given the scenario where the readers are densely deployed, A-IoT Msg1 sent by a device can be received by multiple readers. Then, if no contention occurs, the device successfully completes A-IoT random access procedure to multiple readers, and the device starts transmission of upper layer data to the CN via multiple readers.

We would like to note that the upper layer data is exchanged between the Ambient IoT device and the CN, and the Ambient IoT reader has nothing to do with the NAS message. Therefore, the duplicated paths for upper layer data are not necessary but result in waste of radio resources for readers.

Observation 4.	Reception of A-IoT Msg1 by multiple readers can result in duplicated upper layer data paths which are not necessary but waste of radio resources for Readers.

This issue can be resolved by introducing Reader ID for each Ambient IoT reader. As depicted in Figure 4, the CN configures Reader ID (permanent or temporary), and the Reader attaches its Reader ID in every A-IoT paging message. The Device can send preamble message including Reader ID to report which Reader it is responding to. If the reader configured of different Reader ID receives the A-IoT Msg1, one simple way is, it can just discard the A-IoT Msg1.

Figure 4. Reader ID enables to avoid establishing unnecessary path of upper layer data.

Observation 5.	Ambient IoT reader can save radio resources to allocate to devices responding to other readers if Reader ID is introduced.

Proposal 3.	Introduce permanent or temporary Reader ID to be attached to A-IoT paging message. Ambient IoT devices can include a Reader ID in A-IoT Msg1.

  - Other contents of A-IoT paging message

Type of A-IoT random access procedure

Following agreements that RAN2 [1] made in the last meeting implies that multiple schemes of A-IoT random access procedure, 4-step RA, 2-step CBRA and contention-free access, are studied and potentially supported.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

Considering above agreements, after the Ambient IoT device receives an A-IoT paging message, contents and timing of the first message sent by the device are different as following way.

- In 4-step RA, the device sends A-IoT Msg1 containing a random ID generated by the device at a transmission occasion randomly selected by the device.

- In 2-step CBRA, the device sends A-IoT Msg1 containing upper layer data at a transmission occasion randomly selected by the device.

- In contention-free access, the device sends a message (FFS contents) at a transmission occasion designated by the network.

Therefore, the device has to be aware of which type of A-IoT random access procedure the device is required before it sends the first message in order to determine its contents and timing. Consequently, an indication of A-IoT random access type is needed in the A-IoT paging message.

Observation 6.	Ambient IoT device has to be aware of which type of A-IoT random access is required to determine contents and timing of the first message.

Proposal 4.	Ambient IoT reader indicates type of A-IoT random access procedure in A-IoT paging message.

Resource for D2R message transmission

According to RAN2 agreement in the last meeting, it is FFS how to provide information of resources for D2R message the device next sends.

Agreements (omit) 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. (omit)

As far as we understand, there is no RAN1 conclusion clear enough for RAN2 to start discussion on the detail of resource indication. We can still wait for RAN1 further progress.

Proposal 5.	RAN2 wait for RAN1 further progress on details of resources to transmit A-IoT Msg1.

- Summary and proposal

Observation 1.	Given that security protection is supported in upper layer, upper layer data for a command message provided in an A-IoT paging message cannot be decoded by more than one Ambient IoT devices since the upper layer security is E2E protection.

Proposal 1.	RAN2 assume that only one device can be targeted in A-IoT paging message in “Command only” use case. RAN2 can revisit based on SA2/SA3 conclusion.

Observation 2.	Ambient IoT device will send A-IoT Msg1 for times of the number of nearby Ambient IoT readers, which increases possibility of contention.

Observation 3.	Duplicated transmission of A-IoT Msg1 can be avoided via discarding A-IoT paging messages attached with the same Paging ID.

Proposal 2.	Introduce Paging ID to be attached to the A-IoT paging message. Ambient IoT devices have to respond only once to A-IoT paging messages with the same Paging ID.

Observation 4.	Reception of A-IoT Msg1 by multiple readers can result in duplicated upper layer data paths which are not necessary but waste of radio resources for Readers.

Observation 5.	Ambient IoT reader can save radio resources to allocate to devices responding to other readers if Reader ID is introduced.

Proposal 3.	Introduce permanent or temporary Reader ID to be attached to A-IoT paging message. Ambient IoT devices can include a Reader ID in A-IoT Msg1.

Observation 6.	Ambient IoT device has to be aware of which type of A-IoT random access is required to determine contents and timing of the first message.

Proposal 4.	Ambient IoT reader indicates type of A-IoT random access procedure in A-IoT paging message.

Proposal 5.	RAN2 wait for RAN1 further progress on details of resources to transmit A-IoT Msg1.

- References

[1]	Chair notes, RAN2#126.

[2]	R2-2406150, “LS on RAN2 agreements and assumptions for Ambient IoT,” RAN2#126.