3GPP RAN WG2 Meeting #127 	R2-2407139

# Masstricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.9.3

Source:	InterDigital, Inc.

Title:	Msg3 transmission without msg1/RAR.

Document for:	Discussion

# Introduction

A new WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

Previously, in RAN2#125bis, the following was agreed:

Agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements

In RAN2#126, the following was agreed:

Agreements: - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

Additionally, an LS was sent to RAN1 [3] asking about UL synchronization for contention based msg3 transmission (EDT).

# Overview of existing functionality

## 2.1	MO-EDT

It was agreed that both CP and UP solutions, NB-IoT and eMTC, are in the scope of the study, but only EPS.

The following show MO-EDT procedures for the CP and UP EPS optimizations.

The MO-EDT procedure for Control Plane CIoT EPS optimisation is illustrated in Figure 7.3b- of 36.300 [2]

Figure 7.3b-1: MO-EDT for Control Plane CIoT EPS Optimisation

The MO-EDT procedure for User Plane CIoT EPS optimisation is illustrated in Figure 7.3b-2 of 36.300 [2]

Figure 7.3b-2: MO-EDT for User Plane CIoT EPS Optimisation

## 2.2	Transmission using PUR

The procedure for transmission using PUR for the Control Plane CIoT EPS optimisation is illustrated in Figure 7.3d-2 of 36.300 [2].

Figure 7.3d-2: Transmission using PUR for the Control Plane CIoT EPS/5GS Optimisations

The procedure for transmission using PUR for the User Plane CIoT EPS optimisation is illustrated in Figure 7.3d-3 of 36.300 [2].

Figure 7.3d-3: Transmission using PUR for the User Plane CIoT EPS Optimisation

As can be seen, if PUR is used, it is already possible to transmit Msg3 without Msg1/RAR as this is the whole point of PUR.

Proposal 1: Confirm that it is already possible with the existing PUR feature to transmit Msg3 without Msg1/RAR.

The procedure for PUR configuration request and PUR configuration is common to the Control Plane CIoT EPS/5GS optimisations and the User Plane CIoT EPS/5GS optimisations and is illustrated in Figure 7.3d-1 of 36.300 [2].

Figure 7.3d-1: PUR Configuration Request and PUR Configuration

It should be noted that a PUR resource, although from the UE point of view is a dedicated resource, can be configured by the network to more than one UE. This was discussed during the standardisation of the PUR feature and it is left to eNB implementation whether to configure PUR resources in a shared manner, and how to deal with any contention issue. For example, in case more than one UE uses a PUR resource which has configured, and at least one of those UEs fail, then the application layer can attempt to retry on a future PUR occasion, using RACH based EDT, or using the CIoT optimisation without EDT.

Proposal 2: Confirm that it is already possible with the existing PUR feature to configure PUR resources to more than one UE (shared/contention-based resources).

# Deficiencies with existing functionality and potential enhancements

As can be seen above, the current PUR feature, although it enables Msg3 transmission without Msg1/RAR, and can be configured as contention based, this relies on entering RRC_CONNECTED to be configured with PUR resources which are applicable only in the current cell and when the TA validity conditions are met.

In addition, there is no standardised method to detect a contention, and no standardised method to recover from contention or transmission failure. While introduction of a new transmission scheme may seem attractive on paper, the system and standardisation impact from such a new scheme could be significant.

Proposal 3: To support enhanced contention-based PUR, the following schemes may be considered.

- Introduction of new transmission scheme at layer 1 (Contention Resolution Diversity Slotted Aloha)

- Introduction of contention resolution procedure at L2/3 (e.g. similar to 2 step RA)

Clearly, it is not possible to conclude on (1) without input from RAN1. An LS needs to be sent to RAN1 to ask about feasibility. Until RAN1 can study and conclude on the feasibility, RAN2 may need to assume that a L2/3 based procedure is necessary. This is most likely needed whether a new transmission scheme is used or not, since it is possible that some transmission will in any case fail, and it is possible that CRDSA is not mandatory/supported for all cases.

Proposal 4: RAN2 assume that a L2/3 based contention resolution procedure is necessary.

Proposal 5: Send an LS to RAN1 asking about the feasibility of a new transmission scheme based on (Contention Resolution Diversity Slotted Aloha)

In addition, there is also no method for the network to configure PUR resources without dedicated signalling. For shared resources, system information configuration is the natural choice.

Proposal 6: Shared PUR resources are configured in system information.

For the case of shared (contention based) PUR resources configured by system information, we will also need to discuss whether a UE needs to be authorised/configured by dedicated signalling before being allowed to use shared PUR resources, or whether this is done only using broadcast signalling (e.g. always allowed based on capability, or allowed based on a broadcast configuration like barring).

Proposal 7: For the case of transmission using shared PUR resources, consider the following options for allowing the UE to transmit

- Always allowed based on UE capability and NW enabling the feature

- Allowing the network to select a subset of UEs (e.g. like access barring)

- Dedicated signalling for authorisation, either at AS or NAS.

Finally, it needs to be discussed how the UE should select a resource, when multiple resources are configured by the eNB. This may somewhat depend on whether a new transmission scheme is introduced by RAN1. However, in case multiple configurations/resources are provided by system information, the UE may need to use a selection mechanism, such as selecting a set of resources based on coverage level, UE-ID, etc.

Proposal 8: RAN2 needs to consider how to select a shared resource, when multiple resources are configured by the eNB.

While shared PUR configuration may be beneficial, it may also be beneficial in some cases to use a dedicated resource configuration. For example, to provide some UEs with a more reliable transmission mechanism. However, the signalling overhead from having to perform PUR configuration in every cell is high, therefore it should also be studied whether dedicated resource configuration can be optimised for NTN, such that a PUR configuration can be made in one cell and be used in another cell.

Proposal 9: Study the support of dedicated signalling for configuring PUR resources across cells.

Proposal 10: FFS whether and how shared and dedicated PUR resources can co-exist.

# Conclusion

In this contribution we discuss Msg3 transmission without msg1/RAR and make the following proposals for discussion:

Proposal 1: Confirm that it is already possible with the existing PUR feature to transmit Msg3 without Msg1/RAR.

Proposal 2: Confirm that it is already possible with the existing PUR feature to configure PUR resources to more than one UE (shared/contention-based resources).

Proposal 3: To support enhanced contention-based PUR, the following schemes may be considered.

- Introduction of new transmission scheme at layer 1 (Contention Resolution Diversity Slotted Aloha)

- Introduction of contention resolution procedure at L2/3 (e.g. similar to 2 step RA)

Proposal 4: RAN2 assume that a L2/3 based contention resolution procedure is necessary.

Proposal 5: Send an LS to RAN1 asking about the feasibility of a new transmission scheme based on (Contention Resolution Diversity Slotted Aloha)

Proposal 6: Shared PUR resources are configured in system information.

Proposal 7: For the case of transmission using shared PUR resources, consider the following options for allowing the UE to transmit

- Always allowed based on UE capability and NW enabling the feature

- Allowing the network to select a subset of UEs (e.g. like access barring)

- Dedicated signalling for authorisation, either at AS or NAS.

Proposal 8: RAN2 needs to consider how to select a shared resource, when multiple resources are configured by the eNB.

Proposal 9: Study the support of dedicated signalling for configuring PUR resources across cells.

Proposal 10: FFS whether and how shared and dedicated PUR resources can co-exist.

# References

- RP-234077,  “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”

- R2-2405769, “LS on uplink synchronization for contention based msg3 transmission”