3GPP TSG-RAN WG2 Meeting #127	         R2-2407143

Maastricht, Holland, Aug 19th – 23rd, 2024

Agenda Item	:	8.11.2

Title	:	Random Access in Sub-Band Full Duplex

Source	:	Google

Document for	:	Discussion and Decision

# Introduction

In RAN#104 [1], the following Rel-19 objective has been captured:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this Tdoc, we share our views on the SBFD operation to support random access in SBFD symbols by UEs in RRC _CONNECTED mode and RRC_IDLE/INACTIVE mode.

# Random Access in Sub-band full duplex

- Support of 2-step RACH in SBFD operation

In RAN1#116, the following agreement has been made regarding the support of Type-1 and Type-2 random access procedure for SBFD aware UEs:

Agreement

For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols.

- FFS Type-2 random access procedure (2-step RACH)

The primary objective of SBFD operation is to enhance uplink capacity and coverage and reducing latency. Consequently, it is logical to also support 2-step RACH within UL SBFD symbols, as this mechanism also aims to reduce latency.

- SBFD aware UEs in RRC CONNECTED and RRC IDLE/INACTIVE state support Type-2 random access procedure (2-step RACH) in SBFD symbols.

- Interpretation of msg1-FrequencyStart

In RAN1#117, the following agreement has been made for the adoption of single RACH configuration or two separate RACH configurations for SBFD-aware UEs and non-SBFD-aware UEs:

Agreement

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

msg1-FrequencyStart is the offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP.

The agreement specifies support for RACH configurations Option 1 with Alt1-1 and Option 2. While a UE may be capable of supporting both options, it is not required to do so. Additionally, simultaneous activation of both options on a single UE is not supported.

In Option 1 with Alt 1-1, using the legacy interpretation of msg1-FrequencyStart in rach-ConfigCommon for both SBFD and non-SBFD symbols may result in ROs for SBFD symbols falling outside the UL subband when the UL subband is located in the middle of the BWP.

To address this, we propose utilizing two distinct frequency start references for SBFD and non-SBFD symbols for the interpretation of msg1-FrequencyStart

- Non-SBFD symbols: retain the legacy RACH configuration interpretation, using PRB 0 of the UL BWP as the frequency start reference.

- SBFD symbols: interpret msg1-FrequencyStart as the frequency offset relative to the first full PRB at the start of the UL subband.

This approach resolves the issue, but introduces a constraint:  msg1-FrequencyStart cannot exceed the size of the UL subband on the SBFD symbols as this means the RO will be located outside the UL subband. There are multiple solutions to address this constraint. The UE can ignore the RO occasion if it is located outside the UL subband on the SBFD symbols. It is also possible to use two interpretations and a modulo operation based on the UL subband size can be applied to msg1-FrequencyStart for SBFD symbols only.

- Non-SBFD symbols: frequencyStart = msg1-FrequencyStart

- SBFD symbols: frequencyStart = mod ( msg1-FrequencyStart, UL subband size)

- For Option 1, Alt 1-1, support two different interpretation for msg1-FrequencyStart in rach-ConfigCommon depending if the symbols are SBFD or non-SBFD.

- PRB 0 of the UL BWP as the frequency start reference for the non-SBFD symbols and first full PRB at the start of the UL subband for the SBFD symbols.

- A modulo operation can be used for SBFD symbols to keep the ROs within the UL usable PRBs.

- Power control:

The existing RACH power control mechanisms might not be optimal for SBFD RACH operation. The interference levels in the SBFD symbol, where the UE transmits the RACH preamble, can be significantly different from those in regular uplink symbols. This can lead to either insufficient power for reliable detection at the gNB or excessive power causing interference to the downlink reception in the same sub-band.

Therefore, introducing separate power control parameters for SBFD ROs to account for the varying interference levels in SBFD symbols compared to regular UL symbols is essential. This would allow the network to configure the appropriate transmit power for SBFD RACH preambles, ensuring reliable detection at the gNB while minimizing interference to downlink reception.

For RACH configuration Option 2 using two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration, two separate power control parameters for each RACH configuration are needed (preambleReceivedTargetPower, powerRampingStep, and preambleTransMax in RACH-ConfigGeneric in AdditionalRACH-Config).

- For Option 2, with two separate RACH configurations. support two separate power control parameters for legacy RACH configuration and the additional SBFD RACH configuration (e.g., preambleReceivedTargetPower, powerRampingStep, and preambleTransMax in RACH-ConfigGeneric).

- Random Access triggering events

RAN1 will focus on the potential enhancement to support SBFD random access operation triggered by PDCCH. Other triggering events, which may have no RAN1 impacts, can be discussed in RAN2, including initial access, connection re-establishment, data arrival, handover, …

- RAN2 to evaluate the feasibility and benefits of supporting RACH triggering events within SBFD symbols, analyzing the impact of SBFD operation on each triggering event.

# 3		Conclusions

In conclusion, we have the following proposals:

- SBFD aware UEs in RRC CONNECTED and RRC IDLE/INACTIVE state support Type-2 random access procedure (2-step RACH) in SBFD symbols.

- For Option 1, Alt 1-1, support two different interpretation for msg1-FrequencyStart in rach-ConfigCommon depending if the symbols are SBFD or non-SBFD.

- PRB 0 of the UL BWP as the frequency start reference for the non-SBFD symbols and first full PRB at the start of the UL subband for the SBFD symbols.

- A modulo operation can be used for SBFD symbols to keep the ROs within the UL usable PRBs.

- For Option 2, with two separate RACH configurations. support two separate power control parameters for legacy RACH configuration and the additional SBFD RACH configuration (e.g., preambleReceivedTargetPower, powerRampingStep, and preambleTransMax in RACH-ConfigGeneric).

- RAN2 to evaluate the feasibility and benefits of supporting RACH triggering events within SBFD symbols, analyzing the impact of SBFD operation on each triggering event.

# 4		References

- RP-241614 ‘Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)’	Huawei, RAN#104,  June 17-20, 2024 , Shanghai, China.