3GPP TSG-RAN WG2 Meeting #127                                                                  R2-2407148

Maastricht, Netherlands, 19th – 23th August, 2024 	                      Revision of R2-2404881

Source: 		  ZTE Corporation, Sanechips

Title:                   Further consideration on paging procedure for AIoT

Agenda item:     8.2.3

Document for:   Discussion and Decision

- Introduction

In RAN#102, a study item on solutions for Ambient IoT (Internet of Things) in NR has been agreed and in RAN#103, the SID is further refined in [1].

In RAN2#125bis meeting, RAN2 has had initial discussion for Ambient IoT. The following agreements related to general aspect and protocol stacks have been achieved:

Agreements (General aspect) - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. - RAN2 will support two use cases, “inventory” and “command”. The definition, detailed wording is FFS - Baseline procedure: Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS Step C: The device may perform the data communication with the reader as needed,: Details FFS - We will study the support of both “inventory” and “command” in the same procedure. - FFS if Initial Trigger Message can also include “command”. - RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input. Agreements (Control Plane protocol) - RRC connection management is not supported. FFS how the resource configuration is provided to the device (if needed based on RAN1 progress) - RRM L3 measurement reporting is not supported by Ambient IoT devices. - RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding. - Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals. - RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC Agreements (User Plane protocol) - SDAP is not supported for UP protocol stack. - PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities. - RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. - No HARQ and RLC AM - FFS about the level of visibility required by the reader and what information is necessary for AS layer operations. - RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2 Agreements (Paging) - Legacy paging message for device will not be supported. - Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well). - RAN2 assumes that the device will not support tracking/RAN area update procedure. - For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2

In RAN2#126 meeting, RAN2 has further achieved the following agreements :

Agreements (Stage 2 General aspects) - As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. - From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command) Agreements (Functionality aspects) - Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. - legacy NR BSR/SR is not needed for A-IoT communication. - FFS whether further indication of device message size/status is needed - AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed) Agreements (A-IoT Paging) - RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. - From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details. Agreements (A-IoT Random Access) Agreements on “4 step” RA - A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements - A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) - The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. - “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access. Agreements on 2 step CB RA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

Furthermore, some relevant agreements from RAN1#117 meeting are also cited as below:

9.4.2.2 Frame structure and timing aspects R1-2405379 FL summary #1 on frame structure and timing aspects for Rel-19 Ambient IoT Moderator (vivo) Agreement Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission. Agreement Scheduling information of PDRCH transmission is provided by a corresponding PRDCH. R1-2405380 FL summary #2 on frame structure and timing aspects for Rel-19 Ambient IoT Moderator (vivo) From Thursday session Conclusion RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2. Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D 9.4.2.3 Downlink and uplink channel/signal aspects R1-2404290 FL summary#1 on downlink and uplink channel/signal aspects Moderator (Apple) Agreement For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH Agreement For D2R - PDRCH carries any higher-layer payload - PDRCH carries L1 D2R control information (if defined) - Note: PDRCH carries the response agreed at RAN1#116

In this contribution, based on the progress in both RAN1 and RAN2, we will further discuss the details of paging procedure for air interface inventory and give our proposals.

- Discussion

2.1 General procedure for “command only” use case

In last meeting, RAN2 has agreed the general description on the steps of “inventory only” case and “inventory and command” case. The figure of overall AS procedure has been captured in the TP for TR 38.769 as below:

Figure 6.2.1-1 Overall AS procedures between A-IoT device and reader

It’s still FFS whether and how to support “Command only” use case. Two alternatives for the steps of “Command only” use case have been given as below:

- Initial trigger message from the reader contains the command.  Final feasibility depends on SA2 and SA3 work/conclusions.

- Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

Per our understanding, “Command only” use case refers to that a device is already known to the AF and AF tends to send specific command to the known device (e.g., to read the sensing data from the device or write some information into the device). From service or AF perspective, we think this use case exist. However, from RAN2 perspective, even AF can directly trigger command service, it’s still beneficial to firstly inventory this device. The reason is that, even previously this device has been successfully inventoried, it may be the case that this device may have moved, e.g., no longer in the coverage of this AF. Or as a passive device, it may have exhausted the stored power energy after being inventoried last time and therefore cannot directly receive and process kind of large data packets. Considering all these possibilities, it is necessary to firstly perform an inventory on this device to detect its presence. This can be helpful to avoid unnecessary signalling overhead by sending service data to non-existent device(s). Moreover, by performing an inventory procedure firstly, the device can enter a status of “has been inventoried” and try to accumulate a certain amount of energy which would facilitate the device to be better prepared for receiving or transmitting kind of large service data.

Proposal 1a: At least from reader perspective, e.g., on the A-Uu interface between reader and AIoT device , it’s suggested to use baseline procedure for “inventory and command” (i.e. first triggers inventory procedure and then sends command) for the “Command only” use case.

Moreover, for both “inventory and command” and “Command only” use cases, we assume that, as long as the command service is triggered, the AF will bring the identity information of the device(s) and the command service data for these device(s) to the core network. Further considering that data transmission can be performed during random access procedure (it’s part of the inventory procedure), we recommend that the core network can send the acquired service data directly to the base station. Such process can facilitate the base station to also send the data to the device as soon as possible in the random access procedure. As how to deliver the service data to the base station is in the scope of RAN3 discussion, RAN2 can send LS to RAN3 to indicate the recommendation from RAN2.

Proposal 1b: Send LS to RAN3 to indicate that for “inventory and command” and “Command only” use cases, RAN2 suggest that the core network can send the acquired command service data to the base station as soon as possible, e.g., before the base station triggers the inventory procedure.

2.2 Paging/DL message monitoring

For the inventory performed on the A-Uu interface, the first downlink trigger signalling can provide the identities of one or more AIoT devices and trigger the matched devices to access the network. From functionality perspective, this first downlink trigger signalling is similar as the legacy Paging, so it is tentatively called Paging in the overall AS procedures, which is also for the convenience of the following discussion.

According to the discussion in RAN1 and RAN2 till now, there is kind of general understanding that the AIoT device may not be able to keep stringent time synchronization with the network node, e.g., BS reader. Therefore, RAN2 has agreed that legacy paging occasion and DRX scheme will not be supported by AIoT. In last RAN2 meeting, RAN2 further agreed the assumption that the device can receive AIoT paging as long as there is enough energy. For the monitoring details, RAN2 will wait for RAN1 further progress.

We can see the benefit for the AIoT device to continuously monitor the downlink message, e.g., each time an inventory procedure is trigged, the device can be aware of it timely. Moreover, for this kind of device that naturally does not equipped with a conventional battery and can harvest energy from ambient environment, the continuous downlink signal reception also has the potential benefit of accumulating power energy.

However, during the overall inventory procedure, one or more DL messages will be sent, some messages is for all devices, e.g, the initial paging, while some messages may be used to send data to a single device. The device in continuous monitoring state may demodulate a DL message that is not aimed at it, causing unnecessary power consumption.

Observation 1a: RAN2 has agreed the assumption that the device can receive AIoT paging as long as there is enough energy. Such process is beneficial for the device to monitor paging timely and can also help the device to accumulate power energy. However, the device in continuous DL monitoring may demodulate a DL message that is not aimed at it, causing unnecessary power consumption.

In legacy LTE or 5G NR system, the scheduling and transmission of paging messages is on Paging Channel (PCH) which is generally broadcast in the entire coverage area of the cell. For UE side, it needs to monitor common search space to receive Paging messages addressed with P-RNTI. After completing the paging response, e.g., accessing the network after successful contention resolution for RA procedure, the UE starts to monitor UE-specific search space and mainly acquire the dedicated signalling/data.

However, for AIoT, according to the progress in RAN1, a simple design for downlink and uplink physical channel/signal can be expected. For example, in DL, only one PRDCH channel is design which can carry the R2D (reader to device) link for transmitting R2D data and/or L1 R2D control information. The differentiation between common search space/ broadcast channel and UE-specific search space/unicast channel may no longer exist. In other word, from channel perspective, it is impossible to distinguish whether a DL R2D transmissions on PRDCH channel needs to be monitored by all or a group of devices in entire coverage area or only needs to be monitored by a certain device.

Observation 1b: According to the RAN1 agreements, a simple design for downlink and uplink physical channel/signal can be expected, e.g., for DL, only one PRDCH channel is design which can carry the R2D (reader to device) data and/or control information transmission. It seems the differentiation between common search space/ broadcast channel and UE-specific search space/unicast channel no longer exist.

Even there is no channel differentiation for DL/UL transmission, RAN1 still has discussion on the possible L1 R2D control information which may give help to the device on monitoring/receiving DL R2D transmissions on PRDCH channel.

From RAN2 perspective, for paging transmission during inventory procedure, since all the DL messages will be transmitted on the same PRDCH channel, we understand a cast type, e.g., broadcast or unicast, may need to be introduced for DL R2D transmission to help a device to distinguish whether a R2D transmission is for multiple devices or just one certain device.

Proposal 2a: RAN2 assume a cast type, e.g., unicast or broadcast needs to be introduced for DL transmission on PRDCH channel to help a device to distinguish whether a DL message over R2D transmission is for multiple devices or just one certain device.

Generally, on one hand, all the devices which have not been successfully inventoried need to monitor the paging message over R2D transmission with broadcast type. On the other hand, if a device has already been successfully inventoried, the device can ignore paging message over R2D transmission with broadcast type but just wait for a DL message over R2D transmission with unicast type. Such differentiated processes can be helpful to reduce the unnecessary demodulation process in device side. The implementation of such differentiated processes can be based on a status information within the device, e.g., “has not been inventoried” and/or “has been inventoried”

Proposal 2b: The status of “has not been inventoried” and/or “has been inventoried” may need to be introduced for the device. The device with different status can monitor DL messages over R2D transmission with different cast type.

2.3 Consideration on device ID

Generally, the device ID information needs to be provided within the DL paging message. The main purpose of carrying device ID information is to facilitate the device to compare its stored device ID with the provided device ID and determine whether it needs to respond the AIoT service request. Please note, here device ID information is just a general saying.

In last meeting, RAN2 has agreed the following possible device ID types which can be contained in paging message:

- an ID of a single A-IoT devic:

- a group ID that maps to multiple A-IoT devices.

- without an ID, means all devices that can receive the AIoT message

- (FFS) multiple IDs of A-IoT devices

In last RAN2 meeting, both multiple IDs and a group ID are proposed with the intention to indicate multiple A-IoT devices. RAN2 has no consensus on the necessity of multiple ID and suggest to confirm the need for this use case based on SA2 discussion. However, in last SA2 meeting, no related discussion and agreements can be seen.

Per our understanding, the most general way to indicate multiple devices is to apply “mask/filter” on the device IDs which can finally achieve a device list. To indicate multiple IDs can be seen as a special usage on top of the use of “mask/filter”. It can be assumed that enumerated multiple IDs can be used to indicate multiple devices with discrete ID numbers. Just due to the discrete ID numbers, it’s not easy to indicate these devices with a way of “mask/filter”. We think the scenario for using multiple IDs exists, for example, to indicate several remaining devices that have not yet been inventoried after multiple rounds of inventory on a device list.

The group ID is also needed as it’s another efficient way to indicate multiple devices if these devices previously have been gathered into a same group and assigned with a group ID. To include a single group ID in paging message can trigger all the devices within this group to response the paging.

Proposal 3a: From RAN2 perspective, both multiple IDs and group ID can be considered. The multiple IDs can be used to indicate multiple devices with discrete ID numbers, while group ID can be used to efficiently indicate multiple devices within a pre-defined group.

RAN2 further assume what device ID and group ID and scenarios is depending on SA2 discussion. But in SA2, the format of the AIoT device identifier is still under discussion and no clear agreements yet. Some typical options are cited as below:

- Permanent ID: it is expected to enable uniquely identifying AIoT device. Such ID can contain MNO ID, Enterprise (Owner) ID, Instance ID etc. However, there may be security risk and signalling overhead issue if such long Permanent ID is conveyed on all AS level procedures (e.g. between AIoT device and reader) and/or non-AS level procedures (e.g. between reader and core network).

  - With reference to legacy RFID, an EPC as defined by GS1 in the EPC Tag Data Standard (reference can be found in section 2) can be an option for a Permanent ID.

- Long temporary ID: A long temporary Device ID to be used between the Controller and the Reader. The Controller maintains the mapping between long temporary ID and unique Ambient IoT Device identifier (e.g. based on the Permanent ID) per Device based on notification from the Reader. Such Long temporary ID does not need to be exposed to the Ambient IoT Device.

- Short temporary ID: A short temporary Device ID to be used between the Reader and Ambient IoT Device. The short temporary Device ID can be assigned and provisioned by the Reader to the Ambient IoT Device. The Reader maintains mapping between long temporary ID and short temporary ID per Ambient IoT Device. The Reader notifies the Controller on mapping of long temporary ID per ambient IoT Device (e.g. based on the Permanent ID).

From RAN2 perspective, a brief comparison is listed as below:

- The use of a permanent ID (e.g., the EPC format, can makes it easier to perform business-specific inventory (e.g., inventory for the class of Shipping Container of a sub-class of Shipping Container class), which may be the most common use case for the applications for tag-type devices. Moreover, if the device is typically allocated such permanent ID by the third-party and permanently store it in the physical memory inner the device, the use of a permanent ID would not cause additional storage burden on the device. However, in addition to the signalling overhead and security risk, the other problem may be whether the energy harvested by the device is sufficient to support such long data transmission (not to mention more retransmissions due to possible fragmentation and higher failure possibility).

- On the contrary, the use of shorter temporary ID can partially avoid the problems of transmitting permanent ID. The temporary ID assigned by the core network (which may be similar as the 5G-S-TMSI in 5G) or even reader (maybe a random value as RN16 in RFID) can also contain some node information (e.g., AMF id or BS reader id), which may be helpful to more accurately select the suitable reader for inventory the corresponding devices. However, the main disadvantage may be that core network nodes and even readers need to store a large number of mapping relationships between temporary ID and permanent ID. Moreover, we assume such temporary ID also needs to be assigned to the device and may further increase the storage burden on the device. Finally, it may be not easy to convert business-specific inventory operations into temporary ID-based inventory. How complex a convert scheme may highly rely on how reasonable the serialized structure design for the temporary ID.

Considering some above pros and cons for different types of AIoT device ID may have impacts on air interface, we suggest RAN2 have brief discussion on the type of AIoT device ID and give some general considerations or preferences from RAN2 perspective.

Proposal 3b: RAN2 is suggested to have brief discussion on the type of AIoT device ID, e.g., permanent long ID vs temporary short ID. RAN2 can give our general considerations or preferences and indicate it to SA2.

2.4 Filter/mask scheme for air interface inventory

No matter which type of device ID (long or short) would be used, in several solutions in SA2 study, filter/mask scheme, e.g., filter criteria or bitmask, have been mentioned with intention to filter part of the AIoT devices to be inventoried. Per our understanding, no matter from the service requirement perspective or from the perspective of controlling inventory performance, such filter/mask scheme is needed.

Generally, if the identities of the devices from a same owner are continuous, device identity and simple mask code can be used to indicate a list of devices efficiently. For example, if the last n bits of a device Identity are masked, then 2^n number of UE can be selected.

Observation 2a: In some use case, filter/mask criteria from upper layer may be provided along with device ID information in the service request from AF/CN with intention to filter only part of the AIoT devices to be inventoried.

Although RAN2 can assume kind of upper layer filter/mask scheme will be supported to achieve the purpose of filtering only some devices for inventory, it may be mostly applied according to a business requirement (e.g., to filter the devices belonging to a certain business type) or it may not even be applied (e.g., upper level just want to do an inventory of all the devices). Therefore, it’s still possible for BS reader to receive inventory requests for a large number of devices. This may further cause heavy load for A-Uu air interface inventory and heavy collision for the following random access procedure at the air interface.

Observation 2b: Although kind of upper layer filter/mask scheme may be supported, it’s still possible for BS reader to receive inventory requests for a large number of devices which will further cause heavy load for “paging” and heavy collision for the following random access procedure at the air interface.

Therefore, we believe that the filter/mask mechanism in the RAN level is still needed with intention to reasonably split the inventory of a possible large number of devices from upper layer into several air interface inventories for a smaller group, which can be helpful to reduce the collision probability within each inventory and improves the overall inventory efficiency.

In the following, we will give two angles of analysis to explain the necessity of such an air interface filter/mask mechanism, one is from the potential size of the inventory service from upper layer, and the other is the from the collision probability in the RA procedure after initial paging message in air interface inventory procedure.

- As mentioned in the study report [3, TR 38.848] from RAN plenary, the new Ambient IoT technology will open new markets within 3GPP systems, whose number of connections and/or device density can be orders of magnitude higher than existing 3GPP IoT technologies. Moreover, according to the consolidated potential KPIs in SA1 study report [2, TR 22.840], the maximum connection density target is:

- 150 devices per 100 m2 for indoor scenarios - 20 devices per 100 m2 for outdoor scenarios. RAN WGs will define the 2D or 3D distribution(s) of devices.

It can be seen the consolidated maximum connection density target can be 1,5Million devices per 1 Million m2, e.g., 1km2. In this SA1 study report, the maximum value of “service area dimension” for indoor scenario is 1 700 000 m2 which comes from a flower Auction scenario. And the device density can be as large as 1,3Million/km2. That means totally about 2,2Million devices in the area of 1.7km2.

Such a large indoor environment may require the deployment of 5G macro base stations. Considering that the coverage radius of the macro BS is generally from 100m to 300m, and the coverage area is about 30000m2~280000m2. So at least 5~6 macro base stations may be needed. That may further means there may be between 300,000 ~ 500,000 devices within the coverage area of a base station. Please note the number of UEs that a base station for legacy NB-IoT technology need to support is about 100,000, so this device density requirement for AIoT use case is about 3-5 times the support scale of NB-IoT. Moreover, different from the traffic type in legacy IoT applications where for example only one packet needs to be transmitted per 24 hours, for a typical AIoT inventory service, the overall inventory on hundreds of thousands of devices under a BS reader may need to be completed in a relatively short period of time, e.g., hour level or even shorted.

Observation 3a: The inventory request from upper layer for AIoT, even for a single business, may reach the scale of hundreds of thousands of devices for one time inventory. There may also be stringent time restriction for the entire inventory operation.

- Even for an upper layer inventory for a device group that are not so large, if no any anti-collision scheme is used, heavy collision during the access procedure may still occur.

Here is a roughly calculation. If N devices are inventoried at a same time, the non-collision probability is N!/(N^N), so:

- if only 1 device is inventoried, the non-collision probability is 1

- if 2 devices are inventoried simultaneously, the non-collision probability is 2*1/(2^2)= 50%

- if 3 devices are inventoried simultaneously, the non-collision probability is 3*2*1/(3^3)= 2/9=22.2%

- if 4 devices are inventoried simultaneously, the non-collision probability is 4*3*2*1/(4^4)= 3/32=9.3%

- if 10 devices are inventoried simultaneously, the non-collision probability is 0.03%

- If there are 300,000 ~ 500,000 devices within the coverage area of reader and all devices needs to be inventoried during a certain time period, it will cause very serious collision, again and again inventory may be needed to overcome the collision, which will last for a long time and waste radio resource seriously.

Observation 3b: Even for an upper layer inventory for a certain device group that are not so large, if no any anti-collision scheme is used, heavy collision during the access procedure may still occur.

Therefore,

Proposal 4: It is also necessary for the RAN to apply the mask/filter mechanism with intention to reasonably split the inventory for a large number of devices from the upper layer into several air interface inventories, each for a smaller number of devices.

2.5 Q-algorithm for air interface inventory

With reference to RFID, a Q-algorithm is generally used in the inventory procedure in legacy RFID system for anti-collision. It is a way to resolve collision through time domain distribution. Shortly to say, even a device is filtered out via matching with the provided device ID information, it doesn’t means the device can response to the reader immediately after receiving the paging message. The main steps of a Q-algorithm can be as below:

- A Q value is broadcast to all the devices by the reader;

- For a certain device, it generates a random number between 0~2Q -1 as its response time slot and loads this random slot number into a slot counter in its side. This random slot number will be further reduced when receiving kind of trigger till it equals to 0.

- The device with the slot counter equal to 0 can trigger random access. So this slot counter can be seen as a length of delay before the device can be allowed to trigger random access. If the device successfully access the reader and responses with its device ID, the device can think that it has been successfully inventoried and can exit the operation. It may be possible that several devices have the slot counter equal to 0 at a certain moment, then all of them can trigger random access at the same time, and this collision can be further addressed by other contention resolution methods, e.g., during the random access procedure.

- The reader can send another command to notify the devices to decrease the slot counter. Each time a device receives this command, its slot counter is reduced by 1. By this way, for each time, only part of the devices (devices whose slot counter is reduced to 0) have chance to access the reader.

- Furthermore, if the reader detects some collision in which multiple devices are responding at the same time, it can broadcast a new Q value, and upon receiving this new Q value, all devices will regenerate the random slot number for the slot counter. Usually, in this case, a larger Q value will be provided so that the device can generate a random slot number in a larger value range. On the contrary, it’s also possible that at some point the reader doesn't receive any response from the device. That means some time slots are unnecessary idle, the reader can also provide a new Q value, e.g., with a smaller value, to trigger the devices regenerate random slot number for slot counter that are more tightly distributed.

Per our understanding, in such Q-algorithm, the device does not need to be aware of the exact time position but only needs to decrease a counter according the indication from reader. When the counter is reduced to 0, no matter what the current time it is, the device can trigger access. This way is very suitable to such device as AIoT that cannot have strict time synchronization with the reader.

Proposal 5a: It can be considered to apply kind of anti-collision scheme similar as Q-algorithm in RFID to AIoT.

From RAN2 perspective, the possible impacts on signalling procedure may be that, if kind of anti-collision scheme similar as Q-algorithm in RFID is applied, besides device ID and mask information (e.g., to achieve an inventory for a device list), the paging message also needs to provide an initial Q value.

Proposal 5b: If kind of anti-collision scheme similar as Q-algorithm in RFID is applied, besides the device ID and mask information, the paging message also needs to provide an initial Q value.

Moreover, the reader further needs to send some subsequent DL messages to reduce the slot counter in device side or even adjust the Q value. Different from the initial paging message, the DL message for reducing the slot counter has no need to provide device ID information. So these subsequent DL messages still can be seen within the current paging-like inventory procedure. The end of current paging-like inventory procedure is generally when all devices in the group are successfully inventoried or the maximum time duration for inventory is reached (the devices which give no response can be marked as failed to inventory).

The above mentioned signalling and required information can be found in the Figure 1 in section 2.8.

Proposal 5c: If kind of anti-collision scheme similar as Q-algorithm in RFID is applied, after the initial paging message, the reader also needs to send some subsequent DL messages, e.g., to reduce the slot counter in device side or even adjust the Q value.

2.6 More appropriate inventory area for the devices

Considering the flower Auction scenario mentioned in section 2.4, there may be totally about 2,2Million devices in the auction ground of 1.7km2. Then 5~6 macro base stations may need to be deployed in the area and there may be about 300,000 ~ 500,000 devices within the coverage area of a base station. Obviously, if to inventory all the devices in the entire auction ground in one time, it will mean a very large inventory load.

So a straightforward way is that, each inventory can only be performed on the devices within a certain area, e.g., an area naming inventory area. This inventory area may be similar as the tracking area in legacy paging scheme, but the difference is that, as RAN2 has agreed, no tracking/RAN area update procedure will be supported by the devices, then it may be more possible to rely on network node to setup and maintain such inventory area, e.g., based on the inventory response from AIoT devices during inventory procedure.

Proposal 6a: A concept of inventory area can be introduced with intention that an inventory is mainly performed for the devices within a certain inventory area.

For Topology 1, since AIoT device communicates with BS reader directly, it can be assumed that an AIoT inventory area in TP1 may include at least one BS reader. And the association between a certain device with the BS reader can be set up by network node, e.g., BS reader or CN node. From RAN perspective, one base station (BS reader) can be associated with multiple AIoT devices.

Proposal 6b: It can be assumed that an AIoT inventory area for Topology 1 may include one or more BS readers. The association between a certain device with a BS reader can be set up by network node, e.g., BS reader or CN node. From RAN perspective, one base station (BS reader) can be associated with multiple AIoT devices.

For Topology 2, AIoT device communicates with intermediate node (e.g. NR UE), and the intermediate node will further communicates with the base station. Therefore, it can be assumed that an AIoT inventory area in TP2 may include at least one UE readers. And the UE should associate with a paging area, which should also be known by the base station. The association between a certain device with the UE reader and further with the attached base station can also be set up by network node, e.g., base station or CN node. From RAN perspective, one base station can be associated with multiple UE readers and each of UE reader can be associated with multiple AIoT devices.

Proposal 6c: It can be assumed that an AIoT inventory area for Topology 2 may include one or more UE readers. The association between a certain device with the UE reader and further with the attached base station can also be set up by network node, e.g., base station or CN node. From RAN perspective, one base station can be associated with multiple UE readers and each of UE reader can be associated with multiple AIoT devices.

The above process for setting up association between a certain device with the reader (and may further with the BS) can be done during inventory procedure. Taking TP2 as example, the node A (e.g., the UE reader) that receives the inventory response from AIoT devices, can further deliver the device ID information to the next level network node, node B (e.g., the base station), and attach its own node information (UE id or UE reader id), so that the node B can record the association between each device and the UE reader. Please note, here the node B may receive Uu responses from multiple UE readers which are in its coverage and each Uu response may include responses from multiple AIoT devices. Similarly, the node B can also deliver the association between AIoT devices and the UE readers, and additionally its own node information (base station id) to the node C (e.g., AMF(AIoTF) in core network) in order that the core network node can record the association among the AIoT devices, the UE readers and the base station.

Proposal 6d: The setting up association between a certain device with the reader (and may further with the attached base station) can be done during inventory procedure.

Furthermore, it needs to discuss whether the device is capable to record the AIoT inventory area information, e.g., the BS reader or UE reader where it was successfully inventoried in last time. If yes, the device can only response the inventory from the BS reader or UE reader in the AIoT inventory area. This would be very helpful for reducing the unnecessary response for air interface inventory in device side.

In this way, the reader identifier may need to be provided in the paging message of the inventory procedure.

Proposal 6e: RAN2 can further discuss whether the device is capable to record the BS reader or UE reader where it was successfully inventoried in last time. If yes, the reader identifier may need to be provided to the device in the paging message of inventory procedure.

If the above proposals can be acceptable, RAN2 can further discuss the more complicated scenario, e.g., whether it’s possible that an AIoT inventory area may include more than one readers.

2.7 Contents in the paging message

Based on all the above discussion, RAN2 can consider the following types of DL messages and the possible contents for a paging-like inventory procedure in air interface.

Please note, there is kind of general assumption that some resources information for random access may be also needed in the paging message but without further discussion, as this mainly rely on the output from RAN1 discussion.

Proposal 7a: RAN2 can consider to include the following information in the initial paging message for an air interface inventory procedure:

- Cast type (FFS in L1 control information or in MAC layer)

- Device ID information

- Filter/mask (to achieve a group of devices with number = 1, all or certain value)

- A initial Q value for Q-algorithm-like scheme which is a general time-domain anti-collision scheme in RFID

- The reader identifier

- TDM/FDM/CDM resources for random access (FFS and rely on output of discussion for RA aspect)

Proposal 7b: RAN2 can consider to introduce subsequent DL message and include the following information in it for an air interface inventory procedure:

- Cast type

- Indication for reducing the slot counter for Q-algorithm-like scheme

- The updated Q value for Q-algorithm-like scheme

2.8 Assistance information for reader/base station

- 2.8.1 Assistance information from core network

In previous RAN2 meetings, besides the discussion on the contents in the initial trigger message for the air interface inventory procedure, there is also some discussion on what information needs to be visible to the reader and therefore useful for AS procedure. In other word, such information would give assistance for the reader to perform the signalling procedures in A-Uu interface. The following assistance information has been mentioned in the companies’ contributions:

- Whether single or multiple devices are triggered, or if multiple devices are triggered, the (rough) number of devices to be triggered/paged/inventoried: such information may assist the reader to choose reasonable Q value or allocate suitable random access resources.

- Service type or service request indication (e.g., inventory, command, or detailed command type): different service type may correspond to different service response from the device.

Based on the given justifications, we have sympathy with the above information which needs to be visible to the gNB reader.

For service type indication, we further see the needs as different service type may have different requirements on the DL/UL resources, e.g., "read" command may need large uplink resources allocation while "write" command may need large downlink resources allocation. Therefore, making the reader aware of the detailed type of AIoT services can be beneficial for more reasonable A-Uu resources allocation. For TP2, since the A-Uu interface resources for AIoT service need to be allocated by the base station [5], it’s also beneficial to let the base station be aware of the service type information. So, for both TP1 and TP2, it’s beneficial to let the base station be aware of the AIoT service type information.

Moreover, we also see the need for exposure another information to the base station, e.g., device ID or device ID list of the AIoT device(s) to be inventoried. For TP1, the main usage of letting the base station be aware of the device ID or device ID list of the AIoT device(s) is for delta process. If all device ID (s) are provided to the AIoT devices transparently through the gNB reader and then the inventory responses are transmitted to the core network also transparently through the gNB reader (here we still assume a rough number of devices to be inventoried can be provided to the base station so that the base station can allocate sufficient A-Uu resources), the failed inventories can only be known by core network and only the core network can initiate re-inventory for those failed devices. Such process obviously will be very inefficient and cause waste of resources on end-to-end links. Instead, if gNB reader can be aware of device ID or device ID list of the AIoT device(s) to be inventoried, the gNB reader can directly match the received inventory responses with the device ID (list) and efficiently trigger A-Uu interface re-inventory only for the failed AIoT devices. For TP2, the node that can do the check on inventory responses and trigger re-inventory for the failed AIoT devices still needs to be the base station instead of UE reader since the A-Uu interface resources for re-inventory need to be allocated by the base station.

Observation 4a: If base station can be aware of device ID or device ID list of the AIoT device(s) to be inventoried, the base station can directly match the received inventory responses with the device ID (list) and efficiently trigger A-Uu interface re-inventory only for the failed AIoT devices.

Moreover, there is an additional benefit of exposing device ID or device ID list to the base station in TP2. If the current AIoT service is command service, it’s beneficial to let base station check the inventory responses firstly and the base station will send to the UE reader only the command data for the AIoT device(s) which successfully feedback the inventory response.

Observation 4b: For TP2, there is an additional benefit of exposing device ID or device ID list to the base station. If the current AIoT service is command, the base station can check the inventory responses firstly and send to the UE reader only the command data for the AIoT device(s) which successfully feedback the inventory response.

In a summary, for both TP1 and TP2, it’s beneficial to let the base station be aware of the device ID or device ID list of the AIoT device(s) to be inventoried.

Please note, if the device ID or device ID list of the AIoT device(s) to be inventoried can be visible to the base station, the number of devices can be deduced and no need of another explicit number.

Proposal 8a: For both TP1 and TP2, the following information needs to be visible to the base station to facilitate the base station to allocate suitable A-Uu interface resources and/or trigger appropriate A-Uu interface and Uu interface procedures:

- Identity info of device(s) and/or selection criteria

- AIoT service type information (e.g., inventory, command, or detailed command type)

The above information mainly needs to be provided to the base station by core network node, so RAN2 needs to inform RAN3 the need.

Proposal 8b: Send LS to RAN3 to indicate that RAN2 see the need to make the following information be visible to the base station: Identity info of device(s) and/or selection criteria; AIoT service type information.

- 2.8.2 Information from device

In previous RAN1 and RAN2 meeting, there was discussion that the AIoT device may be unavailable due to fully charging by energy harvesting may need some kind of long time, e.g., up to several tens of seconds.

From RAN2 perspective, as we assume the device can harvest power energy while receiving the DL messages/signalling, we think that the DL reception at the device will generally not be an issue. The only exceptional case (maybe rare case) may be that the device does not receive any DL signal for a long time and the stored power is exhausted.

But for the UL, as the device may need to send kind of long device ID or even large service data, e.g., its sensor data report according to the “read” command, the stored power energy may not be able to support the device to complete the UL transmission. In such case, it may be beneficial to let the device provide its power status information to the reader, e.g., via any UL message (e.g., Msg3) during random access and the following procedures. If the reader can be aware of the power status of one or more devices, the reader may choose to skip or delay the inventory/command service procedures to these devices.

Generally, such power status information can be the remaining power, or the transmission duration/amount of data that can be supported by the current power, or the required time it takes to charge to a sufficient power etc.

Proposal 8c: It’s suggested that a device can report its power status to the gNB/UE reader so that the reader can reasonably schedule the device during the air interface inventory procedure.

2.9 General steps for air interface inventory

Based on the above discussion, a figure for general steps for a basic paging-like inventory procedure can be as below which can be further discussed in RAN2.

Proposal 9: It’s suggested RAN2 to discuss following figure for general steps for an air interface inventory procedure.

Figure 1: Air interface inventory procedure

- Conclusion

According to above discussion for paging-like inventory procedure in air interface, the following proposals are given:

#1: General procedure for “command only” use case

Proposal 1a: At least from reader perspective, e.g., on the A-Uu interface between reader and AIoT device , it’s suggested to use baseline procedure for “inventory and command” (i.e. first triggers inventory procedure and then sends command) for the “Command only” use case.

Proposal 1b: Send LS to RAN3 to indicate that for “inventory and command” and “Command only” use cases, RAN2 suggest that the core network can send the acquired command service data to the base station as soon as possible, e.g., before the base station triggers the inventory procedure.

#2: Paging/DL message monitoring

Proposal 2a: RAN2 assume a cast type, e.g., unicast or broadcast needs to be introduced for DL transmission on PRDCH channel to help a device to distinguish whether a DL message over R2D transmission is for multiple devices or just one certain device.

Proposal 2b: The status of “has not been inventoried” and/or “has been inventoried” may need to be introduced for the device. The device with different status can monitor DL messages over R2D transmission with different cast type.

#3: Consideration on Device ID

Proposal 3a: From RAN2 perspective, both multiple IDs and group ID can be considered. The multiple IDs can be used to indicate multiple devices with discrete ID numbers, while group ID can be used to efficiently indicate multiple devices within a pre-defined group.

Proposal 3b: RAN2 is suggested to have brief discussion on the type of AIoT device ID, e.g., permanent long ID vs temporary short ID. RAN2 can give our general considerations or preferences and indicate it to SA2.

#4: Filter/mask scheme for air interface inventory

Proposal 4: It is also necessary for the RAN to apply the mask/filter mechanism with intention to reasonably split the inventory for a large number of devices from the upper layer into several air interface inventories, each for a smaller number of devices.

#5: Q-algorithm for air interface inventory

Proposal 5a: It can be considered to apply kind of anti-collision scheme similar as Q-algorithm in RFID to AIoT.

Proposal 5b: If kind of anti-collision scheme similar as Q-algorithm in RFID is applied, besides the device ID and mask information, the paging message also needs to provide an initial Q value.

#6: More appropriate inventory area for the devices

Proposal 6a: A concept of inventory area can be introduced with intention that an inventory is mainly performed for the devices within a certain inventory area.

Proposal 6b: It can be assumed that an AIoT inventory area for Topology 1 may include one or more BS readers. The association between a certain device with a BS reader can be set up by network node, e.g., BS reader or CN node. From RAN perspective, one base station (BS reader) can be associated with multiple AIoT devices.

Proposal 6c: It can be assumed that an AIoT inventory area for Topology 2 may include one or more UE readers. The association between a certain device with the UE reader and further with the attached base station can also be set up by network node, e.g., base station or CN node. From RAN perspective, one base station can be associated with multiple UE readers and each of UE reader can be associated with multiple AIoT devices.

Proposal 6d: The setting up association between a certain device with the reader (and may further with the attached base station) can be done during inventory procedure.

Proposal 6e: RAN2 can further discuss whether the device is capable to record the BS reader or UE reader where it was successfully inventoried in last time. If yes, the reader identifier may need to be provided to the device in the paging message of inventory procedure.

#7: Contents in the paging message

Proposal 7a: RAN2 can consider to include the following information in the initial paging message for an air interface inventory procedure:

- Cast type (FFS in L1 control information or in MAC layer)

- Device ID information

- Filter/mask (to achieve a group of devices with number = 1, all or certain value)

- A initial Q value for Q-algorithm-like scheme which is a general time-domain anti-collision scheme in RFID

- The reader identifier

- TDM/FDM/CDM resources for random access (FFS and rely on output of discussion for RA aspect)

Proposal 7b: RAN2 can consider to introduce subsequent DL message and include the following information in it for an air interface inventory procedure:

- Cast type

- Indication for reducing the slot counter for Q-algorithm-like scheme

- The updated Q value for Q-algorithm-like scheme

#8: Assistance information for reader/base station

Proposal 8a: For both TP1 and TP2, the following information needs to be visible to the base station to facilitate the base station to allocate suitable A-Uu interface resources and/or trigger appropriate A-Uu interface and Uu interface procedures:

- Identity info of device(s) and/or selection criteria

- AIoT service type information (e.g., inventory, command, or detailed command type)

Proposal 8b: Send LS to RAN3 to indicate that RAN2 see the need to make the following information be visible to the base station: Identity info of device(s) and/or selection criteria; AIoT service type information.

Proposal 8c: It’s suggested that a device can report its power status to the gNB/UE reader so that the reader can reasonably schedule the device during the air interface inventory procedure.

#9: General steps for air interface inventory

Proposal 9: It’s suggested RAN2 to discuss following figure for general steps for an air interface inventory procedure.

- References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#102

- TR 22.840 V19.0.0: "Study on Ambient power-enabled Internet of Things", 3GPP SA1, Dec. 2023

- TR 38.848 v18.0.0: "Study on Ambient IoT (Internet of Things) in RAN (Release 18)", RAN#101, Sep. 2023

- TR 23.700-13 V0.4.0 (2024-06), Study on Architecture support of Ambient power-enabled Internet of Things, SA2#162, April. 2024

- R2-2407151, Consideration on TP2 for AIoT, ZTE Corporation, RAN2#127, August 2024

- Annex