3GPP TSG-RAN WG2 Meeting #127	R2-2407151

Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item:	8.2.5

Source:	ZTE Corporation, Sanechips

Title:	Considerations on TP2 for Ambient IoT

Document for:	Discussion and Decision

Introduction

In RAN#102, a study item on solutions for Ambient IoT (Internet of Things) in NR has been agreed and in RAN#103, the SID is further refined in [1].

In RAN2#125bis meeting, RAN2 has had initial discussion for Ambient IoT. The following agreements related to general aspect and protocol stacks have been achieved:

Agreements (General aspect) - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. - RAN2 will support two use cases, “inventory” and “command”. The definition, detailed wording is FFS - Baseline procedure: Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS Step C: The device may perform the data communication with the reader as needed,: Details FFS - We will study the support of both “inventory” and “command” in the same procedure. - FFS if Initial Trigger Message can also include “command”. - RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input. Agreements (Control Plane protocol) - RRC connection management is not supported. FFS how the resource configuration is provided to the device (if needed based on RAN1 progress) - RRM L3 measurement reporting is not supported by Ambient IoT devices. - RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding. - Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals. - RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC Agreements (User Plane protocol) - SDAP is not supported for UP protocol stack. - PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities. - RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. - No HARQ and RLC AM - FFS about the level of visibility required by the reader and what information is necessary for AS layer operations. - RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2 Agreements (Paging) - Legacy paging message for device will not be supported. - Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well). - RAN2 assumes that the device will not support tracking/RAN area update procedure. - For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2

In RAN2#126 meeting, RAN2 has further achieved the following agreements:

Agreements (Stage 2 General aspects) - As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. - From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command) Agreements (Functionality aspects) - Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. - legacy NR BSR/SR is not needed for A-IoT communication. - FFS whether further indication of device message size/status is needed - AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed) Agreements (A-IoT Paging) - RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. - From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details. Agreements (A-IoT Random Access) Agreements on “4 step” RA - A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements - A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) - The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. - “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access. Agreements on 2 step CB RA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In RAN2 previous discussion, companies focus on the general aspects of Ambient IoT and the basic procedures for different AIoT services. More of the discussion assumes the basic Topology, e.g., Topology 1.

In this paper, we will give initial analysis on the Topology 2 and give our thoughts for the TP2 related aspects, especially for the interface between the gNB and UE reader.

Discussion

  - General considerations

The scope of the RAN WG SID has indicated that “Deployment scenario 1 with Topology 1” (e.g., Topology 1 with indoor Device and indoor BS) and “Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control” (e.g., Topology 2 with indoor Device, outdoor BS and intermediate UE) will be studied. Topology 1 and Topology 2 will be referred to as TP1 and TP2 respectively hereinafter.

On top of the study for both Topology 1 and Topology 2, we assume that the different topologies can be transparent to the AIoT device since RAN2 has agreed the aim that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2. Therefore, in this paper, we will focus on identifying issues on the (legacy) Uu interface between the UE Reader and the base station for supporting TP2 and the potential enhancements that are necessary. Unless specific issue is identified, we will not discuss enhancements to the AIoT air interface between AIoT device and the UE reader (referred to as A-Uu hereinafter) in the context of separate discussion of TP2.

For TP2, besides reusing the new A-Uu interface which is under discussion for TP1, we firstly assume that the interface between UE reader and the base station should try to leverage legacy 5G NR air interface, e.g., Uu interface. Furthermore, according to the SID, the additional network control on the UE reader needs to be studied.

Proposal 1: For TP2, the interface between the UE reader and the base station should try to leverage legacy Uu interface, with additional network control provision.

  - NAS(-like) layer for both TP1 and TP2

According to the progress in RAN2 study, a simplified and compacted protocol stack for the A-Uu interface between AIoT device and the Reader will be designed. In such protocol stack, at least physical layer and MAC layer are involved. Meanwhile, RAN2 has agreed to assume no support of AS security until SA3 provides further requirements.

As mentioned in the AIoT requirements study in SA1 [2, TR 22.840], generally, the 5G system is required to support operator-controlled alternative authentication methods (i.e. alternative to AKA) with different types of credentials for network access for IoT devices in isolated deployment scenarios (e.g., for industrial automation) [5, TS 22.261], and for AIoT, the alternative authentication method should be light-weight in order make sure that the cost of power consumption caused by the authentication is strictly controlled.

Moreover, as mentioned in SA1 requirements study [2, TR 22.840], one of the consolidated requirements for security and privacy for AIoT is to protect the privacy of information (e.g., location and identity) exchanged during communication between an Ambient IoT device and the 5G network or an Ambient IoT capable UE. Considering that in the description of many use cases, it mentions that mechanisms are needed to prevent the services data of Ambient IoT device from being obtained by other untrusted or unauthorized user, or energy efficient security mechanisms for Ambient IoT devices can include authentication, encryption and data integrity, we understand that, under the strict constraints for the AIoT devices such as weak harvested power and very low complexity, the security we mainly need to implement for AIoT is to protect the security and privacy of the data of the items to which the AIoT device is attached. In other word, we assume the end-to-end protection of Commands (e.g., read, write, kill) and/or the responses to the Commands (e.g., the information stored in the Ambient IoT devices) would be the main security and privacy we need to achieve.

Observation1a: RAN2 assumes the end-to-end protection of the service data of the AIoT devices (e.g., Commands (read, write, disable, kill etc.) and/or the responses to the Command) would be the main security and privacy we need to achieve for AIoT devices.

Meanwhile, from the perspective of RAN2, we understand that another AIoT service, e.g., the inventory is mainly to check the presence or absence of the AIoT devices in the concerned area. As response to the inventory request, the AIoT devices send the identification information to the 5G core network/AF and complete the authentication procedure. Furthermore, 5G core network/AF can trigger other services (e.g., Command) to the AIoT devices that are confirmed to exist and belong to the network/AF. Such process can also help to avoid security risk of sending Command service directly. Based on such understanding, we don’t think that it is necessary to encrypt the identity information (e.g., device ID) of the AIoT devices.

Observation1b: RAN2 assumes the end-to-end protection of the identity information (e.g., device ID) of the AIoT devices is not necessary.

According to the above Observation1a and Observation1b, we see the necessity of an upper layer to provide security for the service data of the AIoT devices. With reference to the legacy LPWA technology, e.g., CP solution for NB-IoT/eMTC, it can be seen that if light-weight security is needed for protecting the IoT data, to only support NAS (-like) layer security is enough. Therefore, it’s also recommended to support NAS (-like) layer security to provide end-to-end protection within 5G network for the service data of the AIoT devices. This makes it feasible to reuse legacy IoT security mechanisms to the extent and potentially reduce the standardization work for AIoT. An example of protocol stack for TP1 is as below. It’s easy to understand that such upper layer protocol design can also be applicable to TP2.

Figure 1. Example of protocol stack with AIoT NAS layer for TP1

Proposal 2a: From RAN2 perspective, for both TP1 and TP2, NAS (-like) layer security can be considered to provide end-to-end protection of the AIoT service data for the AIoT devices (e.g., Commands and/or responses to the Commands) within 5G network. An AIoT NAS layer which can provide NAS (-like) layer security can be introduced.

From RAN2 perspective, we mainly need to ensure that AS layer protocols can support the transmission of AIoT NAS layer data.

Proposal 2b: AS layer protocols need to support the transmission of AIoT NAS layer data.

  - AIoT data transmission to/from UE Reader

From RAN2 perspective, one of the main issues for TP2 is how to transfer the DL AIoT service data from core network to the UE reader in order to facilitate UE reader to trigger the AIoT services for AIoT devices in the A-Uu interface, and how the UE reader deliver the received UL AIoT service data from AIoT devices to the core network.

In the previous discussion in SA2[4, TR 23.700-13] and RAN3[6, R3-243191], there is already much discussion on the architecture aspects and protocol stack for Ambient IoT for TP2, and also on the delivery of AIoT data. There are mainly two types of schemes, one can be seen as Control Plane-based solution (e.g., Solution 5, Solution 8, Solution 17, Solution 21, Solution 30 in [4]) while the other can be seen as User Plane-based solution (e.g., Solution 3, Solution 22, Solution 32 in [4]).

One of the User Plane-based solutions is Solution 3 (Alt1) mentioned in [4]. The example protocol stack for TP2 given in this solution is as below:

In such solution, UE reader establishes a PDU session to send AIoT service data as a GTP-U data to UPF which forwards it to the AIoT aware CN node. The same encapsulation and transmission is performed in the DL direction. In this solution, the AIoT service delivery between AIoT devices and core network can be transparent to the base station.

One of the Control Plane-based solutions is Solution 21 (Alt2) mentioned in [4, TR 23.700-13]. The example protocol stack given in this solution is as below:

In such solution, UE reader uses the NAS between UE reader and AMF and further interface between AMF and AIoT CN node to send AIoT service data (which is already encapsulated in AIoT NAS PDU) to the AIoT aware CN node. The same encapsulation and transmission is performed in the DL direction. Per our understanding, the AIoT service data (encapsulated in AIoT NAS PDU) would be further encapsulated in the legacy NAS PDU between UE reader and AMF.

There is a variant of Control Plane-based solution which can be seen in Solution 33 (Alt3) in [4]. The example protocol stack for TP2 given in this solution is as below:

Per our understanding, this solution is also a Control Plane-based solution. The main point in this solution which is different from that in the NAS-based solution 21 is that AIoT service data (which is already encapsulated in AIoT NAS PDU) could be directly delivered via the RRC signaling between UE reader and the base station.

From RAN2 perspective, we have a brief comparison among the two type schemes as below:

- Feasibility for assisting base station to allocate A-Uu interface resources: As discussed in the following section 2.4, in the context of 3GPP and with licensed spectrum applied, in order to avoid the radio interferences among different UE readers caused by the AIoT services in the A-Uu interfaces, it’s assumed the A-Uu interface resources between UE reader and AIoT devices should be allocated/controlled by the base station. In order to facilitate the base station to allocate the A-Uu interface resources for gNB reader itself or for UE readers, some information needs to be visible to the base station. However, for UP-based solution (Alt1), all the AIoT service information, e.g., the device ID(s) and/or command services data would be encapsulated in the user plane data and cannot be visible to the base station. On the contrary, for CP-based solution (Alt2/3), if needed and allowed (e.g., observation 1b can be common understanding), it would be feasible to explicitly provide the device ID(s) and/or service type information from the core network to the base station and can be further provided to UE reader and AIoT devices. For the command service data, it can be encapsulated in AIoT NAS PDU with NAS layer security and delivered between AIoT devices and core network. Such delivery can be transparent (or contents can be invisible) to the base station and UE readers. Shortly to say, CP-based solution (Alt2/3) is better for achieving a trade-off between AIoT service data security and exposing some information to assist in the operation of reader.

- Efficient command data delivery to UE reader: For UP-based solution (Alt1), the device ID(s) and/or the corresponding command services data are generally encapsulated together in GTP-U data and directly provided to UE reader. it can be under the control of UE reader to firstly trigger inventory and then transmit command data. However, considering that the concerned AIoT devices may not be in the coverage of a UE Reader (e.g., the inventory may be failed), directly providing both device ID (s) and commands to the UE Reader may cause unnecessary resource waste on legacy Uu interface between UE Reader and the base station. Instead, the better way may be to let the base station hold the encrypted command data till it gets inventory results from UE reader/AIoT devices. It’s easy to see CP-based solution (Alt2/3) is more feasible to support such step-by-step inventory and command delivery procedures.

- More suitable association between UE reader and  AIoT devices: Although we generally assume the core network can provide device ID(s) and/or the corresponding command service data to the suitable UE reader according to the association among the AIoT device(s) and network nodes established previously, we still see the possibility that the base station may have more accurate information about the association between UE reader and some AIoT devices. However, for UP-based solution (Alt1), it is almost impossible for the base station to update the list of devices sent to a UE reader. On the contrary, it’s feasible for CP-based solution (Alt2/3) to adjust or even re-assign the list of devices for certain UE reader on top of the assistance information acquired from core network.

- For the alternatives in CP-based solution, we see NAS-involved scheme, e.g., Alt2 may be more in line with the existing Control Plane 5GS Optimization for CIoT mechanism for LPWA, but it seems to cause redundant NAS layer encapsulation. Meanwhile, RRC-involved scheme, e.g., Alt3, may have simple NAS encapsulation, but may involve more signaling transition between different protocol layers.

Observation 2: Based on the above comparison in aspects such as feasibility for assisting base station to allocate A-Uu interface resources, efficient command data delivery to UE reader, more suitable association between UE reader and  AIoT devices, it can be seen that CP-based solution (Alt2/3) outperforms UP-based solution (Alt1).

Therefore, the following proposals are given:

Proposal 3a: Control plane-based solution should be adopted for AIoT service data transmission between UE reader and core network for TP2.

Proposal 3b: Send LS to RAN3 and SA2 to indicate RAN2’s preference on control plane-based solution.

Proposal 3c: There are some small differences between the two alternatives (NAS-involved scheme vs RRC-involved scheme) in control plane-based solution. Both of them have pros and cons. How to choose between these two alternatives can be decided later with coordination between RAN2 and RAN3.

  - Assistance information for UE reader

For TP1, RAN2 has had some initial discussion on the Paging-like Inventory procedure and the contents which need to be sent by the Reader to the AIoT device(s) via initial trigger message during Inventory procedure. Although there are not many relevant agreements yet, the following information has already been mentioned in some previous contributions. Most of them are also mentioned in “2.7 Contents in the paging message” in our contribution [7]:

- Identity info of device(s): e.g., device ID or device ID list of the AIoT device(s)

- Selection criteria: used to indicate which AIoT Device(s) should reply to the inventory request, e.g., by comparing the Mask/Filer/Group information with the device ID owned by the AIoT device(s):

  - the upper layer Mask/Filter/Group information, which is received from the CN.

  - It may consist of device ID with mask or particular address and the corresponding data stored in A-IoT device’s memory.

  - Group information: a) Group information based on particular provider or operator, or particular product type, which may be a static grouping information; b) Group information decided by NAS layer; c) Group information based on paging

- Radio resources information or random access control info (if RACH is needed):

  - scheduling information to indicate the frequency/time resource for the AIoT device to respond;

  - information for access occasion, e.g. Q and N;

  - information about the size of the RN to be generated by the devices;

  - counter or backoff indication to distribute AIoT devices in different slots to trigger the RACH procedure

- Other control info:

  - information about the AIoT service being sought for (e.g., inventory-only, command-only, inventory-and-command)

  - Paging cause

- NAS data: e.g., Command

According to the above Proposal 3a~ Proposal 3b, control plane-based solution is preferred for delivering AIoT service data to or from AIoT devices, we confirm that all the above information cannot be provided directly to the AIoT devices by the core network and is transparent to the reader, instead, it should be provided to the AIoT device BY the reader. This is also aligned with the previous RAN2 agreement that common design for the A-Uu interface between AIoT device and the Reader for both TP1 and TP2 is assumed.

For TP1, the above information can either be acquired by gNB reader from core network (identity info of device(s), selection criteria etc.) or determined by gNB reader itself (e.g., selection criteria, radio resource assignment etc.). However, for TP2, we need further discuss how the UE reader can acquire such information. Some considerations are as below:

- For TP2, we assume that the UE reader can be a regular UE but is capable of perform AIoT services. In order to avoid radio interference among different UE readers and for a certain UE reader, also avoid interference between UE's own 5G services and A-Uu services, the A-Uu interface resources for a UE reader to perform AIoT services (e.g., the radio resources information or random access control info mentioned above) should be allocated by the base station. We think this is straightforward and also very important in the context of 3GPP (licensed spectrum-based system)

- The identity info of device(s) which need to be inventoried by a UE reader also needs to be informed by the base station. Although generally which device(s) will be inventoried by a UE reader can be determined by the core network based the previously established association among AIoT devices, UE readers and the base station] and  transparently transmitted to the UE reader by the base station, we still see the possible use cases that base station may not totally follow the indication from core network and can update or determine by itself the AIoT device (list) which are sent to a UE reader, e.g., when the base station has more information about the location or status of a certain UE reader.

- For TP1, we assume that if the current AIoT service is command service, it’s beneficial to provide the command data to the gNB reader as early as possible in order that gNB can send such command data to a AIoT device as soon as possible when it successfully responses the inventory. However, for TP2, we see the disadvantages to send command data to UE reader early. Different from TP1, to provide command data of AIoT device(s) to a UE reader would consume the resources in legacy Uu interface. If some of the AIoT devices cannot be successfully inventoried by this UE reader, such resources occupation in legacy Uu interface would be waste. Instead, it’s better for the base station to hold the command data and firstly only trigger the inventory procedure. Moreover, the UE reader needs to send back the inventory response from AIoT device(s) to the base station. The base station will send to the UE Reader only the command data/command data list of the AIoT device(s) which feedback the inventory response.

- Please note, according to our proposals in section 2.2, the command data/command data list can be encrypted in AIoT NAS layer, even if the base station provides command data/command data list of some AIoT device(s) to the UE reader, it may be encapsulated in a container for AIoT NAS PDU which can be transparently transmitted to AIoT devices by the UE reader. Meanwhile, the correspondence between the command data(s) and the AIoT device(s) needs to be provided to UE reader.

Proposal 4a: For TP2, new content IE(s) for facilitating UE reader to perform AIoT service in A-Uu interface can be introduced in the legacy Uu interface between the base station and UE reader. The following information can be considered to provide via the new content IE(s):

- Identity info of device(s) and/or selection criteria;

- Radio resource assignment for A-Uu interface for performing AIoT service

- The encrypted command data can be optionally provided and if provided, the correspondence between the command data(s) and the AIoT device(s) also needs to be indicated.

Furthermore, for radio resource assignment for A-Uu interface, the following two alternatives can be considered:

- It can be provided to a connected UE reader via dedicated RRC signaling;

- It's also possible to configure radio resource assignment for A-Uu interface via SIB signaling. That means common A-Uu resources are provided. Later for a certain UE reader, the base station can indicate which parts of the common A-Uu resources can be used, e.g., via dedicated signaling.

Proposal 4b: The radio resource assignment for A-Uu interface can be provided via dedicated signaling or SIB. If common A-Uu resources is provide via SIB, for a certain UE reader, the base station can further indicate which parts of the common A-Uu resources can be used, e.g., via dedicated signaling.

Other assistance information which mainly need to be visible to the base station has been discussed in the [7] and some of them are used by the base station to more suitably allocate resources for the UE reader.

  - Authorization/de-authorization of UE Reader

In [4, TR 23.700-13], there are descriptions in some solutions that a legacy UE needs to be authorized and then can perform AIoT services. Consideration UE needs additional air interface resources to perform AIoT services, we can agree that legacy UE cannot directly perform AIoT services and authorization would be needed.

One straightforward way can be that, UE is authorized/de-authorized to perform as UE reader by core network during registration/de-registration procedure. Furthermore, such Authorization/de-authorization should be based on UE relevant capability. Whether there are other scenarios for UE to be authorized/de-authorized as UE reader can be discussed later.

Proposal 5: From RAN2 perspective, the UE could report its capability of supporting AIoT services to core network and the core network can determine whether the UE can be authorized to act as a UE Reader during the registration procedure.

  - Control on A-Uu resources after UE reader is disconnected

In [4, TR 23.700-13], in solution 4, there is an assumption that in Topology2, when the UE is out of Uu coverage in some area but the AIoT air coverage is still good, the AIoT operation at the UE reader will not stop and continue.

From RAN2 perspective, there may be issue if UE Reader keeps using the previously allocated A-Uu resource (on licensed spectrum) to continue the AIoT services. As the base station may detect disconnection of the UE Reader (e.g., UE_A), release the resources allocated to it, and further allocate the resources to other UE or UE reader (e.g., UE_B), if the UE_A continues to use the previously allocated resources, it is likely to cause interference between each other, causing both UE_A and UE_B to fail to operate properly. Therefore, some coordination/control may be needed on the usage of the A-Uu resources after UE reader is disconnected.

A simple way may be that the base station can further give some indications to the UE Reader when allocating A-Uu interface resources, for example, such indication can indicate whether the previously allocated A-Uu resources can be used after UE Reader disconnects from the base station, or can indicate a time period in which UE Reader can keep using this previously allocated A-Uu resources.

Proposal 6: RAN2 discuss whether A-Uu resources can be continued to be used after a UE reader disconnects from the base station. If yes, RAN2 further discuss whether the base station needs to control this process and how.

  - Example for AIoT service procedure for Topology 2

Based on above discussion, an example for AIoT service procedure for Topology 2 is as following:

Figure 2: AIoT service procedure for Topology 2

Conclusion

This contribution gives the following observations and proposals for supporting TP2 for Ambient IoT:

Observation1a: RAN2 assumes the end-to-end protection of the service data of the AIoT devices (e.g., Commands (read, write, disable, kill etc.) and/or the responses to the Command) would be the main security and privacy we need to achieve for AIoT devices.

Observation1b: RAN2 assumes the end-to-end protection of the identity information (e.g., device ID) of the AIoT devices is not necessary.

Observation 2: Based on the above comparison in aspects such as feasibility for assisting base station to allocate A-Uu interface resources, efficient command data delivery to UE reader, more suitable association between UE reader and  AIoT devices, it can be seen that CP-based solution (Alt2/3) outperforms UP-based solution (Alt1).

Proposal 1: For TP2, the interface between the UE reader and the base station should try to leverage legacy Uu interface, with additional network control provision.

Proposal 2a: From RAN2 perspective, for both TP1 and TP2, NAS (-like) layer security can be considered to provide end-to-end protection of the AIoT service data for the AIoT devices (e.g., Commands and/or responses to the Commands) within 5G network. An AIoT NAS layer which can provide NAS (-like) layer security can be introduced.

Proposal 2b: AS layer protocols need to support the transmission of AIoT NAS layer data.

Proposal 3a: Control plane-based solution should be adopted for AIoT service data transmission between UE reader and core network for TP2.

Proposal 3b: Send LS to RAN3 and SA2 to indicate RAN2’s preference on control plane-based solution.

Proposal 3c: There are some small differences between the two alternatives (NAS-involved scheme vs RRC-involved scheme) in control plane-based solution. Both of them have pros and cons. How to choose between these two alternatives can be decided later with coordination between RAN2 and RAN3.

Proposal 4a: For TP2, new content IE(s) for facilitating UE reader to perform AIoT service in A-Uu interface can be introduced in the legacy Uu interface between the base station and UE reader. The following information can be considered to provide via the new content IE(s):

- Identity info of device(s) and/or selection criteria;

- Radio resource assignment for A-Uu interface for performing AIoT service

- The encrypted command data can be optionally provided and if provided, the correspondence between the command data(s) and the AIoT device(s) also needs to be indicated.

Proposal 4b: The radio resource assignment for A-Uu interface can be provided via dedicated signaling or SIB. If common A-Uu resources is provide via SIB, for a certain UE reader, the base station can further indicate which parts of the common A-Uu resources can be used, e.g., via dedicated signaling.

Proposal 5: From RAN2 perspective, the UE could report its capability of supporting AIoT services to core network and the core network can determine whether the UE can be authorized to act as a UE Reader during the registration procedure.

Proposal 6: RAN2 discuss whether A-Uu resources can be continued to be used after a UE reader disconnects from the base station. If yes, RAN2 further discuss whether the base station needs to control this process and how.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, RAN#103, March, 2024

[2] 3GPP TR 22.840 V19.0.0: "Study on Ambient power-enabled Internet of Things", 3GPP SA1, Dec. 2023

[3] 3GPP TR 38.848 v18.0.0: "Study on Ambient IoT (Internet of Things) in RAN (Release 18)", RAN#101, Sep. 2023

[4] 3GPP TR 23.700-13 V0.4.0 (2024-06): "Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)"

[5] 3GPP TS 22.261: "Service requirements for the 5G system".

[6] 3GPP R3-243191, Architecture aspects and Protocol Stack for AIoT, RAN3#116, May 2024

[7] R2-2407148, Further consideration on paging procedure for AIoT, RAN2#127, August 2024