3GPP TSG-RAN WG2 Meeting #127	R2-2407152

Maastricht, Netherlands, 19th – 23th August, 2024                       Revision of R2-2404882

Agenda Item:	8.9.2

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on S&F operation in IoT NTN

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed and in RAN#103, the WID is further refined in [1]. The main objectives related to support of Store&Forward (S&F) satellite operation are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#125bis meeting, RAN2 has had some high level discussion on this topic and made the following assumptions:

RAN2 assumptions: - S&F implies that at least the full eNB will be onboard - An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this) - The S&F satellite operation is common for NB-IoT and eMTC. - The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

In RAN2#126 meeting, RAN2 has made more agreements for this objective as following:

1. For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): - The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. - When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. - The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). - The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again 2. S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed 3. RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In this paper, we will give further analysis on this objective and try to identify the potential RAN2 impacts for supporting Store&Forward (S&F) satellite operation in IoT NTN.

Discussion

  - General aspects

In SA2 Phase 3 study for satellite network [2], more than 20 solutions have been proposed to address “Key Issue #2: Support of Store and Forward Satellite operation”. Per our understanding, there are generally two classes of solutions, one solution class aims at the deployment scenario with only eNB onboard the satellite, the other one aims at the deployment scenario with whole or partial core network functionalities onboard the satellite. Which solution class will be selected is still under SA2 discussion and no agreements yet. So in the following sections, we will discuss the potential RAN2 impacts of the solution class for each deployment scenario respectively.

In previous RAN2 meeting, there was some discussion about whether to support single-satellite or multi-satellite deployments. For this aspect, as mentioned in[2], “SA2#159 has discussed that, it is important that the S&F Satellite operation mode can properly support multi-satellite deployment scenarios by allowing a UE to be potentially served by multiple satellites, i.e. not restricting the S&F operation for a UE to a single satellite within the constellation. Multi-satellite support is indeed considered critical to be able to reduce the latencies in data transfer delivery, given that the revisit time of a single satellite may be around 12h or longer for typical orbits”.

From RAN2 perspective, we agree multi-satellite deployment should be supported for the sake of reducing latency. In other word, we prefer that the proposed solution can be applicable to both case of using single satellite and case of using multiple satellites. The network node can determine whether to split the data transmission to multiple satellites based on its knowledge on constellation.

Proposal 1: RAN2 assume the proposed solution for supporting S&F Satellite operation can be applicable to both case of using single satellite and case of using multiple satellites.

  - S&F for multiple satellites deployment with only eNB-onboard

In last meeting, RAN2 prioritize the discussion on the solution for deployment scenario with only eNB onboard the satellite. A general procedure has been agreed for accessing such satellite network with non-concurrent service link and feeder link transmission:

- (Step1)The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

- (Step2)When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

- (Step3)The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

- (Step4)The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

    - UL Data transmission

In Step1 of the above general procedure, when an eNB on satellite (for the convenience of following discussion, hereafter it is referred to as eNB-1) moves to a coverage area, it may receive RRC connection request from a UE. If it’s a UE using CP solution, the eNB-1 can firstly complete the air interface procedure (Msg1~Msg5) and get NAS PDU via Msg5 from the UE. Similarly, for UE using UP solution, the eNB-1 can firstly resume (if it has UE context) the air interface and DRB and get UP data from the UE.

Since the eNB-1 has no available feeder link, it needs to store the UL NAS PDU/UP data and maybe some other UE related information (e.g., S-TMSI/Resume ID, the accessed cell ID etc).

In the following, the eNB-1 may move out the coverage area and it has two options to handle the UE:

- Option 1: If it will take long time for eNB-1 to revisit this coverage area, the eNB-1 can choose to release the UE (e.g., Msg5 is followed by a RRC release message).

- Option 2: If it will not take long time for eNB-1 to revisit this coverage area, the eNB-1 can choose to keep the UE in connected state (e.g., no further signaling after Msg5).

  - For UE side, even the UE is still in connected state, it may detect RLF later due to the leaving of the eNB-1. In order to avoid that case, UE can choose to keep in connected state but suspend some AS operations (may similar as the process for GNSS measurements in connected state). By this way, the UE and eNB-1 can keep consistence on the UE state.

It’s easy to understand that which option in above the eNB will take can be left to eNB implementation.

Observation 1: For only eNB onboard the satellite and the eNB using S&F Satellite operation, in step 1, after eNB finishes the air interface establishment/resumption and get NAS PDU or UP data from the UE, it’s possible for eNB to either release the UE or keep the UE in connected mode. This can be left to eNB implementation.

If the UE is kept in connected mode, it’s better for UE to be aware of that the eNB-1 is using S&F Satellite operation, then the UE can decide whether to take some special processes after completing the air interface procedure, e.g., to stay in the connected state but suspend some AS operations. In last meeting, RAN2 has agreed that S&F indication can be provided by SIB and RAN2 assumes that no NAS indication is needed. The details for such indication is still FFS. Based on the above discussion, we prefer to define this indication with meaning of using S&F Satellite operation or availability of the feeder link.

Moreover, sending such indication via SIB, there is another usage, e.g., based on this information, the idle UEs in the coverage of this satellite can choose whether to trigger RRC connection establishment/resumption to this eNB.

Proposal 2: It’s suggested to define the S&F indication in SIB with meaning of “using S&F Satellite operation” or “availability of the feeder link”.

Even generally we assume the eNB-1 can firstly finish the air interface of connection establishment/resumption procedure for the UE, it’s still possible for eNB-1 to reject the RRC connection establishment/resume request if the eNB-1 thinks that it’s not available to provide service for the current area or it’s already overload.

Observation 2: The eNB using S&F Satellite operation may reject the RRC connection establishment/resume request from UE (s). This also can be left to eNB implementation.

By this way, at the current moment we don't see the need to introduce kind of new barring scheme to facilitate eNB with S&F Satellite operation to support more fine-grained access control (e.g., a new access barring scheme that can differentiate the access barring for legacy UEs and R19 IoT NTN UEs). We prefer that the impacts on legacy UE is as small as possible. For example, if a legacy UE has long NAS timer setting, it can be allowed to access the eNB with S&F Satellite operation. If the eNB leave away, the legacy UE can take some processes as that for discontinuous coverage scenario. Moreover, we assume that the R19 IoT NTN UEs supporting S&F operation can also follow the existing accessing barring/rejection scheme.

Proposal 3: It’s unnecessary to introduce new barring scheme to facilitate eNB with S&F Satellite operation to differentiate the access barring for legacy UEs and R19 IoT NTN UEs.

Whether to reject the RRC connection request can be left to eNB implementation, however, in order to avoid the UE initiating another request soon, it’s suggested that eNB-1 can indicates a detailed reject reason to the UE in the RRC reject message. Moreover, if the eNB-1 have enough information about the neighboring satellites, the eNB-1 may recommend other satellite/eNB for UE to access later.

Proposal 4: It’s suggested that, if an eNB rejects UE’s RRC connection request due to using S&F Satellite operation, the eNB can indicate a detailed reject reason to UE in the RRC connection reject message. Moreover, the eNB can give recommendation on other satellite/eNB for UE to access later.

Moreover, for NB-IoT UE using CP solution, in the legacy air interface procedure, after the eNB acquire the UE identifier via Msg3, the eNB can request UE information from the MME (e.g., via Retrieve UE Information/UE Information Transfer function). According to the acquired UE information including QoS Parameters and UE Radio capability, then eNB can provide appropriated configuration in Msg4 that match the capabilities of the UE. This scheme also has the benefit of avoiding the need for UE to report capability via Msg3. However, in the S&F Satellite operation, the eNB has no way to acquire the UE capability from MME during air interface procedure, and one implementation option is to provide kind of default configuration in Msg4 that may downgrade the radio link performance.

Observation 3: For UE using CP solution, in the legacy air interface procedure, eNB can acquire UE Radio capability from MME after Msg3 which can facilitate eNB to provide appropriated configuration in Msg4. However, in the S&F Satellite operation, the eNB has no way to acquire the UE capability from MME after Msg3. One implementation option is to provide kind of default configuration in Msg4 that may downgrade the radio link performance.

If the issue of downgraded radio link performance is thought to be critical, RAN2 can further discuss whether to report some critical UE capabilities via Msg3 is needed.

Proposal 5: RAN2 can discuss whether to report some critical UE capabilities via Msg3 for S&F Satellite operation case if real need is found.

It's also possible for the UE in idle mode to trigger the EDT or PUR procedure to transmit data. Different from normal RRC connection establishment/resumption (in which S1 procedure is triggered after Msg5), in EDT or PUR procedure, even UE’s data is sent in Msg3, the eNB generally sent Msg4 after completing the S1 interface procedure with the intention to also include the potential DL data.

For the S&F Satellite operation case, as there will be no DL data that closely follows the UL data, the eNB can choose to immediately send Msg4 (or maybe Layer 1 ACK for PUR) after reception of Msg3. This can be left to eNB implementation and no additional impacts are seen on air interface signaling and UE process. Buy RAN3 may further check whether more changes are needed for decoupling the air interface procedure and S1 interface procedure.

Observation 4: If the UE in idle mode to trigger the EDT or PUR procedure, the eNB using S&F Satellite operation may need to immediately send Msg4 (or Layer 1 ACK for PUR) after reception of Msg3 (e.g., without triggering S1 procedure). This can be left to eNB implementation and it’s feasible based on RAN2 existing specifications.  Only RAN3 may need to further check whether more changes are needed for decoupling the air interface procedure and S1 interface procedure for EDT or PDU.

Moreover, based on the Proposal 2, if a UE can be aware of whether the eNB is using S&F Satellite operation or not, the UE may be able to make more appropriate decision on whether to trigger EDT/PUR or normal RRC connection establishment/resumption. For example, if UE would expect DL data after UL data, the UE may choose to trigger normal RRC connection in order that the RRC connection can be kept for a while to wait for DL data instead of being released immediately after Msg3.

    - DL Data transmission

When the eNB-1 moves to the area where the feeder link (ground station) is available, the eNB-1 can trigger S1 procedure to setup feeder link with the MME/ground station and furthermore deliver the stored UL data (UL NAS PDU or UL UP data) to core network. From RAN2 perspective, the legacy S1 procedures can be reused as much as possible.

When there is DL data corresponding to the UL data or buffered DL data in MME, generally MME can send the DL data (DL NAS PDU or DL UP data) to the eNB-1 when it receives UL data from eNB-1. However, we can see the possibility that MME may choose another new eNB (can be referred to as eNB-2) and provide the DL data to the eNB-2, for example, if MME can know that eNB-2 will visit the area where the UE locates within short time period, e.g., shorter than the required time length for eNB-1 to revisit the area where the UE locates, MME can send DL data for this UE to eNB-2. No matter which eNB, eNB-1 or eNB-2, is provided the DL data for a certain UE, it needs to store the DL data. Moreover, during the corresponding S1 procedure, this eNB also needs to acquire the relevant UE information, e.g., UE capabilities, and store it together with the data for this UE.

Observation 5: When the source eNB (e.g., eNB-1) moves to the area where the feeder link (ground station) is available, the source eNB can trigger S1 procedure to setup feeder link with the MME/ground station and furthermore deliver the stored UL data to core network. MME/ground station may also deliver the corresponding or buffered DL data to the source eNB. It’s also possible for MME to deliver the corresponding or buffered DL data to a new eNB if MME can know that this new eNB will visit the area where the UE locates within shorter time period.

When the eNB-1 or eNB-2 containing the DL data for the UE (re)visit the area where the UE locates, in more of the cases, the eNB may have no existing connection for this UE, the legacy paging procedure can be used by the eNB to find the UE and deliver the DL data to it. In a few cases, e.g., the source eNB previously keeps the UE in connected mode and revisits the area after a short time period, it may be still possible for the source eNB to use the exiting RRC connection to deliver the DL data to the UE.

Observation 6: When the eNB containing the DL data (re)visit the area where the UE locates, the legacy paging procedure can be used by the eNB to find the UE and deliver the DL data to it. In a few cases, it may be also possible for the source eNB to use the RRC connection which is setup when the eNB previously visited the area to deliver the DL data to the UE.

For DL data delivery, it is easy to figure out a case that when an eNB moves in the coverage of the MME/ground station, the MME/ground station may have stored DL data of several UEs which needs to send to the eNB. It’s also possible that the eNB have sent the stored UL data for several UEs to MME and many of them have corresponding DL data. As a result, the eNB may acquire DL data for several UEs which locate in a same area. Then when next time the eNB (re)visit the area where these UEs locate, the eNB needs to paging several UEs at the same time. Some paging enhancements may be needed to address this case cause by S&F Satellite operation.

Proposal 6: considering that a eNB may acquire DL data for several UEs when it moves in the coverage of the MME/ground station and late need to paging several UEs at the same time when next time it (re)visit the area where these UEs locate, some paging enhancements may be needed to address this case.

    - Processes for connected mode UE

2.2.3.1 The case that feeder link becomes unavailable

For connected mode UE, it may be possible that the feeder link becomes from available to unavailable during a RRC connection. The straightforward process would be that the eNB can proactively release the UE and this can be left to eMB implementation. Furthermore, RAN can discuss whether to introduce a new release reason, e.g., “the feeder link becomes unavailable”, to let the UE be aware of the case and avoid unexpected reconnection.

Observation 7: It may be possible that the feeder link becomes from available to unavailable during a RRC connection. For this case, the eNB implementation can proactively release the UE.

Proposal 7: RAN can discuss whether to introduce a new release reason, e.g., “the feeder link becomes unavailable” for the case that eNB proactively release the UE when feeder link becomes unavailable.

2.2.3.2 The case of RRC connection reestablishment

For a UE in connected mode (e.g., connecting to eNB-0), it may also be possible that RLF occurs due to the RF quality fluctuates and the UE may trigger a RRC connection reestablishment procedure to another base station, e.g., eNB-1 in satellite-1. However, there may be the case that the eNB-1 is with good radio quality (that’s why the UE choose this eNB to request RRC connection reestablishment) but has no available feed link.

Observation 8a: When the RF quality fluctuates, a UE in connected mode may encounter RLF and it may trigger a RRC connection reestablishment procedure to an eNB is with good radio quality but has no available feed link.

For the UE using UP solution, the RRC reestablishment procedure is very similar as RRC connection resume procedure, then the procedures discussed above can be reused. That is, the UE can reestablish the RRC connection with eNB-1 and send UL data to it. The eNB-1 may release the UE when it moves out. Later the UE can get DL data from eNB-1 or another eNB.

However, for the UE using CP solution, as there is additional requirement for RRC connection reestablishment procedure, e.g., MME needs to authenticate the UE's UL CP Security Information in re-establishment request, how to handle the RRC connection reestablishment procedure for CP solution in S&F scenario needs more discussion. For reference, the legacy procedure for RRC connection reestablishment for CP solution is as below:

TS 36.300 19.2.2.30 eNB CP Relocation Indication The eNB CP Relocation Indication procedure is only applicable for NB-IOT UEs using Control Plane CIoT EPS optimisations. The purpose of the eNB CP Relocation Indication procedure is to request the MME to authenticate the UE's re-establishment request as described in TS 33.401 [22], and initiate the establishment of the UE-associated logical S1-connection after the UE has initiated a RRC Re-establishment procedure in an eNB. When the eNB receives the RRCConnectionReestablishmentRequest message, it triggers the eNB CP Relocation Indication procedure including NAS-level security information received from the UE. If the MME authenticates the request, it initiates the Connection Establishment Indication procedure including NAS-level security information to be sent to the UE in the RRCConnectionReestablishment message. In case the MME cannot authenticate the UE's request, the CONNECTION ESTABLISHMENT INDICATION message does not contain security information, and the eNB shall fail the RRC Re-establishment. In case of authentication failure, the eNB and the MME should locally release the allocated S1 resources, if any. Figure 19.2.2.30-1: eNB CP Relocation Indication procedure (highlighted in blue)

As the eNB-1 has no way to immediately trigger eNB CP Relocation Indication procedure upon reception of RRCConnectionReestablishmentRequest message, one simple option may be to let eNB-1 just reject the Reestablishment Request (e.g., to send no response or send RRCConnectionReestablishmentReject). However, the rejection of RRC Reestablishment request would trigger UE leaving RRC_CONNECTED with release cause 'RRC connection failure'. And furthermore, "RRC Connection failure" from the lower layers would trigger NAS layer recovery. Such processes will cause not only the delayed UL data transmission (please note it’s highly possible that UE has data to be sent if it triggers RRC connection Reestablishment) but also the heavy signaling overhead due to NAS layer recovery.

Observation 8b:  For UE using CP solution, the eNB using S&F Satellite operation has no way to immediately trigger eNB CP Relocation Indication procedure if it receives RRCConnectionReestablishmentRequest message. One simple option may be to let eNB-1 just reject the Reestablishment Request. However, such processes will cause not only the delayed UL data transmission but also the heavy signaling overhead due to NAS layer recovery.

Another option may be that, similar as the process for RRC connection establishment, the eNB-1 is still allowed to firstly finish the RRC connection reestablishment. An example procedure can be found as below:

The main processes can include:

1. upon receiving RRCConnectionReestablishmentRequest message, eNB-1 directly response RRCConnectionReestablishment message to the UE to make UE assume the RRC connection reestablishment procedure is completed. And then eNB-1 gets the UL data and UL NAS-level security information, and store all these information.

2. Later, when eNB-1 moves to the coverage of MME/gateway, eNB-1 can trigger an eNB CP Relocation Indication procedure to deliver the UL NAS-level security information and also the UL data to MME. If the MME can authenticate the NAS-level security information, it can also authenticate the received UL data and then deliver it to the core network. Moreover, MME can provide DL NAS-level security information to the eNB-1. If MME can get the DL data from core network, MME also provide the DL data to eNB-1. Afterwards, the MME and eNB-1 can release the S1 interface.

- As legacy, it may be also possible that the eNB CP Relocation Indication procedure is failed. Then the UL data would not be delivered to upper layer. MME and eNB can perform legacy authentication failure procedure.

3. When eNB-1 in Sat-1 moves back to the area where the UE locates, eNB-1 can Paging the UE and setup connection with the UE. The eNB-1 would further provide DL NAS-level security information and also DL data to UE.

- In the authentication failure case, in the air interface, the eNB-1 can indicate to the UE that the previous UL NAS PDU is failed. According to such information, UE can decide whether to re-transmit the previous UL NAS PDU by its implementation,

The above proposed procedure seems like a process of transmitting data first and then doing security authentication. We think in most of the cases, the RRC Connection Reestablishment procedure can be successfully completed and UL/DL data can be transmitted. But in some rare case, the authentication on NAS-level security information may be failed and kind of fallback process may be needed.

Proposal 8: RAN2 discusses how to tailor the RRC Connection Reestablishment procedure for CP solution in the S&F Satellite operation case.

Proposal 8a: It’s suggested that, in the S&F Satellite operation case, after sending RRCConnectionReestablishmentRequest message, UE can handle RRCConnectionReestablishment message without DL security key.

  - S&F for multiple satellites deployment with eNB and MME-onboard

Several solutions in SA2 study [2] give detailed considerations to address the case of S&F for multiple satellites deployment with both eNB and MME-onboard. In the following, we cite Solution#15 as example. Based on it, we give our observation on the potential RAN2 impacts of the solution for this deployment scenario.

In Solution#15, with reference to the architecture Optimized EPS architecture option for CIoT as specified in TS 23.401, a potential distribution/split of the core network (CN) functions is given between the satellite and the ground. Such distribution/split is based on the following principles:

- The set of CN functions on board the satellite can be a subset of the C-SGN functions, with no need for 3GPP to specify the specific split and interfaces between the subset of the C-SGN functions on board the satellite (referred to as C-SGN-SAT in Figure 6.15.1-3) and the remaining subset of the C-SGN functions deployed on ground (referred to as C-SGN-GND in Figure 6.15.1-3). Accordingly, the interface between C-SGN-SAT and C-SGN-GND, referred to as S&F interface in Figure 6.15.1-3, could remain implementation dependent.

- The minimum subset of CN functions that should necessarily form part of the C-SGN-SAT would be the MME functions needed to (1) handle the S1 interface with the onboard eNB and (2) the MME functions needed to terminate NAS protocol signalling from/to UEs via the onboard eNB.

Based on the above principles, the distribution/split between the C-SGN-SAT and C-SGN-GND is further extended to the multi-satellite case, as represented in Fig. 6.15.1-4.

Figure 6.15.1-4: Proposed distribution of CN functions between the satellites and the ground for S&F Satellite operation, considering the Optimized EPS architecture option for CIoT as a baseline, in a multi-satellite deployment scenario

For this solution, the following considerations are given for the initial attach procedure. The main point is that the first attach request can be rejected as the satellite has no enough information. But satellite can indicate next time point (e.g., T1) when UE can re-access. Before that time, the satellite can acquire the UE relevant information from ground station. And when satellite revisit the UE at T1, the satellite would have enough information to complete the whole attach procedure:

6.15.2.2 NAS procedures spanning several satellite passes [2] In a satellite cell operating in S&F Satellite operation mode, if a UE initiates a signalling procedure during a satellite pass (e.g. E-UTRAN Initial Attach procedure) and the procedure cannot be completed within the duration of the pass because some information is missing in the serving satellite (e.g. E-UTRAN Authentication Vectors, AVs, are not available within the satellite and should be retrieved from an HSS on the ground), the initiated procedure should be terminated/suspended with a proper rejection cause and an indication about when/how the UE may re-attempt/resume the procedure (e.g. the initial attach procedure may be rejected with a cause like "procedure suspended due to S&F operation" and an indication such as "re-try in 'X' minutes" could be given for the UE to retry during the next pass of the same or a different satellite where E-UTRAN AVs could have been already uploaded into the satellite to be able to complete the procedure). Figure 6.15.2.2-1: Illustration of E-UTRAN Initial Attach procedure under S&F Satellite operation in a multi-satellite deployment and roaming scenario (i.e. HSS on ground)

Furthermore, the following data delivery procedure is given:

6.15.2.3 Data transfer in S&F Satellite operation mode [2] Following successful registration with the satellite PLMN and establishment of a valid UE/MME context across all or a subset of the satellites of the constellation, data transfer can take place between the UE and any of those satellites when in reach of the UE. Without preventing other approaches, this scenario considers the use of Control Plane CIoT EPS Optimisation as the baseline protocol for data transfer between the UE and satellite cells operating in S&F Satellite operation mode. Data transfer under Control Plane CIoT EPS Optimisation relies on an exchange of ESM DATA TRANSPORT messages between the UE and the MME, encapsulating user data within the User Data Container Information Element (IE), as defined in TS 24.301 [17]. Furthermore, the protocol allows for the exchange of ESM STATUS messages between the UE and MME to report specific error conditions upon the reception of ESM protocol data. Figure 6.15.2.3-1: MO user data transfer

From RAN2 perspective, based on the above example, we can see that if whole or at least partial (e.g., MME) of core network functionalities are onboard the satellite, both S1 interface establishment and NAS protocol signaling from/to UEs can be completed during one time the satellite flies away. And the possible large delay on NAS procedure may not exist. The procedure between UE and MME can be almost same as legacy and therefore no further impacts on air interface can be seen.

Observation 9: For the deployment case that whole or at least partial (e.g., MME) of core network functionalities are onboard the satellite, both S1 interface establishment and NAS protocol signaling from/to UEs can be completed during one time the satellite flies away. The procedure between UE and MME can be almost same as legacy and therefore no further impacts on air interface can be seen.

Conclusion

This contribution gives some observations on the whole procedures for S&F Satellite operation scenario:

#S&F for multiple satellites deployment with only eNB-onboard

Observation 1: For only eNB onboard the satellite and the eNB using S&F Satellite operation, in step 1, after eNB finishes the air interface establishment/resumption and get NAS PDU or UP data from the UE, it’s possible for eNB to either release the UE or keep the UE in connected mode. This can be left to eNB implementation.

Observation 2: The eNB using S&F Satellite operation may reject the RRC connection establishment/resume request from UE (s). This also can be left to eNB implementation.

Observation 3: For UE using CP solution, in the legacy air interface procedure, eNB can acquire UE Radio capability from MME after Msg3 which can facilitate eNB to provide appropriated configuration in Msg4. However, in the S&F Satellite operation, the eNB has no way to acquire the UE capability from MME after Msg3. One implementation option is to provide kind of default configuration in Msg4 that may downgrade the radio link performance.

Observation 4: If the UE in idle mode to trigger the EDT or PUR procedure, the eNB using S&F Satellite operation may need to immediately send Msg4 (or Layer 1 ACK for PUR) after reception of Msg3 (e.g., without triggering S1 procedure). This can be left to eNB implementation and it’s feasible based on RAN2 existing specifications.  Only RAN3 may need to further check whether more changes are needed for decoupling the air interface procedure and S1 interface procedure for EDT or PDU.

Observation 5: When the source eNB (e.g., eNB-1) moves to the area where the feeder link (ground station) is available, the source eNB can trigger S1 procedure to setup feeder link with the MME/ground station and furthermore deliver the stored UL data to core network. MME/ground station may also deliver the corresponding or buffered DL data to the source eNB. It’s also possible for MME to deliver the corresponding or buffered DL data to a new eNB if MME can know that this new eNB will visit the area where the UE locates within shorter time period.

Observation 6: When the eNB containing the DL data (re)visit the area where the UE locates, the legacy paging procedure can be used by the eNB to find the UE and deliver the DL data to it. In a few cases, it may be also possible for the source eNB to use the RRC connection which is setup when the eNB previously visited the area to deliver the DL data to the UE.

Observation 7: It may be possible that the feeder link becomes from available to unavailable during a RRC connection. For this case, the eNB implementation can proactively release the UE.

Observation 8a: When the RF quality fluctuates, a UE in connected mode may encounter RLF and it may trigger a RRC connection reestablishment procedure to an eNB is with good radio quality but has no available feed link.

Observation 8b:  For UE using CP solution, the eNB using S&F Satellite operation has no way to immediately trigger eNB CP Relocation Indication procedure if it receives RRCConnectionReestablishmentRequest message. One simple option may be to let eNB-1 just reject the Reestablishment Request. However, such processes will cause not only the delayed UL data transmission but also the heavy signaling overhead due to NAS layer recovery.

#S&F for multiple satellites deployment with eNB and MME-onboard

Observation 9: For the deployment case that whole or at least partial (e.g., MME) of core network functionalities are onboard the satellite, both S1 interface establishment and NAS protocol signaling from/to UEs can be completed during one time the satellite flies away. The procedure between UE and MME can be almost same as legacy and therefore no further impacts on air interface can be seen.

And the following proposals are given:

#General aspects:

Proposal 1: RAN2 assume the proposed solution for supporting S&F Satellite operation can be applicable to both case of using single satellite and case of using multiple satellites.

#S&F for multiple satellites deployment with only eNB-onboard

Proposal 2: It’s suggested to define the S&F indication in SIB with meaning of “using S&F Satellite operation” or “availability of the feeder link”.

Proposal 3: It’s unnecessary to introduce new barring scheme to facilitate eNB with S&F Satellite operation to differentiate the access barring for legacy UEs and R19 IoT NTN UEs.

Proposal 4: It’s suggested that, if an eNB rejects UE’s RRC connection request due to using S&F Satellite operation, the eNB can indicate a detailed reject reason to UE in the RRC connection reject message. Moreover, the eNB can give recommendation on other satellite/eNB for UE to access later.

Proposal 5: RAN2 can discuss whether to report some critical UE capabilities via Msg3 for S&F Satellite operation case if real need is found.

Proposal 6: considering that a eNB may acquire DL data for several UEs when it moves in the coverage of the MME/ground station and late need to paging several UEs at the same time when next time it (re)visit the area where these UEs locate, some paging enhancements may be needed to address this case.

Proposal 7: RAN can discuss whether to introduce a new release reason, e.g., “the feeder link becomes unavailable” for the case that eNB proactively release the UE when feeder link becomes unavailable.

Proposal 8: RAN2 discusses how to tailor the RRC Connection Reestablishment procedure for CP solution in the S&F Satellite operation case.

Proposal 8a: It’s suggested that, in the S&F Satellite operation case, after sending RRCConnectionReestablishmentRequest message, UE can handle RRCConnectionReestablishment message without DL security key.

Reference

[1] RP-240776 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things MediaTek Inc., RAN#103, March, 2024

[2] 3GPP TR 23.700-29 V19.0.0, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19), 2024-06