3GPP TSG-RAN WG2 Meeting #127	R2-2407153

Maastricht, Netherlands, 19th – 23th August, 2024                       Revision of R2-2404884

Source:	ZTE Corporation, Sanechips

Title:	Further consideration on uplink capacity enhancements in IoT NTN

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed and in RAN#103, the WID is further refined in [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#125bis meeting, RAN2 has had initial discussion to clarify some general aspects for this topic and the following agreements have been achieved:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements

In RAN2#126 meeting, RAN2 has made more agreements as following:

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

In this document, we will give our analysis on CRDSA scheme and other issues of uplink capacity enhancements objective. Based on the analysis, we also give our proposals.

Discsission

  - Feasibility for supporting CRSDA

In last RAN2 meeting, to support Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmission has been proposed in [2].

As mentioned in [2], Contention Resolution Diversity Slotted ALOHA (CRDSA) generally transmits two or more replicas for every packet in randomly selected slots within a TDMA/FDMA frame. And each physical layer packet contains in the payload header information about the location of all the replicas within the allocated TDMA/FDMA frame. At the receiver side, the complete frame is sampled and stored in a digital memory. By using decision-directed interference cancelation techniques, the less or non-interfered packets are initially demodulated, and the interference generated by their replicas on other slots can be cancelled. Therefore the packets on other slots can be further demodulated. So these “replica transmissions” have the objective of providing collision diversity and can improve the uplink capacity. The CRDSA solution has been adopted and practically demonstrated in the satellite DVB-RCS2 standard [3].

With the following figure cited from [4], we can further understand how this scheme works:

In this example, each packet has two replicas. The successive interference cancellation (SIC) is applied at the receiver. The main steps are as below:

- The decoder starts looking for replicas that can be successfully decoded. The first to be found is replica C2.

- As C2 is a collision-free replica, it can be successfully decoded. After successfully decoding, the information on the location of replica C1 is retrieved from the header of C2. Then, the data carried by C2 can be re-encoded, re-modulated, frequency offset and sampling epoch are superimposed on the signal with the contributions from C1.

- Furthermore, the interference caused by C1 on replica A2 also can be removed. The decoder can successfully decode replica A2. Applying the same procedure, the information on the location of replica A1 can be retrieved from the header of A2.

- After removing the interference caused by replica A1, now, replica B1 is collision-free and can be successfully decoded. And the information on the location of replica B2 can be retrieved from the header of B1. All their interference contribution can be removed.

- Also due to the interference caused by replica C1 can be removed, replica D1 becomes collision-free and can be correctly decoded. And then the location information of replica D2 can be retrieved from the header of D1 and their interference contribution can be removed.

- Unfortunately for user 5 and 6, even some of the interferences (e.g., the interference caused by D2) on their replicas can be removed, there are still other reciprocal interferences that cannot be removed. So the replicas E1/E2/F1/F2 cannot be resolved by SIC only.

The performance evaluations in [4] indicates that the throughput of CRDSA can be impressively extended from S ≅0.36 (the peak throughput of slotted ALOHA (SA), measured in average number of successful transmissions per transmission period or packets per slot) up to S ≅0.55. Further performance improvements can be achieved when, 1) more than two replicas per user and per frame are sent (but some other study show that 3 replicas may achieve maximum gain and if the number of replicas increases, the performance starts to degrade as more replicas would increase the system load) 2) difference in received power, due to induced power unbalance or fading, and capture effect are considered.

Bases on the above background information, we have following roughly and qualitatively understanding s for CRDSA:

- Understanding 1: In the study in [2] and [4], it seems the relevant time domain resources are continuously available. So that even for multi users and each user having multi replicas, by randomly selecting the resources location for replica transmission, there is still a certain percentage of replicas who can avoid collision.

- Understanding 2: It can be seen that the SIC scheme at the receiver is very important. SIC exploits the presence of multiple replicas per user for clearing up collisions. Every time a packet is decoded, SIC reconstructs the waveform and subtracts it from all the slot locations selected for transmission by the corresponding user, possibly removing the interference contribution with respect to other packets. We understand such complex interference cancellation scheme would be implemented in physical layer, and some constraints may be needed for achieving good interference cancellation performance. Moreover, even with SIC, there are still some replicas that cannot be demodulated, and other enhanced interference cancellation schemes are needed.

- Understanding 3: According to the simulation results given in both [2] and [4], it can be seen that the performance is greatly affected by the system load, e.g., after reaching a certain load, the throughput drops very much, and the packet loss rate also increases significantly.

Therefore, we at least see the following difficulties that may make it almost infeasible to support CRDSA in R19 IoT NTN:

- In 3GPP system, especially for IoT technology, the allocated random access resources are generally periodical resources. And within a time domain frame, there may be only a few subframes available for the uplink transmission of the random access procedure. Therefore, it is easy to understand that, if replicas are transmitted on such resources, the probability of replicas conflicting with each other will be very high, maybe much higher than the collision probability in “ideal” situation, e.g., continuous resources as mentioned in Understanding 1. Moreover, due to the sparse available resources in a certain time unit, if no or very few collision-free replicas can be found at the beginning, all replicas will become unnecessary interference and additional system load.

- According to Understanding 2, CRDSA cannot be supported with only (or even not mainly) MAC layer work. Physical layer also needs to support complicated interference cancellation scheme. Before introducing an interference cancellation scheme, it is also necessary to do the evaluation on its performance and gain. So we worry about whether the existing TU allocation for IoT NTN in RAN1 can allow such out of WID scope work.

- In traditional satellite communication systems, the number of users is usually very small and the system load is light, so it may be easy to see the expected gain of CRDSA. However, for IoT NTN, it is originally with intention to support many IoT UEs and the system load will generally not be light, so according to Understanding 3, we have doubt on the potentially achievable gain for IoT NTN.

Observation 1: The following difficulties may make it almost infeasible to support CRDSA in R19 IoT NTN:

- In IoT system, the allocated random access resources are generally periodical resources, and within a certain time slot, there may be very few subframes available for the uplink transmission. If replicas are transmitted on such resources, the probability of replicas conflicting with each other will be very high;

- For CRDSA, besides MAC layer work, Physical layer also needs to support complicated interference cancellation scheme. Evaluation on performance and gain for different interference cancellation schemes may be also needed. The existing TU allocation for IoT NTN in RAN1 may not be able to accommodate such work.

- The performance of CRDSA is greatly affected by the system load. IoT NTN needs to support many IoT UEs and the system load will generally not be light, so the expected gain may not be achieved in IoT NTN.

With observation 1, the following proposal is given:

Proposal 1: It’s suggested not to support CRDSA in R19 IoT NTN.

  - Contention-based Msg3 transmission

In last meeting, RAN2 has agreed to focus the study on contention-based Msg3 transmission to complete an EDT-like transaction for R19 IoT NTN. Moreover, UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. RAN2 also send LS to RAN1 and RAN4 to trigger them to discuss the feasibility and requirements for verifying uplink synchronization before sending msg3.

In the following sections, we will further give our analysis on the RAN2 aspects for supporting contention-based Msg3 transmission.

    - Conditions for using contention-based Msg3 transmission

Taking legacy PUR as reference, in legacy PUR, UE needs to firstly fulfill the Timing alignment validation condition (including both PUR TA timer-related criteria and RSRP-related criteria) for triggering PUR as below, then UE can skip the steps of preamble transmission and RAR reception and directly to send Msg3 on the dedicated resources.

<<skip the unrelated parts>> 5.3.3.19 Timing alignment validation for transmission using PUR The UE shall consider the timing alignment value for transmission using PUR to be valid when the following conditions are fulfilled: 1> either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers; and 1> either pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is not configured or the following conditions are fulfilled: 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not increased by more than increaseThresh; and 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not decreased by more than decreaseThresh;

Also in general, UE specific PUR transmission can be performed only when the following conditions are met:

- Cond#1: The TA of the UE in IDLE state keeps valid;

- Cond#2: The UE camps on the cell where pur-Config was provided (if the UE is establishing or resuming an RRC connection in a cell that is different from the cell where pur-Config was provided, pur-Config is released).

In the early stage discussion for R17 IoT NTN, in [5], we indicate that, due to the following reasons, the legacy PUR which is based on dedicated resources may not be always applicable to IoT NTN, especially for the IoT over LEO case:

- Firstly, due to the movement of satellite, even for stationary UEs, the TA of UE in IDLE state may change frequently and the stored TA may not be able to always keep valid during the relative long pur-TimeAlignmentTimer period.

- Secondly, for moving cell of IoT over LEO and fixed cell of IoT over LEO with different PCI cases, satellite change would cause frequent cell change. Then it would be very easy or frequent to occur that the cell where UE camps is different from the cell where pur-Config was provided. As mentioned above, if UE initiates RACH on a new cell, the UE should release the PUR resources. That means, in such case cell changes frequently, the allocated PUR resources may hardly have chance to be used.

However, with the new objective in R19 IoT NTN, e.g., to support contention-based or shared resources-based Msg3 transmission, the restriction in legacy PUR that PUR can only be used in the last cell would be no longer needed. This will further make the contention-based Msg3 transmission applicable to not only the case of fixed cell of IoT over LEO with same PCI but also the cases of moving cell of IoT over LEO and fixed cell of IoT over LEO with different PCI.

Observation 2: To support contention-based Msg3 transmission may make the restriction that legacy PUR can only be used in the last cell no longer needed. This will further make the contention-based Msg3 transmission function applicable to not only the case of fixed cell of IoT over LEO with same PCI but also the cases of moving cell of IoT over LEO and fixed cell of IoT over LEO with different PCI.

Based on the Observation 2, the following proposal is given:

Proposal 2a: If contention-based Msg3 transmission is supported, when the camping cell of the UE changes, the UE doesn’t need to stop using this function.

Furthermore, for IoT NTN, TA compensation has been supported, it may make it easier for IoT NTN UE in IDLE state to maintain TA validation than IoT UE in TN network. Moreover, if RAN1 and RAN4 can confirm the issue mentioned in LS [6], e.g., RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2, we think the timing alignment validation condition (including both PUR TA timer-related criteria and RSRP-related criteria) in legacy PUR would not be needed for contention-based Msg3 transmission in R19 IoT NTN..

Proposal 2b: If RAN1 and RAN4 can confirm that RRC Idle UE with a pre-compensated TA can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2, the timing alignment validation condition in legacy PUR (including both PUR TA timer-related criteria and RSRP-related criteria) would not be needed for contention-based Msg3 transmission in R19 IoT NTN.

    - The resources configuration for contention-based Msg3 transmission

For legacy PUR, NW needs to provide UE with dedicated resources configuration via RRC connection release message. For reference, the resources configuration is copied as below (taking NB-IoT as example):

– PUR-Config-NB The IE PUR-Config-NB is used to specify PUR configuration. PUR-Config-NB information element -- ASN1START PUR-Config-NB-r16 ::= SEQUENCE { pur-ConfigID-r16 PUR-ConfigID-NB-r16 OPTIONAL, --Need OR pur-TimeAlignmentTimer-r16 INTEGER (1..8) OPTIONAL, --Need OR pur-NRSRP-ChangeThreshold-r16 SetupRelease {PUR-NRSRP-ChangeThreshold-NB-r16} OPTIONAL, --Need ON pur-ImplicitReleaseAfter-r16 ENUMERATED {n2, n4, n8, spare} OPTIONAL, --Need OR pur-RNTI-r16 C-RNTI OPTIONAL, --Need ON pur-ResponseWindowTimer-r16 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64} OPTIONAL, --Need ON pur-StartTimeParameters-r16 SEQUENCE { periodicityAndOffset-r16 PUR-PeriodicityAndOffset-NB-r16, startSFN-r16 INTEGER (0..1023), startSubframe-r16 INTEGER (0..9), hsfn-LSB-Info-r16 BIT STRING (SIZE(1)) } OPTIONAL, --Need ON pur-NumOccasions-r16 ENUMERATED {one, infinite}, pur-PhysicalConfig-r16 SEQUENCE { carrierConfig-r16 CarrierConfigDedicated-NB-r13, npusch-NumRUsIndex-r16 INTEGER (0..7), npusch-NumRepetitionsIndex-r16 INTEGER (0..7), npusch-SubCarrierSetIndex-r16 CHOICE { khz15 INTEGER (0..18), khz3dot75 INTEGER (0..47) }, npusch-MCS-r16 CHOICE { singleTone INTEGER (0..10), multiTone INTEGER (0..13) }, p0-UE-NPUSCH-r16 INTEGER (-8..7), alpha-r16 ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}, npusch-CyclicShift-r16 ENUMERATED {n0, n6}, npdcch-Config-r16 NPDCCH-ConfigDedicated-NB-r13 } OPTIONAL, -- Need ON ..., [[ pur-PhysicalConfig-v1650 SEQUENCE { ack-NACK-NumRepetitions-r16 ACK-NACK-NumRepetitions-NB-r13 } OPTIONAL --Need ON ]], [[ pur-PhysicalConfig-v1700 SEQUENCE { pur-UL-16QAM-Config-r17 SetupRelease {PUR-UL-16QAM-Config-NB-r17} OPTIONAL, -- Need ON pur-DL-16QAM-Config-r17 SetupRelease {NPDSCH-16QAM-Config-NB-r17} OPTIONAL -- Need ON } OPTIONAL -- Need ON ]] } PUR-NRSRP-ChangeThreshold-NB-r16 ::= SEQUENCE { increaseThresh-r16 NRSRP-ChangeThresh-NB-r16, decreaseThresh-r16 NRSRP-ChangeThresh-NB-r16 OPTIONAL --Need OP } PUR-UL-16QAM-Config-NB-r17 ::= SEQUENCE { uplinkPowerControlDedicated-r17 UplinkPowerControlDedicated-NB-v1700 } NRSRP-ChangeThresh-NB-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1} -- ASN1STOP ------

With reference to above PUR-Config IE, from RAN2 perspective, we understand at least the following resources types need to be configured for contention-based Msg3 transmission:

- Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of NPUSCH occasions

- Frequency domain resources, e.g., Carrier, SubCarrier

- TBS

- NPDCCH search space window

It can be expected that, although some of the resources for contention-based Msg3 transmission may be similar as the ones in legacy pur-Config, it’s still possible that the detailed configuration contents will be somewhat different from the legacy PUR-Config. For example, the resources for contention-based Msg3 may also have a “start time” element which is similar as the corresponding one in PUR-Config. For dedicated PUR, the start time of PUR resources is relative to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config, but it’s highly possible this would not be the case for the definition of start time in the resources for contention-based Msg3.

Observation 3: Even shared resources configuration for contention-based Msg3 transmission may have some resources IEs which are similar as the ones in legacy PUR-Config, it’s still possible that the detailed configuration contents in the resources configuration for contention-based Msg3 transmission will be somewhat different from the one in legacy PUR resources configuration.

RAN1 is also discussing how to define and apply OCC (orthogonal cover codes) sequence to NPRACH/NPUSCH to facilitate more UEs multiplexing. We understand further CDMA scheme will be supported for the contention-based Msg3 transmission with intention to alleviate the collision caused by shared time/frequency domain resources. From RAN2 perspective, the OCC sequence will be a new resource type which needs to be introduced in the resources for contention-based Msg3 transmission.

Proposal 3a: With reference to legacy PUR configuration, RAN2 can take the following resources types as start point for the discussion on resources configuration for contention-based Msg3 transmission:

- Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of NPUSCH occasions

- Frequency domain resources, e.g., Carrier, SubCarrier

- TBS

- NPDCCH search space window

- OCC resource

As for the configuration way, a straightforward way for providing resources for contention-based Msg3 transmission is to provide a common resource configuration via SIB (Alt1) that can be shared by all the UEs. This way may have highest collision probability.

Taking into account the potential contents mentioned above, we think it’s also possible to provide resources for contention-based Msg3 transmission via dedicated signaling, e.g., via RRC connection release message (Alt2). By this way, NW is allowed to configure some different resources for different UEs (e.g., different OCC sequence/index) while the other resources still can be same for all the UEs.

Another usually used configuration way is to provide multiple sets of resources for contention-based Msg3 transmission in the SIB instead of only one set, and then NW can indicate index of the resource set for a specific UE via the dedicated signaling (Alt3). By this way, the UEs seem as being divided into several groups. The UEs within a group share same set of resource while the UEs in different groups using different resource set.

Both Alt2 and Alt3 are better ways for reducing the collision, e.g., better than Alt1, but may need more resources provision. And another disadvantage of Alt2 and Alt3 is that they may still need the restriction that the resources can only be used in the last cell.

It suggests RAN2 to discuss/evaluate the following three alternatives:

Proposal 3b: RAN2 is suggested to discuss/evaluate the following alternatives for configuring resources for contention-based Msg3 transmission:

- Alt1: To provide a totally shared resource via SIB.

- Alt2: To provide (partially) shared or dedicated resources via dedicated signaling. It can be allowed to configure some different resources for different UEs while the other resources may be same for all the UEs.

- Alt3: To provide multiple sets of shared resources via the SIB and NW can indicate index of the resource set for a specific UE via the dedicated signaling.

    - RNTI and contention resolution

RNTI is another aspect that needs to be discussed. It’s mainly related to contention resolution and scheduling of Msg4.

In legacy PUR, PUR-RNTI is used which is allocated by eNB and provided to UE within dedicated pur-Config. At least in UE side, it can be seen as a UE-specific RNTI. The DL transmission using PUR resources are addressed by this dedicated PUR-RNTI and no other contention resolution scheme is needed.

For contention-based Msg3 transmission, it’s easy to understand such RNTI is used not only for address the DL transmission, but also maybe with the intention to resolve at least part of the collisions.

The following alternatives can be considered for the RNTI design (now still call it as PUR-RNTI for discussion convenience, but exact naming can be late decided) for contention-based Msg3 transmission:

- Common PUR-RNTI (Alt1): That means only one PUR-RNTI may be configured in the shared Msg3 transmission resources configuration. All the UEs using shared Msg3 transmission resources can use this common PUR-RNTI to address DL transmission. Such common PUR-RNTI may be similar as P-RNTI or SI-RNTI and just for identifying the contention-based Msg3/Msg4 transmission. It seems that no benefit of collision resolution can be expected.

- A UE-specific PUR-RNTI allocated by NW (Alt2): Even the shared Msg3 transmission resources can be configured via SIB, it still can be possible to let NW separately configure a use-specific PUR-RNTI via dedicated signalling. This UE-specific PUR-RNTI seems similar as that in dedicated PUR, but the difference is that it’s used in a shared resources space.

- A resource-specific PUR-RNTI (Alt3): Such RNTI may be similar as RA-RNTI. UE and eNB can respectively calculate the PUR-RNTI according to the resources used/selected by the UE for Msg3 transmission. The eNB use the calculated RA-RNTI to address the DL transmission (e.g., PDCCH/Msg4). The UE only need to handle the DL transmission addressed by its resource-specific PUR-RNTI. However, as it’s still possible that some UEs use same shared resources for Msg3 transmission and will expect same resource-specific PUR-RNTI, the other contention resolution may still be needed.

As this RNTI design is also applicable to the scheduling of Msg4, the proposal will be given under the discussion for Msg4.

  - Efficient delivery of Msg4

In this section, we will discuss RAN2 aspects for supporting efficient delivery of Msg4 which corresponds to the contention-based Msg3 transmission.

    - Msg4 types

In legacy PUR, the following types of “Msg4” are possibly received by UE in the normal process (the abnormal cases, e.g., the PUR fallback or PUR failure indicated from lower layer are not mentioned):

- Physical layer L1 ACK or Timing Advance Command MAC CE: if no DL data needs to be transmitted to the UE, NW may send the L1 ACK to acknowledge the successful Msg3 transmission. It’s also possible for NW to only send a TAC MAC CE to update the TA for the UE in idle. Upon reception one of these two indications, UE can assume the transmission using PUR was successful

- RRCEarlyDataComplete or RRCConnectionRelease with data: if only one DL data needs to be sent to UE, NW can send RRCEarlyDataComplete or RRCConnectionRelease message containing the DL data. Upon reception of such signalling, UE also can think the transmission using PUR was successful.

- RRCConnectionSetup or RRCConnectionResume for fallback: if more data need to be transmitted, NW can send RRCConnectionSetup or RRCConnectionResume messages to fall back UE to connected mode. A newUE-Identity (C-RNTI) may be provided for UE to use after moving to RRC_CONNECTED mode.

Generally, on one hand, we think the above Msg4 types are already enough to cover the main transmission scenarios, so at least now no need can be seen to introduce additional Msg4 signaling. On the other hand, all above types of Msg4 need to be supported for contention-based Msg3 transmission scheme to handle different scenarios.

Moreover, the very simple Physical layer L1 ACK has already been introduced in legacy PUR and it’s still applicable to contention-based Msg3 transmission scheme to achieve the most efficient Msg4 transmission.

Proposal 4: The existing types of “Msg4” for normal PUR process, e.g., Physical layer L1 ACK, Timing Advance Command MAC CE, and RRCEarlyDataComplete/RRCConnectionRelease/RRCConnectionSetup/ RRCConnectionResume, need to be supported for contention-based Msg3 transmission scheme. The very simple Physical layer L1 ACK can be used in suitable cases to achieve the most efficient Msg4 transmission.

    - The scheduling of Msg4

In legacy PUR, UE in IDLE mode monitors Msg4 addressed by PUR-RNTI within a search space which is specifically configured for UE. But for contention-based Msg3 transmission scheme, we assume only common search space can be used. RAN1 can give further information on this aspect.

Observation 4: For contention-based Msg3 transmission scheme, we assume only common search space can be used. This can be further discussed based on RAN1 progress.

Based on the discussion in section 2.2.3, RAN2 can discuss the following alternatives for scheduling of Msg4.

Proposal 5: RAN is suggested to discuss the following alternatives for scheduling of Msg4:

- Alt1: Common PUR-RNTI configured in the shared resources configuration.

- Alt2: UE-specific PUR-RNTI allocated by NW that is similar as the one for dedicated PUR.

- Alt3: Resource-specific PUR-RNTI which can be respectively calculated by UE and eNB according to the resources used/selected by the UE for Msg3 transmission.

    - Contention resolution

As mentioned above, it’s highly possible that the collision caused by using shared resources for Msg3 transmission cannot be completely resolved by RNTI design. The additional contention resolution scheme is still needed.

The straightforward way is to reuse the contention resolution scheme in legacy random access procedure. That is, after recpetion of Msg3, eNB can send a DL MAC PDU in which a Contention Resolution Identity is included. For UE side, it needs to check whether the UE Contention Resolution Identity included in the DL MAC PDU matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 6: The contention resolution scheme in legacy random access procedure can be reused for contention-based Msg3 transmission, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

  - Other issues

For now, the following other issues are identified for contention-based Msg3 transmission:

- Coexistence of dedicated PUR and contention-based Msg3 transmission

It may be possible for eNB to configure both dedicated PUR for specific UEs and broadcast shared resources for contention-based Msg3 transmission. In such case, we assume UE will prioritize the use of dedicated PUR. Only when the trigger condition is not fulfilled or the dedicated PUR becomes invalid, UE can consider to use shared resources for contention-based Msg3 transmission.

- Backoff scheme when Msg3 collision occurs

When the collision or congestion is detected by eNB, the eNB may provide kind of backoff indication via the Msg4 (or DL MAC CE before Msg4) to alleviate the situation. That generally means the UE needs to delay a backoff time and then can choose another time domain resources within the shared resources for Msg3 transmission. RAN2 can firstly discuss the need for backoff scheme after Msg3 and then further discuss the detailed scheme.

Proposal 7: RAN2 is suggested to discuss the following other issues:

- Issue#1: How to handle the coexistence of dedicated PUR and contention-based Msg3 transmission

- Issue#2: Whether a backoff scheme is needed when Msg3 transmission collision occurs

Conclusion

Based on the above analysis, the following observations and proposals are given for uplink capacity enhancements for IoT NTN:

Observation 1: The following difficulties may make it almost infeasible to support CRDSA in R19 IoT NTN:

- In IoT system, the allocated random access resources are generally periodical resources, and within a certain time slot, there may be very few subframes available for the uplink transmission. If replicas are transmitted on such resources, the probability of replicas conflicting with each other will be very high;

- For CRDSA, besides MAC layer work, Physical layer also needs to support complicated interference cancellation scheme. Evaluation on performance and gain for different interference cancellation schemes may be also needed. The existing TU allocation for IoT NTN in RAN1 may not be able to accommodate such work.

- The performance of CRDSA is greatly affected by the system load. IoT NTN needs to support many IoT UEs and the system load will generally not be light, so the expected gain may not be achieved in IoT NTN.

Observation 2: To support contention-based Msg3 transmission may make the restriction that legacy PUR can only be used in the last cell no longer needed. This will further make the contention-based Msg3 transmission function applicable to not only the case of fixed cell of IoT over LEO with same PCI but also the cases of moving cell of IoT over LEO and fixed cell of IoT over LEO with different PCI.

Observation 3: Even shared resources configuration for contention-based Msg3 transmission may have some resources IEs which are similar as the ones in legacy PUR-Config, it’s still possible that the detailed configuration contents in the resources configuration for contention-based Msg3 transmission will be somewhat different from the one in legacy PUR resources configuration.

Observation 4: For contention-based Msg3 transmission scheme, we assume only common search space can be used. This can be further discussed based on RAN1 progress.

Proposal 1: It’s suggested not to support CRDSA in R19 IoT NTN.

Proposal 2a: If contention-based Msg3 transmission is supported, when the camping cell of the UE changes, the UE doesn’t need to stop using this function.

Proposal 2b: If RAN1 and RAN4 can confirm that RRC Idle UE with a pre-compensated TA can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2, the timing alignment validation condition in legacy PUR (including both PUR TA timer-related criteria and RSRP-related criteria) would not be needed for contention-based Msg3 transmission in R19 IoT NTN.

Proposal 3a: With reference to legacy PUR configuration, RAN2 can take the following resources types as start point for the discussion on resources configuration for contention-based Msg3 transmission:

- Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of NPUSCH occasions

- Frequency domain resources, e.g., Carrier, SubCarrier

- TBS

- NPDCCH search space window

- OCC resource

Proposal 3b: RAN2 is suggested to discuss/evaluate the following alternatives for configuring resources for contention-based Msg3 transmission:

- Alt1: To provide a totally shared resource via SIB.

- Alt2: To provide (partially) shared or dedicated resources via dedicated signaling. It can be allowed to configure some different resources for different UEs while the other resources may be same for all the UEs.

- Alt3: To provide multiple sets of shared resources via the SIB and NW can indicate index of the resource set for a specific UE via the dedicated signaling.

Proposal 4: The existing types of “Msg4” for normal PUR process, e.g., Physical layer L1 ACK, Timing Advance Command MAC CE, and RRCEarlyDataComplete/RRCConnectionRelease/RRCConnectionSetup/ RRCConnectionResume, need to be supported for contention-based Msg3 transmission scheme. The very simple Physical layer L1 ACK can be used in suitable cases to achieve the most efficient Msg4 transmission.

Proposal 5: RAN is suggested to discuss the following alternatives for scheduling of Msg4:

- Alt1: Common PUR-RNTI configured in the shared resources configuration.

- Alt2: UE-specific PUR-RNTI allocated by NW that is similar as the one for dedicated PUR.

- Alt3: Resource-specific PUR-RNTI which can be respectively calculated by UE and eNB according to the resources used/selected by the UE for Msg3 transmission.

Proposal 6: The contention resolution scheme in legacy random access procedure can be reused for contention-based Msg3 transmission, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 7: RAN2 is suggested to discuss the following other issues:

- Issue#1: How to handle the coexistence of dedicated PUR and contention-based Msg3 transmission

- Issue#2: Whether a backoff scheme is needed when Msg3 transmission collision occurs

Reference

[1] RP-240776 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things MediaTek Inc., RAN#103, March, 2024

[2] R2-2405202, Discussion on Msg3-EDT enhancements, ESA, discussion, Rel-19, RAN2#126, May, 2024

[3] ETSI EN 301 545-2, “Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard”

[4] Enhancing Contention Resolution ALOHA using Combining Techniques, Federico Clazzer, Student Member, IEEE, Christian Kissling, Mario Marchese, Senior Member, IEEE, Computer Science - Information Theory, 10.1109/TCOMM.2017.2759264

[5] R2-2103357, SIB and IoT features applicability for IoT over NTN, ZTE Corporation, Sanechips, RAN2#113bise, April, 2021

[6] R2-2405769, LS to RAN4 and RAN1 on TA for Msg3, ZTE Corporation, Sanechips, RAN2#126, May, 2021