3GPP TSG-RAN WG2 Meeting #127			         R2-2407156

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	8.4.2

Source: 	LG Electronics Inc.

Title:         	LP-WUS operation in RRC_IDLE and RRC_INACTIVE

Document for: 	Discussion and Decision

Introduction

In this contribution, we will continue the discussion on the procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, based on the agreements made by RAN1, RAN2, and RAN4 so far.

Discussion

- LP-WUS related configuration

In the last meeting, RAN2 agreed what information is included in the LP-WUS related configuration as follows:

RAN2#126

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

However, it is still FFS if the configuration of the entry/exit condition for LP-WUS monitoring is mandatory or optional. Some companies argued that the entry/exit conditions do not always need to be configured by network and the optional configuration of it would be beneficial. The proponents of the optional entry/exit condition are divided into two camps, each with a different intended UE behaviour when the exit/entry condition is absent in the LP-WUS related configuration.

The first camp argues that smart UEs can make sensible UE implementation when to start/stop LP-WUS monitoring. However, if UE conservatively determines the entry/exit condition to ensure reliable LP-WUS reception, the MR may lose the opportunity to enter the ultra-deep-sleep and the power saving gain from LP-WUS monitoring will somewhat decreases. Conversely, if the threshold for the entry condition is set too low, or if the threshold for the exit condition is set too high, the UE will monitor LP-WUS with low reliability and may miss paging. Since only a few bits are expected to configure the entry/exit condition, it seems unnecessary to tolerate such potential issues.

Observation 1	If UE determines inappropriate entry/exit condition by itself, the power saving gain from LP-WUS or the paging reception performance can be reduced.

Another camp argues that in the deployment of LP-WUS coverage same as cell coverage, NW is not required to provide the entry/exit condition, and UE always assumes the entry condition is met. However, even if LP-WUS is operated to have the same coverage as the cell, it does not mean that good SSB reception quality guarantees good LP-WUS reception quality. LP-WUS and SSB are transmitted on different channel bandwidth, so UE can experience a degradation in LP-WUS quality without a deterioration in cell quality. UE should be able to stop monitoring LP-WUS, if the reception quality of LP-WUS is deemed poor in any deployment scenarios.

Observation 2	Even if LP-WUS is transmitted to have the same coverage as the associated cell, LP-WUS quality can deteriorate while the cell is fine, because they are transmitted on different channels.

Based on the observation 1 and 2, we concluded that the configuration of the entry/exit condition of LP-WUS monitoring should be mandatory present in the LP-WUS configuration.

Proposal 1	The configuration of the entry/exit condition of LP-WUS monitoring is mandatory present in the LP-WUS related configuration.

Regarding the LP-WUS configuration, there is another FFS if dedicated configuration is needed. It is obvious the LP-SS/LP-WUS configuration is cell specific. There is no reason to support dedicated signalling-based configuration for them. However, the LP-WUS configuration may need to be activated/de-activated per UE. For instance, if a UE has a very high paging probability, the power saving gain the UE can achieve through LP-WUS monitoring would be minimal. Additionally, if the LP-WUS is transmitted every time a paging message is sent to such UE, it will cause frequency false alarm for other UEs belonging to the same paging subgroup, significantly reducing their power saving gain. The network can increase the power saving gain of UEs by disabling the WUS monitoring for UEs with a high paging probability. Therefore, we propose followings:

Proposal 2	Do not support LP-WUS related configuration using dedicated signalling.

Observation 3	By disabling WUS monitoring for UEs with a high paging probability, the power saving gain of other UEs using LP-WUS can be increased.

Proposal 3	The LP-WUS monitoring can be activated/de-activated per UE, e.g. via RRC Release message.

- Entry/exit condition of LP-WUS monitoring

Regarding the entry/exit condition for LP-WUS monitoring, followings were agreed:

RAN2#126

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

To ensure the UE doesn't miss paging, UE should be allowed to use LP-WUS only when the LP-WUS is sufficiently reliable. LP-SS is being designed to allow UE to measure the channel quality of LP-WUS using LR, and it is transmitted using the same frequency and bandwidth as LP-WUS. Therefore, no matter how strong the channel correlation between SSB and associated LP-WUS is, the reliability of LP-WUS is definitely better indicated by LP-SS measurement using LR rather than SSB measurements using MR.

Observation 4	The LP-SS measurement using LR is better to indicate the reliability of LP-WUS than SSB measurements using MR.

When not monitoring LP-WUS, i.e., before the entry condition is met or after the exit condition is met, the UE may not be required to perform LP-SS measurement using LR. However, the power saving from not performing LP-SS measurements using LR outside the LP-WUS coverage would be minimal, since the target coverage of LP-WUS and LP-SS is the coverage of PUSCH for message3 as described in WID.

Observation 5	Since the cell coverage and the LP-WUS coverage are likely to be similar, the power saving from not performing LP-SS measurements using LR outside the LP-WUS coverage would be marginal.

Therefore, UE may perform LP-SS measurements using LR even when not performing the LP-WUS monitoring to precisely evaluate whether the reception quality of LP-WUS is reliable. Therefore, we propose that if LP-SS based LR measurement is available, the UE determines whether to start LP-WUS monitoring based on the LP-SS based LR measurements, not MR measurements. If UE is not performing the LP-SS based LR measurement, the UE should be able to evaluate the entry condition of using LP-WUS based on the MR measurements.

Observation 6	It is UE implementation whether to perform LP-SS based measurements using LR while MR is awake.

Proposal 4	If the LR measurement is available, UE determines whether to start LP-WUS monitoring based on the LR measurement, not MR measurement.

Proposal 5	If the LR measurement is not available, UE determines whether to start LP-WUS monitoring based on the MR measurements.

MR can wake up while performing LP-WUS monitoring using LR for various reasons, e.g. due to the detection of LP-WUS, for performing RRM measurement, for SI request/reception, or for transitioning to RRC_CONNECTED. While MR is active, LR may not need to monitor LP-WUS. However, except the case when transitioning to RRC_CONNECTED, the activated MR will rapidly accomplish its task, e.g. monitoring PO or RRM measurement, then return to sleep. Hence, UE does not need to stop monitoring LP-WUS in such cases. If LR is turned off while MR is active, and then is turned back on after the MR activity ends, MR cannot transition to sleep mode until LR is ready to monitor LP-WUS. This could delay the entry of MR into sleep state. Therefore, unless significant benefit of stopping LP-WUS monitoring during MR activity is revealed, MR activity should not be specified as an exit condition of LP-WUS monitoring.

Proposal 6	The MR activation, e.g. for measurements, is not specified as the exit condition of LP-WUS monitoring.

Proposal 7	It is UE implementation whether to stop the LP-WUS monitoring based on the MR activation.

- LP-WUS subgrouping

RAN2 agreed that the PEI subgrouping methods are taken as baseline for LP-WUS subgrouping, and it is up to CN to assign the LP-WUS subgroup ID to the UE as in R17 PEI, but the details of the UE_ID based LP-WUS subgrouping is FFS. There is no reason to modify the existing formula for UE_ID based PEI subgrouping to apply it to the LP-WUS subgrouping, so we propose to reuse the existing formula as it is for UE_ID based LP-WUS subgrouping.

Proposal 8	For UE_ID based LP-WUS subgrouping, the existing formula for UE_ID based PEI subgrouping is reused as it is.

In the PEI subgrouping, the CN assigned subgroup is prioritized over the UE_ID based subgroup. That is, UE determines the UE_ID based subgroup ID only when the CN assigned subgroup ID is not assigned, or the serving cell support the UE_ID based subgrouping. There is no difference between the PEI subgrouping and the LP-WUS subgrouping, except they are transmitted on different channel, so the existing principles used for PEI subgrouping can be directly applied to the LP-WUS subgrouping.

Proposal 9	If the LP-WUS subgroup ID is not assigned by CN, or if the serving cell supports only UE_ID based LP-WUS subgrouping, UE uses a LP-WUS subgroup ID based on UE_ID.

Proposal 10	If a LP-WUS subgroup ID is assigned by CN, and if the serving cell supports the CN assigned LP-WUS subgrouping, UE uses the LP-WUS subgroup ID assigned by CN.

The correlation between the LP-WUS subgrouping and the PEI subgrouping is still unclear. The UE supporting both LP-WUS subgrouping and PEI subgrouping will monitor the paging occasion using MR only when both the LP-WUS subgroup ID and the PEI subgroup ID are indicated in LP-WUS and PEI, respectively. Therefore, different subgroup ID for LP-WUS subgrouping and PEI subgrouping can be useful to reduce the false alarm. In addition, the LP-WUS subgrouping is used to determine whether to wake up MR while the PEI subgrouping is used to decide whether to monitor PEI-O/PO using MR. since their usage are different, they do not need to be correlated. Therefore, LP-WUS subgrouping and PEI subgrouping should be configured independently of each other.

Proposal 11	The LP-WUS subgroup and the PEI subgroup are completely independent of each other, e.g. in terms of subgrouping method.

Conclusion

Regarding LP-WUS related configuration:

Observation 1	If UE determines inappropriate entry/exit condition by itself, the power saving gain from LP-WUS or the paging reception performance can be reduced.

Observation 2	Even if LP-WUS is transmitted to have the same coverage as the associated cell, LP-WUS quality can deteriorate while the cell is fine, because they are transmitted on different channels.

Proposal 1	The configuration of the entry/exit condition of LP-WUS monitoring is mandatory present in the LP-WUS related configuration.

Proposal 2	Do not support LP-WUS related configuration using dedicated signalling.

Observation 3	By disabling WUS monitoring for UEs with a high paging probability, the power saving gain of other UEs using LP-WUS can be increased.

Proposal 3	The LP-WUS monitoring can be activated/de-activated per UE, e.g. via RRC Release message.

Regarding entry/exit condition for LP-WUS monitoring:

Observation 4	The LP-SS measurement using LR is better to indicate the reliability of LP-WUS than SSB measurements using MR.

Observation 5	Since the cell coverage and the LP-WUS coverage are likely to be similar, the power saving from not performing LP-SS measurements using LR outside the LP-WUS coverage would be marginal.

Observation 6	It is UE implementation whether to perform LP-SS based measurements using LR while MR is awake.

Proposal 4	If the LR measurement is available, UE determines whether to start LP-WUS monitoring based on the LR measurement, not MR measurement.

Proposal 5	If the LR measurement is not available, UE determines whether to start LP-WUS monitoring based on the MR measurements.

Proposal 6	The MR activation, e.g. for measurements, is not specified as the exit condition of LP-WUS monitoring.

Proposal 7	It is UE implementation whether to stop the LP-WUS monitoring based on the MR activation.

Regarding LP-WUS subgrouping:

Proposal 8	For UE_ID based LP-WUS subgrouping, the existing formula for UE_ID based PEI subgrouping is reused as it is.

Proposal 9	If the LP-WUS subgroup ID is not assigned by CN, or if the serving cell supports only UE_ID based LP-WUS subgrouping, UE uses a LP-WUS subgroup ID based on UE_ID.

Proposal 10	If a LP-WUS subgroup ID is assigned by CN, and if the serving cell supports the CN assigned LP-WUS subgrouping, UE uses the LP-WUS subgroup ID assigned by CN.

Proposal 11	The LP-WUS subgroup and the PEI subgroup are completely independent of each other, e.g. in terms of subgrouping method.