3GPP TSG-RAN WG2 Meeting #127			         R2-2407157

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	8.4.3

Source: 	LG Electronics Inc.

Title:         	RRM relaxation and RRM offloading

Document for: 	Discussion and Decision

Introduction

Regarding the RRM relaxation and offloading, RAN2 agreed followings in the last meeting:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

In this contribution we will discuss the criteria/condition for neighbor cell measurement relaxation, serving cell measurement relaxation, and serving cell measurement offloading.

Discussion

- Neighbor cell measurement relaxation

For R16 measurement relaxation on neighbor cell, two relaxed measurement criteria were introduced as follows:

- Relaxed measurement criterion for UE with low mobility

- (SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

- Srxlev = current Srxlev value of the serving cell (dB).

- SrxlevRef = reference Srxlev value of the serving cell (dB),

- Relaxed measurement criterion for UE not at cell edge

- Srxlev > SSearchThresholdP, and,

- Squal > SSearchThresholdQ, if SSearchThresholdQ is configured,

UE performs measurements on neighbor cells using relaxed measurement requirement, if at least one of above criteria is met.

For R19 further relaxed measurement on neighbor cell, following three options can be considered:

- Option 1. Defining a new criterion

- Option 2. Defining a new measurement requirement, i.e. further relaxed measurement requirement

- Option 3. Enhancing the existing criteria so that they can be evaluated based on the LR measurements

The first option is defining an additional criterion beyond the R16 criteria, allowing UE to perform measurements using the existing relaxed requirement defined by RAN4 in R16. However, since criteria for measurement relaxation were thoroughly discussed in R16, it seems very challenging to define an additional criterion that really help with further relaxed measurement.

Alternatively, an additional threshold can be introduced in the existing criteria, which is associated with further relaxed measurement requirement. That is, if the existing criterion with the additional threshold is met, UE is allowed to apply further relaxed measurement requirements. However, to achieve this, RAN4 should specify the further relaxed measurement requirements for intra-frequency/inter-frequency/inter-RAT cells. This is out of RAN2 scope.

The option 3 is to reuse the R16 relaxation criteria and the R16 relaxed measurement requirement. The R16 relaxation criteria are based on the MR measurement of serving cell. If the serving cell measurement is fully offloaded to LR, the UE has no MR measurements for serving cell and cannot evaluate the R16 relaxation criteria. If R16 criteria are modified so that UE can evaluate them based on the LR measurements, the UE can relax the neighbor cell measurements even while the serving cell measurement is fully offloaded to LR. In general, the condition for serving cell measurement offloading to LR will be more stringent than that for relaxing neighbor cell measurement. That is, if the condition for serving cell measurement offloading to LR is met, the condition for neighbor cell measurement relaxation is also likely to be met. Therefore, the UE should be able to evaluate the relaxed measurement criteria for neighbor cell based on the LR measurements for serving cell.

The neighbor cell measurements already can be relaxed sufficiently by R16 relaxed measurement. The option 3 seems the best option RAN2 can take for further measurement relaxation related to LP-WUS/LR. Therefore, we propose followings:

Proposal 1	For further neighbor cell measurement relaxation, do not define additional criterion over the existing criteria, i.e. one for UE with low mobility and another for UE not at cell edge.

Proposal 2	For further neighbor cell measurement relaxation, enhance the existing criteria, i.e. one for UE with low mobility and another UE not at cell edge, to enable UE to evaluate them based on the LR measurements when the serving cell measurement is offloaded to LR.

- Serving cell measurement relaxation

UE in RRC_IDLE/INACTIVE is required to measure serving cell more frequently than any neighbor cells for cell re-selection, so if the serving cell measurement can be relaxed, i.e. measurement with longer periodicity, it would be very effective for UE power saving. In Rel-16 neighbor cell measurement relaxation, RAN4 defined relaxed measurement requirements for intra-frequency, inter-frequency, and inter-RAT cells, while RAN2 specified two criteria, i.e. one for UE with low mobility and another for UE not at the cell edge, to determine whether to apply the relaxed measurement requirement.

Since the serving cell measurement is more critical than neighbor cell measurement, more stringent conditions may be required for serving cell measurement relaxation compared to neighbor cell measurement. However, this can be done by configuring higher/lower thresholds in the existing criteria without modifying them. Therefore, it is proposed for the serving cell measurement relaxation to reuse the R16 relaxed measurement criteria.

Proposal 3	For serving cell measurement relaxation, reuse the existing criteria for neighbor cell measurement relaxation, i.e. ‘UE with low mobility’ and ‘UE not at cell edge’.

Proposal 4	The discussion/decision on the relaxed measurement requirement for serving cell is up to RAN4.

- Serving cell measurement offloading

If the serving cell measurement is offloaded to LP-WUR, the UE no longer obtains MR measurement results for serving cell, which is more accurate than LR measurements, so UE should be allowed to offload serving cell measurement to LR only when more stringent conditions are met, compared to the serving cell measurement relaxation.

In RRC_IDLE/INACTIVE state, the UE periodically performs serving cell and neighbour cell measurements for cell reselection according to measurement rules. The measurement interval for serving cell varies depending on the SMTC periodicity, DRX cycle, and UE’s power class. When the interval is shortest, the serving cell measurement should be performed at least once every 2.56 seconds. In contrast, the minimum measurement period for neighbour cells on the intra-frequency is 10.24 seconds, and for inter-frequency neighbour cells, it is 15.36 seconds. Therefore, if serving cell measurements are offloaded to LR, MR can maintain ultra-deep sleep for at least 10 seconds.

The target cell that the UE needs to measure depends on the quality of the serving cell, and the measurement cycle varies according to the frequency priority of the target cells. Hence, the duration that MR can sleep without neighbour cell measurements varies based on the quality of the serving cell.

Table 1 Measurement rules for cell re-selection

| Serving cell quality | Measurement target |
| --- | --- |
| If Srxlev > SIntraSearchP and Squal > SIntraSearchQ | Serving cell + Higher priority cells |
| If Srxlev < SIntraSearchP or Squal < SIntraSearchQ, and if Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Serving cell + Higher priority cells + intra-frequency neighbour cell |
| If Srxlev < SnonIntraSearchP and Squal < SnonIntraSearchQ | Serving cell + Higher priority cells + intra- frequency + inter-frequency with an equal or lower priority |

Even if the quality of the serving cell is excellent (i.e., if the serving cell fulfills Srxlev > SIntraSearchP and Squal > SIntraSearchQ), periodic measurements of higher priority cells should be made using MR. However, once a higher priority cell has been measured but not reselected, the UE does not need to measure that cell for 60 seconds thereafter. Therefore, while UE is required to measure only serving cell and higher priority cells, great power saving can be achieved through the serving cell measurement offloading to LR.

In the cell reselection to a higher priority frequency, the UE selects the highest ranked cell among the cells on the highest priority frequency(ies). Since it is not based on the cell-ranking criterion of the serving cell, the UE is not required to perform ranking using LR measurements though the serving cell measurement is offloaded to LR while the serving cell fulfills Srxlev > SIntraSearchP and Squal > SIntraSearchQ.

Observation 1	If the serving cell measurement is offloaded to LR while the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ,

- a great power saving can be achieved by allowing MR to sleep for 60 seconds, and

- it does not affect the cell-ranking criterion Rs.

If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE should perform measurements on identified cells on the intra-frequency using MR every 10.24 to 30.72 seconds. Consequently, MR needs to wake up much more frequently than when UE is not required to perform intra-frequency measurements, and the UE power saving gain achievable through the serving measurement offloading to LR is significantly reduced.

The intra-frequency and equal priority inter-frequency cell reselection is performed based on comparing the ranking of serving cell and neighbour cells on intra-/equal priority inter-frequency. This means the UE needs to calculate ranking of serving cell using LR measurements and compare it with MR measurements based ranking of neighbour cells to select the highest ranked cell among the serving cell and neighbour cells. If the target cell for reselection is selected based on the relatively inaccurate LR measurement results, the UE may perform incorrect cell reselection.

Observation 2	If the serving cell measurement is offloaded to LR while the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ,

- The power saving gain is significantly reduced, compared to the case when Srxlev > SIntraSearchP and Squal > SIntraSearchQ is satisfied, and

- the highest ranked cell is selected based on LR measurements and it may lead to incorrect cell reselection.

The mobility performance should not be compromised for power saving, and the mobility decision should be made based on reliable MR measurements. Therefore, when UE initiates (or before initiating) intra-frequency measurements, the serving cell measurement offloading to LR should be halted, and MR should measure serving cell.

Proposal 5	The serving cell RRM measurement can be offloaded to LR only while the UE is not required to perform intra-frequency measurements.

Proposal 6	Do not support cell-ranking based on LR measurements.

Conclusion

Regarding the neighbor cell measurement relaxation

Proposal 1	For further neighbor cell measurement relaxation, do not define additional criterion over the existing criteria, i.e. one for UE with low mobility and another for UE not at cell edge.

Proposal 2	For further neighbor cell measurement relaxation, enhance the existing criteria, i.e. one for UE with low mobility and another UE not at cell edge, to enable UE to evaluate them based on the LR measurements when the serving cell measurement is offloaded to LR.

Regarding the serving cell measurement relaxation

Proposal 3	For serving cell measurement relaxation, reuse the existing criteria for neighbor cell measurement relaxation, i.e. ‘UE with low mobility’ and ‘UE not at cell edge’.

Proposal 4	The discussion/decision on the relaxed measurement requirement for serving cell is up to RAN4.

Regarding the serving cell measurement offloading

Observation 1	If the serving cell measurement is offloaded to LR while the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ,

- a great power saving can be achieved by allowing MR to sleep for 60 seconds, and

- it does not affect the cell-ranking criterion Rs.

Observation 2	If the serving cell measurement is offloaded to LR while the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ,

- The power saving gain is significantly reduced, compared to the case when Srxlev > SIntraSearchP and Squal > SIntraSearchQ is satisfied, and

- the highest ranked cell is selected based on LR measurements and it may lead to incorrect cell reselection.

Proposal 5	The serving cell RRM measurement can be offloaded to LR only while the UE is not required to perform intra-frequency measurements.

Proposal 6	Do not support cell-ranking based on LR measurements.