3GPP TSG-RAN WG2 Meeting #127			         R2-2407159

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	8.5.3

Source: 	LG Electronics Inc.

Title:         	On-demand transmission of SIB1

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss all remaining issues to complete the study on on-demand SIB1, such as need of WUS configuration acquisition from NES cell, need of SIB1 validity, and whether/how to support case 3 defined in RAN1.

Discussion

- OD-SIB1 before camping on the NES cell (Case 2 & 3)

In the last meeting, RAN2 discussed the case 3 (where UE transmits UL WUS to Cell A and receives SIB1of NES Cell from Cell A) but failed to reach any decision.

RAN2#126

Proposal 1 (modified): RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

- Postponed.

However, RAN1 agreed to prioritize Case2 and Case 3.

RAN1#117

Agreed to prioritize RAN1 discussions on Case 2 (Options 1+B+X) and Case 3 (Options 2+B+Y) where the options 1/2/A/B/X/Y are defined below:

    - On target cell of UL WUS transmission:

      - Option 1: UE transmits UL WUS to NES Cell

      - Option 2: UE transmits UL WUS to Cell A

    - On configuration provision for UL WUS transmission

      - Option A: UE obtains the UL WUS configuration from NES Cell

      - Option B: UE obtains the UL WUS configuration from Cell A

    - On receiving of SIB1

      - Option X: UE receives on-demand SIB1 from NES Cell

      - Option Y: UE receives on-demand SIB1 from Cell A

Before discussing Case 3, to clarify each Case, we need to clearly define Cell A and NES Cell from the perspective of RAN2. The definition of Cell A defined by RAN 1 is a cell that is periodically transmitting at least its own SIB1. UE in RRC_IDLE/INACIVE can receive SI message from serving cell only. Hence, from RAN2 perspective, the Cell A in the case 2 and 3 should be the serving cell for the UE. UE in RRC_IDLE/INACTIVE should not be required to receive SI message from a neighbor cell to acquire the WUS configuration. As RAN2 agreed that UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES Cell, the NES Cell in the case 2 and case 3 should be a neighbor cell. Therefore, we propose to clarify the ‘Cell A’ and ‘NES cell’ in multi-carrier scenarios as follows:

Proposal 1	In Case 2 and 3 defined by RAN1, ‘Cell A’ is serving cell, and ‘NES Cell’ is neighbour cell.

For SIB1 request before camping on NES cell in multi-carrier scenario, RAN2 agreed to start study on Case2 (defined by RAN1) that the UE receives the WUS configuration from Cell A, transmits WUS to NES cell, and receives SIB1 from the NES cell. In case 3, the NES cell does not need monitor UL WUS transmitted by UE and does not need to wake up even for SIB1 transmission, so much NES gain can be achieved compared to case 2. Since the Cell A in case 3 is in active state, the transmission of the NES cell’s SIB1 will not significantly increase the energy consumption of Cell A. One thing to consider in case 3 is that Cell A should keep the up-to-date SIB1 of the NES cell. It may require additional signaling between the Cell A and the NES cell. However, if they belong to the same gNB, Cell A can obtain SIB1 of the NES cell without inter-node signaling.

Observation 1	If Cell A and NES cell belong to the same gNB, much NES gain can be achieved in case 3 than in case 2, without any increase in complexity of NW/UE.

In the inter-gNB scenario, Case 2, which RAN2 have already agreed to support, might be more appropriate. Therefore, we propose to complete the study on Case2 and 3 in this meeting, and discuss both cases in the WI phase.

Proposal 2	For on-demand SIB1 before camping on the NES Cell, discuss both case 2 and case 3 in WI phase.

In Case 3, Cell A should be able to transmit SIB1 of NES Cell when it is requested by UE. If SIB1 of NES Cell is contained in SIB X of Cell A, UE can request SIB1 of NES Cell using the existing SI request procedure. Although the target cell to which the UE transmits UL WUS differs in Case2 and 3, the triggering condition of WUS transmission does not need to be different. the triggering condition of WUS transmission RAN2 has discussed for Case 2 can be directly applied to Case 3.

Proposal 3	In case 3, SIB1 of NES cell is transmitted via SIB X of Cell A.

Proposal 4	In case 3, UE requests SIB1 of NES Cell using the existing SI request procedure from Cell A.

A single Cell A may be associated with both a NES Cell belonging to the same gNB and another NES Cell belonging to neighbor gNB. If observation 1 is acceptable, a single Cell A should be able to simultaneously support Case 2 and 3, and UE needs to know whether the NES Cell of which it wants to receive SIB1 belongs to Case2 or Case 3.

Proposal 5	A single Cell A can simultaneously support Case 2 and Case 3.

If P1 is agreed, UE performs the random access on neighbor cell, i.e. NES Cell, in Case 2 to request SIB1 transmission. Until now, there has been no case where UE in RRC_IDLE/INACTIVE performs the random access procedure on neighbor cell. UE in RRC_IDLE/INACTIVE should monitor periodically the serving cell for paging reception. Since the paging reception is more important than acquisition of neighbor cell’s SIB1, the UL WUS transmission in Case 2 should not interfere with the paging reception from serving cell.

Proposal 6	In Case2, the UL WUS transmission to NES Cell should not interfere with paging reception from Cell A.

- OD-SIB1 after camping on the NES Cell

RAN2 has discussed on-demand SIB only for the scenario that UE requests SIB1 before camping on the NES cell. However, since SIB1 also contains the scheduling of other system information, radio resource configuration information that is common for all UEs and barring information, UE in RRC_IDLE/INACTIVE needs to request and acquire SIB1 even after camping on the NES cell. For instance, if the UE wants to acquire a SI message after camping on the NES cell, the UE should first request and acquire SIB1 of the NES serving cell to check whether the SI message is being broadcast or not. Even when an SI is updated, the NES Cell may not transmit SIB1 if no UL WUS is received. Then, upon receiving SI change indication from serving NES Cell, the UE needs to request SIB1 to check whether SI of interest is changed or not.

Observation 2	After camping on a NES cell, the UE still needs to request SIB1 of the serving NES cell, e.g. when SI request is triggered or SI is updated.

Therefore, RAN2 also needs to study and support the case that UE requests SIB1 of a serving NES cell after camping on the NES Cell. After camping on NES Cell, UE can obtain the WUS configuration from the serving NES Cell via SI message, and we need to study only following case. The different from Case 1 is NES Cell in Case1 is a neighbor cell but NES Cell in this case is a serving cell.

- Case 5: UE obtains the WUS configuration from serving NES Cell, transmits UL WUS to serving NES Cell, and receives on-demand SIB1 from serving NES Cell.

Proposal 7	For on-demand SIB1 after camping on NES Cell, define case 5 that UE obtains the WUS configuration from serving NES Cell, transmits WUS to serving NES Cell, and receives on-demand SIB1 from serving NES Cell.

The NES Cell can provide the WUS configuration required for requesting its SIB1 via its SIB. The WUS configuration can be updated, so UEs camping on the NES Cell should keep the up-to-date WUS configuration of the serving NES Cell.

Proposal 8	NES cell provides the WUS configuration required for requesting its SIB1 via its SIB Y.

- SIB1 validity

In the last meeting, RAN2 discussed whether to support SIB1 validity to prevent UE from requesting SIB1 transmission if it has valid one. To enable UE to check the validity of the network configuration, we generally use validity timer, value tag, and/or validity area. If the timer based validity check is applied to the on-demand SIB1, the network cannot update SIB1 for a predetermined period of time, i.e. while the validity timer is running. Since SIB1 includes dynamically changing information like the value tag of other SIBs, the validity timer is not suitable.

The only feasible solution seems to be a validity check using the value tag. If P3 is agreed, i.e. if SIB1 of NES Cell is transmitted via SIB X of Cell A in Case 3, the validity of SIB1 of NES Cell can be checked by the value tag of SIB X. That is, no enhancement is needed for SIB1 validity in Case 3. The value tag in MIB seems not acceptable, so for Case 2, we can consider the value tag in WUS configuration. However, NES Cell is required to inform Cell A of the value tag of SIB1 whenever it is updated. Considering that a single NES Cell can be associated with more than one Cell A, this also does not seem acceptable.

In Case 2 or 5, UE would not frequently request SIB1 of NES Cell, if RAN2 well defines the triggering condition of WUS transmission. For example, UE can be allowed to send the WUS only when the NES Cell is the best cell. Since the cost of ensuring the SIB1 validity in Case 2 or 5 is too high, it seems better to focus on making the condition for UE to send the UL WUS as strict as possible, rather than supporting SIB1 validity in an effort to minimize the SIB1 transmission of NES Cell.

Observation 3	If SIB1 of NES Cell is transmitted via SIB X of CellA in Case 3, the SIB 1 validity can be check using the existing value tag.

Proposal 9	Do not support SIB1 validity for Case 2 and 5.

Conclusion

Regarding OD-SIB1 before camping on the NES cell

Proposal 1	In case 2 and 3 defined by RAN1, ‘Cell A’ is serving cell, and ‘NES Cell’ is a neighbour cell.

Observation 1	If Cell A and NES cell belong to the same gNB, much NES gain can be achieved in case 3 than in case 2, without any increase in complexity of NW/UE.

Proposal 2	For on-demand SIB1 before camping on the NES Cell, discuss both case 2 and case 3 in WI phase.

Proposal 3	In case 3, SIB1 of NES cell is transmitted via SIB X of Cell A.

Proposal 4	In case 3, UE requests SIB1 of NES Cell using the existing SI request procedure from Cell A.

Proposal 5	A single Cell A can simultaneously support Case 2 and Case 3.

Proposal 6	In Case2, the UL WUS transmission to NES Cell should not interfere with paging reception from Cell A.

Regarding OD-SIB1 after camping on the NES Cell

Observation 2	After camping on a NES cell, the UE still needs to request SIB1 of the serving NES cell, e.g. when SI request is triggered or SI is updated.

Proposal 7	For on-demand SIB1 after camping on NES Cell, define case 5 that UE obtains the WUS configuration from serving NES Cell, transmits WUS to serving NES Cell, and receives on-demand SIB1 from serving NES Cell.

Proposal 8	NES cell provides the WUS configuration required for requesting its SIB1 via its SIB Y.

Regarding SIB1 validity

Observation 3	If SIB1 of NES Cell is transmitted via SIB X of CellA in Case 3, the SIB 1 validity can be check using the existing value tag.

Proposal 9	Do not support SIB1 validity for Case 2 and 5.