# 3GPP TSG RAN WG2 Meeting #127                          	   R2-2407161

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.5.2

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SSB SCell Operation

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The WID for “Enhancements of network energy savings for NR” was approved at RAN #102, and further updated at RAN # 103 with the follows [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss on-demand SSB transmission scenarios and on-demand SSB cases for on-demand SSB Scell operations and the potential RAN2 specification work.

# Discussion

## Use cases and Scenarios for On-demand SSB

At RAN1 #116, the following aggreements were made on the following use cases and scenarios for further study [2].

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission

Additionally, the following scenarios with Case #1 (no always-on SSB) and Case #2 (always-on SSB) respectively were agreed at RAN1 #116-bis [3].

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

On-demand SSB may be cell defining (i.e., SSB with associated SIB1) or non-cell defining (i.e., SSB without associated SIB1).

At RAN1#116-bis, the following agreement was made for always-on SSB (Case #2) and no always-on SSB (Case #1).

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, down select between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

However, it is still not concluded if on-demand SSB can be cell defining or non-cell defining or both for no always-on SSB (Case #1).

The WID has a note that “On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation.”

Furthermore, on-demand SSB in Case #1 (no always-on) is expected to be transmitted in a bursty or semi-persistent manner over a short period. The bursty or semi-persistent cell-defining SSB is not useful for both legacy UEs’ and Release 19 UEs’ operations, at RRC Idle or Inactive state, which rely on periodic cell-defining SSB.

On the other hand, non-cell defining on-demand SSB has no impact to both legacy UEs’ and Release 19 UEs’ operations at RRC Idle/Inactive state.

Observation 1: Cell-defining on-demand SSB for Case #1 (no always-on SSB) has negative impact to both legacy UEs’ and Release 19 UEs’ operations at RRC Idle or Inactive state.

Proposal 1: RAN2 may wait for RAN1’s further concludes before discussing on-demand SSB’s impact to UE operations at RRC Idle or Inactive state.

## Indication for On-demand SSB transmission

On-demand SSB is transmitted when it is triggered, e.g., for SCell time/frequency synchronization, L1/L3 measurements, SCell selection or activation, etc.

There are two options for triggering on-demand SSB transmissions: triggered by the network or triggered by a UE.

At RAN1#116, the option of network triggering was agreed [2].

Agreement Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE

Additionally, the following options were agreed at RAN1 #116-bis [3] for on-demand SSB signaling from gNB.

Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. FFS: Details on On-demand SSB transmission indication

Furthermore, the following agreement on gNB’s indication of on-demand SSB transmission was agreed at RAN1 #117 [4].

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. FFS: Scenarios where the above signalings are applicable

Figure 1: On-demand SSB Cases and Scenarios

As shown above in Figure 1, to support on-demand SSB, UE may expect on-demand SSB transmissions at least for the following on-demand SSB transmission scenarios for both on-demand SSB cases.

- On-demand SSB Case #1 (i.e., without always-on SSB)

- On-demand SSB transmission Scenarios 2: before UE receives SCell activation command

- On-demand SSB transmission Scenario 3A: after UE receives SCell activation command

- On-demand SSB transmission Scenario 3B: after SCell is activated

- On-demand SSB Case #2 (i.e., with always-on SSB)

  - On-demand SSB transmission Scenario 3A: after UE receives SCell activation command

To support on-demand SSB transmission Scenario 3A for both on-demand SSB Case #1 and Case #2, a single signaling in which both SCell activation/deactivation and on-demand SSB transmission indication (i.e., Option 2) is more efficient to align Scell activation/deactivation and on-demand SSB transmission in time.

To support on-demand SSB transmission Scenario 2 or Scenario 3B for on-demand SSB Case #2, separate signaling between SCell activation/deactivation and on-demand SSB transmission indication (i.e., Option 1) is needed.

Observation 2: Extending the current Scell activation MAC CE for OD-SSB transmission indication is more efficient for supporting SSB transmission Scenario 3A for both on-demand SSB Case #1 and Case #2.

Observation 3. A new MAC CE for OD-SSB transmission indication is needed for supporting on-demand SSB transmission Scenario 2 or Scenario 3B for on-demand SSB Case #2.

Proposal 2.  RAN2 discusses the following options for MAC-CE based signaling to indicate on-demand SSB transmission for both on-demand SSB Case #1 and case #2.

- Option 1. Extending the current Scell activation MAC CE for OD-SSB transmission indication.

- Option 2. New MAC CE for OD-SSB transmission indication.

# Conclusion

In this contribution, we discussed on-demand SSB transmission scenarios and on-demand SSB cases for on-demand SSB Scell operations and potential RAN2 specification work and concluded with the following observations and proposals.

Use cases and scenarios for On-demand SSB

Observation 1: Cell-defining on-demand SSB for Case #1 (no always-on SSB) has negative impact to both legacy UEs’ and Release 19 UEs’ operations at RRC Idle or Inactive state.

Proposal 1: RAN2 may wait for RAN1’s further concludes before discussing on-demand SSB’s impact to UE operations at RRC Idle or Inactive state.

Indication for on-demand SSB transmission

Observation 2: Extending the current Scell activation MAC CE for OD-SSB transmission indication is more efficient for supporting SSB transmission Scenario 3A for both on-demand SSB Case #1 and Case #2.

Observation 3. A new MAC CE for OD-SSB transmission indication is needed for supporting on-demand SSB transmission Scenario 2 or Scenario 3B for on-demand SSB Case #2.

Proposal 2.  RAN2 discusses the following options for MAC-CE based signaling to indicate on-demand SSB transmission for both on-demand SSB Case #1 and case #2.

- Option 1. Extending the current Scell activation MAC CE for OD-SSB transmission indication.

- Option 2. New MAC CE for OD-SSB transmission indication.

# References

[1] RP-240170 “Revised WID for Enhancements of network energy savings for NR”, RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024.

[2] “Chair notes RAN1#116 eom0” RAN1 #116	 Athens, Greece, February 26th – March 1st, 2024.

[3] “Chair notes RAN1#116bis eom0” RAN1 #116bis	 Changsha, China, April 15th – 19th, 2024

[4] “Chair notes RAN1#117 eom0” RAN1 #117 Fukuoka, Japan, May 20th – 24th, 2024, 2024