# 3GPP TSG RAN WG2 Meeting #127                          	   R2-2407162

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.5.3

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The WID for “Enhancements of network energy savings for NR” was approved at RAN #102, and further updated at RAN # 103 with the follows [1].

The objectives of the work item are the following: - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

In this contribution, we discuss scenarios or use cases for on-demand SIB1 and potential impact to specification work.

# Discussion

## RAN1 Case 3

For on-demand SIB1 for UEs in Idle or Inactive state, the following agreements were made at RAN1 #116 for further study [2].

Agreement For discussion purpose, the following assumption will be used in RAN1 - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options. On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A Other options are not precluded.

Furthermore, the following cases were agreed at RAN1 #115-bis [3], as illustrated in Figure 1.

Agreement For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A

Figure 1: RAN1 Cases for On-demand SIB1

At RAN2 #123-bis [4], the following agreement was made for on-demand scenarios based on RAN1’s agreement made at RAN1 #116 [2].

Agreement Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed.

At RAN2 #125-bis [5], the scenarios illustrated in Figure 2 were discussed and the following agreement was made.

Figure 2: RAN2 Cases for On-demand SIB1

Agreements on on-demand SIB1:

- At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

Comparing with RAN1’s cases, RAN2’s Scenario 1a is the same as RAN1’s Case 2, RAN2’s Scenario 3 is the same as RAN1’s Case 1, and RAN2’s Scenario 1b is similar to RAN1’s Case 3 except that NES Cell’s on-demand SIB1 is transmitted from Cell A (not NES Cell as in Scenario 1b).

For the following options for supporting on-demand SIB1 of a NES Cell.

- Option 1 (RAN2 Scenario 1b): A UE transmits UL WUS to a Cell A for a NES Cell’s on-demand SIB1 and receives the NES Cell’s on-demand SIB1 from the NES Cell.

- Option 2 (RAN1 case 3): A UE transmits UL WUS to a Cell A for a NES Cell’s on-demand SIB1 and receives the NES Cell’s on-demand SIB1 from the Cell A.

Figure 3: Options for On-demand SIB1

For Option 1 (RAN2 Scenario 1b) as illustrated in Figure 3, the Cell A receiving an uplink WUS for on-demand SIB1 of a NES Cell needs to communicate the on-demand SIB1 request to the NES Cell where the communication may depend on whether the Cell A and NES Cell share the same CU or not. In this case, the NES Cell doesn’t need to monitor uplink WUS signals from different UEs, but still needs to transmit the on-demand SIB1 to different UEs accordingly.

For Option 2 (RAN1 case 3) as illustrated in Figure 1, a UE may directly communicate with the Cell A for the on-demand SIB1(s) of NES Cell(s) which may require the least specification work. In this case, the NES Cell(s) may have the most power saving since it doesn’t need to monitor uplink WUS from different UEs and transmit the on-demand SIB1 to different UEs respectively. In this case, the Cell A needs to know NES Cells’ SIB1s which may result more traffic on F1 or Xn interface (e.g., traffic with SIB1 contents).

Observation 1: For the Option 1 where A UE transmits UL WUS to a Cell A for a NES Cell’s on-demand SIB1 and receives the NES Cell’s on-demand SIB1 from the NES Cell, it may still require the specification work for uplink WUS and cause some power consumption for a NES Cell transmitting on-demand SIB1.

Observation 2: For the Option 2 where a UE transmits UL WUS to a Cell A for NES Cell’s on-demand SIB1 and receives one or more NES Cells’ on-demand SIB1s from the Cell A, it may require least specification work and save more power for a NES Cell.

Proposal 1: RAN2 discusses how to support the RAN1 case 3 and sends an LS to RAN1 and RAN3 based on the decision.

For RAN1 case 3, one or more NES cells’ on-demand SIB1s may be transmitted by Cell-A via a new SIBx (i.e., a container for NES cell’s SIB1). In this case, on-demand SIB1 for RAN1 case 3 is very similar to the current on-demand SI. For example, the current on-demand SI configuration in Cell-A SIB1 may be extended to support on-demand SIBx which is the container of a NES cell’s SIB1.

Another option can be a unified design for UL WUS configuration for both RAN2’s scenario 1a and RAN1’s case 3 with a new SIBy.

Proposal 2: RAN2 discusses the following options for supporting RAN1 case 3 and leave the design details to stage 3.

Option 1: Extending the current on-demand SI configuration in Cell-A’s SIB1 for supporting a NES cell’s on-demand SIB1 for RAN1 Case 3.

Option2: Add a new SIB for UL WUS configuration for supporting a NES cell’s on-demand SIB1 in both RAN2’s scenario 1a and RAN1’s case 3.

## SIB1 validation

For on-demand SIB1, a UE needs to send UL WUS to trigger NES cell’s on-demand SIB1 transmission which consumes power for both network and UE. To avoid unnecessary on-demand SIB1 operation, e.g., UE had acquired the on-demand SIB1 of a NES cell (i.e., NES1) while camped on a Cell-A or the NES cell (NES1) and UE desires to reselect or reenter the NES cell (NES1) while currently camping on a Cell-A or another NES cell (i.e., NES2), the UE may use its stored on-demand SIB1 of the NES cell (NES1) if it’s still valid.

Observation 3: On-demand SIB1 validation is beneficial for both network’s and UE’s power saving when a UE wants to reselect or reenter a NES cell using its stored on-demand SIB1.

Proposal 3: Support on-demand SIB1 validation.

Currently, SI validation is supported via valueTag in SIB1. If valueTag based validation is extended to the on-demand SIB1 of a NES cell (NES1), a UE needs to check the valueTag at the Cell-A or NES cell (NES2) that the UE is currently camped, this requires that the Cell-A or the NES cell (NES2) can support valueTag.

Proposal 4: RAN2 discusses how to supports on-demand SIB1 validation.

## Uplink WUS acquisition at NES cell

For on-demand SIB1, a UE needs to send UL WUS to trigger NES cell’s on-demand SIB1 transmission. Currently, it’s agreed that a Cell-A may provide UL WUS configuration for a UE camped at the Cell-A. However, if the UE is camped at a NES cell (i.e., NES1) and wants to select a best cell which is a NES cell (i.e., NES2), the UE has to go to a Cell-A to acquire UL WUS configuration in order to trigger the on-demand SIB1 transmission from the NES cell (NES2). Always returning to Cell A for UL WUS is not efficient and may cause more power consumption for both network and UE.

Observation 4: UL WUS acquisition at a NES cell is beneficial for both network’s and UE’s power saving when a UE is camped on a NES cell (i.e., NES1) and wants to select or enter another NES cell (i.e., NES2).

Additionally, when a UE leaves a NES cell that it’s currently camped, the UE may receive the latest UL WUS configuration for the NES cell’s on-demand SIB1 via the NES cell’s SIB or RRC release. Since the UL WUS configuration is not expected to change frequently, the UE may store the UL WUS for triggering on-demand SIB1 when the UE wants to reselect or reenter the NES cell at a later time if its stored on-demand SIB1 is not valid.

Proposal 5: Support that a NES cell provides UL WUS configuration via SI or RRC release where the UL WUS configuration includes the UL WUS configuration for its own and/or other NES cells.

# Conclusion

In this contribution, we discussed scenarios or options for on-demand SIB1 operation and concluded with the following observations and proposals.

RAN1 Case 3

Observation 1: For the Option 1 where A UE transmits UL WUS to a Cell A for a NES Cell’s on-demand SIB1 and receives the NES Cell’s on-demand SIB1 from the NES Cell, it may still require the specification work for uplink WUS and cause some power consumption for a NES Cell transmitting on-demand SIB1.

Observation 2: For the Option 2 where a UE transmits UL WUS to a Cell A for NES Cell’s on-demand SIB1 and receives one or more NES Cells’ on-demand SIB1s from the Cell A, it may require least specification work and save more power for a NES Cell.

Proposal 1: RAN2 discusses how to support the RAN1 case 3 and sends an LS to RAN1 and RAN3 based on the decision.

## On-demand SIB1 validation

Observation 3: On-demand SIB1 validation is beneficial for both network’s and UE’s power saving when a UE wants to reselect or reenter a NES cell using its stored on-demand SIB1.

Proposal 3: Support on-demand SIB1 validation.

Proposal 4: RAN2 discusses how to supports on-demand SIB1 validation.

Uplink WUS acquisition at NES cell

Observation 4: UL WUS acquisition at a NES cell is beneficial for both network’s and UE’s power saving when a UE is camped on a NES cell (i.e., NES1) and wants to select or enter another NES cell (i.e., NES2).

Proposal 5: Support that a NES cell provides UL WUS configuration via SI or RRC release where the UL WUS configuration includes the UL WUS configuration for its own and/or other NES cells.

# References

[1] RP-240170 “Revised WID for Enhancements of network energy savings for NR”, RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024.

[2] “Chair notes RAN1#116 eom0”, RAN1 #116,	 Athens, Greece, February 26th – March 1st, 2024.

[3] “Chair notes RAN1#116bis eom0”, RAN1 #116bis	, Changsha, Hunan Province, China, April 15th – 19th, 2024.

[4] “RAN3_123bis_agenda_20240419_EOM”, RAN3 meeting #123bis, Changsha, Hunan Province, China, April 2024

[5] “R2_125bis_SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM”, RAN2 #125bis, Changsha, Hunan Province, China, April 15th – 19th, 2024.

[6] “R2_126_R18 SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM”, RAN2 #126, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.