3GPP TSG-RAN WG2 Meeting #127	R2-2407164

Maastricht, Netherlands, August 19th -23rd, 2024

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Remaining issues in LCM for UE-sided model

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the following remaining aspects to progress this topic.

- FFS points from RAN2 post email discussion

- NW-side additional condition

- Performance monitoring

# FFS points from RAN2 post email discussion

Post RAN2#126 meeting, RAN2 had a long email discussion to progress LCM discussion for UE sided model, i.e., [POST126][032][AI/ML PHY] LCM (Intel/Samsung). The definition of functionalities and basic LCM procedure were concluded as a baseline.

In this contribution, we provide our view on remaining FFSs in order to identify information/discussion that need to progress LCM design.

FFS #1: optionality of NW- side additional condition

Proposal 2: “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) network may provide NW-side additional condition FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS the content of configuration.

Optionality means that UE needs to provide applicable functionality without NW-side additional condition. Given that NW-side additional condition is introduced to ensure consistency between training and inference, it is not clear how UE identifies applicable functionality that can provide reasonable performance with UE side model. In our view, RAN1 may decide on a default behavior if feasible. Therefore, it is reasonable to ask RAN1 on whether/how UE could determine applicable functionality if there is no NW-side additional condition.

Proposal 1: RAN2 ask RAN1 whether/how the UE could determine applicable functionality without NW-side additional condition.

FFS #2: inference configuration in Step 3 in Proposal 2 [2]

It was commented that it should be able to include inference configuration in Step 3 [2]. This point is related to two questions according to RAN2 email discussion [2].

- Whether additional inference configuration is needed for UE to identify applicable functionality to report?

- Whether it is feasible for gNB to provide inference configuration  in the first configuration without receiving applicable functionality from UE?

For the first question, RAN1 input is needed. Therefore, RAN2 could wait until RAN1 provides whether additional information on top of associated ID for UE to determine applicable functionalities.

For the second question, there could be two different cases i.e. inference configuration will be used to activate as applicable functionality or inapplicable. If we assume that inference configuration is not applicable, this could be just reference configuration for UE to provide reactive reporting i.e. it is provided to get UE reporting than to activate functionality. It is feasible but it is not so essential given that gNB should update inference configuration upon UE applicability reporting.

More important question would be whether inference configuration  is used as applicable functionality i.e. for UE to activate the inference configuration right upon step-3 RRC configuration. If there is any default option/functionality that gNB can decide without UE reporting, gNB provides inference configuration for applicable functionality in Step 3. Otherwise, it would be not feasible. Therefore, it is necessary to get more information from RAN1 if gNB can determine applicable functionality without UE reporting.

Proposal 2:  RAN2 ask RAN1 whether/how gNB could determine applicable functionality (i.e., inference configuration) without UE reporting of applicable functionality.

FFS #3: other configuration is used for applicable functionalities

Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can be considered by UE.

This FFS might be similar to the first question in FFS #2 i.e. whether additional inference configuration is needed for UE to identify applicable functionality to report. If additional inference configuration is provided, the UE would consider it in reporting applicable functionality.

Proposal 3:  RAN2 wait until RAN1 provides other configuration that is needed for determination of applicable functionality.

FFS #4: RRC signaling message to transmit UE report on applicable functionality

Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

The following reasons to support RRCReconfigurationComplete are summarized by the moderator.

1) NW-side additional condition and/or inference configuration are provided over RRCReconfiguration message, where UE can response immediately.

2) UAI cannot support reporting of neighbouring cells and handover scenario.

3) Avoid additional reporting

The third reason is already invalid given that proposal 4 in offline discussion (as highlighted yellow)  already suggests to use UAI to indicate applicable functionality , whenever UE detects the update of applicable functionality (e.g., due to UE-side additional conditions and model availability).

Regarding the second reason, we are not sure why UAI cannot support neighbouring cells and handover scenario. The existing UAI works well in handover scenario and if needed, new RRC parameters can be defined to allow neighbor cell related information.

Regarding the first reason, UAI can also be initiated upon RRC configuration as shown in the following example.

5.7.4.2 Initiation A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

Based on the above observation, we think that UAI can be used in all cases for reporting of applicable functionality.

Proposal 4:  RAN2 agree to use UAI to report applicable functionality as response to NW-side additional condition.

FFS #5: Initial activation and additional L1/L2 signaling for activation

Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

Regarding initial activation, this is related to FFS #2 whether gNB could determine applicable functionality without UE reporting of applicable functionality. Again, for example, if there is generic model concept and UE supports, it might be possible for gNB to configure applicable functionality (i.e. inference configuration) and could be activated in Step 3. If there is no generic model and UE supports, RAN2 could consider the way that gNB provides inference configuration but they are not activated and used for reactive reporting.

Proposal 5:  RAN2 postpone discussion on initial activation until it is concluded on whether/how gNB could configure functionality without UE reporting of applicable functionality.

Regarding L1/L2 signaling, we understand that it should be discussed in RAN1 how/whether to activate/deactivate functionality for inference. RAN2 can postpone work on this issue pending  RAN1 conclusion.

Proposal 6:  RAN2 wait for RAN1 conclusion on the need of additional L1/L2 signaling for activation/deactivation.

FFS #6: Positioning use case

Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

Both FFSs would be the similar discussion as for BM use case.

Proposal 7:  RAN2 to request RAN1 input on the following questions related to Positioning use case 1.

- whether/how the UE could determine applicable functionality without NW-side additional condition.

- whether/how LMF could determine applicable functionality without UE reporting of applicable functionality

FFS#7: Need of configured functionality

Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed. Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

During the email discussion, the companies have diverged view on need of defining configured functionalities as summarized below.

- Yes: Configured functionalities may be superset of applicable functionalities in case of reactive reporting

- No: the definition is too obvious because anyway gNB/LMF will configure for functionalities. gNB configures applicable functionalities i.e. configured functionalities means applicable functionalities.

The main discussion point would be whether the initial configuration of functionality is immediately activated. This should be related to FFS #2 and FFS #5 above. Therefore, RAN2 could postpone for now until FFS #2 is resolved.

Proposal 8:  RAN2 postpone discussion on configured functionality until it is concluded on whether/how gNB could configure functionality without UE reporting of applicable functionality.

FFS#8: available functionality

Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

During phase 1 discussion, the companies have quite diverged views on defining available functionalities separately from applicable functionalities.

- Separate definition: available functionalities are the super set of applicable functionalities e.g. due to NW sided additional conditions. As a starting point, we could have two different definitions to discuss further details between UE and NW.

- Not to define available functionalities: Applicability already imply that it has at least one available model. Applicability is determined by UE i.e. applicability should not be defined from NW point of view only. Need further discussion about specific cases and relationship between applicability and availability especially the case where there is no applicable functionality despite having available models.

From our observation, available functionality is the term to describe applicable functionalities from UE side, while applicable functionality should be for the functionality the one gNB configures to enable inference. This is correct observation but it is not necessary to define the term “available functionality” explicitly because the important part is what is reported in UE reporting (applicable functionality reporting) and what is configured by gNB (applicable functionality configuration). From procedure and signalling point view, there is not much confusion even if we use applicable functionality in both UE and NW side.

Proposal 9:  RAN2 agree that there is no need to explicitly define available functionality.

# NW-sided additional condition

Based on RAN1 discussion, it is understood that NW-side additional condition should be provided to ensure consistency between training and inference and one option agreed is using associated ID. That is, associated ID can be used for UE categorizing the measurements (as model inputs/labeling during training) and associated ID can also be used in the inference operation to identify proper models (i.e. applicable functionalities).

Observation 1: Associated ID is needs to be provided in both training and inference phase.

It means that NW-sided additional conditions would mean different configurations to affect model training e.g. size of set A / set B, composition of set A and set B, prediction window, gNB beam codebook, gNB antenna configuration, etc. Some of configurations would not be explicitly indicated because it is related to gNB implementation. In addition, it is FFS in RAN1 how to configure the associate ID i.e. how it is transmitted with respect to Set A/Set B resource configuration.

Observation 2: RAN1 is still discussing how to configure associate ID with respect to another configuration (e.g. Set A/Set B resource configuration).

RAN1 is discussing how to define associated ID and made following working assumption.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

We understand that RAN1 is discussing whether associated ID is cell specific or multiple-cell specific. Main consideration would be the granularity of categorizing the measurement. Cell specific would mean that UE categorize data per cell and per associated ID, which would require more complexity/burden in model training at UE side. In order to reduce complexity/burden, multiple cell specific ID is proposed, which can give further information to UE categorizing measurement results for similar properties.

Observation 3: RAN1 is still discussing whether associate ID is cell specific or multiple cells specific.

From signaling point of view, there could be two options to define associate ID to be applied for multiple cells.

- Area specific associated ID: associated ID(s) is provided with a certain area ID similar to system information area ID in NR. Different area ID can be used to differentiate the same associated ID. Therefore, the actual associated ID is the combination of associated ID and area ID. With this approach, the same associated ID can be applied across multiple cells with the same area ID. Allocation and mapping of area IDs to specific areas/cells should be left to implementation. At least, we can assume that area ID should be unique within a PLMN.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN/SNPN.

- Globally unique associated IDs within PLMN: associated IDs are consistently unique as long as the cell belongs to the same PLMN. Allocation and mapping of associate IDs to specific NW sided additional conditions should be left to implementation.

The two options are quite similar from operation point of view i.e. requiring pre-allocation by operators to make it unique in PLMN. Area specific associated ID would require more specification efforts to update associated IDs with the received area ID. Nevertheless, both options are feasible from signaling point of view. Given that RAN1 is discussing the feasibility for multiple cells, it would be useful for RAN2 to indicate that it is feasible to support area/PLM specific associated ID from signaling point of view.

Proposal 10: RAN2 agree that it is feasible to support area/PLMN specific associated ID from signaling point of view.

# Performance monitoring

For beam management use case, as captured in TR 38.843, RAN1 is considering two types of performance monitoring, where type 1 is NW to control LCM operation of UE-side model and type 2 is UE to control the LCM operation.

Figure 1. LCM procedure for UE-side AI/ML model.

Contents of performance monitoring

Until now, there is no RAN1 agreement that is specific to performance monitoring other than RAN1 LS on Data Collection Requirements and Assumptions [3].

Based on the LS, we could consider two types of reporting.

- Actual prediction/measurement results: UE provides L1-RSRP and/or beam-ID(s) to gNB. The size can be variable depending on what measurement is provided and number of resources to report.

- Performance metric/event/management decision: in case of type 1/option 2, UE calculates the metric and provides to gNB. In this case, the amount data size is small (10s of bits). In case of Type2, it could be just UE’s decision outcome of model selection/activation/deactivation/switching/fallback.

Observation 4: For performance monitoring reporting, the level of data size can be different depending on whether UE report actual prediction/measurement results or performance metric/event/management decision.

Delay requirement of performance monitoring

As usual, three different signaling options (L1, MAC, RRC) are on the table in RAN1. From delay perspective, RAN1 concluded that the delay requirement of reporting for performance is near real time and provided that Near-real-time is e.g., several tens of msecs to a few seconds [3]. All signaling options are feasible to meet near-real-time requirement.

Observation 5: For performance monitoring reporting, all signaling options (L1, MAC, RRC signaling) are feasible to meet near-real-time requirement.

Signaling Flexibility

Another aspect to consider is signaling flexibility. Although L1/MAC CE can provide less delay, the format is not so flexible as RRC signaling. In beam management use case, multiple beams/L1-RSRP results may be included for multiple configured resources. If the number of configured resources to report is dynamic, more flexible signaling format would be required.

On the other hand, in case of performance metric/event/management decision, the less delay would be more beneficial to provide event situation/management decision immediately. In addition, performance metric/event/management decision requires small size of reporting.

Observation 6: For performance monitoring reporting, depending on the size and characteristics of reporting for performance monitoring, it may require more flexible format.

Since RAN1’s discussion on options for measurement results and performance metric, it would be early for RAN2 to identify which signaling option is more suitable.

Proposal 11: RAN2 wait for RAN1 inputs to identify signaling option for performance monitoring.

# Conclusion

We discussed the following three topics and provide the following proposals.

Topic 1: FFS points from RAN2 post email discussion

Proposal 1: RAN2 ask RAN1 whether/how the UE could determine applicable functionality without NW-side additional condition.

Proposal 2:  RAN2 ask RAN1 whether/how gNB could determine applicable functionality (i.e., inference configuration) without UE reporting of applicable functionality.

Proposal 3:  RAN2 wait until RAN1 provides other configuration that is needed for determination of applicable functionality.

Proposal 4:  RAN2 agree to use UAI to report applicable functionality as response to NW-side additional condition.

Proposal 6:  RAN2 wait for RAN1 conclusion on the need of additional L1/L2 signaling for activation/deactivation.

Proposal 7:  RAN2 to request RAN1 input on the following questions related to Positioning use case 1.

- whether/how the UE could determine applicable functionality without NW-side additional condition.

- whether/how LMF could determine applicable functionality without UE reporting of applicable functionality

Proposal 8:  RAN2 postpone discussion on configured functionality until it is concluded on whether/how gNB could configure functionality without UE reporting of applicable functionality.

Proposal 9:  RAN2 agree that there is no need to explicitly define available functionality.

Topic 2: NW-side additional condition

Observation 1: Associated ID is needs to be provided in both training and inference phase.

Observation 2: RAN1 is still discussing how to configure associate ID with respect to another configuration (e.g. Set A/Set B resource configuration).

Observation 3: RAN1 is still discussing whether associate ID is cell specific or multiple cells specific.

Proposal 10: RAN2 agree that it is feasible to support area/PLMN specific associated ID from signaling point of view.

Topic 3: Performance monitoring

Observation 4: For performance monitoring reporting, the level of data size can be different depending on whether UE report actual prediction/measurement results or performance metric/event/management decision.

Observation 5: For performance monitoring reporting, all signaling options (L1, MAC, RRC signaling) are feasible to meet near-real-time requirement.

Observation 6: For performance monitoring reporting, depending on the size and characteristics of reporting for performance monitoring, it may require more flexible format.

Proposal 11: RAN2 wait for RAN1 inputs to identify signaling option for performance monitoring.

# Reference

- R2-2407165	Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung) – Phase 1	Samsung

- R2-2406381	Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2	Intel Corporation

- R2-2311720  Reply LS on Data Collection Requirements and Assumptions (LS to RAN2)