3GPP TSG-RAN WG2 Meeting #127	R2-2407167

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Consideration on UL capacity enhancement for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As one target of WID[1], support of capacity enhancement for UL needs to be studied and specified. One of the motivations is to reduce the necessary uplink and downlink signalling to complete an Early Data Transmission (EDT) transaction. More specifically, it includes enhancement to transmit Msg3 without Msg1/Msg2 and enhancement to efficiently deliver Msg4.

- Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2-125bis and RAN2-126 meeting, a list of agreements has been reached. On top of the EDT enhancements, it was agreed to continue the discussion on DSA and CRDSA for Msg3-EDT transmissions.

Agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

In this contribution, we provide our view on possible enhancements for EDT transaction, as well as our considerations on the support of DSA/CRDSA for IoT NTN.

# 2	Discussion

## 2.1	EDT enhancements

Mobile originated early data transmission (EDT) is supported in Rel-15, which allows for early UL data transmission in Msg3. The UE initiates the MO-EDT procedure and selects a random access preamble configured for EDT. In Rel-16, preconfigured uplink resource (PUR) was introduced to further enhance the small data transmission. The PUR builds on the EDT concept but allows to skip the Msg1 and Msg2 steps of the random access procedure if the UE can confirm the serving cell is same as the one which configured the PUR and optionally that Timing Advance is valid/RSRP is within a certain range, and instead directly initiate the data transmission using the PUR resource in Msg3. The PUR is configured in a previous RRC connection (i.e., in RRCConnectionRelease message) and enables further potential energy savings compared to EDT.

In RAN2-126 meeting, RAN2 agreed to study Contention-Based Msg3 transmission based on Shared resources (CBS-Msg3 transmission) to complete an EDT-like transaction without Msg1/Msg2. However, according to the discussions in last meeting, it is not clear whether the UE estimated TA (which is used for Msg1 transmission for RRC Idle UE) is accurate enough for the direct Msg3 transmission (without Msg1/Msg2). Hence RAN2 decided to send LS [2] to RAN1 and RAN4 to clarify the feasibility of direct Msg3 transmission.

RAN2 kindly asks RAN4 and RAN1: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

Observation 1: RAN4 and RAN1 are expected to feedback whether a pre-compensated TA used for Msg1 transmission can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2, and if not, how the TA requirement can be satisfied.

In our view, the feasibility also depends on whether the UE is served by GEO or LEO satellites. The support of PUR towards GEO satellites is already specified in Release 17, while support of LEO satellites was excluded. One major challenge is the frequent cell switch, which occurs in LEO independently of the UE mobility state.

The legacy PUR configuration is received by UE in RRCConnectionRelease message and it is only valid in the cell where the configuration was received. For IoT NTN, due to the frequently changed serving cell in LEO, it is necessary to study how the NW provides the CBS-Msg3 configuration to UEs in a new cell.

For the CBS-Msg3 shared resource, multiple UEs may transmit Msg3 simultaneously, thus cause collision which will bring some adverse consequence such as the increased signalling overhead, additional UE power consumption and waste of the resources. If the configuration is provided via dedicated signalling, it allows the network to control the number of UEs that use the shared resource, thus control or predict the possible Msg3 collision. However, it is necessary to study whether a CBS-Msg3 shared resource configuration can be applicable to other (LEO-based) cells than the serving cell, which provided it. This is essential to avoid the UE entering RRC Connected mode to get the configuration in each new cell. Instead, if the configuration is provided in system information, the solution is valid for all kinds of UEs including the UE accessing the network for the first time. If the CBS-Msg3 configuration is provided to UE via SIB, the NW has no idea how many idle UEs will use the resource simultaneously which may cause unexpected Msg3 collision. Therefore, the NW should have a mechanism to control the possible Msg3 collision and to resolve the Msg3 collision as efficiently as possible.

Observation 2: If the shared CBS-Msg3 resource is provided via SIB, a mechanism to control and/or resolve the Msg3 collision should be supported.

Observation 3: If the shared CBS-Msg3 resource is provided via dedicated signaling, a mechanism to enable the CBS-Msg3 configuration can be applicable to other cells than the serving cell should be supported.

In GEO-based cells the UEs may not experience the change of a serving cell, unless the UEs are mobile. Therefore, the use of dedicated signaling for CBS-Msg3 configuration may be applicable to GEO cells. On the other hand, UEs in LEO cells will experience a cell change, independently of whether Earth-fixed or Earth-moving cells are deployed. In this LEO scenario, the use of SIB may be preferred.

Observation 4: Use of dedicated signaling for CBS-Msg3 resource configuration may be preferred in GEO-based deployments.

Observation 5: Use of System Information for CBS-Msg3 resource configuration may be preferred in LEO-based deployments.

Proposal 1: RAN2 to study whether the configuration of shared CBS-Msg3 resource should be provided to UE via System Information or dedicated signaling, based on whether both GEO and LEO should be supported for CBS-Msg3.

In legacy 4-step RACH the collisions are handled using the contention resolution scheme, which relies on uniquely identifying the UEs in Msg3 and NW acknowledgement in Msg4. However, in legacy PUR mechanism (Msg3 transmission without Msg1/RAR), there is no need for contention resolution since the PUR resource allocated via RRCConnectionRelease message is dedicated to a specific UE (i.e., contention free resource). For CBS-Msg3 transmission in shared resource, there may occur Msg3 collision between UEs. Therefore, the contention resolution scheme should be supported. The most straightforward way is to reuse the contention resolution scheme in 4-step RACH.

Proposal 2: Reuse the contention resolution scheme in legacy 4-step RACH for CBS-Msg3 transmission.

In legacy, the use of the PUR configuration depends on the Timing Advance being valid and possibly also a RSRP condition. In the LEO scenario, the Timing Advance may be very different between a target cell and the serving cell, which provided the CBS-Msg3 configuration, and thus RAN2 shall discuss if it is still applicable. In our view, for TA validation, we can first await feedback from RAN1 and RAN4 regarding the UE’s ability to properly pre-compensate the Msg3 transmission. For the RSRP conditions, in legacy PUR, the RSRP Change Threshold is defined for the purpose of TA validation. The change threshold can be reused for GEO. However, for LEO, due to the frequent change of the satellite & cell, it looks not suitable especially for a UE entering a new cell provided by a new satellite. However, we think the RSRP condition remains valid, because it will ensure the uplink configuration in terms of MCS and resources is suitable for the current radio conditions. In this case, instead of the RSRP change threshold, the absolute RSRP threshold should be defined.

Proposal 3: RAN2 to await LS response is received from RAN1/4 before considering TA condition for CBS-Msg3 applicability.

Proposal 4: RAN2 to discuss the use of a RSRP condition for the UE to confirm the CBS-Msg3 is applicable with the current radio conditions on the current cell, based on whether both GEO and LEO should be supported for CBS-Msg3.

The CBS-Msg3 procedure is subject to failures due to, e.g., TA validity, interference or collision. Thus, one important issue to consider is the UE behavior in case of a CBS-Msg3 failure or consecutive CBS-Msg3 failures. For example, conditions for the UE to keep using CBS-Msg3 or to fall back to using, e.g., the legacy 4-step RACH procedure after a CBS-Msg3 failure or consecutive CBS-Msg3 failures need to be studied and specified. However, it is important to ensure that the UE may have a successful EDT within a time limit, assuming that the UE has periodic EDT as applied for the legacy PUR. This should be considered for fallback option after a CBS-Msg3 failure or consecutive CBS-Msg3 failures.

Proposal 5: RAN2 to discuss UE behavior in case of CBS-Msg3 procedure failure, including fallback options for the UE to ensure that the UE may have a successful EDT within a time limit.

On the objective of efficient delivery of msg4, according to the WID [1], the intention is to identify methods to de-couple the UL from the DL as much as possible hence unlock the additional UL capacity potential. We understand it is to avoid the PDCCH/PDSCH congestion which may impact the UL scheduling as well as the UL capacity. Therefore, how to reduce the Msg4 overhead should be studied.

In legacy, a dedicated Msg4 is always needed for the corresponding Msg3 for contention resolution. To reduce the Mgs4 overhead, the enhancement should reduce the PDCCH usage for Msg4 scheduling and/or the PDSCH usage for Msg4 delivery.   For example, it seems a possible optimization way-forward to support one Msg4 to deliver contention resolution result for multiple UEs or preconfigure the PDSCH for Msg4 delivery to save the scheduling PDCCH at least.

Proposal 6: To support efficient delivery of Msg4, RAN2 to study on how to reduce the PDCCH usage for Msg4 scheduling and/or the PDSCH usage for Msg4 transmission.

## 2.2	Consideration on DSA/CRDSA support

For DSA (Diversity Slotted ALOHA) and CRDSA (Contention Resolution Diversity Slotted ALOHA), RAN2 agreed to continue the discussion on DSA/CRDSA for Msg3-EDT transmissions on the shared UL resource. For DSA, as described in [3],  the UE transmits multiple copies of the same packet. The multiple copies can be either transmitted simultaneously on different frequency channels (frequency diversity) or they may be transmitted on a single channel but spaced apart by random time intervals (time diversity). In [4], it describes that DSA can improve the conventional SA (slotted ALOHA) throughput by about a factor of 6 at a packet error rate of 10-2 without any modification in the demodulator. However, in IoT NTN, the Msg3 repetition has been supported in PUR in which the NW can configure the number of continuous PUSCH repetition and activate the frequency hopping for the PUSCH transmissions. It is not clear to what extend the similar gain can be achieved by applying the diversity of the repetitions for Msg3.

Observation 6: It is not clear what gain can be achieved by applying DSA compared to Msg3 repetitions in PUR.

For CRDSA, it has been specified as the random-access methods for satellite communication in DVB-RCS2 [5]. On the transmission side, each terminal generates multiple copies of the same packet. The terminal will randomly select the slots for transmission within a predefined frame based on the predefined function specified in [5]. This random selection ensures that replicas are spread out, reducing the likelihood of all replicas colliding in the same slot. Each packet replica is tagged with information that allows the receiver to identify and correlate the replicas. On the reception side, the receiver (e.g., satellite gateway) monitors the slots for incoming packets. If a slot contains a single packet, it may be successfully decoded. If a slot contains multiple packets, a collision is detected. In this case, interference cancellation should be performed in the receiver. In other words, the receiver uses the correctly received replicas from other slots to reconstruct and decode the collided packets. This process, known as successive interference cancellation, allows the recovery of collided packets using the diversity of the replicas.

If we want to support CRDSA on the shared PUSCH in IoT NTN, a similar mechanism as the satellite communication should be designed for UE and eNB. The UE shall transmit multiple replicas in the selected subframes within a predefined window. The eNB shall store the received packets within the window and then perform the interference cancellation for the UEs. Due to the limited memory in eNB, the window length for buffering should be carefully designed. In our understanding, RAN2 may discuss how the UE determines the number of replicas, how to determine the subframes selected for replica transmissions as well as the contention resolution for Msg3. However, it should be decided/evaluated by RAN1 on whether and how the eNB can perform the interference cancellation based on current physical layer design. Furthermore, according to the simulation in [4], the CRDSA may improve the system throughput about 80 to 100 times higher than the legacy Slotted ALOHA in the satellite communication. However, the detailed simulation assumptions are not fully discussed hence RAN1 should be involved to evaluate the CRDSA performance gain.

Observation 7: For CRDSA, RAN2 may discuss how the UE determines the number of replicas, how to determine the subframes selected for replica transmissions as well as the contention resolution for Msg3. However, whether and how the eNB can perform the interference cancellation should be decided/evaluated by RAN1.

Observation 8: The CRDSA performance gain should be evaluated by RAN1.

Based on above observations, it seems not a straight-forward task for RAN2 to decide whether the DSA or CRDSA can be supported for Rel-19 IoT NTN. How to incorporate the DSA/CRDSA-like techniques into contention-based EDT transmission within the 3GPP NTN framework should be studied by RAN1 as well. According to WID[1], DSA/CRDSA is not in current working scope for the objective of Msg3-EDT transmissions enhancement. Nothing related to CRDSA was mentioned in the WID even in the justification part. Furthermore, RAN1 is not indicated as the working group for the EDT enhancement objective.

Observation 9: The CRDSA is not in the scope of current WID for the Msg3-EDT enhancement.

Therefore, considering the requirement of RAN1 involvement as well as the efforts to further study the performance gain and impact caused by DSA/CRDSA, we believe whether to support DSA/CRDSA in Rel-19 IoT NTN should be decided by RAN plenary.

Proposal 7: Whether to support DSA/CRDSA in Rel-19 IoT NTN should be decided by RAN plenary.

# 3	Conclusion

This document has made the following observations:

Observation 1: RAN4 and RAN1 are expected to feedback whether a pre-compensated TA used for Msg1 transmission can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2, and if not, how the TA requirement can be satisfied.

Observation 2: If the shared CBS-Msg3 resource is provided via SIB, a mechanism to control and/or resolve the Msg3 collision should be supported.

Observation 3: If the shared CBS-Msg3 resource is provided via dedicated signaling, a mechanism to enable the CBS-Msg3 configuration can be applicable to other cells than the serving cell should be supported.

Observation 4: Use of dedicated signaling for CBS-Msg3 resource configuration may be preferred in GEO-based deployments.

Observation 5: Use of System Information for CBS-Msg3 resource configuration may be preferred in LEO-based deployments.

Observation 6: It is not clear what gain can be achieved by applying DSA compared to Msg3 repetitions in PUR.

Observation 7: For CRDSA, RAN2 may discuss how the UE determines the number of replicas, how to determine the subframes selected for replica transmissions as well as the contention resolution for Msg3. However, whether and how the eNB can perform the interference cancellation should be decided/evaluated by RAN1.

Observation 8: The CRDSA performance gain should be evaluated by RAN1.

Observation 9: The CRDSA is not in the scope of current WID for the Msg3-EDT enhancement.

And proposed the following:

Proposal 1: RAN2 to study whether the configuration of shared CBS-Msg3 resource should be provided to UE via System Information or dedicated signaling, based on whether both GEO and LEO should be supported for CBS-Msg3.

Proposal 2: Reuse the contention resolution scheme in legacy 4-step RACH for CBS-Msg3 transmission.

Proposal 3: RAN2 to await LS response is received from RAN1/4 before considering TA condition for CBS-Msg3 applicability.

Proposal 4: RAN2 to discuss the use of a RSRP condition for the UE to confirm the CBS-Msg3 is applicable with the current radio conditions on the current cell, based on whether both GEO and LEO should be supported for CBS-Msg3.

Proposal 5: RAN2 to discuss UE behavior in case of CBS-Msg3 procedure failure, including fallback options for the UE to ensure that the UE may have a successful EDT within a time limit.

Proposal 6: To support efficient delivery of Msg4, RAN2 to study on how to reduce the PDCCH usage for Msg4 scheduling and/or the PDSCH usage for Msg4 transmission.

Proposal 7: Whether to support DSA/CRDSA in Rel-19 IoT NTN should be decided by RAN plenary.

# Reference

[1] RP-241624 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] R2-2405769 LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

[3] G. Choudhury and S. Rappaport, "Diversity ALOHA - A Random Access Scheme for Satellite Communications," in IEEE Transactions on Communications, vol. 31, no. 3, pp. 450-457, March 1983.

[4]	R2-2405202 Discussion on EDT enhancements, ESA

[5] ETSI EN 301 545-2, “Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard”