3GPP TSG-RAN WG2 #127	R2-2407176

# Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:	7.4.2

Source:	Ericsson

Title:	Remaining issues related to LTM

Document for:	Discussion

# 1	Introduction

In this contribution we address some remaining issues about LTM which concerns the handling of measurement gap, the configuration of the UL skipping feature, and when the UE starts to monitor the PDCCH after an LTM cell switch procedure.

# 2	Discussion

## 2.1	Handling of measurement gaps for LTM

In the last RAN2 meeting, the handling of measurement gaps for LTM has been discussed, but no decision has been taken.

R2-2405527	Discussion on measurement gaps for LTM in NR-DC	Samsung	discussion

Moved from 7.4.2

DISCUSSION

- 	ZTE think the L3 measurent gaps are sufficient.

- 	Ericsson think same samples are used for L1 measurements as for L3 measurements

-	Samsung think that no of cells can be different between L1 and L3 meas.

-	Xiaomi think 3 in the table may need disc, vivo think this is then a R4 discussion.

-	Apple think there could be an issue, and MN cannot fully understand SN situation.

-	Samsung expect that if we address this, it will impact Inter-node-signalling

-	Session Chair: can have time to check, until next meeting.

Postponed

The concern raised during the meeting has been that SN needs to know whether MN has configured LTM and on which frequencies the LTM candidate cell have been configured. The main reason for this is because there could be scenario on when the MN or SN would not configure gaps if those are only based on L3 measurements.

According to this, we have two understanding on this topic. The first one is that when RAN4 defined the measurement gaps capabilities for LTM, the intention was not to ask RAN2 to have a new signalling so that measurements gap can be configured independently for L1 and L3 measurements. Their understanding is rather to re-use the measurement gap framework which is already in RRC.

RAN4 intention is re-used the current measurement gap framework also for LTM, without the new of new signalling.

The second understanding we have is that the need for gap is related to the frequency and not by the fact on whether L1 and L3 measurements are configured. In this sense, there is no scenario where measurement gaps are needed for L1 measurements but not needed for L3 measurement, and vice versa.

The need for gaps is related to the frequencies in which the UE should perform the measurements, but not whether the UE performs L1 or L3 measurements.

According to this, the existing measurement gaps framework can be re-used for LTM and no new signalling needs to be introduced. Thus, we propose:

The existing measurement gaps framework is re-used for LTM without the need to introduce new signalling.

## 2.2	UE behaviour when UL skipping is configured

When a UE performs a RACH-less LTM cell switch, the UE determines the successful completion of the LTM cell switch based on that network successfully receives the first UL data transmission from the UE in the target cell. How the UE determines the successful reception was agreed in RAN2#123bis as follows:

For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission as first UL transmission. Can be either DL assignment or UL grant addressed to same HARQ process for the “new transmission”.

For LTM in the MCG and the SCG configured with SRB3, the first UL data in the target cell will be the RRC Reconfiguration complete message. However, for the case of LTM in the SCG for a UE not configured with SRB3, uplink user plane data may be required to trigger the first UL transmission in the target cell.

However, there could be cased when the UE may not have any uplink data to transmit at the time when the UE shall indicate to the target that the cell switch has happened. This is not a problem in normal situations since the UE can send a transmission comprising padding to the target and such transmission can serve the purpose of UE indicating that the UE has performed the cell switch. However, when the skip uplink feature is enabled, the UE shall refrain from performing uplink transmissions when the UE has no uplink data to transmit, or in other words, the UE would not perform transmissions if the uplink transmission would only contain padding.

When the skip uplink feature is enabled, the UE shall refrain from performing uplink transmissions when the UE has no uplink data to transmit and the uplink transmission only contain padding.

This means that the UE will not formally complete the LTM cell switch procedure on the SCG and it will perform eventually the RRC re-establishment procedure since the supervision timer that is used during an LTM cell switch procedure will expire. The RRC re-establishment procedure means a longer interruption timer and data loss.

According to this, two possible solutions may solve this issue:

- Option 1: If UL skipping is configured, the UE overriding the skip uplink feature for the first transmission in the target cell after the LTM cell switch in order to enable the UE and network to determine successful completion of the cell switch.

- Option 2: the network either de-configures the skip uplink features before or during the RACH-less cell switch or ensures that RACH-less cell switch is not triggered for a UE configured with the skip uplink feature.

Between the two options, we think that Option 2 means that an RRC reconfiguration may need to be sent before triggering the LTM cell switch procedure, or that network would not be able to trigger a RACH-less LTM cell switch if the UL skipping feature is configured at the UE. On the other side, Option 1 is simpler and require limited implementation efforts. For this reason, we propose:

If UL skipping is configured, the UE ignore the UL skipping indication for the first transmission in the target cell during an LTM cell switch procedure, i.e. the UE always performs the first transmission.

## 2.3	When the UE starts to monitor PDCCH

According to TS 38.321, the UE considers itself to be in an ongoing RACH-less LTM cell switch when it receives and LTM cell switch command with a valid TA. Also, according to clause 5.3.1 and 5.4.1 the UE should do the following action upon the reception of the DL assignment or a UL grant:

2>	if there is an ongoing RACH-less LTM cell switch; and

2>	if the downlink assignment/ uplink grant has been received on the PDCCH for the MAC entity's C-RNTI after the first PUSCH transmission to the Serving Cell; and

2>	if the downlink assignment/ uplink grant is for a new transmission on the same HARQ process used for the first PUSCH transmission to the Serving Cell:

3>	consider the LTM cell switch to be successfully completed and indicate it to upper layers

However, it is not clear from this text at which point in time the UE will start to monitor the PDCCH after receiving the LTM cell switch MAC CE. If we look at Figure 1, after receiving the LTM cell switch MAC CE the UE would need to send an ACK, but then there are three regions where is not clear at which point the UE starts to monitor PDCCH.

Since the network, if a CG grant is not provided, can either pre-schedule the UE with a DG or can wait the UE to send first an SR, in order to do the pre-scheduling it would be useful to understand at which point the UE start to monitor PDCCH.

RAN2 to clarify at which point, after the reception of the LTM cell switch MAC CE, the UE starts to monitor the PDCCH.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN4 intention is re-used the current measurement gap framework also for LTM, without the new of new signalling.

Observation 2	The need for gaps is related to the frequencies in which the UE should perform the measurements, but not whether the UE performs L1 or L3 measurements.

Observation 3	When the skip uplink feature is enabled, the UE shall refrain from performing uplink transmissions when the UE has no uplink data to transmit and the uplink transmission only contain padding.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The existing measurement gaps framework is re-used for LTM without the need to introduce new signalling.

Proposal 2	If UL skipping is configured, the UE ignore the UL skipping indication for the first transmission in the target cell during an LTM cell switch procedure, i.e. the UE always performs the first transmission.

Proposal 3	RAN2 to clarify at which point, after the reception of the LTM cell switch MAC CE, the UE starts to monitor the PDCCH.

# 4	TP to 38.331

TP for proposal 2:

The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR; and 2> if this is not a grant for a first uplink transmission following a RACH-less handover or LTM cell switch: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR; and 2> if this is not a grant for a first uplink transmission following a RACH-less handover or LTM cell switch: 3> not generate a MAC PDU for the HARQ entity.