3GPP TSG-RAN WG2 #127	R2-2407177

# Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:	7.4.2

Source:	Ericsson

Title:	Summary of RRC proposals for feMob

Document for:	Discussion

# 1	Introduction

In this contribution is provide a summary of the RRC proposals for FeMob with some possible conclusion for each of them.

# 2	Discussion

Hereafter the contributions submitted to AI 7.4.2 are listed and a possible outcome for what is proposed is suggested:

| Tdoc # | Company | Proposal(s) | Suggested conclusion |
| --- | --- | --- | --- |
| R2-2406332 | CATT | P1: For SCPAC configured in MN format, subsequent CPAC execution condition update is not applied for subsequentCondExecutionCond. P2: Upon subsequent CPAC execution, the logged measurement configuration is not released by UE P3: When the servingSecurityCellSetId is not configured, the securityCellSetId can be configured for all the candidate configurations for subsequent CPAC or for none of them | P1: To be agreed. Seems a straightforward change. P2: To be agreed. In principle this is okay. I think we can align to the case of normal handover. P3: To be discussed. I don’t think this is really needed but it is true that in case of CPA scenario there would be a need to include an indication about what security key to use. It could then perhaps be fine to mention that in the field description |
| R2-2406337 | MediaTek | P1: RAN2 to clarify that if there is only one TCI state in ltm-CandidateConfig for the candidate cell indicated by LTM Cell Switch Command MAC CE, the network ensures this TCI state is identical with the TCI state indicated by LTM Cell Switch Command MAC CE P2: In the field description of spcellInclusion, clarify that it cannot be set to True if SpCell resources are not included in LTM-CSI-ResourceConfig. P3: In the field description of unifiedTCI-StateType, specify that the UE applies value separate if unifiedTCI-StateType is absent in LTM-TCI-Info. In addition, change the need code of unifiedTCI-StateType to Need S. P4: In clause 5.3.5.18.6, correct the procedural text so that the UE stops the UE-based TA measurements for the LTM candidate which becomes the current SpCell. P5: In clause 5.3.5.18.6, correct the procedural text so that the UE does not start the UE-based TA measurements for the LTM candidate which becomes the current SpCell. P6: In clause 5.3.5.18.6, remove the unnecessary procedural text to stop the UE-based TA measurements when the LTM candidate which becomes the current SpCell does not have field ltm-UE-MeasuredTA-ID. P7: RAN2 to discuss if and how to capture handling of the UE-based TA measurements for the scenario where the SpCell changes by L3 mobility procedure (handover/PSCell change) while LTM is configured | P1: Not Pursued. As also clarified in the contribution, is already clear from the RAN1 specification what the UE behaviour will be. In this case a network restriction seems totally unnecessary. P2: To be discussed. In principle, the UE can use the legacy resource for L1 measurements on the serving cell, does not need explicit resource. What is needed is that the serving cell has a LTM candidate configuration ID. But not strong view, also what is mentioned in the proposal should be okay. P3: To be agreed. The intention from RAN1 to always use the unified TCI state framework is not crystal clear. However, we can assume that if this field is not present than the UE can assume that separate TCI states for DL and UL are provided. P4-P5-P6. To be discussed. There are other proposals on this topic. P7: Not pursued. The UE-based TA was never thought to be for normal L3 handover. So, there is no support for it. |
| R2-2406355 | MediaTek | P1: For the LTM capabilities with the granularity of per band pair per band combination: - Add the target band list into FeatureSetsDownlink/Uplink for each capability. - Include the corresponding components for each target band within the target band list. The ASN.1 structures as described above can be considered as a starting point. P2: maxBands is used for the maximum size of the target band list. P3: The target band lists of these capabilities are filtered in accordance with the potential candidate bands indicated in frequencyBandListFilter by network. P4: Clarify that the capability value of ltm-FastProcessingConfig-r18 indicates the maximum number across all the supported bands, rather than the current band P5: Add separate optional markers for the three components of the feature R4 39-7 (ltm-FastUE-Processing). | P1-P5. To be discussed in the capabilities offline |
| R2-2406418 | ZTE | P1: In order to ensure that UE capabilities on L1 measurement are not exceeded, the MN indicates the maximum number of L1 measurement resources/configurations the SN is allowed to configure for SCG LTM, including: - The max number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement; - The max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps; - The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement; - The max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement; - Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement; - Maximum number of LTM CSI report configs, including aperiodic configs, periodic configs, and semi-persistent configs, respectively; - Maximum number of RRC configured candidate cells for intra- and inter-frequency L1-RSRP measurement P2: The SN can also request the MN for new maximum values of the number of L1 measurement resources/configurations (as listed in proposal 1) that the SN can configure for SCG LTM. And it’s up to the MN whether to accommodate the SN request P3: Regarding the capabilities across MCG and SCG per frequency layer, e.g.: - The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps; - The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps; - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps; - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps. RAN2 to discuss the following options for inter-node coordination: - Option 1: the MN indicates a hard-split value of the maximum number of resources per frequency that the SN is allowed to configure. And the SN can request the MN for a new maximum value of the number that the SN can configure. I.e. similar to P1 and P2 above. - Option 2: Based on option 1, the MN and the SN further exchange the frequency layer information configured for LTM by each node. The value indicated by the MN is only applicable to frequency layers where both the MN and the SN configures LTM. P4: Inter-node coordination is not supported for UE capability that defined as maximum number of resources configured in one slot, i.e. the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement. P5: For each coordinated capability above, the MN indicates a single value to the SN and vice versa. And the value is applicable to all BCs in the allowed BC list P6: In order to ensure that the L1 measurement layer configured by the SN has a corresponding L3 measurement layer, the indicated max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps should be lower than or equal to the indicated maximum number of NR inter-frequency carriers for L3 measurement P7: RAN2 confirms that gaps for L3 measurements are sufficient for LTM L1 measurements, i.e. no need to make the additional inter-node coordination for measurement gap to meet the L1 measurement requirements | P1-P7: To be discussed in the capabilities offline. I think the proposals raise good points and some inter-node RRC signalling is needed to accommodate the RAN4 capabilities. About P3, in principle Option 1 would be preferable in order to reduce signalling, even if it costs a bit from the flexibility point of view. Same goes for P5, good if we can reduce the signalling, so this is a good approach. |
| R2-2406438 | Vivo | P1: UE should always perform L3 measurement for LTM candidate cells even if s-Measure criterion is satisfied. | P1: Not Pursued. I think that for L3 measurements the UE should follow what the network configures. If the network wants the UE to performs L3 measurements on an LTM candidate cell, it should configure the L3 measurement configuration and s-measure accordingly. |
| R2-2406476 | Intel | Report of [Post126][514][R18MobE] UE capabilities Open Issues (Intel) | To be discussed in the capabilities offline |
| R2-2406477 | Intel | CR from [Post126][514][R18MobE] UE capabilities Open Issues (Intel) | To be discussed in the capabilities offline |
| R2-2406478 | Intel | CR from [Post126][514][R18MobE] UE capabilities Open Issues (Intel) | To be discussed in the capabilities offline |
| R2-2406531 | OPPO | P1: Add the restriction that the NW does not include discardOnPDCP and reestablishRLC for SRB3 in case of SCPAC in MN format P2: Capture the agreement that the servingSecurityCellSetId is not present when there is no SCG in the field description for servingSecurityCellSetId | P1: Need clarification. The proposal is not clear. How something that is for SRB3 being received via SRB1? Some clarification is needed. P2: Not pursued. The current text seems fine. All the SCG configurations are generated by the SN, so current text is in line with the agreement. |
| R2-2406552 | Google | P1: RAN2 sends a LS to RAN3 to request RAN3 to support the scenario where the CU instructs the serving DU to initiate an LTM cell switch for the UE | P1: Not pursued. In the last meeting we agreement that LTM can also be triggered based on L3 measurements and that we would not optimize anymore this case. |
| R2-2406726 | Apple | P1: RAN2 clarify whether the nested ltm-Config is supported or not P2: If the nested ltm-Config is supported, RAN2 confirm the understanding in Observation 3 and Observation 4 on how to handle the nested ltm-Config. P3: Confirm that that network may provide the ltm-Config without ltm-CSI-ResourceConfig | P1-P2: To be discussed. My interpretation is that we don’t limit this case, but no strong view on this. P3: To be agreed. This is a consequence of the RAN2 agreement we took in the last meeting. Even if this agreement has no specification impact. |
| R2-2406847 | Nokia | P1: NW indicates which cells (e.g. by providing a list of cells) are subject to early ASN.1 decoding. This indication is included in the RRCReconfiguration message used to configure LTM P3: RAN2 confirms repetitions are supported for msg1 transmission for early TA acquisition P4: The configuration of msg1 repetitions for early TA acquisition is provided in the earlyUL-SyncConfig IE, as proposed in the TP in Annex C P5: RAN2 to agree that if network decides to stop the UE based TA estimation process, it must stop the unnecessary UE based TA estimation immediately without waiting for the network to trigger cell switch execution P6: The SSB that is associated with the TRS that is associated with the TCI State ID should be considered for the UL grant transmission. P7: Provide an explicit description of the UE behaviour when receiving an LTM cell switch command, where the TAC is set as FFF, and the UE-based TA measurement is either not configured or not successfully performed for the indicated target cell | P1: To be discussed. This topic has been postponed in the last meeting and there are companies who propose to not have the support for this. P3-P4: Not pursued. The early TA acquisition is a network triggered procedure via the PDCCH order. If the preamble is not received is the network who is responsible for the retransmission. P5: To be discussed. Other proposals from other companies on this topic. P6: To be agreed but with a clarification. Leave how and whether to capture this in the MAC offline. P7: To be discussed in the MAC offline |
| R2-2406852 | Google | P1: Clarify in the field description that maxNumberConfigs-r18 indicates the maximum number of LTMCandidateConfigs for which the UE can perform early ASN.1 decoding and validity check as specified clause 6.3 in TS 38.133 P2: Clarify the meaning of maxNumberConfigs-r18 in TS 38.306 using one of the following options: - Option 1: maxNumberConfigs-r18 represents the total number of LTMCandidateConfigs and ltm-ReferenceConfiguration for which the UE can perform early ASN.1 decoding and validity check. - Option 2: maxNumberConfigs-r18 represents the maximum number of LTMCandidateConfigs for which the UE can perform early ASN.1 decoding and validity check, as described in TS 38.133. The UE indicating the maxNumberConfigs-r18 shall perform early ASN.1 decoding and validity check for the ltm-ReferenceConfiguration if received. P3: maxNumberStoredConfigCells-r18 indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s), and Scell(s) in LTMCandidateConfig(s) and ltm-ReferenceConfiguration(s) for MCG and SCG, that UE can store the configurations. P4: RAN2 should discuss and clarify whether UEs not supporting LTM fast processing should indicate maxNumberStoredConfigCells-r18. | P1-P4: To be discussed in the capabilities offline. |
| R2-2407050 | Samsung | P1: Update the 2TA Condition in ‘CandidateTCI-State’ as follows: “This field is mandatory present if tag2 is present in the SpCellConfig in ltm-CandidateConfiguration and unifiedTCI-StateType in the ltm-TCI-Info in the ltm-CandidateConfiguration is set to joint. It is absent, Need R, otherwise.” P2: Add condition ‘JointTCI1’ for pathlossReferenceRS-Id-r18 in ‘CandidateTCI-State’ as follows: JointTCI1: This field is mandatory present, if unifiedTCI-StateType in the ltm-TCI-Info in the ltm-CandidateConfiguration is set to joint. It is absent, Need R, otherwise. P3: Add condition ‘JointTCI’ for ul-powerControl-r18 in ‘CandidateTCI-State’ as follows: JointTCI: This field is optionally present, Need R, if unifiedTCI-StateType in the ltm-TCI-Info in the ltm-CandidateConfiguration is set to joint. It is absent, Need R, otherwise P4: add clarification to Section 10.1 of TS37.340 as “In case of PSCell addition/change, the MCG/SCG LTM candidate configurations can be kept/added/modified/released” P5: RAN2 is kindly asked to confirm that L2 reset is also applicable for the intra-DU LTM case, and an LS can be sent to RAN3. P6: Upon LTM cell switch, UE doesn’t release/clear logged measurement configuration P7: Upon subsequent CPAC configured by MCG, UE doesn’t release/clear logged measurement configuration P8: In NR-DC, MN considers the LTM configuration in SN (and MN) and the capabilities 39-2 and 39-3 while configuring measurement gaps P9: SN can inform the NR-ARFCN, physical cell-id (PCI) and SSB configuration of its LTM candidate cells to MN in CG-Config INM P10: Clarify that RRCReconfiguration applied is identified by Target Configuration ID in RRC as follows P11: Capture the handling of SRB5 during Subsequent CPAC execution | P1-P3: To be agreed. This is in line with what we have for legacy. P4: Not pursued. This is not really needed as for the legacy PSCell addition/change we follow the legacy principle of setupRelease in the ASN.1. I think we should not state the obivious. P5: To be agreed. We never limited in RAN2 the L2 reset only to the inter-DU LTM cell switch. Whether to send the LS or not can be discussed. Eventually RAN3 can simply read our agreement. P6: To be agreed. P7: To be agreed. P8-P9: To be discussed. There are other proposals from other companies on this topic. P10: Not pursued. I see no point in clarify this as the existing text is already fine. Also, there is no Target Configuration ID in RRC, so the change it will create even more confusion. P11: To be discussed. This is a co-existence issue which was never discussed. |
| R2-2407072 | Nokia | P1: In SK-CounterConfiguration-r18 , the size of sk-CounterConfigToReleaseList-r18 and sk-CounterConfigToAddModList-r18 should be updated as 1..maxSecurityCellSet-r18-1 P2: For the SCPAC scenario where initial serving SN/PSCell is not part of SCPAC , the Network should ensure that the servingCellSetID is set to a unique value different than the security-cell-set-ID of the candidates. P3: The description for securityCellSetId should be as follows (red part is the additional text) This field is used to determine whether UE should perform security update when conditional reconfiguration containing subsequentCondReconfig is executed. If the field servingSecurityCellSetId is configured, this field is configured for all the candidate configurations for subsequent CPAC. If the subsequent CPAC for inter-SN CPA is configured, then this field is configured for all the candidate configurations P4: Allow R18 EMR to be configured without T331 timer P5: Add field description for validityStatus as proposed in the Annex 2. | P1: Not pursued, I guess 9 is because there are cases where the serving cell is not one of the subsequent CPAC configuration. P2: To be agreed, but is already the understanding. P3: To be discussed. I think that how the network set (or whether it needs to set) this ID needs to be discussed. P4: Not Pursued. The Rel-18 EMR is built on the existing framework and there the timer T331 is mandatory. P5: Not Pursued. It was decided in the last meeting to delete this field description as the procedural text already described quite well what this field is and how the UE needs to handle it. |
| R2-2407091 | Ericsson | Misc stage 2 corrections for subsequent CPAC | To be discussed in a stage 2 offline |
| R2-2407175 | Ericsson | Misc RRC corrections for the FeMob WI | This CR will be used to capture the agreements from this meeting. |
| R2-2407176 | Ericsson | P1: The existing measurement gaps framework is re-used for LTM without the need to introduce new signalling. P2: If UL skipping is configured, the UE ignore the UL skipping indication for the first transmission in the target cell during an LTM cell switch procedure, i.e. the UE always performs the first transmission P3: RAN2 to clarify at which point, after the reception of the LTM cell switch MAC CE, the UE starts to monitor the PDCCH | P1-P3: To be discussed. |
| R2-2407200 | Huawei | P1a: Confirm the understanding that the information to lower layers that the UE is configured with UE-based TA measurements for a candidate remains valid after LTM cell switch, if there is no new explicit information. P1b: Discuss whether to clarify that the UE shall not do any UE-based TA measurement for a candidate whose SpCell is the current UE SpCell (current procedure text says to inform lower layers that the UE is configured for UE-based TA measurements for this candidate). P1c: Discuss whether there is a need to clarify that after LTM cell switch to a candidate configuration for which the UE was configured with UE-based TA measurements, the UE considers itself as configured with UE-based TA measurements for the candidate configuration whose PCell is the previous UE PCell. P2: Capture in the LTM candidate configuration addition/modification procedures that, when ltm-UE-MeasuredTA-ID is not present for a LTM candidate configuration, the UE informs lower layers that UE-based TA measurement is not configured. P3: Capture that when the network configures LTM candidates with ltm-UE-MeasuredTA-ID, the network ensures that the UE has a value stored in ltm-ServingCellUE-MeasuredTA-ID. P4: In the field description of unifiedTCI-StateType in LTM-TCI-Info, capture that the network always configures this field. P5: The network does not indicate which LTM candidate configurations can be fast processed by the UE. Fast RRC processing for LTM candidate configurations is up to UE implementation | P1a: To be agreed. I think this was the intention from the beginning. P1b-P1c-P2-P3: To be discussed. Several companies have proposals for this aspect. P4: Not pursued. I think that MTK proposal is maybe better, also because whether only the unified TCI state framework is used for LTM in Rel-18 is not crystal clear. P5: To be discussed. This is a postponed issue from the last meeting. Need to be discussed. |
| R2-2407370 | Qualcomm | P1: RAN2 to agree adding a new IE ltm-tdd-UL-DL-ConfigurationCommon-r18 in LTM-Candidate-r18 | P1: RAN1 is discussing this and it seems that more parameters are needed. We should wait the outcome from RAN1 discussions. |
| R2-2407410 | Sharp | A change to improve the UE-based TA measurement procedure | P: To be discussed together with the other proposals on this topic. |
| R2-2407449 | Lenovo | P1: The UE stops evaluating the execution condition(s) for CHO when a PSCell change is triggered by SCG LTM. And the UE resumes evaluating execution condition(s) for CHO once UE completes SCG LTM. | P1: To be agreed. This is in principle fine but to we need to capture anything explicitly? |

According to what is discussed above, the following proposals are made.

The following proposals are suggested to be agreed:

Subsequent CPAC

In the field description of subsequentCondExecutionCond clarify that SCPAC configured in MN format, subsequent CPAC execution condition update is not applied for subsequentCondExecutionCond.

Upon subsequent CPAC execution, the logged measurement configurations are not released by UE.

For the SCPAC scenario where initial serving SN/PSCell is not part of SCPAC, the Network should ensure that the securityCellSetId is set to a unique value different than the securityCellSetId of the candidates. No specification impact.

LTM

In the field description of unifiedTCI-StateType, specify that the UE applies value separate if unifiedTCI-StateType is absent in LTM-TCI-Info. In addition, change the need code of unifiedTCI-StateType to Need S.

The network may provide the ltm-Config without any LTM-CSI-ResourceConfig.

Update the 2TA condition according to the Proposal 1 in R2-2407050.

Introduce new conditional presence for the fields pathlossReferenceRS-Id and ul-powerControl-r18 according to proposal Proposals 2 and 3 in R2-2407050.

The L2 reset IDs are applicable for both intra- and inter-DU LTM.

Upon an LTM cell switch execution, the logged measurement configurations are not released by the UE.

The information to lower layers that the UE is configured with UE-based TA measurements for a candidate remains valid after LTM cell switch, unless otherwise indicated by the network.

The following proposals are suggested to be discussed ONLINE:

RAN2 to discuss whether the existing measurement gap framework can be re-used for LTM.

RAN2 to discuss whether a nested LTM-Config (an LTM-Config within an LTM-Config) is supported or not.

RAN2 to discuss whether the network indicated to the UE on which LTM candidate configuration the early ASN.1 decoding is expected.

RAN2 to discuss the co-existence of subsequent CPAC and QoE.

RAN2 to discuss whether the UE should ignore the UL skipping indication (if configured) upon an LTM cell switch procedure.

RAN2 to clarify at which point the UE starts to monitor PDCCH upon an LTM cell switch procedure.

The remaining proposals are suggested to be discussed OFFLINE:

Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline, except for stage2 and capabilities.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

The following proposals are suggested to be agreed:

Subsequent CPAC

Proposal 1	In the field description of subsequentCondExecutionCond clarify that SCPAC configured in MN format, subsequent CPAC execution condition update is not applied for subsequentCondExecutionCond.

Proposal 2	Upon subsequent CPAC execution, the logged measurement configurations are not released by UE.

Proposal 3	For the SCPAC scenario where initial serving SN/PSCell is not part of SCPAC, the Network should ensure that the securityCellSetId is set to a unique value different than the securityCellSetId of the candidates. No specification impact.

LTM

Proposal 4	In the field description of unifiedTCI-StateType, specify that the UE applies value separate if unifiedTCI-StateType is absent in LTM-TCI-Info. In addition, change the need code of unifiedTCI-StateType to Need S.

Proposal 5	The network may provide the ltm-Config without any LTM-CSI-ResourceConfig.

Proposal 6	Update the 2TA condition according to the Proposal 1 in R2-2407050.

Proposal 7	Introduce new conditional presence for the fields pathlossReferenceRS-Id and ul-powerControl-r18 according to proposal Proposals 2 and 3 in R2-2407050.

Proposal 8	The L2 reset IDs are applicable for both intra- and inter-DU LTM.

Proposal 9	Upon an LTM cell switch execution, the logged measurement configurations are not released by the UE.

Proposal 10	The information to lower layers that the UE is configured with UE-based TA measurements for a candidate remains valid after LTM cell switch, unless otherwise indicated by the network.

The following proposals are suggested to be discussed ONLINE:

Proposal 11	RAN2 to discuss whether the existing measurement gap framework can be re-used for LTM.

Proposal 12	RAN2 to discuss whether a nested LTM-Config (an LTM-Config within an LTM-Config) is supported or not.

Proposal 13	RAN2 to discuss whether the network indicated to the UE on which LTM candidate configuration the early ASN.1 decoding is expected.

Proposal 14	RAN2 to discuss the co-existence of subsequent CPAC and QoE.

Proposal 15	RAN2 to discuss whether the UE should ignore the UL skipping indication (if configured) upon an LTM cell switch procedure.

Proposal 16	RAN2 to clarify at which point the UE starts to monitor PDCCH upon an LTM cell switch procedure.

The remaining proposals are suggested to be discussed OFFLINE:

Proposal 17	Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline, except for stage2 and capabilities.