3GPP TSG-RAN WG2 Meeting #127	R2-2407183
Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.5.3

Source:	InterDigital

Title:	On-demand SIB1 request and reception

Document for:	Decision

# Introduction

RAN approved a new work item on network energy saving (NES) [1]. The WID includes the following objective accordingly:

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

RAN2 made the following agreements in RAN2#126 meeting [2]

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

This contribution discusses various aspects to support on-demand SIB1 for UEs in idle/inactive mode.

# Background for On-Demand SIB1 for Idle/Inactive UEs

The design of 5G NR is quite efficient from the perspective of minimizing network transmissions of always-on signals when there is no data. However, there is still some common signals and channels are transmitted/received at fixed periodicities, which consumes considerable energy at the network, especially when not transmitting for other activities such as baseband digital beamforming processing. Such idle power consumption can be considerable in dense networks, e.g., during low cell load conditions. Network energy consumption can therefore be reduced when the network does not exchange transmissions with the UE. Depending on cell load, it can be more beneficial to adapt the periodicities at which SSBs, SIBs (e.g., SIB1), RACH, and paging channels are transmitted/received.

The Rel-18 NES WI mostly focused on enhancements targeting connected mode and low load scenarios, while avoiding any impacts on the IDLE/INACTIVE or legacy UE operation. Rel-18 kept its focus on reducing the network energy consumption associated with the transmission of user specific signals and channels. The NES gains from cell DTX/DRX operation can be limited, as the gNB still needs to be ready for the transmission of common signals/channels such as periodic SSB, SIB1 etc, and RACH reception. More energy savings are thus possible by considering NES techniques when supporting UEs operating in RRC idle/inactive states.

In 5G systems, SIB1 is always broadcast with SSB. SIB1 information is essential for the idle/inactive UEs as it provides the information to camp on a cell and provides necessary information on how to obtain additional system information. From energy saving perspective, the network may choose a cell to transmit reduced periodicity SIB1 transmissions or completely stop transmitting SIB1 at all. This then requires a mechanism through which idle or inactive mode UEs in these cells can request on-demand SIB1 transmission. Following sections discuss the deployment scenarios for on-demand SIB1, configuration acquisition for trigger signals and signalling to trigger on-demand SIB1.

# Deployment Scenarios for On-demand SIB1

To target energy savings by reduced transmission of SIB1 or SIB1 turned off, different types of network deployments may be considered. The network deployments may be based upon micro or capacity cells overlaid over macro cells, cells deployments over different carrier frequencies and/or different bands, cells operating in FR1/FR2, etc. To study the implementations and procedural updates for the feature of on-demand SIB1, RAN1 agreed the following:

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

RAN2 has agreed in RAN2#125Bis meeting to study scenario 1a (RAN1 case 2) as captured in the following agreement:

Agreements on on-demand SIB1:

- At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

In the previous RAN2 meeting, there was some discussion on RAN1 case 3. In RAN1 case 3, UE acquires the UL WUS configuration from Cell A, transmits UL WUS to cell A and obtains the SIB1 for NES cell from Cell A as well. This scenario may have certain drawbacks:

- One issue is related to the fact that UE will be camping over the NES cell and if the UE needs to re-acquire SIB1, the UE may need to re-select to Cell A, acquire SIB1 and then camp on NES cell again. This will result in additional overhead and increased UE power consumption. Further, once a WUS configuration is no longer valid for a given NES cell and the UE requests OD-SIB1 for such cell, the UE is forced to perform cell reselection to another cell A to acquire OD-SIB1 on the NES cell, thus also increasing UE power consumption.

- Another issue is related to the deployments where NES cell and cell A are not co-located. If the UE is located close to NES cell, it still needs to acquire the configuration, transmit UL-WUS to cell A, which may have worse coverage than the NES cell. This will result in increased UE power consumption, an increased probability of RACH failure, and may still have coverage issues.

- RAN1 case 3 may have limitations on the network side as well. If the Cell A is supporting multiple NES cells, it will be receiving UL-WUS and transmitting SIB1 for multiple NES cells. This may result in considerable PRACH overhead for cell A. Depending on the cell geometry and where the UE is located, the UE may select different cell As.

Proposal 1: 	RAN1 case 3 (OD-SIB1 of NES Cell is requested and received from Cell A) is not pursued further in Rel-19.

# UL-WUS configuration for On-demand SIB1

According to the WID for enhanced NES, the triggering for on-demand SIB1 will use UL WUS signal based upon existing signal/channel. One important step to enable on-demand SIB1 transmissions is the provisioning of UL WUS configuration for the UEs which are currently in idle/inactive mode.

When Cell A is providing the UL-WUS configuration for one or more NES cells, e.g., to support the scenario 1a, the Cell A may transmit UL-WUS configuration as part of the system information. Although the UL-WUS configuration can be included in one of the legacy SIBs, the overhead will scale with the number of NES cells, which can be cumbersome of existing SIBs. A cleaner design would thus be to define a new SIB where UL-WUS configuration for one or more NES cells is transmitted by the Cell A.

Proposal 2: 	Support defining a new SIB carrying UL-WUS configuration for (neighboring) NES cell(s).

UL-WUS configuration will allow the UE to identify the NES cell. This is captured in the following RAN2#126 agreement through inclusion of PCI and frequency as part of the UL-WUS configuration:

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Based upon the received UL-WUS configuration, the UE can transmit the UL WUS to the NES cell. Thus, at minimum, the UL-WUS configuration provides the RACH configuration for the NES cell. The RACH configuration should provide the suitable RO and preamble determination for the UE based upon the SSBs being transmitted by the NES cell. This was agreed in RAN2#125Bis.

In order to transmit Msg1, the UE needs to know the mapping of SSB to RO. The reception of RAR in response to the UL-WUS transmission will require that UE knows a CORESET, e.g., CORESET 0, where it can receive and decode the response. The CORESET configuration for RAR can be the CORESET 0 or a different configuration, which can be investigated in RAN2. Whatever that configuration is should be part of the UL-WUS configuration so that the UE is able to transmit UL-WUS and receive the RAR.

Proposal 3: 	Support UL-WUS configuration comprising of at least: applicable NES cell(s),  RACH configuration (including resource partition, power control, SSB to RO mapping etc), and CORESET configuration to receive RAR.

One aspect related to the UL-WUS RACH configuration is whether the RACH configuration is as per legacy or if there is a separate RACH configuration to trigger SIB1. Although RACH configuration triggering SIB1 transmissions shared with legacy RACH functionality can work, a separate RACH configuration as trigger can be more advantageous, contention free from legacy UEs, and can be used for all UEs in the cell. It is not likely to need too many preambles for requesting OD-SIB1, as in legacy request for other SI. Legacy UEs are also not likely to be camped on such SIB1-less cell. Further, with dedicated RACH functionality, multiple NES cells may support the same trigger configuration.

From our understanding, a dedicated RACH configuration is basically the intention from the following RAN1 agreement made in RAN1#116Bis meeting.

RAN2 can adopt the support of dedicated RACH configuration for UL WUS for further work in line with the RAN1 agreement.

Proposal 4:	Support a dedicated RACH configuration for requesting OD-SIB1, different from common RACH configuration for UL WUS.

# Conditions to request On-demand SIB1

RAN2#126 had a discussion on whether if there is a new RSRP based condition for triggering of UL-WUS transmission is needed, in addition to the already agreed conditions. In our understanding, an RSRP based condition for UL-WUS transmission is useful to avoid the scenarios where a UE is in poor coverage at the NES cell and is transmitting UL-WUS waking up the cell. The RSRP based condition can be implemented as part of the RACH configuration within UL-WUS configuration.

Proposal 5:	Support an RSRP based condition for the UE to transmit UL-WUS to an on-demand SIB1 based cell.

If the RSRP based condition is not met, the UE will not transmit UL-WUS to the NES cell. This will result in UE not able to receive SIB1 from the NES cell and will treat the NES cell as barred. In such a case, the UE may perform cell re-selection to other cells.

# SIB1 Validity

One signalling aspect for UL WUS transmission is related to the target cell recipient of the UL WUS signal. If the UL WUS configuration is available, the NES cell should be the target receiver for the UL WUS signal. If the UL WUS is transmitted directly to the NES cell, the NES cell may transmit SIB1 after detecting the PRACH signal. For the situations when the anchor cell is not good enough for the UEs, and these UEs are located much closer to the NES cell than an anchor cell, having the possibility of transmitting the UL WUS to the NES cell can be more beneficial.

In RAN2#117, it was discussed to have a )D-SIB1 configuration validity criterion, such that if a UE has a valid version of SIB1 stored, UE does not initiate the on-demand SIB1 request. When the UE camps on a NES cell, the network may not know when the UE acquired SIB1. For the cells not transmitting SIB1, it may be important to maintain a valid version of SIB1 at the UEs. Following the validity functionality for other SIBs, time-based validity for SIB1 can be introduced.

Proposal 6:	Time based validity can be introduced for OD-SIB1 and UL WUS configuration maintenance.

In the scenario where the UE has a valid UL WUS configuration and is camped on the NES cell, if the UE needs to acquire SIB1 from the NES cell, e.g., after a certain validity period, the UE may transmit UL WUS to the NES cell.

Proposal 7:	If the UE does not have a valid SIB1 stored for the NES cell and an UL WUS configuration is available, the UE transmits UL WUS towards the NES cell once triggered.

In the scenario where the UE does not have a valid UL WUS configuration and is camped on the NES cell, the UE should then attempt to re-acquire a valid configuration; if no configuration is available, the UE can perform cell reselection.

Proposal 8:	If no valid WUS configuration is available, the UE attempts to acquire an UL WUS configuration. If the UE cannot acquire a valid WUS configuration for the camped NES cell, the UE performs cell reselection.

# Handling for Legacy UEs in On-demand SIB1 based Cells

When a network operator is employing energy savings by cells where only on-demand SIB1 is being transmitted, the operator may like to block the legacy UEs accessing these cells. The legacy UEs may for example receive and decode an on-demand SIB1 transmission and try to camp on this NES cell. This may result in UE power waste and also make network not able to sleep as per the intention.

The following agreement was reached in RAN2#126:

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

One solution can be to indicate on-demand SIB1 based cells as barred cells through the MIB indication. This will stop the legacy UEs from accessing the cells whether or not they receive SIB1. This will then require some understanding for Rel-19 NES capable UEs to still camp on such a cell despite cell barred MIB based indication. In one implicit design, if the UE knows that a cell is on-demand SIB1 based cell, it may expect receiving “cell barred” in MIB but treat the cell as not barred. Another solution can be to add an explicit indication in the (on-demand) SIB1 cell being not barred for Rel-19 NES capable UEs.

Proposal 9:	Legacy UEs can be barred through legacy MIB “cell barred” indication. For Rel-19 NES capable UEs, RAN2 to consider the following options for allowing R19 NES UEs to camp on the NES cell:

  - Receiving a separate explicit indication to enable camping on the NES cell (e.g. similar to R18 cell DTX barring) form the NES cell or Cell A;

  - if the UE doesn’t detect SIB1 on the NES cell and determines that the cell is on-demand SIB1 based cell, e.g. by receiving a UL WUS configuration from cell A for the NES cell, the UE can camp on the cell.

# Conclusions

The following proposals are made in this document for on demand SIB1 acquisition in IDLE/Inactive states:

Proposal 1: 	RAN1 case 3 (OD-SIB1 of NES Cell is requested and received from Cell A) is not pursued further in Rel-19.

Proposal 2: 	Support defining a new SIB carrying UL-WUS configuration for (neighboring) NES cell(s).

Proposal 3: 	Support UL-WUS configuration comprising of at least: applicable NES cell(s),  RACH configuration (including resource partition, power control, SSB to RO mapping etc), and CORESET configuration to receive RAR.

Proposal 4:	Support a dedicated RACH configuration for requesting OD-SIB1, different from common RACH configuration for UL WUS.

Proposal 5:	Support an RSRP based condition for the UE to transmit UL-WUS to an on-demand SIB1 based cell.

Proposal 6:	Time based validity can be introduced for OD-SIB1 and UL WUS configuration maintenance.

Proposal 7:	If the UE does not have a valid SIB1 stored for the NES cell and an UL WUS configuration is available, the UE transmits UL WUS towards the NES cell once triggered.

Proposal 8:	If no valid WUS configuration is available, the UE attempts to acquire an UL WUS configuration. If the UE cannot acquire a valid WUS configuration for the camped NES cell, the UE performs cell reselection.

Proposal 9:	Legacy UEs can be barred through legacy MIB “cell barred” indication. For Rel-19 NES capable UEs, RAN2 to consider the following options for allowing R19 NES UEs to camp on the NES cell:

  - Receiving a separate explicit indication to enable camping on the NES cell (e.g. similar to R18 cell DTX barring) form the NES cell or Cell A;

  - if the UE doesn’t detect SIB1 on the NES cell and determines that the cell is on-demand SIB1 based cell, e.g. by receiving a UL WUS configuration from cell A for the NES cell, the UE can camp on the cell.

# References

RP-240170, Revised WID: Enhancements of network energy savings for NR, Mar 2023

RAN2 Chairman’s note, RAN2 #126, May 2024.

RAN2 Chairman’s note, RAN2 #125Bis, April 2024.

RAN1 Chairman’s note, RAN1 #116Bis, April 2024.

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023