3GPP TSG-RAN WG2 Meeting #127	R2-2407185
Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.5.2

Source:	InterDigital

Title:	On demand SSB transmission for SCell

Document for:	Discussion, Decision

# Introduction

The Rel-19 New WID on Network Energy Savings for NR has the following objective on on-demand SSB SCell [1]:

The WID includes the following objective accordingly:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The agreements made in RAN1#116 meeting are provided in the appendix. In this contribution, we discuss the enhancements for enabling on-demand SSB SCell.

# Discussion

For CA deployments, SCells with SSB-less Scell has been supported since Rel-15 for both FR1 and FR2. In Rel-18 the support for SSB-less Scell is extended for collocated cells in FR1 that are in inter-band CA (e.g. within max receive time, frequency and power difference). In the inter-base CA case, the NW signals to the UE which cell is the reference cell (e.g. a Pcell or another Scell), where the reference cell acts as the timing reference and AGC source of the SSB-less Scell.

Supporting inter-band CA for FR2 comes with challenges, e.g. achieving synchronization, L1/L3 measurements and beam management, especially when the cells are non-collocated and only the Scells operate in FR2 while other cells in FR1. For energy saving purposes in CA, it is important for the Scells to operate in a NES mode as long as possible without or with limited SSB transmission.

## Signalling and reporting for OD-SSB reception

During Rel-18 SI, It was discussed that the Scells which are used as capacity cells and operating in NES mode are not expected to transmit SSBs or only transmit sparse SSBs. gNB initiated triggering of OD-SSB in the context of Scell activation procedure has been agreed by R1. In addition to gNB initiated triggering, triggering OD-SSB transmission from the Scells only when there is a need from the UE (e.g. based on channel conditions when the UE moves into coverage, specifically when cells are not geolocated) is also practical. The following options were discussed in RAN1 for the indication of reception of OD-SSB; the selection of which option was left for RAN2:

- MAC CE is separate from the legacy MAC CE for SCell (de)activation

- MAC CE can be also used for SCell (de)activation

- MAC CE is the same as the legacy MAC CE for SCell (de)activation with enhancements

Since RAN1 agreed to support OD-SSB before SCell activation, at least a MAC CE separate from the legacy MAC CE is needed. Further, since the activation of the SCell itself is based on the network’s decision, i.e. based on the reception of UE measurement and L1 CSI, the activation decision may lag the indication for the reception of an OD-SSB. Therefore, it is proposed to use a MAC CE separate from the legacy MAC CE for SCell (de)activation for the indication of a OD-SSB.

Proposal 1:	Reception of on-demand SSB on an SCell is indicated by MAC CE signalling separate from the legacy MAC CE for SCell (de)activation. It is up to R1 whether L1 signalling is also supported for OD-SSB indication.

Proposal 2:	The OD-SSB indication MAC CE signalling at least includes the SCell index, FFS whether there is a need to indicate anything else

Proposal 3:	The OD-SSB indication MAC CE signalling does not indicate SCell (de)-activation. Reuse legacy MAC CE for SCell (de)activation.

RAN1 already agreed to support L1 measurements based on the received OD-SSB, including periodic, semi-persistent, and aperiodic L1 measurements based on existing CSI framework. Whether the UE also supports L3 measurements based on was left for RAN2 to handle. Generally speaking, it is beneficial for the network to receive some feedback regarding the OD-SSB, especially since the SCell can be deactivated prior to the OD-SSB transmission instance. Such feedback can be in the from of a measurement report or any other information that can help the network with (or whether to) activate the SCell, including L3 measurements or preferred SSBs. Given the OD-SSB structure is expected to be periodic, the UE is able to leverage such SSBs for L3 measurements as well, as those can be useful for the network.

Proposal 4:	Support L3 measurements and reporting on OD-SSBs, if configured.

In terms of what can be configured by RRC in advance, some parameters can be configured for the structure and expected OD-SSB parameters. Parameters may be added in addition to those used for legacy SCell activation, per RAN1’s discussions and agreements:

Proposal 5:	The following parameters can be configured by RRC to support OD-SSB reception on SCells:

  - Frequency of the OD-SSB

  - SSB positions within an OD-SSB burst

  - Periodicity of OD-SSB

  - Sub-carrier spacing of the OD-SSB

  - Physical Cell ID of the OD-SSB

  - Location of on-demand SSB burst (e.g. offset, half frame)

  - Downlink transmit power of OD-SSB

  - How many OD-SSB burst(s) are transmitted once indicated

  - Gap length between SSB bursts

  - L1 and L3 measurement configuration for measurements on OD-SSBs

In addition to OD-SSB reception triggered by MAC CE, the request for on-demand SSB can be conveyed by the UE, as likely the conditions for requesting such demand is known at the UE. The UE can therefore send the UL signal to gNB, corresponding to UL wake-up signal (WUS), to request for OD-SSB transmission. Using UL WUS as a triggering method for OD-SSB transmission also results in considerable NES gains. The Rel-18 TR [2] mentions that with sparse SSB of 160ms periodicity transmitted before gNB reception of WUS, 5 sources show technique A-3-1 of UE WUS triggering gNB for SSB/SIB/RACH, with the assumption of ideal detection of UL WUS, could achieve BS energy savings by 6.2% – 80.7% in range with UPT loss by 0%-24.2%, where the gain increases as a function of the WUS period. Given the benefits in terms of low delay, energy savings and low UPT losses, UL WUS can be supported as a triggering method for OD-SSB at SCell.

Options considered in the R18 SI for UL WUS include using PRACH resource and PUCCH resource (e.g. SR). Using a PUCCH resource for triggering on demand SSBs can be useful for UEs in connected mode, but this may require introducing a new procedure for triggering the transmission and ensuring sufficient reliability/coverage when receiving at Scell and hence, less preferable for UL WUS. Using PRACH is thus simpler, since the gNB monitors PRACH on the PCell regardless of NES states.

contention-based PRACH resources for UL WUS can be configured to consist of a set of PRACH preambles and RACH occasions. Given there is no CBRA on the Scell, the UL WUS would have to be transmitted by the UE on the PCell.

The resources for UL WUS can be provided by the PCell to the NES-capable UEs as a separate configuration or a sub-configuration of the RACH config.

Proposal 6:	Support using PRACH preamble on the PCell as UL WUS for triggering OD-SSB on the SCell. FFS: Conditions for triggering UL WUS

## Assumption on OD-SSB cells and transmission

RAN1 already identified some assumptions on the SCell for which OD-SSB transmission is possible, and agreed to the following cases for when on demand SSB can be received:

    - No always-on SSB on the cell

    - Always-on SSB is periodically transmitted on the cell

transmitting always-on SSBs results in lower NES gains even when using longer SSB periodicity (e.g. 160ms) compared to considering the cell as SSB-less. With a prolonged SSB periodicity on the SCell, the impacts to legacy UEs can be higher when the SSBs are not transmitted per legacy periodicity.

With regards to the timing of the OD-SSB, the following options above can be broadly categorized as follows:

-	Category 1: Periodic OD-SSB transmission with single periodicity

-	Category 2: Periodic OD-SSB transmission with periodicity switching

-	Category 3: OD-SSB is transmitted once or a-periodically

For Category 1, the Scell is assumed to transmit OD-SSBs regularly with certain periodicity. Category 1 was agreed for the case for when on demand SSB is be received in a cell with no always-on SSB. This option also makes sense for when the SCell is activated, there will be a need for recurring SSBs, as the UE is in connected mode and will periodically report channel measurements. The applicability of this category is thus suitable for scenario where the UE has received an SCell activation command. Category 2 requires predefining instances in time at which SSB transmission assumptions will change. However, predefining such instances should be specific for different scenarios and UEs, and thus will be tough to configure or specify. Category 3 can be suitable when the SSB periodicity is already too long, and an on demand SSB occasion is needed. In the case of Category 3, the network has more flexibility transmit OD-SSB in a limited time period only when there is a need (e.g. for measurements at UE) and hence, enables higher NES gains. The aperiodic/semi-persistent OD-SSB transmission can be useful for all assumed deployment scenarios, including the case where the Scell is already activated and is applying NES.

Proposal 7:	The following assumption/options for OD-SSB transmission timing are considered:

  - UE expects that on-demand SSB is periodically transmitted, with single periodicity

  - UE expects that on-demand SSB is transmitted once or a-periodically

# Conclusion

The following proposals are provided for on demand SSB reception on SCells:

Proposal 1:	Reception of on-demand SSB on an SCell is indicated by MAC CE signalling separate from the legacy MAC CE for SCell (de)activation. It is up to R1 whether L1 signalling is also supported for OD-SSB indication.

Proposal 2:	The OD-SSB indication MAC CE signalling at least includes the SCell index, FFS whether there is a need to indicate anything else

Proposal 3:	The OD-SSB indication MAC CE signalling does not indicate SCell (de)-activation. Reuse legacy MAC CE for SCell (de)activation.

Proposal 4:	Support L3 measurements and reporting on OD-SSBs, if configured.

Proposal 5:	The following parameters can be configured by RRC to support OD-SSB reception on SCells:

  - Frequency of the OD-SSB

  - SSB positions within an OD-SSB burst

  - Periodicity of OD-SSB

  - Sub-carrier spacing of the OD-SSB

  - Physical Cell ID of the OD-SSB

  - Location of on-demand SSB burst (e.g. offset, half frame)

  - Downlink transmit power of OD-SSB

  - How many OD-SSB burst(s) are transmitted once indicated

  - Gap length between SSB bursts

  - L1 and L3 measurement configuration for measurements on OD-SSBs

Proposal 6:	Support using PRACH preamble on the PCell as UL WUS for triggering OD-SSB on the SCell. FFS: Conditions for triggering UL WUS

Proposal 7:	The following assumption/options for OD-SSB transmission timing are considered:

  - UE expects that on-demand SSB is periodically transmitted, with single periodicity

  - UE expects that on-demand SSB is transmitted once or a-periodically

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023

# Appendix: Agreements on OD-SSB SCell operation

RAN1#116 Agreements

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

RAN1#116bis Agreements

- Scenarios where OD-SSB can be triggered by gNB

  - Scenario #2 with Case #1 (no always-on SSB) and Case #2 (always-on SSB is periodically transmitted)

  - Scenario #2A with Case #1 and Case #2

  - FFS: Scenario 3A with Case 1 & 2

  - FFS: Scenario 3B with Case 1 & 2

    - For Case #1, once OD-SSB is triggered, its transmission is in a periodic manner

    - Notes:

      - Scenario #2A: When UE receives SCell activation command

      - Scenario #3A: After UE receives SCell activation command until SCell activation is completed

      - Scenario #3B: When SCell activation is completed, or after SCell activation is completed

- Type of SSBs for OD-SSB

  - Down-select between the following:

    - Alt-1: It is up to gNB implementation whether OD-SSB is CD-SSB or not

    - Alt-2: OD-SSB is limited to NCD-SSB.

      - FFS: Further limitations to OD-SSB

- Type of measurements

  - L1 and/or L3 measurements based on OD-SSB is supported for the cell

  - FFS further details on L1 and/or L3 measurement (e.g. timing)

- Study signalling for indicating OD-SSB

  - Option 1: Separate signalling for providing SCell (de)activation with legacy signalling (e.g., RRC, MAC CE) and for providing OD-SSB transmission indication

  - Option 2: A single signalling providing both SCell (de)activation and OD-SSB transmission indication

RAN1#117 Agreements

- Type of signaling for indicating OD-SSB

  - Support RRC based signaling

  - Support MAC based signaling

  - FFS: DCI based signaling

    - DCI does not provide Scell (de)activation

    - If supported, details on whether to use UE-specific or group-common DCI

  - FFS: Scenarios where the signaling is applicable

- Configuration info provided for OD-SSB:

  - Frequency of the OD-SSB

  - SSB positions within an OD-SSB burst (by using signaling similar to ssb-PositionsInBurst)

  - Periodicity of OD-SSB

  - Sub-carrier spacing of the OD-SSB

  - Physical Cell ID of the OD-SSB

  - Location of on-demand SSB burst (e.g. offset, half frame)

  - Downlink transmit power of OD-SSB

  - FFS: Further config details:

    - Whether more than one OD-SSB configs can be configured for the cell to UE

    - Whether RRC parameters are newly introduced or existing are reused

  - FFS: Other parameters, e.g.

    - How many OD-SSB burst(s) are transmitted once indicated

    - Offset between HARQ-ACK corresponding to OD-SSB transmission indication signaling and OD-SSB transmission

    - Gap length between SSB bursts

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

- Timing of OD-SSB transmission upon indication by MAC CE

  - Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted OD-SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signaling from gNB to indicate OD-SSB transmission

    - SSB time domain positions of OD-SSB burst are configured by gNB.

  - FFS: Details of value of T (≥ 0) including possibility of T comprising of multiple components

    - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

- Support L1 measurements on OD-SSB

  - Periodic, semi-persistent [and aperiodic] L1 measurements based on existing CSI framework

  - FFS: Potential enhancements of CSI report config and/or triggering/activation mechanisms for L1 measurements on OD-SSB