3GPP RAN WG2 Meeting #127	R2-2407188

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	8.8.4

Source:	InterDigital

Title:	Support for broadcast service in non-terrestrial networks

Document for:	Discussion, Decision

# Introduction

An objective for Rel-19 NR non-terrestrial networks (NTN) is to provide enhanced signaling support for multicast/broadcast services (MBS). MBS provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, requiring enhancements to notify the service area of a Broadcast service.

Detailed objectives from the WID [1] are as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

This contribution discusses necessary enhancements to support broadcast service in non-terrestrial networks, specifically the impact of NTN characteristics on broadcast services such as cell size and cell movement.

# Discussion

## Broadcast service area description

Multicast/Broadcast Service (MBS) is a point-to-multipoint service in which data is transmitted from a single source entity to multiple recipients either via broadcast or multicast transmission. For broadcast communication service, the same service and content data are provided simultaneously to all UEs in an MBS service area, which are defined via a cell ID list or Tracking area ID (TAI) list. Since terrestrial cells are fixed and the coverage area is relatively limited, a geographical area or civic address information can be readily translated into a list of cells (performed by the NEF/MBSF, see TS 23.247 [2]).

Alternatively, NTN cell sizes range from 100km to 3500km diameter and can cover multiple countries. Defining a geographic location in terms of cells is therefore unsuitable since it is likely at least some portion of a cell will cover a region where data from certain MBS session cannot be transmitted/received (e.g., due to regulatory requirements).

Figure 1: An MBS session area in terrestrial networks (left) vs non-terrestrial networks (right).

In RAN2#125bis it was agreed to model service area geographically [3] either via a circle (i.e., a reference point and radius) or a geographical area (i.e., a polygonal shape). The two agreed options allow a simple/low overhead solution (via circles) and a high accuracy solution (polygons). RAN2 should focus future discussion on which format is preferred.

Proposal 1:	No additional geographic area formats are considered to model the intended service area.

Further agreements discuss how the geographical service area interacts with cell coverage [3]:

- ”We will cover at least the case where the indicated intended service area covers a portion of a NTN cell”

- ”The intended service area can cover the area of more than one NTN cells (or portions thereof)”

Knowing which cell(s) serve the intended service area is important for content reception and service continuity. In legacy MBS the service area was defined by a list of cell IDs/tracking areas so there was no issue, however, it is unclear how the new geographic description of service area will be incorperated into the existing procedure. For example, if the geographic information and cell ID list are both available for a service area, the UE can reintepret the cell ID list as meaning the cells serving the intended service area instead of defining it. If the intention was to have the geographical description replace entirely the cell ID list then additional information may be needed.

Observation 1:	How the geographic description of service area interacts with the legacy MBS cell ID/TAI list needs to be clarified (e.g., whether the UE reinterprets the cell ID list or it is replaced entirely).

As a first step, RAN2 should clarify that at least which cells are serving the geographic service area are known to the UE. Whether this is via the existing procedure or some other means can be discussed further, possible pending progress on how the geographic area is signalled to the UE.

Proposal 2:	Which cell(s) serve a geographic service area is indicated to the UE. FFS whether the legacy cell ID list is reused, or new signalling is introduced.

In RAN2#126 it was agreed that both (quasi)earth fixed and earth moving cells are supported for MBS broadcast service. Supporting NGSO satellites requires consideration that the cells move continuously relative to earth. For example, a cell within the boundaries of a country at time T1 may cross into a region where the MBS session is not applicable at time T2. This means that the cells which make up an MBS session area will change over time due to satellite movement, requiring methods to dynamically update the MBS session area.

Observation 2:	Satellite movement in NSGO deployments could cause the cell(s) serving a service area to constantly change.

Figure 2: Due to satellite movement, the cells within a session area will change over time.

To avoid possible service continuity issues, the UE should be aware of how long each cell is serving the intended service area. Whether the existing mechanisms (e.g., t-Service) are sufficient can be further studied, as well as where this information is provided (e.g., within the MBS cell ID list or separately via SIB19).

Proposal 3:	In NGSO deployments, the duration a cell serves a geographic service area is indicated to the UE. FFS whether t-Service is reused, or new signalling is introduced.

The Rel-17/18, UE can trigger various actions (e.g., measurements) prior to cell coverage expiry to help the UE find a suitable cell before coverage disappears. RAN2 may also study whether similar enhancements are needed in NGSO deployments to ensure service continuity during a broadcast service. It is noted that the service continuity discussion may also require coordination with RAN3 (e.g., for network signaling aspects).

Proposal 4:	Study whether existing procedures are sufficient to ensure broadcast service continuity in NGSO deployments.

## Service content reception

Regarding how the service content is provided within the intended service area, the following was agreed [4]:

”RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area”

A consequence of describing a service area geographically and not preventing signalling outside of the area means that even though a serving cell ID belongs to a service area, the UE may not be located in the right area of the cell to access the service (e.g., if the cell footprint covers a larger area than the service area).

Observation 3:	Describing a service area geographically means that even though a serving cell ID belongs to a service area, the UE may not be located in the right area of the cell to access the service.

This may raise two possible issues:

- UE location information is needed to determine which service area(s) apply to the UE; and

- UE location may need to be verified by the network.

To access service content, the UE may first acquire updated location information to determine whether it is within a particular service area or area(s). Additional restrictions may be specified to ensure the UE only receives content if it is located within the service area outlined by the geographic description. The UE may also need to successfully complete the network verified UE location procedure (e.g., to ensure the reliability of the reported location information) prior to accessing the service content.

Observation 4:	Defining service area(s) within a cell may require updated UE location information to be acquired and/or verified to determine which service area(s) are applicable for the UE.

UEs with unverified location or locations outside of the service area can be prevented from accessing service content by using multicast communication service, which is used when not all UEs in the MBS service area are authorized to receive the data.

Alternatively, the UE may be prevented from accessing service content by controlling when the UE may establish/release MRB(s) associated with a service. This was heavily discussed in the RAN2#126 meeting, and concluded with the following agreements:

”Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)”

”Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)”

It is quite clear that if the UE is not in the intended service area, then the signalling is not intended for the UE. If the signalling is not intended for the UE, then the UE should not receive it. Managing the associated MRBs accordingly (e.g., establishing only while inside the intended service area, and releasing outside the intended service area) is a relatively straightforward way of preventing the UE from receiving data which is not intended for it. Leaving how the UE detects it is not in the intended service area up to UE implementation also provides some flexibility in when the UE establishes/releases bearers to not be overly restrictive on the UE.

It is therefore proposed that a simple and flexible way to prevent the UE from receiving MBS signalling outside of the intended service area is to control accordingly how the UE handles the MRBs when entering and exiting the intended service area.

Proposal 5:	A UE should not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area.

Proposal 6:	A UE should release established MRB(s) associated with a service limited to an intended service area when exiting the area.

# Conclusion

In this contribution the following observations and proposals are made regarding support for MBS in NTN

Observation 1:	How the geographic description of service area interacts with the legacy MBS cell ID/TAI list needs to be clarified (e.g., whether the UE reinterprets the cell ID list or it is replaced entirely).

Observation 2:	Satellite movement in NSGO deployments could cause the cell(s) serving a service area to constantly change.

Observation 3:	Restricting broadcast transmission to an intended service area is highly limiting to implementation, complex, and suboptimal for DL coverage.

Observation 4:	Describing a service area geographically means that even though a serving cell ID belongs to a service area, the UE may not be located in the right area of the cell to access the service.

Observation 5:	Defining service area(s) within a cell may require updated UE location information to be acquired and/or verified to determine which service area(s) are applicable for the UE.

Proposal 1:	No additional geographic area formats are considered to model the intended service area.

Proposal 2:	Which cell(s) serve a geographic service area is indicated to the UE. FFS whether the legacy cell ID list is reused, or new signalling is introduced.

Proposal 3:	In NGSO deployments, the duration a cell serves a geographic service area is indicated to the UE. FFS whether t-Service is reused, or new signalling is introduced.

Proposal 4:	Study whether existing procedures are sufficient to ensure broadcast service continuity in NGSO deployments.

Proposal 5:	A UE should not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area.

Proposal 6:	A UE should release established MRB(s) associated with a service limited to an intended service area when exiting the area.

# References

RP-240775 "Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3", Thales, CATT

3GPP TS 23.247

RAN2#125bis chair notes

RAN2#126 chair notes