3GPP RAN WG2 Meeting #127	R2-2407189

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	Applicable functionality reporting

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback.

Part of this general framework includes a mechanism for signaling applicable functionalities. This has been heavily discussed in the [POST126][032] email discussion [2], where the following procedure for reporting applicable functionalities is proposed:

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. Step 3: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) It is optional for network to provide NW-side additional condition in OtherConfig. Check whether companies are ok to change it as mandatorily signalled. 3) FFS on full inference configuration of supported functionalities. UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device. Step 4: UE considers to report applicable functionality in the following scenarios: 1) Upon change of applicable functionality change via UAI 2) As response to NW-side additional condition, FFS via UAI or RRCReconfigurationComplete, etc Step 5: 1) Network configures full inference configuration to UE after applicable functionality reporting, if inference configuration is not provided in Step 3 (i.e. full inference configuration is provided in Step 5). 2) If full inference configuration is provided to Step 3, it is up to network implementation whether to provide an updated configuration or not. The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

This contribution discusses the above procedure for applicable functionality determination and reporting, specifically which information is required by the UE to determine applicable functionalities.

# Discussion

## What is needed determine functionality applicability?

Phase 2 of [POST126][032] [2] discusses a framework to report (reactively or proactively) applicable AI/ML functionalities. Information used by the UE to determine and report applicable functionalities includes information internal to the UE (e.g., UE-side additional conditions and model availability) and external information and/or configurations provided by the network.

In the proposed procedure [2], external information and configurations are provided by the network in “Step 3”, including the following:

(Proposed) Step 3: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) It is optional for network to provide NW-side additional condition in OtherConfig. Check whether companies are ok to change it as mandatorily signalled. 3) FFS on full inference configuration of supported functionalities.

Apart from the configuration to support UAI reporting, there are two aspects possibly provided by the network in Step 3 for applicable functionality determination: NW-side additional conditions and inference configuration information. Whether one or both are mandatorily provided, or if the full inference configuration is needed or just some aspects of the configuration (e.g., beam information for set A and set B) is currently under discussion.

Observation 1:	Along with UE-side additional conditions and model availability, other information may be needed for the UE to determine applicable functionalities, such as:

- NW side additional conditions

- Full or partial inference configuration information

When considering the information already internally known at the UE, the following combinations of information for applicable functionality determination can be summarized in Table 1 below:

Table 1: Different information considered by the UE to determine functionality applicability.

|  | UE-side additional conditions | Model availability at UE | NW-side additional conditions | Inference configuration information |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Full | Partial |
| Option 1 | X | X |  |  |  |
| Option 2 | X | X | X |  |  |
| Option 3 | X | X |  | X |  |
| Option 4 | X | X |  |  | X |
| Option 5 | X | X | X | X |  |
| Option 6 | X | X | X |  | X |

As can be seen from the above, there are many options regarding what information is necessary to determine applicable functionalities, which should be concluded prior to finalizing any signalling procedure. Furthermore, definition of NW-side additional conditions and whether the Associated ID can include or imply certain aspects of the inference configuration is still under discussion.

For the purposes of the email discussion, the Rapporteur has captured the following note:

NOTE: In this email discussion and summaries, rapporteur assumes NW-side additional condition as associated ID (which is referred by majority of companies), and full inference configuration (e.g. CSI-RS resource configuration, etc) is considered separately from NW-side additional condition, i.e. it is not considered as part of NW-side additional condition in below proposals.

Although this assumption simplifies RAN2 discussion, it may not necessarily be in line with RAN1 understanding. If the above understanding is correct (i.e., Associated ID does not include/imply aspects of the inference condition) then additional inference configuration information may need to be separately provided. If RAN1 does conclude that inference configuration information is included/implied within the “Associated ID” (e.g., signaled along with NW side additional conditions) then how and when inference configuration information is provided in the above procedure will be impacted.

Observation 2:	How RAN1 defines Associated ID and NW-side additional conditions may impact whether other configuration information is needed to determine functionality applicability.

Furthermore, it seems quite difficult to conclude whether NW-side additional conditions are mandatorily provided or not in “Step 3” prior to a final definition of what NW-side additional conditions are in RAN1.

## Providing information for functionality applicability determination

Conclusion on the above aspects by RAN1 will greatly simplify discussion in RAN2 regarding which information is needed for applicable functionality reporting and at what step in the procedure it should be provided. For example, if NW-side additional conditions are required to determine functionality applicability (e.g., based on RAN1 definition), then it shall be mandatorily provided in Step 3 (e.g., prior to applicability determination). Alternatively, if NW-side additional conditions are not needed to determine/report applicable functionalities, then when the information is available to the UE (or provided at all) may be more flexible. To conclude the discussion in RAN2, progress is first needed from RAN1.

Similarly, if inference configuration information is needed to determine applicable functionalities (e.g., either because it is not included as part of Associated ID or a full inference configuration is needed), then such information should be provided in “Step 3” prior to the UE reporting functionality applicability. If separate inference configuration information is not needed, then to avoid restricting network implementation the configuration information could be provided at Step 3 or Step 5, (as currently supported by the proposed procedure). Again however, when and what aspects of the inference configuration is needed by the UE to determine applicable functionalities will require confirmation from RAN1.

To summarize, before RAN2 can progress and conclude on the applicable functionality reporting procedure, RAN1 must address the following questions:

Q1:	Does the Associated ID include information other than NW-side additional conditions (e.g., inference configuration information)?

Q2:	Is information other than Associated ID (i.e., including NW-side additional conditions) required for UE to determine applicable functionalities? (e.g., separate/additional inference configuration information)

Q3: 	If “Yes”, what information is needed? (e.g., the full inference configuration, or a subset of the full inference configuration)

Observation 3:	Before RAN2 can conclude the applicable functionality reporting procedure, RAN1 must progress on the following aspects:

- Does the Associated ID include information other than NW-side additional conditions (e.g., inference configuration information)?

- Is information other than Associated ID (i.e., including NW-side additional conditions) required for UE to determine applicable functionalities? (e.g., separate/additional inference configuration information)

- If “Yes”, what information is needed? (e.g., the full inference configuration, or a subset of the full inference configuration)

Considering the many open aspects in RAN2 which could be concluded relatively easily based on RAN1 input, it is proposed that discussion be postponed until the above be addressed by RAN1.

Proposal 1:	Postpone conclusion on applicable functionality reporting procedure until RAN1 progresses on: 1) definition of NW-side additional conditions; 2) relationship of Associated ID and inference configuration information; and 3) required configuration(s) for applicable functionality determination.

## Functionality non-applicability

A functionality may be non-applicable for many reasons outside of network control, such as UE speed, lack of model training at the UE etc. However, if a functionality is not applicable due to missing information from the network, this may be easily corrected (e.g., via a UE request/response procedure, or via proactive reporting etc.). Complimentary to (or as part of) the applicable functionality reporting procedure, RAN2 may consider introducing a “non-applicability” reporting option where the UE may indicate which functionality is non-applicable and the reason for non-applicability (e.g., not trained, missing information etc.). This may however be revisited after further progress from RAN1 regarding what information is needed to determine model applicability.

# Conclusion

In this contribution the following observations and proposals are made regarding applicable AI/ML functionality determination and reporting:

Observation 1:	Along with UE-side additional conditions and model availability, other information may be needed for the UE to determine applicable functionalities, such as:

- NW side additional conditions

- Full or partial inference configuration information

Observation 2:	How RAN1 defines Associated ID and NW-side additional conditions may impact whether other configuration information is needed to determine functionality applicability.

Observation 3:	Before RAN2 can conclude the applicable functionality reporting procedure, RAN1 must progress on the following aspects:

- Does the Associated ID include information other than NW-side additional conditions (e.g., inference configuration information)?

- Is information other than Associated ID (i.e., including NW-side additional conditions) required for UE to determine applicable functionalities? (e.g., separate/additional inference configuration information)

- If “Yes”, what information is needed? (e.g., the full inference configuration, or a subset of the full inference configuration)

Proposal 1:	Postpone conclusion on applicable functionality reporting procedure until RAN1 progresses on: 1) definition of NW-side additional conditions; 2) relationship of Associated ID and inference configuration information; and 3) required configuration(s) for applicable functionality determination.

# References

RP-240774 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (Qualcomm)

R2-240xxxx: Report of [POST126][032][AI/ML PHY] LCM Phase 2 (Intel Corporation)