3GPP TSG-RAN WG2 #127       			      		 R2-2407191

Maastricht, Netherlands – August 19 - 23, 2024		      Revision of R2-2405298

Agenda item:	8.10.2

Source: 	Kyocera

Title: 	SON/MDT reports for LTM

Document for:	Discussion

# Introduction

For Rel-19 the WID included further enhancement of data collection for SON/MDT for LTM, CHO with candidate SCGs, subsequent CPAC [1].  In this contribution, information that should be needed in the LTM RLF report to assist the network with LTM HO optimization is discussed.

# Discussion

The MRO enhancement for Rel-19 objective is described as follows, which includes the enhancement for LTM.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

During RAN2#126 the following agreements regarding LTM were reached:

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

2.1.	Enhancements for LTM

For Rel-18, the procedure for LTM is depicted in the following Figure 1.

Figure 1	 Signalling procedure for LTM (Figure 9.2.3.5.2-1 in [2])

With reference to Figure 1, one of the key features in LTM is the availability of a TA value prior to the cell switch to the target cell.  Currently, there are two ways for the UE to acquire the early TA values associated with each candidate cell.  The serving cell may send the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch or the UE performs TA measurement for the candidate cells after being configured by RRC. Considering the possibility of LTM failure due to inaccurate or invalid TA value(s), RAN2 should consider including the TA value(s) of all candidate cells or at least the TA associated with the target cell.

- Proposal 1	To support LTM optimization, RAN2 should consider if all available early TA values or just the early TA values associated with the target cell should be included in the RLF-report upon LTM failure.

Since the TA values may be provided to the UE during Cell Switch Command or calculated by the UE based on its own measurements, the RLF-Report should also differentiate between the two cases.

- Proposal 2	RAN2 should discuss whether it will be helpful to the NW to differentiate between TA values provided by Cell Switch Command and UE-based TA measurements.

In our view, the accuracy of the TA measurements (esp. for the case of UE-based TA measurement) is confined within the cyclic prefix used by each cell.  The cyclic prefix used in calculating the UE-based TA value.  CP is typically determined based on different frequency bands and deployment scenarios (e.g., macrocells would require larger CP to account for the larger delay spread).

- Observation 	The cyclic prefix used by each cell will impact the accuracy of the early TA values.

Although, the NW may also have available this information, it may be more convenient to have the UE provide this information together with the early TA values, regardless of whether they are provided to the UE or UE-based TA measurements.  In some cases, the network may dynamically change the numerology or CP length based on channel conditions or other factors.  If the network decides to change the CP length (e.g., due to mobility or interference), it informs the UE through updated SIB2. It indicates whether the cell uses normal cyclic prefix (NCP) or extended cyclic prefix (ECP).

- Proposal 3	RAN2 should discuss whether it will be helpful to include the cyclic prefix (e.g., NCP vs ECP) used for calculating the early TA values and the associated numerology.

In addition to logging the early TA values, RAN2 should also consider the elapsed time between the Cell Switch Command (step 6 in Figure 1) and the RACH Procedure (step 7 in Figure 1), which should also include the RACH-less procedure when early TA is available.  For the successful LTM case, this would be equivalent to logging the stop time associated with T304.  However, in case of RACH failure, the elapsed time should also be included in the RLF-Report.

- Proposal 4	RAN2 should discuss whether the elapsed time for LTM between the RRC Reconfiguration and the RACH procedure.

With regards to the early TA measurements, it would also be helpful to the NW to know whether the early TA measurement (esp. for the UE-based TA measurement) is completed before the UE receives the Cell Switch Command or the elapsed time between the completion of the early TA measurement and Cell Switch Command reception.  Depending on whether the UE has completed the early TA measurement prior to receiving the Cell Switch Command, the UE may end up with various options for access attempt.  Therefore, we think the following information should be included in RLF report.

- The type of TA value (if any) used for the LTM execution:

  - Early TA (with CFRA)

  - UE-based TA measurement

  - Configured TA value (i.e., TA=0 or TA=source)

- If early TA value was used, include the elapsed time between the early TA measurement completion and the reception of the Cell Switch Command.

- The type of access attempt used for the LTM execution:

  - RACH-less

  - CFRA

  - CBRA

- Proposal 5	RAN2 should include in the RLF report the type of TA value used for the LTM execution, the elapsed time between the early TA measurement completion and the reception of the Cell Switch Command and the type of access attempt used in the LTM execution.

# Conclusion

In this contribution, we addressed some potential enhancements to the RLF Report in support of Rel-18 LTM.  RAN2 is kindly asked to take into account the observation and proposals below:

- Proposal 1	To support LTM optimization, RAN2 should consider if all available early TA values or just the early TA values associated with the target cell should be included in the RLF-report upon LTM failure.

- Proposal 2	RAN2 should discuss whether it will be helpful to the NW to differentiate between TA values provided by Cell Switch Command and UE-based TA measurements.

- Observation 	The cyclic prefix used by each cell will impact the accuracy of the early TA values.

- Proposal 3	RAN2 should discuss whether it will be helpful to include the cyclic prefix (e.g., NCP vs ECP) used for calculating the early TA values and the associated numerology.

- Proposal 4	RAN2 should discuss whether the elapsed time for LTM between the RRC Reconfiguration and the RACH procedure.

- Proposal 5	RAN2 should include in the RLF report the type of TA value used for the LTM execution, the elapsed time between the early TA measurement completion and the reception of the Cell Switch Command and the type of access attempt used in the LTM execution.

# References

- RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, CMCC

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”,