3GPP TSG-RAN2 Meeting #127	R2-2407192

Maastricht, Netherlands, Aug 19-23, 2024

Agenda item:	8.11.2

Source:	Qualcomm Incorporated

Title:	Views on random access for SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. One of the objectives is to specify SBFD operation to support random access in SBFD symbols for UE in RRC-connected UE while studying the benefit of extending the random-access operation in SBFD symbols to UE(s) in RRC idle and inactive state.

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] …

In this contribution, we would like to discuss the configuration of random access resource in SBFD symbols for UE in RRC_CONNECTED and RRC_IDLE/INACTIVE.

# Discussion

There were already several basic agreements regarding supporting SBFD random access operation in RAN1.

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

It was agreed to support Type-1 random access procedure (4-step RACH) in SBFD symbols and further discuss Type-2 random access process (2-step RACH) after finalizing the design of 4-step RACH. The working assumption to support both CBRA and CFRA in SBFD symbols were confirmed in the later RAN1 meetings, since both CBRA and CFRA in SBFD symbols bring benefits of access latency reduction, PRACH collision probability reduction and improved coverage of PRACH.

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols and at least support 4-step RACH and FFS on 2-step RACH.

Regarding the RACH configuration for SBFD operation, there were two options under RAN1 discussion. One option is to use single RACH configuration from legacy RACH configuration with modification, and another option is to have two separate RACH configurations including one legacy RACH configuration and one additional RACH configuration to configure ROs within UL subband in SBFD symbols which could be valid for SBFD-aware UE.

It can be observed that single RACH configuration would enable random access for SBFD operation for both SBFD-aware UE and legacy UE without extra signaling overhead and separate RACH configuration for SBFD operation may enable more flexible configuration on the PRACH format, PRACH time/frequency resource, and so on.

In last RAN1 meeting, RAN1 confirmed the working assumption on supporting both options by network configuration but UE is not required to support both options.

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

For a SBFD-aware UEs in RRC_CONNECTED state, it may indicate whether supporting single or separate RACH configurations via UE capabilities. It is up to network to determine single or separate RACH configurations and network provides one RACH configuration for all UEs in the serving cell. For example, network may configure separate RACH configuration. However, if UE only supports single RACH configuration, UE has to use the legacy RACH resource to perform RACH

Proposal 1: For a SBFD-aware UE in RRC_CONNECTED state, network configures single RACH configuration or two separate RACH configurations for SBFD operation. The UE SBFD RACH operation may happen if UE reported SBFD-RACH related capability is matching gNB configuration.

There are still more details on supporting two options of RACH configuration for SBFD operation discussed in RAN1. RAN2 should wait for RAN1 further inputs on the detailed RACH configuration parameters.

Proposal 2: RAN2 waits for RAN1 further inputs on the detailed RACH configuration parameters for SBFD operation.

For RACH occasion (RO) selection, SBFD-aware UE may be configured with separate SSB mapping to SBFD-ROs and legacy TDD-ROs. From latency reduction perspective, the SBFD-aware UE can select the earliest RO to reduce the overall RACH latency. Thus, if there is no indication from network, it is up to UE implementation to select SBFD-RO or TDD-RO. It could be the nearest RO, either SBFD-RO or TDD RO or based on criteria, such as the RSRP of the selected SSB.

However, there are other considerations that network may identify the type of UE (i.e., SBFD-aware UE) via the selected ROs, and further considerations that SBFD-ROs may be less congested than TDD-ROs and the probability of RACH collision can be reduced. In that sense, network may indicate which type of RO that SBFD-aware UE is allowed to initiate RACH.

Proposal 3: Upon initiation of RACH procedure for a SBFD-aware UE, if network configures the SBFD-RO and no additional indication is from network, it is up to UE implementation to select either the SBFD-RO or TDD-RO. Otherwise, UE should follow network indication on the RO selection.

In case of PRACH attempt failures, UE may perform PRACH re-attempt. The SBFD-aware UE will increase the PRACH transmission power and should re-transmit the preamble in the earliest RO. UE may be stick to the same type of RO for PRACH re-transmission or may switch another type of RO after certain criteria evaluation. RAN2 can study the fallback procedure to address the multiple times failure on the same type of RO, e.g., SBFD-RO, and UE may then be fallback to select the other type of RO.

Proposal 4: RAN2 to study the fallback procedure on RACH occasion selection in RACH re-attempt for the SBFD-aware UE.

For the SBFD RACH configuration for RRC_IDLE/INACTIVE UE, the same design principle in RRC-CONNECTED state could be leveraged. The SBFD-aware UE obtains the information of SBFD time and frequency locations from system information.

In addition, the SBFD-aware UE should be able to determine whether gNB is adopting single RACH configuration or separate RACH configuration for SBFD operation.

Proposal 5: The design principles of RACH configuration for SBFD operation for RRC_CONNECTED state SBFD-aware UE is applied to the SBFD RACH configuration for RRC_IDLE/INACTIVE UE, and such SBFD RACH configuration is configured in system information.

It is important for the gNB to know the UE SBFD-awareness capability in early stages of the random-access procedure to leverage the SBFD random access operation. The SBFD-aware early indication is useful to inform the UE SBFD capability at the earliest time during RACH. The similar studies were done for many features in the previous release. The design is also heavily depending on the solution of RACH resource configuration for SBFD.

Proposal 6: RAN2 to study the SBFD-awareness indication for RRC_IDLE/INACTIVE UE.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols and at least support 4-step RACH and FFS on 2-step RACH.

Proposal 1: For a SBFD-aware UE in RRC_CONNECTED state, network configures single RACH configuration or two separate RACH configurations for SBFD operation. The UE SBFD RACH operation may happen if UE reported SBFD-RACH related capability is matching gNB configuration.

Proposal 2: RAN2 waits for RAN1 further inputs on the detailed RACH configuration parameters for SBFD operation.

Proposal 3: Upon initiation of RACH procedure for a SBFD-aware UE, if network configures the SBFD-RO and no additional indication is from network, it is up to UE implementation to select either the SBFD-RO or TDD-RO. Otherwise, UE should follow network indication on the RO selection.

Proposal 4: RAN2 to study the fallback procedure on RACH occasion selection in RACH re-attempt for the SBFD-aware UE.

Proposal 5: The design principles of RACH configuration for SBFD operation for RRC_CONNECTED state SBFD-aware UE is applied to the SBFD RACH configuration for RRC_IDLE/INACTIVE UE, and such SBFD RACH configuration is configured in system information.

Proposal 6: RAN2 to study the SBFD-awareness indication for RRC_IDLE/INACTIVE UE.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei