3GPP TSG-RAN2 Meeting #127	R2-2407194

Maastricht, Netherlands, Aug 19-23, 2024

Agenda item:	8.11.3

Source:	Qualcomm Incorporated

Title:	Other aspects of SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. The objectives include to specify the semi-static indication of time and frequency domain locations of the SBFD subbands to the UE and specify UE transmission, reception and measurement procedures in SBFD and non-SBFD symbols.

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

The other objectives of the work item are to study the enhancements for CLI handling, and RAN2 will focus on the signaling design for UL resource muting for PUSCH, and L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD.

In this contribution, we would like to discuss the configuration of SBFD time and frequency locations, the potential RAN2 impacts on transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols and enhancement for CLI handling.

# Discussion

RAN1 has agreed to support at least cell-specific configuration of SBFD time and frequency and will further discuss the need for additional UE-specific SBFD time and frequency configuration.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

Regarding the frequency location of SBFD sugband, the following option was agreed in RAN1.

Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

For the time location of SBFD subbands, it is agreed that SBFD subband time locations are configured within a period.

Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. FFS details Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

To accommodate SBFD operation for RRC_IDLE/INACTIVE, UE-dedicated frequency configurations are not applicable and then cell-specific configuration frequency configuration are needed. For semi-static indication of SBFD subbands time location, it is desirable to configure together with the existing parameter with modification, i.e., TDD-UL-DL-ConfigCommon. Thus, the cell-specific configuration on time and frequency location of SBFD subbands should be configured in system information.

Proposal 1: The cell-specific configuration on time and frequency location of SBFD subbands is indicated in system information.

Regarding the UE-specific configuration, it is still under RAN1 discussion and RAN2 can wait for RAN1 decision.

Proposal 2: RAN2 wait RAN1 further decision on whether additional UE-specific configuration is supported.

SBFD symbols can be configured in DL and/or FL symbols indicated by TDD-UL-DL-ConfigCommon. The SBFD-aware UE is half duplex UE which can either transmit or receive within the SBFD symbols. UE may utilize the UL resource configured in SBFD symbols to enjoy the benefits of the transmission latency reduction and low UL resource congestion. The resource configuration of the UL physical channels like SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols should be studied.

For example, for PUCCH resource configuration, there could be two options to have PUCCH transmission across SBFD and non-SBFD symbols in different slots. From resource configuration perspective, there could be separate PUCCH resource/set configurations for PUCCH transmission in SBFD and non-SBFD slots, or same PUCCH resource/set configuration for PUCCH transmission across SBFD and non-SBFD slots.

The similar principle of the resource configuration (single vs. separate) across SBFD and non-SBFD symbols can be applied to those other PHY channels also, like CG-PUSH, SPS-PDSCH, SRS and so on. Different resource configuration schemes may have impacts on the MAC layer setting and procedure of MAC features, like SR, CG, and PHR.

Thus, RAN2 should study the MAC impacts on the MAC features with SBFD operations considering RNA1 further conclusions.

Proposal 3: RAN2 to study the MAC impacts on the corresponding MAC features like SR and CG with enabling the SBFD operation.

Regarding the enhancement for CLI handling, RAN plenary approved RAN1 agreed down-selected schemes for CLI handling and revised the WID in [1].

Specifically, the L1 based UE-to-UE CLI measurement resource and reporting based on existing CSI framework should be specified. The configuration of periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource should be supported.

Proposal 4: For L1 based UE-to-UE CLI measurement configuration, the configuration of periodic, semi-persistent or aperiodic measurement resource (set), i.e., SRS-RSRP resource and CLI-RSSI resource should be supported. FFS on the detailed configuration parameters.

For L1 based UE to UE CLI reporting, new report quantities for CLI reporting should be introduced in the reporting configuration, e.g., L1-SRS-RSRP, L1-CLI-RSSI. Thus, we have following proposal.

Proposal 5: For L1 based UE-to-UE CLI reporting configuration, the configuration of new report quantities of L1-SRS-RSRP and L1-CLI-RSSI should be supported.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: The cell-specific configuration on time and frequency location of SBFD subbands is indicated in system information.

Proposal 2: RAN2 wait RAN1 further decision on whether additional UE-specific configuration is supported.

Proposal 3: RAN2 to study the MAC impacts on the corresponding MAC features like SR and CG with enabling the SBFD operation.

Proposal 4: For L1 based UE-to-UE CLI measurement configuration, the configuration of periodic, semi-persistent or aperiodic measurement resource (set), i.e., SRS-RSRP resource and CLI-RSSI resource should be supported. FFS on the detailed configuration parameters.

Proposal 5: For L1 based UE-to-UE CLI reporting configuration, the configuration of new report quantities of L1-SRS-RSRP and L1-CLI-RSSI should be supported.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei