3GPP TSG-RAN WG2 Meeting #127 	R2-2407195

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.3

Source:	Panasonic

Title:	Reference resource configuration for L1 measurement event

Document for:	Discussion

# Introduction

In last meeting, RAN2 achieved the following agreements on measurement enhancement for LTM.

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this paper, we focus on addressing the yellow-highlighted part in the above agreements. In particular, we discuss what beams of the serving and what beams of the candidate cell(s) for event evaluation, and how to ensure the same reference signal type (SSB vs. CSI-RS) for both serving and candidate cells. In our companion paper [1], the configuration of event conditions to support both early synchronization and cell switch (related to the above agreements 1 and 6) is discussed.

# Discussion

## What beams to evaluate

The discussion can be separated for serving cell and candidate cell. For serving cell, since UE is constantly performing beam management as configured and indicated by gNB, the current serving beam is expected to be a good reference for LTM event evaluation. The current serving beam means the current indicated TCI state that is used for communication in the serving cell. Or the beam selected by UE during random access procedure if no TCI state is indicated. During the online discussion in the previous meeting, there was a view that the best beam (instead of the current serving beam) should be used for LTM event evaluation concerning serving cell. In our opinion, if we assume the beam management procedure is configured and working properly, the best beam and the current serving beam should be the same in most cases. If not, the network might consider other factors such as load balancing for choosing the current serving beam. Even in this case, we think it is more reasonable to consider the current serving beam for LTM mobility decision as well.

Proposal 1. For LTM event evaluation involving serving cell, the current serving beam (current indicated TCI state in the serving cell or current UE selected beam through random access procedure if no TCI state is indicated) should be used.

For the candidate cell(s), on the other hand, the UE can be configured with a set of reference beams for measurement event evaluation. Those reference beams can be TCI states or reference signals directly. We will discuss both options in details below.

- Option 1: configured TCI state as reference beam for measurement event evaluation concerning candidate cell

In Rel-18 LTM, it is already supported that for each candidate cell UE can be configured with one or more TCI states (called candidate TCI state). See, e.g. the following IE CandidateTCI-State. As shown, candidate TCI states already support either SSB or CSI-RS as QCL reference signal.

-- ASN1START

-- TAG-CANDIDATETCI-STATE-START

CandidateTCI-State-r18 ::=           SEQUENCE {

tci-StateId-r18                      TCI-StateId,

qcl-Type1-r18                        LTM-QCL-Info-r18,

qcl-Type2-r18                        LTM-QCL-Info-r18                    OPTIONAL,   -- Need R

pathlossReferenceRS-Id-r18           PathlossReferenceRS-Id-r17          OPTIONAL,   -- Need R

tag-Id-ptr-r18                       ENUMERATED {n0,n1}                  OPTIONAL,   -- Cond 2TA

ul-powerControl-r18                  Uplink-powerControlId-r17           OPTIONAL,   -- Need R

...

}

LTM-QCL-Info-r18 ::=                 SEQUENCE {

referenceSignal-r18                  CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

},

qcl-Type-r18                         ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-CANDIDATETCI-STATE-STOP

-- ASN1STOP

The original purpose of candidate TCI state in Rel-18 was for UE to achieve early DL synchronization before switching to the candidate cell to save the cell switching delay. More specifically, after candidate TCI states are configured, UE can be activated with a subset of configured candidate TCI states which UE needs to maintain early DL synchronization. Then in the cell switch command, UE can be indicated with one of the activated TCI states that UE will be using directly as the serving beam after UE switches to the target cell (one of the candidate cells).

Option 1 is to use candidate TCI states as reference beams for LTM measurement event evaluation. As agreed in the last meeting (see section 1), the purpose of LTM measurement event is to achieve 1) Select the candidate beam/cell to trigger early synchronization, and 2) Select the target beam/cell and trigger LTM cell switch procedure. Both purposes fit well with the intention of candidate TCI states. Therefore, it seems reasonable to define candidate TCI states as the reference beam for measurement event evaluation for candidate cells.

- Option 2: Configured RS as reference beam for measurement event evaluation concerning candidate cell

Instead of using candidate TCI states, Option 2 is to directly configure RS resource set as reference beam for measurement event evaluation involving candidate cells. In Rel-18, the LTM measurement resource configuration has been introduced. See the following IE LTM-CSI-ResourceConfig which only supports SSB as reference signal.

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

For event triggered L1 measurement, the CSI-RS needs to be supported. It seems feasible to add CSI-RS resource set into the IE LTM-CSI-ResourceConfig for such purpose.

Proposal 2. For LTM event evaluation involving candidate cells, RAN2 to further discuss the following two options:

- Option 1: Configured candidate TCI state as reference beam

- Option 2: Configured RS as reference beam

## How to ensure the same RS type

First, we suggest to confirm the working assumption from previous meeting i.e. Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5, because otherwise, the measurements are not comparable between serving and candidate cells. Secondly, if proposals 1 and 2 are taken, the same RS type can be ensured by following the QCL RS associated with the indicated TCI state in the serving cell.

In more details, for option 1 (candidate cells use configured candidate TCI states), UE should first select the candidate TCI states that are associated with the same QCL RS as that in the indicated TCI state of the serving cell, and then performs measurement on the QCL RS for the selected candidate TCI states and QCL RS of the indicated TCI state of the serving cell.

For Option 2, UE measures the QCL RS of the indicated TCI state of the serving cell, and for candidate cells, UE measures the configured RS(s) with the same RS type as the QCL RS of the indicated TCI state of the serving cell.

Proposal 3. RAN2 confirms the WA related to same RS type for measurement event evaluation.

Proposal 4. The RS type for event evaluation should follow the QCL RS associated with the indicated TCI state in the serving cell; or if no TCI state is indicated in the serving cell, RS type is SSB.

# Conclusion

In this paper, we discuss reference resource configuration for L1 measurement event. The proposals are as follows:

Proposal 1. For LTM event evaluation involving serving cell, the current serving beam (current indicated TCI state in the serving cell or current UE selected beam through random access procedure if no TCI state is indicated) should be used.

Proposal 2. For LTM event evaluation involving candidate cells, RAN2 to further discuss the following two options:

- Option 1: Configured candidate TCI state as reference beam

- Option 2: Configured RS as reference beam

Proposal 3. RAN2 confirms the WA related to same RS type for measurement event evaluation.

Proposal 4. The RS type for event evaluation should follow the QCL RS associated with the indicated TCI state in the serving cell; or if no TCI state is indicated in the serving cell, RS type is SSB.

# Reference

[1] R2-2407208, Event triggered reporting enhancements for LTM, Panasonic