3GPP TSG RAN WG2 Meeting #127	R2-2407198

Maastricht, Netherlands, 19 - 23 August, 2024

Agenda Item:	7.4.3

Source:	Huawei, HiSilicon

Title:	MAC CR rapporteur summary

Document for: Discussion and Decision

# 1	Introduction

This document summarizes all the corrections proposed to TS 38.321 for LTM and indicates the points to make decisions for correction or not.

# 2	Discussion

The following documents were submitted:

| R2-2406331 | Corrections to TS 38.321 for LTM | CATT |
| --- | --- | --- |
| R2-2406349 | MAC corrections for LTM | Samsung Electronics Co., Ltd |
| R2-2406439 | Discussion on MAC open issue for LTM | vivo |
| R2-2406517 | Discussion on fallback RACH for LTM | ASUSTeK |
| R2-2406530 | Discussion on carrier selection for RACH-LESS LTM | OPPO |
| R2-2406853 | Corrections of MAC issues for RACH-less LTM | NEC |
| R2-2407199 | Issues for RACH-less LTM cell switch | Huawei, HiSilicon |
| R2-2407433 | Consideration on Remaining Issues on LTM UP Aspect | ZTE Corporation |
| R2-2407176 | Remaining issues related to LTM | Ericsson |

Note: the last document was submitted to 7.4.2, but it has one TP for TS 38.321, which is included in this document.

## 2.1	Straightforward corrections

The MAC CR rapporteur believes that some (or all) of the proposed corrections in [1], [2], [3] and [8] are sufficiently straightforward to be discussed as comments to a CR, so the rapporteur has integrated them in a revision of the MAC CR.

Proposal 1: Companies can provide comments on the corrections included in the MAC CR from the rapporteur [10] (no need to take corrections one by one in an online session).

## 2.2	Corrections that may need discussion

### 2.2.1	Valid UL grant reception during RACH-less LTM cell switch

R2-2406853 raises that 5.8.2 indicates which grants are valid for RACH-less LTM and consequently proposes to do the following change:

For each Serving Cell and each configured uplink grant, if configured and activated and available for use and valid as specified in clause 5.8.2, the MAC entity shall:

The rapporteur's understanding is the validity of UL grants for RACH-less LTM cell switch is specified in the same way like CG-SDT which is in Rel-17 already, so this should perhaps be discussed together with it.

Proposal 2: Postpone discussions on the need to add "valid" after "if configured and activated and available for use" (because it seems to affect CG-SDT).

### 2.2.2	DL assignment and RACH-less LTM cell switch completion

R2-2407199 raises that, according to 5.1, the UE declares RACH-less LTM cell switch as complete upon reception of a DL assignment for a new transmission after the first PUSCH transmission, but the UE cannot judge whether a DL assignment is for a new transmission from 5.1, it is only in 5.3.2.2 after it was delivered to the HARQ entity, and anyway, during RACH-less LTM cell switch, the network will not send any DL assignment before the first PUSCH transmission and it should anyway make LTM cell switch as complete.

Proposal 3: Discuss whether the UE should consider RACH-less LTM cell switch complete upon reception of a DL assignment address to the MAC entity C-RNTI, without any extra condition.

### 2.2.3	UL grant and RACH-less LTM cell switch completion

R2-2407199 raises that:

-	At RACH-less LTM cell switch, the UE could transmit 2 MAC PDUs to the target cell before RACH-less LTM cell switch is complete

-	The target DU cannot know whether the first MAC PDU successfully received from the UE is the first MAC PDU sent by the UE, it will consider LTM cell switch as complete upon receiving any MAC PDU and notify the CU.

-	Even if the first MAC PDU received by the network is not the first MAC PDU sent by the UE and does not contain the RRCReconfigurationComplete message, there is no issue to consider LTM cell switch as complete.

-	According current 38.321, if no DL assignment is received, the UE only considers RACH-less LTM cell switch as complete if the network schedules a new transmission on the HARQ process of the first MAC PDU, even though the network scheduled a new transmission on the HARQ process of another MAC PDU.

Proposal 4: Discuss whether the UE should consider RACH-less LTM cell switch as complete upon reception of a UL grant for a new transmission on a HARQ process for which there was a previous (not necessarily the first) transmission to the target cell.

### 2.2.4	CG usage during RACH-less LTM cell switch

R2-2407199 raises that it is unclear in TS 38.321 whether or not the UE is restricted to use a single HARQ process of the CG, and only if there was not a previous transmission to the target cell with a dynamic grant (on another HARQ process).

Proposal 5: Discuss whether, during RACH-less LTM cell switch, the UE can use all HARQ processes of the CG configured for RACH-less LTM cell switch.

### 2.2.5	cg-RRC-RetransmissionTimer

In [6], it is raised that, during RACH-less LTM cell switch, upon receiving an uplink grant scheduled with the MAC entity’s C-RNTI in the target cell for a HARQ process for which a CG was already delivered, but that was not used for the first PUSCH transmission, the UE should not stop the cg-RRC-RetransmissionTimer.

However, there is no explanation why it should not be so. Besides, if the conclusion of the discussion in proposal 4 is that LTM cell switch is complete in this case, the CG is no more valid so this timer is anyway stopped.

Proposal 6: Discuss whether, in cases where the UE considers RACH-less LTM cell switch as complete, whether there is a need to explicitly stop certain, or all, running cg-RRC-RetransmissionTimer or not.

In [7], another case is shown:

When the MAC entity has a C-RNTI, Temporary C-RNTI, CS-RNTI, G-RNTI or G-CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI, or G-RNTI configured for multicast MTCH:

[...]

2>	stop the cg-RRC-RetransmissionTimer, if it is running, for the corresponding HARQ process for initial transmission of RACH-less handover or for the first PUSCH transmission at RACH-less LTM cell switch;

Here it is unclear why this timer should be used in the case a DL assignment is received with the MAC's entity G-RNTI. Besides, if it is concluded that there is no need to stop any cg-RRC-RetransmissionTimer upon RACH-less LTM cell switch completion (proposal 6), and that the LTM RACH-less cell switch is complete upon reception of any DL assignment with the MAC entity's C-RNTI (proposal 3), then there may be no need to stop this timer.

Proposal 7: Discuss whether, during RACH-less LTM cell switch, the UE should stop the cg-RRC-RetransmissionTimer upon reception of DL assignment with the MAC entity's C-RNTI or G-RNTI scheduling a DL HARQ process that has the same number like the UL HARQ process used for the first PUSCH transmission, as specified currently.

### 2.2.6	SR during RACH-less LTM cell switch

[8] raises that, if there are no SR resources during RACH-less LTM cell switch, a pending SR does not trigger RACH but, once RACH-less LTM cell switch is complete, as the SRs are still pending, the UE will trigger RACH. Therefore [8] proposes to capture that SRs triggered while RACH-less LTM cell switch is ongoing are not pending.

However, the rapporteur wonders whether the consequence would not be that, if there are SR resources, SR is not triggered during RACH-less LTM cell switch, which may not be the intention.

Proposal 8: Discuss whether to change anything about SR triggered during RACH-less LTM cell switch (e.g., they are not pending or they are cancelled when RACH-less LTM cell switch is complete).

## 2.3	Enhancements

### 2.3.1	UL skipping and RACH-less LTM cell switch

[9] raises that, for SCG LTM without SRB3, there may be no uplink signalling or data to send to the target PSCell at RACH-less LTM cell switch, then if UL skipping is configured, the UE will not perform any initial UL transmission in the case of RACH-less LTM cell switch.

One solution could be not to configure uplink skipping in SCG LTM candidate configurations, to avoid this issue. [9] proposes that, if the UE is configured with UL skipping, the UE ignores the UL skipping indication for the first transmission in the target cell during an LTM cell switch procedure i.e., the UE always performs the first transmission.

One comment from the rapporteur is that this seems to imply that, if the UE uses a CG first and receives a DG (or vice-versa), the UE will skip the second grant, even though RACH-less LTM cell switch is not complete and the SCG might have not receive the first transmission, so perhaps "while RACH-less LTM cell switch is ongoing".

Proposal 9: Discuss whether there is a need to support that the UE configured with UL skipping does not skip the first grant, or does not skip any grant, when RACH-less LTM cell switch is ongoing.

### 2.3.2	RACH fallback for LTM

[4] proposes that the UE falls back to RACH-based LTM in the target Cell when the SS-RSRP of the SSB corresponding to the configured uplink grant that has the same SSB index as the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE is lower than a threshold.

The rapporteur's recollection is that was discussed already and, for RACH-based LTM, the UE is anyway required to use a preamble associated with the SSB of the indicated TCI state, so that if the DL beam is wrong, the UE will also not get the RAR.

Proposal 10: Discuss whether any change is needed for the case where the SSB used as QCL RS for the DL/joint TCI state indicated in the LTM cell switch MAC CE is below a threshold.

### 2.3.3	NUL/SUL selection for RACH-less LTM cell switch using CG

[5] proposes to add a threshold to select NUL or SUL for RACH-less LTM cell switch using CG. The rapporteur's understanding is that, according to current specifications, the UE will use the first CG occasion, regardless whether it is on NUL or SUL.

Proposal 11: Discuss whether there is a need to change current specification for selection of NUL or SUL for RACH-less LTM cell switch with CG.

### 2.3.4	Candidate LTM configuration with MIMO 2TA

[3] has the following proposals:

1)	Support the co-existence between RACH-based LTM and R18 MIMO two TA, i.e., the LTM candidate cell could be configured with R18 MIMO two TA.

2)	RAN2 to confirm the mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch MAC CE is associated to one TAG while the TA value in the RACH RAR is associated to another TAG, assuming the co-existence between RACH-based LTM and R18 MIMO two TA is supported.

3)	RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH based LTM procedure are associated to different TAGs:

Option 2: Target DU sends a new TCI state in RACH Msg 4.

Option 3: If the mismatch issue occurs, UE follows the TCI state associate with the RACH based LTM procedure. Otherwise, UE follows the indicated TCI-state in the LTM cell switch command.

Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with indicated TCI state in LTM Cell Switch Command MAC CE during the RACH based LTM procedure.

The rapporteur's understanding is that it is already clear that this co-existence is supported, the mismatch can occur but may not be so likely, and as explained in option 1, the network can solve the problem by sending a PDCCH order to trigger another RACH.

Proposal 12: Discuss whether any enhancement is needed for RACH-based LTM to a target LTM candidate cell with MIMO 2TA.

# 3	Conclusion

This contribution makes the following proposals:

Straightforward corrections

Proposal 1: Companies can provide comments on the corrections included in the MAC CR from the rapporteur [10] (no need to take corrections one by one in an online session).

Corrections that may need discussion

Proposal 2: Postpone discussions on the need to add "valid" after "if configured and activated and available for use" (because it seems to affect CG-SDT).

Proposal 3: Discuss whether the UE should consider RACH-less LTM cell switch complete upon reception of a DL assignment address to the MAC entity C-RNTI, without any extra condition.

Proposal 4: Discuss whether the UE should consider RACH-less LTM cell switch as complete upon reception of a UL grant for a new transmission on a HARQ process for which there was a previous (not necessarily the first) transmission to the target cell.

Proposal 5: Discuss whether, during RACH-less LTM cell switch, the UE can use all HARQ processes of the CG configured for RACH-less LTM cell switch.

Proposal 6: Discuss whether, in cases where the UE considers RACH-less LTM cell switch as complete, whether there is a need to explicitly stop certain, or all, running cg-RRC-RetransmissionTimer or not.

Proposal 7: Discuss whether, during RACH-less LTM cell switch, the UE should stop the cg-RRC-RetransmissionTimer upon reception of DL assignment with the MAC entity's C-RNTI or G-RNTI scheduling a DL HARQ process that has the same number like the UL HARQ process used for the first PUSCH transmission, as specified currently.

Proposal 8: Discuss whether to change anything about SR triggered during RACH-less LTM cell switch (e.g., they are not pending or they are cancelled when RACH-less LTM cell switch is complete).

Enhancements

Proposal 9: Discuss whether there is a need to support that the UE configured with UL skipping does not skip the first grant, or does not skip any grant, when RACH-less LTM cell switch is ongoing.

Proposal 10: Discuss whether any change is needed for the case where the SSB used as QCL RS for the DL/joint TCI state indicated in the LTM cell switch MAC CE is below a threshold.

Proposal 11: Discuss whether there is a need to change current specification for selection of NUL or SUL for RACH-less LTM cell switch with CG.

Proposal 12: Discuss whether any enhancement is needed for RACH-based LTM to a target LTM candidate cell with MIMO 2TA.

# 4	References

- R2-2406331	Corrections to TS 38.321 for LTM	CATT

- R2-2406349 MAC corrections for LTM	Samsung Electronics Co., Ltd

- R2-2406439	Discussion on MAC open issue for LTM	vivo

- R2-2406517 Discussion on fallback RACH for LTM	ASUSTeK

- R2-2406530 Discussion on carrier selection for RACH-LESS LTM	OPPO

- R2-2406853 Corrections of MAC issues for RACH-less LTM	NEC

- R2-2407199 Issues for RACH-less LTM cell switch	Huawei, HiSilicon

- R2-2407433 Consideration on Remaining Issues on LTM UP Aspect	ZTE Corporation

- R2-2407176	Remaining issues related to LTM	Ericsson

- R2-24xxxxx CR to 38.321 Rel-18, User plane corrections for LTM, Huawei, HiSilicon