3GPP TSG-RAN WG2 Meeting #127	R2-2407199

Maastricht, Netherlands, 19 - 23 August, 2024

Agenda Item:	7.4.3

Source:	Huawei, HiSilicon

Title:	Issues for RACH-less LTM cell switch

Document for: Discussion and Decision

# 1	Introduction

In this document, we raise a number of issues for 38.321, showing actions for RACH-less LTM cell switch that may lead to LTM cell switch completion failure or include procedure that is unnecessary.

# 2	Discussion

## 2.1	DL assignments addressed to the UE C-RNTI

RAN2 agreed how the UE determines RACH-less LTM cell switch to be complete:

For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission as first UL transmission. Can be either DL assignment or UL grant addressed to same HARQ process for the “new transmission”

The procedure is currently specified in section 5.1 for DL assignments is as follows, where 3 necessary conditions to declare LTM cell switch completion are highlighted in yellow:

When the MAC entity has a C-RNTI, Temporary C-RNTI, CS-RNTI, G-RNTI or G-CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell: 1> if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI, or G-RNTI configured for multicast MTCH: [...] 2> if the downlink assignment has been received on the PDCCH for the MAC entity's C-RNTI after the first PUSCH transmission to the Serving Cell; and 2> if the downlink assignment is for a new transmission: 3> if there is an ongoing RACH-less handover procedure: 4> consider the RACH-less handover to be successfully completed and indicate it to upper layers. 3> else if there is an ongoing RACH-less LTM cell switch: 4> consider the LTM cell switch to be successfully completed and indicate it to upper layers. 2> indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

The second condition is that the DL assignment is for a new transmission, but how the UE determines that a DL assignment is for a new transmission is specified in 5.3.2.2, which is only executed after the UE has delivered the DL assignment to the HARQ entity i.e., not when this condition is evaluated. In any case, according to 5.3.2.2, the very first DL assignment is always considered as a new transmission, so if the network sends the first DL assignment addressed to the UE C-RNTI while the first condition is met, the UE will consider LTM cell switch as complete.

According to the above procedure, the only case in which the UE would not consider LTM cell switch as complete upon receiving a DL assignment addressed to the UE C-RNTI is if the DL assignment has a different NDI bit value, but this can only be the case if a previous DL assignment was received for the same HARQ process, or if the DL assignment would come before the UE has made any uplink transmission to the serving cell.

However, it is unclear what the use of sending a DL assignment to a UE from which the network has not received anything is. Moreover, the network cannot know exactly when the UE is ready to receive it. Also, if the UE has a configured grant, the network cannot know when the UE is ready to use it exactly, so it has no way to send a DL assignment while making sure the UE does not consider LTM cell switch as complete. If the UE would consider LTM cell switch as complete while the network did not receive any uplink transmission and cannot receive them, it will take extra time (as compared with T304 expiry) to detect a failure.

Observation 1: At RACH-less LTM cell switch, the network will not send any DL assignment addressed to the UE C-RNTI before having received an UL transmission from the UE.

Observation 2: The UE only determines whether a DL assignment is for a new transmission after the DL assignment is delivered to the HARQ entity (in 5.3.2.2, not in 5.3.1).

Observation 3: A DL assignment addressed to the UE C-RNT received during RACH-less LTM cell switch is always for a new transmission.

Therefore, it would be clearer to remove the unnecessary conditions (first PUSCH transmission to the serving cell, new transmission)

Proposal 1: Correct the procedure text in 5.3.1 so that the UE considers RACH-less LTM cell switch complete upon reception of a DL assignment addressed to the UE's C-RNTI, without any extra condition.

The above procedure also includes the following:

When the MAC entity has a C-RNTI, Temporary C-RNTI, CS-RNTI, G-RNTI or G-CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI, or G-RNTI configured for multicast MTCH:

[...]

2>	stop the cg-RRC-RetransmissionTimer, if it is running, for the corresponding HARQ process for initial transmission of RACH-less handover or for the first PUSCH transmission at RACH-less LTM cell switch;

Accordingly, during RACH-less LTM cell switch, upon receiving a DL assignment addressed to the UE C-RNTI, temporary C-RNTI or G-RNTI, the UE is required to stop the cg-RRC-RetransmissionTimer "for the corresponding [UL] HARQ process, but corresponding to what? To the DL HARQ process ID of the DL assignment?

This timer is only for the CG for RACH-less handover or RACH-less LTM cell switch and its effect is to allow autonomous UE retransmissions when it is configured but not running.

During RACH-less LTM cell switch:

-	when the UE receives a DL assignment addressed to the UE C-RNTI, the UE considers RACH-less LTM cell switch as complete. In this case, the CG for RACH-less LTM cell switch is no more used (as captured in Note 1b in 5.8.2), so whether this timer is stopped or not makes no difference at all;

-	the UE has no temporary C-RNTI;

-	the UE may have a G-RNTI, but if so, it is unclear why receiving something on the UE G-RNTI should have any effect on the cg-RRC-RetransmissionTimer.

Observation 4: There is no use to stop cg-RRC-RetransmissionTimer upon reception of a DL assignment addressed to the UE C-RNTI during RACH-less LTM cell switch, because the UE will anyway not use the associated CG anymore (because LTM cell switch is complete).

Observation 5: There is no reason to stop cg-RRC-RetransmissionTimer upon reception of a DL assignment addressed to the UE G-RNTI during RACH-less LTM cell switch

Proposal 2: Remove the action to stop cg-RRC-RetransmissionTimer in 5.3.1 upon reception of a DL assignment address to the UE C-RNTI or G-RNTI during RACH-less LTM cell switch.

## 2.2	RACH-less LTM completion upon reception of a UL grant

At RACH-less LTM cell switch, the target cell can send a dynamic grant to the UE even if the UE has a configured grant for RACH-less access to this target cell. As discussed above, the network does not know when the UE is ready to use CG occasions, so, for the dynamic grant, the network should use a HARQ process ID that is not allocated to the configured grant (otherwise, the UE would declare LTM cell switch complete even though the network may not have received any UL UE transmission).

In this case, it is possible that the UE uses a first grant (e.g., the configured grant) to transmit a first MAC PDU and a second grant (e.g., a dynamic grant) is available while RACH-less LTM cell switch is not yet complete, so the UE transmits a second MAC PDU. In this case, if the HARQ transmission of the second MAC PDU is successful while HARQ transmission of the first one is not, the DU cannot know that this is not the first uplink transmission, it only knows that the UE has successfully accessed the target cell and will notify this to the CU, so the network will consider LTM cell switch to be successful. The first MAC PDU sent by the UE may contain the RRCReconfigurationComplete message, so its processing is a little delayed, but this is not a serious problem.

Observation 6: At RACH-less LTM cell switch, the UE could transmit 2 MAC PDUs to the target cell before RACH-less LTM cell switch is complete.

Observation 7: The target DU cannot know whether the first MAC PDU successfully received from the UE is the first MAC PDU sent by the UE, it will consider LTM cell switch as complete upon receiving any MAC PDU and notify the CU.

Observation 8: Even if the first MAC PDU received by the network is not the first MAC PDU sent by the UE and does not contain the RRCReconfigurationComplete message, there is no issue to consider LTM cell switch as complete.

According to 5.4.1, the UE considers RACH-less LTM cell switch as complete upon reception of a UL grant as follows:

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

[....]

2>	if the uplink grant has been received on the PDCCH for the MAC entity's C-RNTI after the first PUSCH transmission to the Serving Cell; and

2>	if the uplink grant is for a new transmission on the same HARQ process used for the first PUSCH transmission to the Serving Cell:

3>	if there is an ongoing RACH-less handover procedure:

4>	consider the RACH-less handover to be successfully completed and indicate to upper layers.

3>	else if there is an ongoing RACH-less LTM cell switch:

4>	consider the LTM cell switch to be successfully completed and indicate it to upper layers.

Accordingly, in the above case, the DU may send a UL grant for the HARQ process of the second MAC PDU but since it is not "the same HARQ process used for the first PUSCH transmission to the Serving Cell", the UE will not consider RACH-less LTM cell switch to be complete.

Observation 9: According current 38.321, if no DL assignment is received, the UE only considers RACH-less LTM cell switch as complete if the network schedules a new transmission on the HARQ process of the first MAC PDU, even though the network scheduled a new transmission on the HARQ process of another MAC PDU.

Proposal 3: The UE considers RACH-less LTM cell switch as complete upon receiving PDCCH indicating a new transmission on a HARQ process for which there was a previous transmission to the target cell.

## 2.3	Configured grants during RACH-less LTM cell switch

During RACH-less LTM cell switch, if the UE is configured with a UL grant for RACH-less LTM cell switch, the target cell will monitor UL transmissions on all configured grant occasions as long as it has not successfully received a UL transmission from the UE.

According to 5.4.1, the UE processes configured grants as follows (parts not applicable to RACH-less LTM cell switch are omitted):

For each Serving Cell and each configured uplink grant, if configured and activated and available for use as specified in clause 5.8.2, the MAC entity shall:

1>	if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or

1>	if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured and cg-SDT-RetransmissionTimer is not configured, and cg-RRC-RetransmissionTimer is not configured (i.e. new transmission):

[...]

3>	if the configured uplink grant is for the first PUSCH transmission during an on-going LTM cell switch; or [...]:

4>	consider the NDI bit for the corresponding HARQ process to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

[else: no action i.e., grant is not delivered]

2>	else if the cg-RRC-RetransmissionTimer is configured and not running for the corresponding HARQ process:

3>	if the configured uplink grant is for the first PUSCH transmission at RACH-less LTM cell switch (i.e., initial new transmission); or [...]:

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	if RACH-less LTM cell switch is not successfully completed and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for first PUSCH transmission at RACH-less LTM cell switch or for its retransmission (i.e., retransmission for initial transmission at RACH-less LTM cell switch):

4>	consider the NDI bit to have not been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

[else: no action i.e., grant is not delivered]

The procedure text distinguishes the case where repetition is not configured (cg-RRC-RetransmissionTimer is not configured) and the case where it is configured (cg-RRC-RetransmissionTimer is configured).

In the case where repetition is not configured, a wording like "the configured uplink grant is for the first PUSCH transmission during an on-going LTM cell switch" could be understood in different ways e.g.,

alt. 1)	the configured grant is a configured grant that is for the first PUSCH transmission during an on-going RACH-less LTM cell switch i.e., that is configured in cg-LTM-Configuration;

alt.	2)	there was no previous PUSCH transmission to the Serving Cell and the configured grant is configured in cg-LTM-Configuration.

In alt. 1), before LTM cell switch is complete, if the UE transmitted a first MAC PDU using a dynamic grant, the UE can transmit a second MAC PDU using the grant configured in cg-LTM-Configuration, and if that grant is associated with more than 1 HARQ process, the UE can even transmit one MAC PDU per HARQ process.

In alt. 2), before LTM cell switch is complete, if the UE transmitted a first MAC PDU using a dynamic grant, the UE will not transmit any further MAC PDU using the grant configured in cg-LTM-Configuration.

In the case where repetition is configured, the procedure has similar ambiguities and similar unclear wordings.

Observation 10: During RACH-less LTM cell switch, it is unclear in TS 38.321 whether or not the UE is restricted to use a single HARQ process of the CG, and only if there was not a previous transmission to the target cell with a dynamic grant (on another HARQ process).

If the UE is restricted to a single HARQ process for the CG configured in cg-LTM-Configuration, there seems to be no point in configuring more than one HARQ process for it.

Some companies previously argued that, if there was a CG transmission already and RACH-less LTM cell switch is not complete, the UE shall ignore any receive DG because "completing RACH-less LTM cell switch is way more important than transmitting user data" (see [1]). However, as discussed above, completion could be declared first by confirming the successful reception of any uplink MAC PDU, so any further uplink transmission can actually help for the completion of RACH-less LTMC cell switch.

Observation 8: There is no gain of not using valid CG occasions during RACH-less LTM cell switch, this reduces uplink data rate and may even delay RACH-less LTM cell switch completion.

Proposal 4: During RACH-less LTM cell switch, the UE can use all HARQ processes of the CG configured for RACH-less LTM cell switch.

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1: The UE considers RACH-less LTM cell switch complete upon reception of a DL assignment addressed to the UE's C-RNTI, without any extra condition.

Proposal 2: In 5.3.1, remove the action to stop cg-RRC-RetransmissionTimer upon reception of a DL assignment address to the UE C-RNTI or G-RNTI during RACH-less LTM cell switch.

Proposal 3: The UE considers RACH-less LTM cell switch as complete upon receiving PDCCH indicating a new transmission on a HARQ process for which there was a previous transmission to the target cell.

Proposal 4: During RACH-less LTM cell switch, the UE can use all HARQ processes of the CG configured for RACH-less LTM cell switch.

# 4	Reference

- R2-2405331, UL grant handling during RACH-less LTM cell switch, LG Electronics Inc, Lenovo, ZTE Corporation.

# 5	TP