3GPP TSG-RAN WG2 Meeting #127	R2-2407200

Maastricht, Netherlands, 19 - 23 August 2024

Agenda Item:	7.4.2

Source:	Huawei, HiSilicon

Title:	RRC issues for R18 LTM

Document for: Discussion and Decision

# 1	Introduction

In this document, we discuss RRC issues for Rel-18 LTM, including UE-based TA measurement, unifiedTCI-StateType configuration in LTM-TCI-Info, and early ASN.1 decoding.

# 2	Discussion

## 2.1	UE-based TA measurement determination

According to the TS 38.331 v18.2.0 [1], when ltm-UE-MeasuredTA-ID is present for one LTM candidate configuration, it should also be present for other LTM candidate configurations.

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18               LTM-CandidateId-r18,

---omitted---

ltm-UE-MeasuredTA-ID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)  OPTIONAL, -- Need M

...

}

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config. This field is absent if tag2 is present for this LTM candidate configuration.

We consider four cases:

1)	ltm-ServingCellUE-MeasuredTA-ID is stored and ltm-UE-MeasuredTA-ID is configured for all LTM candidate configurations

2)	ltm-ServingCellUE-MeasuredTA-ID is stored but ltm-UE-MeasuredTA-ID is not configured for any LTM candidate configurations

3)	ltm-ServingCellUE-MeasuredTA-ID is not stored but ltm-UE-MeasuredTA-ID is configured for all LTM candidate configurations

4)	ltm-ServingCellUE-MeasuredTA-ID is not stored and ltm-UE-MeasuredTA-ID is not configured for any LTM candidate configurations

In case 1), we consider the scenario that cell 1 is the source cell when the UE receives ltm-ServingCellUE-MeasuredTA-ID = 1 and the following LTM candidate are configured:

-	Candidate 1, PCell = Cell 1, ltm-UE-MeasuredTA-ID = 1,

-	Candidate 2, PCell = cell 2, ltm-UE-MeasuredTA-ID = 1,

-	Candidate 3, PCell = cell 3, ltm-UE-MeasuredTA-ID = 2.

When the UE receives the LTM configuration while on cell 1, the UE executes the LTM candidate configuration addition procedure:

##### 5.3.5.18.3 LTM candidate configuration addition/modification The UE shall: 1> for each ltm-CandidateId value included in the ltm-CandidateToAddModList: 2> if the current UE configuration contains an LTM-Candidate with the ltm-CandidateId value: 3> reconfigure the corresponding LTM-Candidate in accordance with the received LTM-Candidate; 2> else: 3> add the received LTM-Candidate; 2> if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID: 3> if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID: 4> inform lower layers that the UE is configured with UE-based TA measurements for this LTM-Candidate; 3> else: 4> inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate.

Accordingly, the UE informs lower layer that it is configured with UE-based TA measurements for configuration 1 (same PCell as the current UE PCell) and configuration 2, and not for configuration 3.

Observation 1: The LTM candidate configuration addition/modification procedure informs lower layers that the UE is configured with UE-based TA measurements for the candidate whose PCell is the current PCell, even though there is probably not action.

If the UE receives an LTM cell switch command MAC CE indicating configuration 2, the UE executes the LTM cell switch execution procedure:

##### 5.3.5.18.6 LTM cell switch execution [---omitted---] 1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID: 2> if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID: 3> replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID; 3> for each LTM-Candidate IE in ltm-Config: 4> if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID: 5> inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate; 4> else: 5> inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate; 1> else if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-UE-MeasuredTA-ID: 2> inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate.

Because the value of ltm-UE-MeasuredTA-ID of configuration 2 is 1, like the value of ltm-ServingCellUE-MeasuredTA-ID, there is no new information to lower layer whether the UE is configured or not with UE-based TA measurements for any LTM candidate.

For configuration 3, the information sent to lower layers at the configuration is clearly still valid. For configuration 1, according to the procedure text, there was also an information to lower layers that the UE is configured with UE-based autonomous TA measurement for it, but that information was somehow not relevant as the PCell of configuration 1 is the current UE PCell, so the UE shall not do any UE-based TA measurement for this candidate.

Observation 2: After switching to another candidate for which UE-based TA measurement was configured, the LTM cell switch procedure does not inform lower layers that the UE is configured with UE-based TA measurements for the candidate whose PCell is the previous PCell.

One may consider that lower layers retain the information received at configuration, although it was irrelevant when it was received.

Proposal 1a: Confirm the understanding that the information to lower layers that the UE is configured with UE-based TA measurements for a candidate remains valid after LTM cell switch, if there is no new explicit information.

Proposal 1b: Discuss whether to clarify that the UE shall not do any UE-based TA measurement for a candidate whose SpCell is the current UE SpCell (current procedure text says to inform lower layers that the UE is configured for UE-based TA measurements for this candidate).

Proposal 1c: Discuss whether there is a need to clarify that after LTM cell switch to a candidate configuration for which the UE was configured with UE-based TA measurements, the UE considers itself as configured with UE-based TA measurements for the candidate configuration whose PCell is the previous UE PCell.

Case 2) can happen if the network has initially configured all LTM candidates with ltm-UE-MeasuredTA-ID, then the UE has stored ltm-UE-MeasuredTA-ID and, based on new L3 measurement reports, the gNB decides to release all the candidates and add them again, this time without ltm-UE-MeasuredTA-ID. This could also happen after L3 handover to another gNB, that also configures LTM with the same candidate IDs.

The UE may have initially informed lower layers that the UE was configured with UE-based TA measurements for several candidates. However, according to existing procedures, the UE will not inform lower layers that the UE is no more configured with UE-based TA measurements for these candidates.

Observation 3: If the UE is configured with UE-based TA measurements for certain LTM candidates and all LTM candidates are released are added (e.g., at inter-gNB handover) without ltm-UE-MeasuredTA-ID, the UE does not inform lower layers that the UE is not configured with UE-based TA measurements for any LTM candidate, in particular for the ones for which lower layers were informed that the UE is configured with UE-based TA measurements.

To solve the problem raised in Observation 3, a branch should be added to the the scenario when ltm-UE-MeasuredTA-ID is not present in LTM candidate configurations should be added.

Proposal 2: Capture in the LTM candidate configuration addition/modification procedures that, when ltm-UE-MeasuredTA-ID is not present for a LTM candidate configuration, the UE informs lower layers that UE-based TA measurement is not configured. (See Annex TP)

If agreed, proposal 2) also handles case 4).

In case 3), since ltm-ServingCellUE-MeasuredTA-ID is not stored, it is unclear how the UE compares the value of ltm-ServingCellUE-MeasuredTA-ID and the value of ltm-UE-MeasuredTA-ID. While it is probably assumed that such a case is excluded, this is not captured.

Proposal 3: Capture that when the network configures LTM candidates with ltm-UE-MeasuredTA-ID, the network ensures that the UE has a value stored in ltm-ServingCellUE-MeasuredTA-ID.

## 2.2	unifiedTCI-StateType in LTM-TCI-Info

In Rel-18 LTM, the TCI state indication in LTM cell switch MAC CE and TCI state activation for candidate cells are based on the unified TCI state framework. In this framework, the network needs to configure the UE with unifiedTCI-StateType to tell the UE whether the indicated TCI state is joint TCI state or separate UL/DL TCI state. In this case, unifiedTCI-StateType is mandatory for the UE. However, according to the TS 38.331 v18.2.0, the unifiedTCI-StateType in LTM-TCI-Info is optional need R field.

Since ASN.1 is frozen, it is a bit late to change it to a mandatory field. RAN2 can clarify that the network always configures unifiedTCI-StateType in LTM-TCI-Info.

Proposal 4: In the field description of unifiedTCI-StateType in LTM-TCI-Info, capture that the network always configures this field. (See Annex TP)

## 2.3	Early ASN.1 decoding

In RAN2 #126 meeting, RAN2 discussed whether the network can indicate that the UE should perform fast RRC processing for specific LTM candidate configurations [2]. The background is that RAN4 agreed that the UE can report a maximum number of LTM candidate configurations for which the UE can support fast RRC processing. Some companies argued that this UE capability is useful if the network knows for which candidate configurations the UE can do fast processing, then the network can schedule the UE with low latency for cell switch to them, e.g., in dynamic grant based RACH-less procedure.

However, the LTM cell switch processing time varies for different UEs, so the network is not able to precisely know the exact cell switch processing completion time. Some UEs process a lot faster than RAN4 requirement. If the network intends to realize a low interruption LTM cell switch, an easier way is that the network schedules the UE earlier. Even though some PDCCH may not successfully be received by the UE, this is not a serious drawback if the network wants to achieve low interruption.

Proposal 5: The network does not indicate which LTM candidate configurations can be fast processed by the UE. Fast RRC processing for LTM candidate configurations is up to UE implementation.

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1a: Confirm the understanding that the information to lower layers that the UE is configured with UE-based TA measurements for a candidate remains valid after LTM cell switch, if there is no new explicit information.

Proposal 1b: Discuss whether to clarify that the UE shall not do any UE-based TA measurement for a candidate whose SpCell is the current UE SpCell (current procedure text says to inform lower layers that the UE is configured for UE-based TA measurements for this candidate).

Proposal 1c: Discuss whether there is a need to clarify that after LTM cell switch to a candidate configuration for which the UE was configured with UE-based TA measurements, the UE considers itself as configured with UE-based TA measurements for the candidate configuration whose PCell is the previous UE PCell.

Proposal 2: Capture in the LTM candidate configuration addition/modification procedures that, when ltm-UE-MeasuredTA-ID is not present for a LTM candidate configuration, the UE informs lower layers that UE-based TA measurement is not configured. (See Annex TP)

Proposal 3: Capture that when the network configures LTM candidates with ltm-UE-MeasuredTA-ID, the network ensures that the UE has a value stored in ltm-ServingCellUE-MeasuredTA-ID. (See Annex TP)

Proposal 4: In the field description of unifiedTCI-StateType in LTM-TCI-Info, capture that the network always configures this field. (See Annex TP)

Proposal 5: The network does not indicate which LTM candidate configurations can be fast processed by the UE. Fast RRC processing for LTM candidate configurations is up to UE implementation.

# 4	Reference

- NR; Radio Resource Control (RRC) protocol specification V18.2.0.

- On Early Processing of LTM candidates, Power Control Parameters after LTM Cell Switch and coexistence with NES, Nokia.

# 5	Annex TP

##### 5.3.5.18.3	LTM candidate configuration addition/modification

The UE shall:

1>	for each ltm-CandidateId value included in the ltm-CandidateToAddModList:

2>	if the current UE configuration contains an LTM-Candidate with the ltm-CandidateId value:

3>	reconfigure the corresponding LTM-Candidate in accordance with the received LTM-Candidate;

2>	else:

3>	add the received LTM-Candidate;

2>	if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID:

3>	if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

4>	inform lower layers that the UE is configured with UE-based TA measurements for this LTM-Candidate;

3>	else:

4>	inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate.

2>	else:

3>	inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate;

##### 5.3.5.18.7	LTM configuration release

For the cell group for which the LTM configuration release procedure is triggered, the UE shall:

1>	remove ltm-ServingCellNoResetID from VarLTM-ServingCellNoResetID;

1>	remove ltm-ServingCellUE-MeasuredTA-ID from VarLTM-ServingCellUE-MeasuredTA-ID;

1>	release ltm-Config.

### 6.3.2	Radio resource control information elements

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID This field is absent if tag2 is present for this LTM candidate configuration.

#### –	LTM-TCI-Info

The IE LTM-TCI-Info is used to configure TCI related information for an LTM candidate configuration to be used during activation of TCI state(s) and/or upon the reception of the LTM Cell Switch procedure.

LTM-TCI-Info information element

-- ASN1START

-- TAG-LTM-TCI-INFO-START

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                          OPTIONAL,    -- Need R

...

}

-- TAG-LTM-TCI-INFO-STOP

-- ASN1STOP

LTM-TCI-Info field descriptions

unifiedTCI-StateType Indicates the unified TCI states type the UE is configured for this LTM candidate configuration. The value separate means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StatesToAddModList for UL TCI states. The value joint means this LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation. The network always configures this field.