3GPP TSG-RAN WG2 Meeting #127 	R2-2407203

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.3

Source:	Panasonic

Title:	Ambient IoT identifiers for A-IoT paging

Document for:	Discussion

# Introduction

In last meeting, RAN2 achieved the following agreements on A-IoT paging.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In this paper, we further discuss the ID usage in the A-IoT paging.

# Discussion

## Identifier for paging

In order to reach a particular device, ID must be included in the A-IoT paging message. One option is to use the Device ID. Although the exact format is still under discussion in SA2, the Device ID is expected to enable uniquely identifying an A-IoT device at least in the 3GPP system. For example, Device ID can be uniquely allocated by 3GPP network. Or as another example, if EPC (Electronic Product Code) is used as Device ID, it is designed to be unique across all physical objects in the world, over all time, and across all categories of physical objects.

Therefore, the Device ID is expected to have large size due to unambiguous identification purpose. As an example, the size of EPC could be as large as 96 bits. Further, the Device ID is expected to be “permanent”, in the sense that it can be pre-configured and stored in the non-volatile memory of the device such that it can be kept even after device energy is exhausted.

However, using the Device ID for communication with the A-IoT device may risk exposure of the A-IoT device identifier and thus unauthorized device tracking. Moreover, the large size of Device ID may cause unnecessary signaling overhead and energy usage. Therefore, at least for air interface (e.g. between reader and device), a temporary ID with shorter length could be a more reasonable option. It belongs to RAN2’s scope to discuss/define such temporary ID and its related procedures.

Proposal 1. Temporary ID should be supported to address a targeted device in A-IoT paging.

## Assignment of temporary identifier

The temporary ID can be assigned via an A-IoT random access procedure. In the previous meeting, RAN2 discussed random access procedures and achieved the following agreements.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4 	The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5	“Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

Taking the 4-step RA as an example, the temporary ID assignment can be performed based on the following Fig.1.

Fig.1. Assigning temporary ID using 4-step RA

There could be two alternatives for device to obtain the temporary ID, which is related to the step 8 of Fig.1. The first option is that the random ID generated by device (and transmitted to reader via A-IoT Msg1) is directly used as temporary ID after the completion of RA. In this case, the step 8 is not needed. The second option is that the reader generates a temporary ID for the device and sends to the device by step 8. Comparing these two options, the second option is more robust because the reader can ensure the uniqueness of the temporary ID within its coverage.

Proposal 2. Reader assigns a temporary ID to an A-IoT device during the A-IoT RA procedure.

Besides the reader, CN can also be considered to allocate the temporary ID for the device. In this case, the temporary ID may be used not only at AS level, but also at NAS level. CN can also ensure that the allocated temporary ID is unique across multiple base stations. The scope of uniqueness can be controlled by CN. However, such aspects might be better to discuss at SA2 rather than RAN2. Nevertheless, RAN2 can progress independently on the “AS-level” temporary ID discussion. Depending on the progress of SA2, we can later decide on whether to make sense to have two levels of temporary IDs or merged them into one temporary ID.

Proposal 3. RAN2 to discuss on temporary ID usage in AS level without waiting for SA2.

## Life time of temporary identifier

After the temporary ID is indicated by the reader to the device, it can be used for service request and response between reader and device, for example, included in the A-IoT paging message to address the targeted device. On the other hand, the communication between reader and CN can rely on Device ID (potentially with encryption). The mapping between Device ID and temporary ID can be managed by the reader or within NW. In this way, the Device ID is not exposed to the air interface. For the case that the reader manages the mapping between temporary ID and Device ID, the operation is illustrated in the following Fig.2.

Fig.2. Service request and response using temporary ID

However, it is to be discussed how long the temporary ID can sustain. Generally speaking, it is preferable to keep the temporary ID as long as possible to avoid power consumption and signaling overhead caused by re-assigning the temporary ID. However, the life time of temporary ID may also depend on the device types which be one of the following:

- Type-1: Temporary ID is kept in device as long as “sustainable operation time”.

- Type-2: Temporary ID is kept in device longer than “sustainable operation time”

- Type-3: Temporary ID can be always kept in device once assigned

The sustainable time is currently under discussion in RAN1. One way to define such time is to use a timer. In more details, after A-IoT paging or A-IoT Msg1 of A-IoT RA, the timer starts. Before timer expires, the device may receive R2D or transmit D2R. After timer expires, R2D/D2R are not possible. The content of volatile memory is lost. For Type-1 device mentioned above, the temporary ID is stored in the volatile memory. For the Type-2 device, the temporary ID is stored in volatile memory but with separate capacitor to keep the energy longer than energy storage for sustainable operation. For the Type-3 device, the temporary ID is stored in the non-volatile memory. All three types of devices should be supported to operate with a temporary ID.

Note: for sustaintable operation time, instead of timer, our view in RAN1 is some device power consumption model may be required as the power consumption for reception and transmission can be so different. If timer is used, the timer value may be required to set based on the highest power consumption value, which is not desirable.

Proposal 4. The temporary ID should be supported for following types of devices:

- Type-1: Temporary ID is kept in device as long as “sustainable operation time”.

- Type-2: Temporary ID is kept in device longer than “sustainable operation time”

- Type-3: Temporary ID can be always kept in device once assigned

FFS whether and how the network should differentiate the above types of devices in terms of temporary ID operation.

# Conclusion

In this paper, we have discussed the use of temporary ID and the proposals are as follows.

Proposal 1. Temporary ID should be supported to address a targeted device in A-IoT paging.

Proposal 2. Reader assigns a temporary ID to an A-IoT device during the A-IoT RA procedure.

Proposal 3. RAN2 to discuss on temporary ID usage in AS level without waiting for SA2.

Proposal 4. The temporary ID should be supported for following types of devices:

- Type-1: Temporary ID is kept in device as long as “sustainable operation time”.

- Type-2: Temporary ID is kept in device longer than “sustainable operation time”

- Type-3: Temporary ID can be always kept in device once assigned

FFS whether and how the network should differentiate the above types of devices in terms of temporary ID operation.