3GPP TSG RAN WG2 #127	R2-2407204

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	8.13.2
Source:	Toyota ITC
Title:	Discussion on relay discovery and (re)selection for NR sidelink multi-hop relay

Document for:	Discussion and Decision

# Introduction

In RAN #104 meeting, the WID for Rel-19 NR sidelink multi-hop relay was approved [1]. One of the objectives of the WID is to specify relay discovery and (re-)selection for NR sidelink multi-hop relay as shown below:

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this contribution, we provide our views on relay (re-)selection for NR sidelink multi-hop relay.

# Discussion

In this section, we discuss relay (re-)selection for NR sidelink multi-hop relay. Rel-17 U2N relay was specified to support only single-hop U2N relay (i.e., single Relay UE) as shown in Figure 1. According to the WID of Rel-19 NR sidelink multi-hop relay, it will support up to two additional hops of relay on top of Rel-17 U2N relay. Figure 2 shows an example of multi-hop U2N relay with two additional hops from Rel-17 U2N relay.

Figure 1. Example of Rel-17 single-hop U2N relay.

Figure 2. Example of multi-hop U2N relay with two additional hops from Rel-17 U2N relay.

## Discussion on relay (re-)selection

As specified in TS 38.331 Clause 5.8.15.3, Rel-17 U2N relay (re-)selection procedure uses the SD-RSRP(s) of U2N Relay UE(s) that Remote UE has discovered to determine suitable U2N Relay UE(s) as shown below. More specifically, for candidate U2N Relay UE(s) that Remote UE has discovered, Remote UE considers the candidate U2N Relay UE(s) for which SD-RSRP exceeds sl-RSRP-Thresh by sl-HystMin has met the AS criteria. Then, if Remote UE detects any suitable U2N Relay UE(s) that meet the AS criteria and higher layer criteria, one of the available suitable U2N Relay UE(s) can be selected, and it is up to Remote UE implementation to choose one U2N Relay UE if there are multiple available suitable U2N Relay UEs. Otherwise, no U2N Relay UE will be selected.

#### 5.8.15.3 Selection and reselection of NR sidelink U2N Relay UE A UE capable of NR sidelink U2N Remote UE operation that is configured by upper layers to search for a NR sidelink U2N Relay UE shall: 1> if the UE has no serving cell; or 1> if the RSRP measurement of the cell on which the UE camps (for L2 and L3 U2N Remote UE in RRC_IDLE or RRC_INACTIVE)/ the PCell (for L3 U2N Remote UE in RRC_CONNECTED) is below threshHighRemote within sl-RemoteUE-Config: 2> if the UE does not have a selected NR sidelink U2N Relay UE; or 2> if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is available and is below sl-RSRP-Thresh; or 2> if the UE has a selected NR sidelink U2N Relay UE, and SL-RSRP of the currently selected NR sidelink U2N Relay UE is not available, and SD-RSRP of the currently selected U2N Relay UE is below sl-RSRP-Thresh; or NOTE 1: U2N Remote UE uses SL-RSRP measurements for relay reselection trigger evaluation when there is data transmission from U2N Relay UE to U2N Remote UE, and it is left to UE implementation whether to use SL-RSRP or SD-RSRP for relay reselection trigger evaluation in case of no data transmission from U2N Relay UE to U2N Remote UE. If SD-RSRP is used, the discovery procedure will be performed between the U2N Remote UE and the selected U2N Relay UE. 2> if the UE has a selected NR sidelink U2N Relay UE, and upper layers indicate not to use the currently selected NR sidelink U2N Relay UE; or 2> if the UE has a selected NR sidelink U2N Relay UE, and upper layers request the release of the PC5-RRC connection; or 2> if the UE has a selected NR sidelink U2N Relay UE, and sidelink radio link failure is detected on the PC5-RRC connection with the current U2N Relay UE as specified in clause 5.8.9.3: 3> perform NR sidelink discovery procedure as specified in clause 5.8.13 in order to search for candidate NR sidelink U2N Relay UEs; 4> when evaluating the one or more detected NR sidelink U2N Relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same U2N Relay UE ID and using the sl-FilterCoefficientRSRP in SIB12 (if in RRC_IDLE/INACTIVE), the sl-FilterCoefficientRSRP in sl-ConfigDedicatedNR (if in RRC_CONNECTED) or the preconfigured sl-FilterCoefficientRSRP as defined in 9.3 (out of coverage), before using the SD-RSRP measurement results; 4> consider a candidate NR sidelink U2N Relay UE for which SD-RSRP exceeds sl-RSRP-Thresh by sl-HystMin has met the AS criteria; 3> if the UE detects any suitable NR sidelink U2N Relay UE(s): 4> consider one of the available suitable NR sidelink U2N relay UE(s) can be selected; NOTE 2: A candidate NR sidelink U2N Relay UE which meets all AS layer criteria defined in 5.8.15.3 and higher layer criteria defined in TS 23.304 [65] can be regarded as suitable NR sidelink U2N Relay UE by the NR sidelink U2N Remote UE. If multiple suitable NR sidelink U2N Relay UEs are available, it is up to Remote UE implementation to choose one NR sidelink U2N Relay UE. The details of the interaction with upper layers are up to UE implementation. NOTE 3: For L2 U2N Remote UEs in RRC_IDLE/INACTIVE and L3 U2N Remote UEs, the cell (re)selection procedure and relay (re)selection procedure run independently. If both suitable cells and suitable NR sidelink U2N Relay UEs are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE. Furthermore, L3 U2N Remote UE's selection on both cell and NR sidelink U2N Relay UE is also based on UE implementation. 3> else: 4> consider no NR sidelink U2N Relay UE to be selected.

There are some technical issues when supporting Rel-19 multi-hop U2N relay based on Rel-17 U2N relay procedure. In Rel-17 U2N relay (re-)selection procedure, Remote UE uses only the SD-RSRP of a candidate U2N Relay UE(s) that Remote UE has discovered. When the candidate U2N Relay UE provides multi-hop U2N relay, Remote UE does not consider the property and quality (e.g., number of hops, RSRPs of indirect links, etc.) of indirect links of a multi-hop relay path that the candidate U2N Relay UE provides. For example, as shown in Figure 3, Remote UE only considers the SD-RSRP of 3rd-hop Relay UE (a candidate U2N Relay UE) for relay (re-)selection, and thus it does not take into account the property and quality of the indirect links of the relay path that the 3rd-hop Relay UE provides. Due to Remote UE’s lack of knowledge about the property and quality of the indirect links, it may result in inappropriate (re-)selection of U2N Relay UE that does not meet Remote UE’s QoS requirements. Therefore, some mechanisms should be introduced to take into account the property and quality of each relay path in relay discovery and (re-)selection.

Figure 3. Issues of Rel-17 U2N relay (re-)selection procedure in multi-hop U2N relay scenarios.

Observation 1: Rel-17 U2N relay supports only single-hop relay and does not consider the property and quality of indirect links of each multi-hop relay path in relay (re-)selection. In multi-hop U2N relay scenarios, this may result in inappropriate (re-)selection of U2N Relay UE that does not meet Remote UE’s QoS requirements.

Proposal 1: RAN2 considers how to take into account the property and quality of indirect links of each multi-hop relay path (e.g., number of hops, RSRPs of indirect links, etc.) in relay discovery and (re-)selection.

# Conclusion

In this contribution we discussed our views on relay (re-)selection for NR sidelink multi-hop relay. Our observation and proposals are summarized as follows:

Observation 1: Rel-17 U2N relay supports only single-hop relay and does not consider the property and quality of indirect links of each multi-hop relay path in relay (re-)selection. In multi-hop U2N relay scenarios, this may result in inappropriate (re-)selection of U2N Relay UE that does not meet Remote UE’s QoS requirements.

Proposal 1: RAN2 considers how to take into account the property and quality of indirect links of each multi-hop relay path (e.g., number of hops, RSRPs of indirect links, etc.) in relay discovery and (re-)selection.

# Reference

- RP-241609, “New WID on NR sidelink multi-hop relay,” RAN #104, June 2024.